IEEE P802.11
Wireless LANs

| Normative Text Enabling Neighbor Report for Multiple ESSs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen Wang | Motorola, Inc. | 8000 W Sunrise Boulevard, Plantation, FL 33322 | 954-723-8124 | Stephen.Wang@motorola.com |
| Tim Wilson | Motorola, Inc. | 1301 E Algonquin Road, Schaumburg, IL 60196 | 847-576-5296 | Tim.Wilson@motorola.com |
| Floyd Simpson | Motorola, Inc. | 8000 W Sunrise Boulevard, Plantation, FL 33322 | 954-723-5269 | Floyd.Simpson@motorola.com |
| Steve Emeott | Motorola, Inc. | 1301 E Algonquin Road, Schaumburg, IL 60196 | 847-576-8268 | Steve.Emeott@motorola.com |
| Walter Johnson | Motorola, Inc. | 1301 E Algonquin Road, Schaumburg, IL 60196 | 847-576-2352 | Walter.Johnson@motorola.com |

Abstract

This document presents normative text enabling a STA to optionally include SSID information elements in a neighbor report request frame in order to receive a neighbor report for neighbor APs of interest in a multi-ESS system.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 7. Frame formats

## 7.4 Management Actions

### 7.4.2 Radio Measurement Action Details

Replace clause 7.4.2.5 with tables and figures included therein as shown:

#### 7.4.2.5 Neighbor Report Request frame format

The Neighbor Report Request frame uses the Action frame body format and is transmitted by STA requesting information in the Neighbor Report about neighboring AP’s. The format of the Neighbor Report Request frame body is shown in Figure k33.

|  | Category | Action | Dialog Token | Neighbor Report Request Types | SSID element |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable |

Figure k33—Neighbor Report Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to the value indicating Neighbor Report Request, as specified in Table k13 in 7.4.2.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the measurement request to identify the request/report transaction.

The Neighbor Report Request Types field shall be one octet in length and shall contain the subfields as shown in Figure k34.

|  | B0 | B1 B7 |
| --- | --- | --- |
|  | Neighbor TSF Information Request | Reserved |
| Bits: | 1 | 7 |

Figure k34—Neighbor Report Request Types Subfield

- Neighbor TSF Information Request – When this bit is set to 1, it indicates the Neighbor TSF Information field, which contains the TSF Offset and the Beacon Interval subfields, may be included in the Neighbor Report.

- Reserved – Bit 1 – 7.

The SSID element is defined in 7.3.2.1. One or more SSID elements may be included to request a neighbor report for specific ESSs. The absence of a SSID element indicates neighbor report for the current ESS.

# 11 MLME

Modify clause 11.85 as shown:

## 11.8 Usage of the Neighbor Report

A Neighbor Report is sent by an AP and it contains information on known neighbor AP’s. A Neighbor Report may not be exhaustive either by choice, or due to the fact that there may be neighbor APs not known to the AP. The Neighbor Report contents shall be derived from the MIB table dot11RRMNeighborReportTable. The mechanism by which the contents of this table are determined is outside the scope of this amendment, but it may include information from measurement reports received from the STA’s within the BSS, information obtained via the management interface, or the DS.

### 11.8.1 Purpose of a Neighbor Report

The purpose of the Neighbor Report is to enable the STA to optimize aspects of neighbor BSS transition. The Neighbor Report element contains information on APs, which the STA may use as candidates for a BSS transition. A Neighbor Report element shall only contain entries of neighboring APs that are legitimate members of ESSs requested by STA in Neighbor Report Request.

Since the information in the Neighbor Report may be stale, it should be considered advisory; information obtained through a scan or other sources may also be considered, possibly overriding information in the Neighbor Report. For example, where information contained within a Neighbor Report is contradicted by information in the Beacon/Probe Response, the Beacon/Probe Response information may be considered; similarly, where information is available within a standardized security handshake (for example the 802.11i 4-way handshake), it may be considered.

NOTE—Determination of the neighboring APs can be accomplished be several means, including:

- Configuring an AP with a list of BSSIDs that are neighbors.

- Utilizing the Beacon Report in order to determine which APs can be heard by STAs in the service area. To guard against pollution of the neighbor report by an erroneous (but not malevolent) STA, an AP may only utilize information corroborated by Beacon Reports from multiple STAs. In addition, an AP should validate that BSSIDs returned in a Beacon Report represent legitimate members of an ESS  that is known and trusted by the AP before including them in a neighbor report.

Determination of which neighboring APs are legitimate members of known and trusted ESSs can be accomplished by several means, including:

- Configuring APs with a list of BSSIDs that are mapped to SSIDs of ESSs known and trusted by the AP. Only those neighbor APs, whose BSSIDs are in the list, may be reported in neighbor reports.

- Utilizing a secure IAPP protocol such as IEEE 802.11F, in order to determine whether a Reassociation Request has been sent to a legitimate member of a known and trusted ESS.

### 11.8.2 Requesting a Neighbor Report

An associated STA requesting a Neighbor Report shall send a Neighbor Report Request frame to its associated AP. A STA may optionally include one or more SSID elements in the Neighbor Report Request if it wishes to receive information on neighbor APs for the corresponding ESSs. A STA shall not include any SSID element in the Neighbor Report Request if it is only interested in neighbor APs that are in the same ESS as the STA.

A STA that is not yet associated may send an Association Request frame with a Request information element containing the element ID for the Neighbor Report or the Neighbor Report w/ Neighbor TSF Information in order to request a Neighbor Report in the Association Response frame. If the Request element in the Association Request contains the element ID for the Neighbor Report, then the Neighbor Report in the Association Response frame shall not contain the Neighbor TSF Information in the Neighbor Report. If the Request element in the Association Request frame contains the element ID for the Neighbor Report w/ Neighbor TSF Information, then the Neighbor Report returned in the Association Response frame shall also contain the Neighbor TSF Information, if available from the AP.

### 11.8.3 Receiving a Neighbor Report

An AP receiving a Neighbor Report Request shall respond with a Neighbor Report Response frame containing zero or more Neighbor Report elements. If the Neighbor Report is requested using the Request element in an Association Request frame, an AP receiving the request shall respond with the Neighbor Report in the corresponding Association Response frame. If SSID information elements are specified in the corresponding Neighbor Report Request frame, the Neighbor Report elements shall only contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that are members of the current ESS or ESSs identified by SSID elements contained within the Neighbor Report Request. Otherwise, the Neighbor Report elements shall contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs belonging to the same ESS as the requesting STA.

A serving AP shall include a TSF Information field in the Neighbor List Entry only if it is able to guarantee an accumulated error of ±1.5 TU or better on the TSF Offset subfield. The error budget (±1.5 TU) can be broken down as follows:

- Delays by the measuring STA in transmitting the first bit of the Beacon Report after receiving the last bit of a neighbor AP’s Beacon or Probe Response (±0.5 TU).

- Error caused by rounding to the nearest TU boundary when converting Neighbor TSF Offset from microseconds to TUs (±0.5 TU). 2

- Delays by the serving AP between reception of the last bit of the Beacon Report and transmission of the first bit of the Neighbor Report (±0.5 TU).