IEEE P802.11
Wireless LANs

| Proposed DS Service Interface |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-03-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Darwin Engwer | Nortel Networks | 4655 Great America Pkwy, Santa Clara, CA 95054 | 408-495-7099 | dengwer@nortelnetworks.com |
| Johnny Zweig |  |  |  | johnny@ieee.org |

Abstract

Clarification of the DS SAP interface and draft text that describes the DS SAP Service Interface.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

As part of the APF investigative work it was noted that IEEE 802.11-1999 does not define or describe the service interface between the DS and the MAC SAP (see IEEE 802.11-1999 clause 5.8, Figure 11).  Also, since the MAC entity is a component within an AP, the DS interface really terminates at the AP SAP.  The AP then interfaces to the MAC SAP of its MAC. Therefore, what is really needed is a description of the interface between the DS and the AP SAP.  Of course the DS also interfaces to the portals, so while one could describe the interface in terms of the AP SAP, it makes more sense to describe the interface in terms of the DS itself, i.e. the DS SAP, which can connect to either an AP or a portal.

[insert diagram here]

Note that describing the DS itself or the functions thereof is out of scope.  But adding a description of the DS SAP interface is not, in fact it is even more important to do so.

A DS SAP interface specification needs to describe the primitives required to get MSDUs in/out of the DS and update the DS’s STA-to-AP map.

That is, the DS interactions (with the other entities of the system with which it interfaces) are:

accepts MSDUs from APs and portals

delivers MSDUs to APs and/or portals

accepts STA-to-AP mapping updates from the APs

Once the MSDUs have been delivered to an AP, then the AP determines when and how to deliver the MSDUs to the AP’s MAC (via the MAC SAP).

A high level sketch of the DS interface would include these primitives:

1. DS-UNITDATA.request (msdu (i.e. SA/DA/DATA), enum:SRC_AP/ SRC_PORTAL)

2. DS-UNITDATA.indication (msdu (i.e. SA/DA/DATA), enum:DEST_AP/ DEST_PORTAL)

3. DS-STA-NOTIFY.request (STA_macaddr, enum:ADD/MOVE/DELETE)

Draft text that describes the DS SAP Service Interface is shown below.

Editor: Add the following text as a new informative annex:

# Distribution System SAP Service Specification

The following primitives describe the DS SAP service interface.

The DS-UNITDATA primitives accept and deliver MSDUs, including all the parameters and data as defined in clause 6.2.1.1.2.  These tuples are called Distribution System Service Data Units (DSSDUs).

## DS-UNITDATA.request

### Function

This primitive requests distribution of a DSSDU across the DS.

### Semantics of the Service Primitive

The primitive parameters are as follows:

DS-UNITDATA.request		(

Dssdu,

SourceType

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dssdu | 802.11 MSDU | Any valid data unit per clause 6.2.1.1.2, including data and all parameters | Specifies the DSSDU to be distributed via the DS. |
| SourceType | Enumeration | SRC_AP, SRC_PORTAL | Specifies the type of entity that is requesting distribution of a DSSDU. |

### When Generated

This primitive is generated by an AP or a portal to submit a DSSDU to the DS for distribution.

### Effect of Receipt

This primitive initiates distribution of the DSSDU through the DS.

## DS-UNITDATA.indication

### Function

This primitive indicates delivery of a DSSDU to either an AP or a portal.

### Semantics of the Service Primitive

The primitive parameters are as follows:

DS-UNITDATA.indication	(

Dssdu,

DestinationType

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dssdu | 802.11 MSDU | Any valid data unit per clause 6.2.1.1.2, including data and all parameters | Specifies the DSSDU that is being delivered by the DS. |
| DestinationType | Enumeration | DEST_AP, DEST_PORTAL | Specifies the type of entity to which the DSSDU is being delivered. |

### When Generated

This primitive is generated by the DS to deliver a DSSDU to an AP or a portal.

### Effect of Receipt

This primitive delivers a DSSDU to an AP or a portal.

## DS-STA-NOTIFY.request

### Function

This primitive requests an update to the DS’s STA-to-AP map.

### Semantics of the Service Primitive

The primitive parameters are as follows:

DS-STA-NOTIFY.request		(

STAAddress,

UpdateType

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| STAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the STA whose association status with the AP has changed. |
| UpdateType | Enumeration | ADD, MOVE, DELETE | Specifies the DS mapping update operation to be performed. |

### When Generated

This primitive is generated by an AP to update to the DS’s STA-to-AP map.

### Effect of Receipt

This primitive updates the DS’s STA-to-AP map, which controls to which AP the DS delivers DSSDUs that are destined for a given STA.

Editor: End of inserted text for new annex.

References:

1. IEEE 802.11-1999

2. submission 11-05-0120-09-0apf-ap-functional-description.doc

3. submission 11-04-1225-08-0apf-ap-function-summary.xls

4. submission 11-04-0639-01-000m-clause-10-primitive-updates.doc

5. ITU-T Recommendation X.210, 1993 Edition (dated 11/93), "INFORMATION TECHNOLOGY—OPEN SYSTEMS INTERCONNECTION—BASIC REFERENCE MODEL: CONVENTIONS FOR THE DEFINITION OF OSI SERVICES"; identical to ISO/IEC International Standard 10731.

6. submission 11-05-0115-00-0apf-ds-service-interface.doc

When the capabilities and functions defined by 802.11i are active, this MSDU processing by the AP may require specification of virtual MAC entities (or virtual ports) like those described in submission 11-05-0115-00-0apf-ds-service-interface.doc.