IEEE P802.11
Wireless LANs

| Draft TGu Terms and Definitions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-01-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | Siemens Roke Manor | Roke Manor Research Ltd Old Salisbury Lane Romsey Hampshire SO51 0ZN United Kingdom | +44 1794 833341 | stephen.mccann@roke.co.uk |

Abstract

This submission contains terms and definitions for TGu.

Major Contributions from:

| Eleanor Hepworth | Siemens Roke Mannor |
| --- | --- |
| Mike Moreton | STmicroelectronics |
| Cheng Hong | Panasonic |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Document Version History

| Revision | Comments | Date | Authors | Editor |
| --- | --- | --- | --- | --- |
| r0 | Initial Version | April 8,2005 | Eleanor Hepworth, Stephen McCann |  |
| r1 | Comments from TGu Teleconference | April 20, 2005 | Eleanor Hepworth |  |
| r2 | More commets | April 26, 2005 | Sabine Demel, Mike Moreton |  |
| r3 | Comments by email | May 3, 2005 | Mike Moreton |  |

Table of Contents

Terminology and Definitions Document Organization	3

Introduction	3

Acronyms	3

Core Terms & Definitions	5

Supplementary Terms & Definitions	7

References	8

# Terminology and Definitions Document Organization

This document is arranged in the following organization:

- the “core” terms and definitions that are to be used to describe 802.11u core concepts

- the important “supplementary” terms and definitions that could be used to help to describe some proposed system architecture

# Introduction

This document includes a collection of terms and definitions related to 802.11u. The purpose of this document is to promote consistent use of new terminology to describe 802.11u technology. The definitions in this document may eventually be integrated into the TGu amendment draft.

The following two figures describe the concepts and a sample configuration of the core terms and definitions that are proposed by the ad hoc team, respectively.

## Acronyms

The following acronyms are used in this document:

3G				Third Generation

3GPP				3G Partnership Project

3GPP2				3G Partnership Project 2

AAA				Authentication, Authorization, and Accounting

AN				Access Network

AP				Access Point

BSS				Basic Service Set

BSSID   			Basic Service Set Identifier

ESS        			Extended Service Set

DS				Distribution System

DSS				Distribution System Services

ESS				Extended Service Set

GPRS				General Packet Radio Service

GSM				Global System for Mobile Communication

IEEE				Institute of Electrical and Electronics Engineers

IETF				Internet Engineering Task Force

IP				Internet Protocol

LLC            			Logical Link Control

MAC				Medium Access Control

MLME        			MAC Layer Management Entity

MN				Mobile Node

MS				Mobile Station

MT		   		Mobile Terminal

PHY            			Physical Layer

PLME				PHY Layer Management Entity

QoS				Quality of Service

SAP             			Service Access Point

STA				Station

TSPEC				Traffic Specification

WLAN				Wireless Local Area Network

WMAN			Wireless Metropolitan Area Network

## Core Terms & Definitions

The following core terms are used to describe IEEE 802.11u basic concepts.

Accounting : The act of collecting information on resource usage for the purpose of trend analysis, auditing, billing, or cost allocation [2].

Authentication : The act of verifying a claimed identity, in the form of apre-existing label from a mutually known name space, as the originator of a message (message authentication) or as the end-point of a channel (entity authentication) [2].

Authorization : The act of determining if a particular right, such as access to some resource, can be granted to the presenter of a particular credential.

AAA : Authentication, Authorization, Accounting

AAA Proxy : An entity that acts as both a client and a server.  When a request is received from a client, the proxy acts as a AAA server.  When the same request needs to be forwarded to another AAA entity, the proxy acts as a AAA client. [2]

AAA Server (AAAS) : Server providing AAA functionality.

Authorisation Information:

- Policy that should be applied to user’s traffic in terms of routing provision.

- User Profile Information : Specifies the set of services that the user can access and what policy should be applied to their user data.  This includes:

  - basic connectivity service they are authorised to use in the local network, e.g. what QoS they are allowed.

  - what accounting policy should be applied by the local network.

  - what TOE services the users are allowed to access within which correspondent network.

Basic Connectivity Service : Service provided by the local network over which other services (e.g. Internet access) are provided.

c-plane : control plane that represents AAA exchange between the STA and the AAAS

Correspondent Network: the destination/source network for the user plane traffic travelling to and from the STA.  TOEs reside in the correspondent network.

Guarantor: The entity with which the Local Network has a relationship that provides authentication services for a particular station.  The Guarantor may be an SSPN, or a Proxy Network.

IEEE 802.11 AN : DS with IEEE 802.11 Access Points

Note : wireless local area network system: The WLAN system includes the distribution system (DS),

access point (AP), the AP's station (STA) and portal entities. It is also the logical location of distribution and

integration service functions of an extended service set (ESS). A WLAN system contains one or more APs

and zero or more portals in addition to the DS [3].

(Note : Perhaps should state that the CAPWAP scope is entirely within here) : Appears to be the same as TGs DS entity.

Note : wireless local area network system: The WLAN system includes the distribution system (DS),

access point (AP), the AP's station (STA) and portal entities. It is also the logical location of distribution and

integration service functions of an extended service set (ESS). A WLAN system contains one or more APs

and zero or more portals in addition to the DS [3].

Integration: The service that enables delivery of medium access control (MAC) service data units (MSDUs) between the distribution system (DS) and an existing, non-IEEE 802.11 local area network (LAN) (via a portal) [3].

Local Network : Network that interconnects IEEE 802.11 ANs and provides AAA Proxy and Gateway functionality.

Native Service: A user accessible service that is supported directly by the network in question.  For an 802.11 WLAN, the only native service is MSDU transfer, while for a cellular network, voice will usually be the native service.

Non-Native Service: A user accessible service that is not directly provided by the network in questions.  It can be supported on the network in question by providing an application independent description of the application requirements (for example, a TSPEC for a voice call over 802.11).

Proxy Network: a network that can not be accessed directly by a STA, but that exists mainly to provide trusted relationships between large numbers of home and visited networks.  A STA will know the identity of intermediate networks that can provide this service for its own home network – not sure I agree with this last bit – the STA does not necessarily have to know about the presence of an intermediate network, although the access network will in order to route AAA appropriately…

Roaming: where a user with a subscription to one service provider uses services offered by another (supported by a roaming agreement between service providers).  The roaming agreements provide users access to the underlying basic connectivity service by supporting AAA relationships between operators.  Roaming agreements specify the types of services that can be provided to users.

Subscription Service Provider (SSP): an organisation (operator) offering connection to network services, usually for a fee.  The user has a contractual relationship with the service provider.

Subscription Service Provider Network (SSPN): the network with which a STA has an established relationship.  The network maintains user subscription information, and is always the same for a given user identity.

“The Other End” (TOE): the termination point for a user data exchanged by the STA and another entity in the network.  Examples include web servers, correspondent nodes, the other end of a VoIP exchange etc.

u-plane : user plane that represented the application data between the STA and TOE

User Plane Gateway : Offers policy enforcement and mapping services to user plane traffic.

Figure 1 : Top Level Scenario

## Supplementary Terms & Definitions

The following supplementary terms are used to provide more additional descriptions to TGu system architecture.

# References

[1]	11-04-1477-02-000s-tgs-terms-and-definitions.doc

[2]	B. Aboba et al, “RFC 2989 - Criteria for Evaluating AAA Protocols for Network Access”, RFC 2989

[3]	P802.11REV-ma-D1.0.pdf