IEEE P802.11
Wireless LANs

| 802.11 TGw Protocol Security Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-05-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | Kapil.Sood@intel.com |
| Emily H Qi | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-7799 | Emily.H.Qi@intel.com |
| Jesse Walker | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-1849 | Jesse.Walker@intel.com |

Abstract

This document specifies a sebset of TGw protocol security requirements, for discussion and adoption as a guideline for developing proposals.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Table of Contents

1	General	4

2	Requirements	4

2.1	Req 100: Forgery Protection	4

2.2	Req 110: Confidentiality Protection	4

2.3	Req 120: Negotiating Security Protection	4

2.4	Req 130: Protect the Negotiation	4

2.5	Req 140: Compatibility with 802.11i/r Key Hierarchy	5

2.6	Req 150: Unicast-Broadcast-Multicast Protection	5

2.7	Req 160: Different schemes for Unicast and Broadcast	5

2.8	Req 170: Incremental Inclusion of new Management Frames	5

2.9	Req 180: Selected Deployment of categories of Management Frames protection	6

2.10	Req 190: Protection only after Key establishment	6

2.11	Req 200: Disregard Frames without keys	6

2.12	Req 210: Fragmentation	6

List of Figures

Error! No table of figures entries found.

# General

The motivation of this submission is to generate thought and discussion in the group towards the security requirements of the mechanism for protection of management frames.

Establishing consistent requirements upfront leads to fast protocol design convergence of multiple solutions, and hance a faster time-to-market and relevance to the efforts of this group.

# Requirements

This section lists the recommended requirements.

## Req 100: Forgery Protection

This protocol shall provide protection against forgeries.

Rationale:

This is the ultimate rationale for doing 802.11w.  Protection against forgeries is needed by almost every amendment to 802.11 (e, h, k, r, s, v), with potentially others in the future.

## Req 110: Confidentiality Protection

This protocol shall provide confidentiality protection.

Rationale:

There are selected subsets of management frames that require this property, including 802.11k Location Report Message. In addition, the work being proposed in 802.11v/k to access SNMP MIBs must support same or greater level of security policy implemented by higher level protocols, like SNMPv3.

## Req 120: Negotiating Security Protection

This protocol shall negotiate the security properties to be enforced for the protection of management frames.

Rationale:

This allows operation with legacy STAs that do not implement management frame protection.

## Req 130: Protect the Negotiation

There shall be a mechanism to protect negotiation of the security properties to be enforced for the protection of management frames.

Rationale:

Protect against MITM and downgrade attacks.

## Req 140: Compatibility with 802.11i/r Key Hierarchy

Management frame protection mechanism shall be compatible both with the 802.11i and TGr (Fast Transitioning) key hierarchies and key management schemes.

- Extend the 802.11i/r key hierarchies

- Derive additional keys from 802.11i/r key hierarchies

This mechanism shall operate alongside 802.11i and TGr protocols.

Rationale:

Designing, managing, implementing, and deploying key hierarchies is costly and complex.

Constrained environments and their lower end computational and memory sizes make this approach costly to adopt and widely deploy.

There will be deployments and implementations that will support one or both of 802.11i and 802.11r

## Req 150: Unicast-Broadcast-Multicast Protection

This protocol shall provide protection for unicast, multicast, and broadcast management frames.

Rationale:

Management frame are increasingly widely used for 802.11 purposes, including but not limited to:

- TGs Mesh routing

- TGkBeacon Report Request frames

- Disassociate and Deauthenticate frames

## Req 160: Different schemes for Unicast and Broadcast

The mechanism may be different for protecting unicast and multicast/broadcast management frames.

Rationale:

Unicast and multicast/broadcast requirements tend to have different security requirements.

Multicast/Broadcast confidentiality key must be changed everytime someone leaves or joins the group.  For integrity protection for broadcast/multicast, the 802.11i broadcast key is applicable only to stateless/idempotent messages, and not for stateful messages like mesh routing.  For integrity protection solution, the frames must be delivered before the integrity keys are derived.

## Req 170: Incremental Inclusion of new Management Frames

This protection mechanism shall provide for incremental inclusion of management frames, as new management frames are developed by 802.11.

Rationale:

The scheme must work with legacy STAs that do not implement management frame protection.  A generic scheme for protection of management frames will greatly improve applicability and deployment.

## Req 180: Selected Deployment of categories of Management Frames protection

This protocol shall provide for selected deployment of categories of management frames for protection.

Rationale:

Generic scheme will increase adoption, applicability, and deployment.  This will lend greater policy flexibility, at the expense of usability.

## Req 190: Protection only after Key establishment

This protocol shall protect management frames only after the establishment of transient session keys for protection of management frames.

Rationale:

Else, how do we get keys in place?  There just isn’t a way to protect the entire suite of management frames, and we should carefully avod the catch-22 scenario!

## Req 200: Disregard Frames without keys

A STA must treat any non-static unicast integrity protected management frames it received before the STA gets the integrity key, as a forgery.

Rationale:

The STA does not have the keys, and as such, the STA cannot verify the replay status of this message.  Therefore, the STA must assume that these messages are replay messages.

## Req 210: Fragmentation

This protocol shall handle fragmented management frames.

Rationale:

The 802.11 base specification allows management frames to be fragmented.