IEEE P802.11
Wireless LANs

| Normative Text Changes for Advanced Antennas |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-JUL-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4327 | joekwak@mindspring.com |

Abstract

This document contains normative text changes to TGk measurement reports to accommodate the use of advanced antennas.

This contribution addresses 13 LB73 comments:  #299, 465, 756, 757, 868, 1013, 1233, 1452, 1502, 1558, 1594, 1601, and 1602.

.

Modify clauses of TGk draftD2.0 as indicated below in red and blue highlights:

# 3. Definitions

Insert the following new definitions in alphabetical order, renumbering as necessary:

3.51 neighbor AP: Any AP to which one or more STAs in the current BSS could successfully associate from its current location.

3.52 non-serving channel: A channel that is not the operating channel of the BSS in which the STA is a member.

3.53 received channel power indicator (RCPI): An indication of the total channel power (signal, noise, and interference)of a received 802.11 frame as measured at the currently-in-use receiving antenna connector.

3.54 serving channel: The operating channel of the BSS of which the STA is a member.

3.55 location configuration information (LCI): As defined in IETF RFC 3825, includes latitude, longitude, and altitude, with resolution indicators for each. The reference datum for each is also included.

3.56 currently in use antenna: The antenna used for a particular noise or frame measurement.  For frame measurements of RCPI, the currently in use antenna is the antenna used to measure the reported RCPI for the body of the frame.

# 7. Frame formats

## 7.2 Format of individual frame types

### 7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Change the order 11, 14 and 18 information fields, and insert the order 22 field as follows:

Table 5—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Country | The Country element shall be present if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true |
| 14 | Power Constraint | Power Constraint element shall be present if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true |
| 18 | TPC Report | TPC Report element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true |
| 22 | AP Channel Report | The AP Channel Report element shall be present only if dot11RadioMeasurementEnabled is true. |
| 23 | Antenna Information | The Antenna Information element shall be present if dot11RadioMeasurementEnabled is true. |

#### 7.2.3.9 Probe Response frame format

Change the order 10, 13 and 17 information fields, insert the order 22 information field and change the starting order of the Requested information elements field in Table 12 as follows:

Table 12—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Country | Included if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 13 | Power Constraint | Shall be included if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true. |
| 17 | TPC Report | Shall be included if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 22 | AP Channel Report | Shall be included if dot11RadioMeasurementEnabled is true. |
| 23 | Antenna Information | The Antenna Information element shall be present if dot11RadioMeasurementEnabled is true. |
| 2224-n | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |

### 7.3.2 Information Elements

Insert Element ID 51-53 into Table 20 and change the Reserved row accordingly:

Table 20—Element IDs

| Information Element | Element ID |
| --- | --- |
| AP Channel Report | 51 |
| Neighbor Report | 52 |
| RCPI | 53 |
| Neighbor Report w/ TSF Information | TBD |
| Antenna Information | TBD |
| Reserved | 5154-255 |

##### 7.3.2.22.5 Noise Histogram Report

The format of the Measurement Report field of a Noise Histogram Report is shown in Figure k1.

|  | Channel Number |  |  | Regulatory Class |  | Actual Measurement Start Time |  |  | Antenna ID |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 |  |  | 1 |  | 8 |  |  | 1 |  |  |  |  |
|  |  | RPI 0 Density | RPI 1 Density |  | RPI 2 Density |  | RPI 3 Density | RPI 4 Density |  | RPI 5 Density | RPI 6 Density |  | RPI 7 Density |
| Octets: |  | 1 | 1 |  | 1 |  | 1 | 1 |  | 1 | 1 |  | 1 |

Figure k1—Measurement Report field format for a Noise Histogram Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Regulatory Class indicates the frequency band for which the measurement report applies. Valid values of Regulatory Class are shown in Error: Reference source not found.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

The Antenna ID field contains the identifying number for the antenna used for this measurement.  The valid range for the Antenna ID is 1 through 254. The value 0 indicates that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction.

Measurement Duration shall be set equal to the duration over which the Noise Histogram Report was measured, expressed in TUs.

The Noise Histogram Report shall contain the RPI densities observed in the channel for the eight RPI levels defined in Table k1. To compute the RPI densities, the STA shall measure the RCPI in the specified channel as a function of time over the measurement duration when NAV is equal to 0 (when virtual CS mechanism indicates idle channel). The time resolution of the RCPI measurements shall be in microseconds. The RCPI measurements are converted to a sequence of RPI values by quantizing the measurements according to Table k8. The RPI densities are then computed for each of the eight possible RPI values using Ceiling (255 * [Duration receiving at RPI value (microseconds) / (1024 * Measurement Duration)]). The sum of the RPI densities will be approximately 255, but could be up to 262 beacuse of rounding effects.

Table k1—RPI Definitions for a Noise Histogram Report

| RPI | RPI Measured Power (dBm) |
| --- | --- |
| 0 | RPI  92 |
| 1 | RPI -87 |
| 2 | -87  RPI -82 |
| 3 | -82  RPI -77 |
| 4 | -77 RPI -72 |
| 5 | -72 RPI -67 |
| 6 | -67 RPI -62 |
| 7 | -62 RPI -57 |
| 8 | -57 RPI |

##### 7.3.2.22.6 Beacon Report

The format of the Measurement Report field corresponding to a Beacon Report is shown in Figure k2.

|  |  | Actual Measurement Start Time |  |  | Measurement Duration |  |  | PHY Type | RCPI |  | BSSID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: |  | 8 |  |  | 2 |  |  | 1 | 1 |  | 6 |  |  |  |  |  |  |  |  |  |
|  | Antenna ID |  |  | Parent TSF |  |  | Target TSF |  |  | Beacon Interval |  | Capability Information |  |  |  |  | Received Elements |  |  |  |
| Octets | 1 |  |  | 4 |  |  | 8 |  |  | 2 |  | 2 |  |  |  |  | variable |  |  |  |

Figure k2—Measurement Report field format for a Beacon Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Table k1.

Regulatory Class indicates the frequency band for which the measurement report applies. Valid values of Regulatory Class are shown in Table k1.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Beacon Report was measured, expressed in TUs.

PHY Type indicates the physical medium type of the beacon or probe response frame being reported. Valid entries are coded according to the value of dot11PHYType.

RCPI indicates the received channel power of the beacon or probe response frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type.

The BSSID field contains the BSSID from the Beacon or Probe Response frame being reported.

The Antenna ID field contains the identifying number for the antenna used for this measurement.  If different antennas are used to receive the beacon preamble and the beacon frame body, this Antenna ID shall contain the identifying number for the antenna used to receive the beacon frame body. The valid range for the Antenna ID is 1 through 254. The value 0 indicates that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas. that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction.

The Parent TSF field shall contain the lower 4-octets of the measuring STA's TSF timer value at the time the Beacon or Probe Response frame being reported was received.

The Target TSF field shall contain the Timestamp field from the Beacon or Probe Response frame being reported.

The Beacon Interval field shall contain the Beacon Interval field from the Beacon or Probe Response frame being reported.

The Capability Information field shall contain the Capability Information field from the Beacon or Probe Response frame being reported.

The Received Elements portion of the Beacon report contains a number of information elements from the received Beacon, or Probe Response. All information elements that are present in the reported frame shall be included. TIM elements shall be truncated such that only the first 4 octets of the element are reported.

##### 7.3.2.22.7 Frame Report

The format of the Measurement Report field corresponding to a Frame Report is shown in Figure k3—Measurement Report field format for a Frame Report.

|  | Channel Number | Regulatory Class | Actual Measurement Start Time | Measurement Duration | Frame Report Entry |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 | 16 |

Figure k3—Measurement Report field format for a Frame Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Table k1.

Regulatory Class indicates the frequency band for which the measurement report applies. Valid values of Regulatory Class are shown in Table k1.

Actual Measurement Start Time shall be set to value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Frame Report was measured, expressed in TUs.

|  | Transmit Address | BSSID | RCPI | Antenna ID | Number of Frames |
| --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 6 | 1 | 1 | 1 |

Figure k4—Frame Report Entry format

Transmit Address contains the 6-octet MAC address of the transmitter STA of the measured frame.

The BSSID field contains the BSSID from the frames being reported.

RCPI indicates the received channel power of the received frame in dBm, as defined in the RCPI measurement clause for the PHY Type. This field shall be the RCPI value for the most recently received frame.

The Antenna ID field contains the identifying number for the antenna used for the RCPI measurement included in this report. .  If different antennas are used to receive the frame preamble and the frame body, this Antenna ID shall contain the identifying number for the antenna used to receive the frame body.The valid range for the Antenna ID is 1 through 254. The value 0 shall indicate that the antenna identifier is unknown.  The value 255 shall indicate that this measurement was made with multiple antennas. that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction.

Number of Frames is a count of the individual frames received with the indicated Transmit Address and BSSID during the measurement duration.

#### 7.3.2.29 Antenna Information element

The Antenna Information element contains the Antenna ID and Antenna Count fields as shown in Figure k9.

|  | Element ID | Length | Antenna ID |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure k5 — Antenna Information element format

The length field shall be set to 2.

The Antenna ID field contains the identifying number for the antenna used to transmit the frame containing this Information element. The valid range for the Antenna ID is 1 through 254. The value 0 shall indicate that the antenna identifier is unknown.  The value 255 shall indicate that this frame was transmitted using multiple antennas. that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction.

#### 7.4.2.4 Link Measurement Report Frame Format

The Link Measurement Request frame uses the Action frame body format and is transmitted by a STA in response to a Link Measurement Request frame. The format of the frame is shown in Figure k6.

|  | Category | Action | Dialog Token | TPC Report element | Receive Antenna ID | Transmit Antenna ID |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 | 1 | 1 |

Figure k6—Link Measurement Report frame body format

The Category field shall be set to indicate the Radio Measurement category according to Table 19a in 7.3.1.11.

The Action field shall be set to indicate a Link Measurement Report according to Error: Reference source not found in 7.4.2.

The Dialog Token field shall be set equal to the Dialog Token value in the corresponding Link Measurement Request frame.

The TPC Report element shall be set as described 7.3.2.16.

The Receive Antenna ID field contains the identifying number for the antenna used to receive the corresponding Link Measurement Request frame. The valid range for the Antenna ID is 1 through 254. The value 0 shall indicate that the antenna identifier is unknown.  The value 255 shall indicate that this frame was transmitted using multiple antennas. that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction.

The Transmit Antenna ID field contains the identifying number for the antenna used to transmit this Link Measurement Report frame. The valid range for the Antenna ID is 1 through 254. The value 0 shall indicate that the antenna identifier is unknown.  The value 255 shall indicate that this frame was transmitted using multiple antennas. that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction.

# Annex A - PICS

Add new row to PICs table inAnnex A as shown below:

### A.4.13 Radio Resource Measurement extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRMTBD | Antenna Information elements included in Beacon and Probe Reponse frames | 7.2.3.1, 7.2.3.9, 7.3.2.29, | CFk:M | Yes, No |

### ANNEX D

Modify dot11BeaconReportTable as follows:

-- ********************************************************************

-- * dot11NoiseHistogramReport TABLE

-- ********************************************************************

dot11NoiseHistogramReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Noise Histogram report that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 2 }

dot11NoiseHistogramReportEntry OBJECT-TYPE

SYNTAX Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11NoiseHistogramReportTable

Indexed by dot11NoiseHistogramRprtIndex."

INDEX { dot11NoiseHistogramRprtIndex }

::= { dot11NoiseHistogramReportTable 1 }

Dot11NoiseHistogramReportEntry ::=

SEQUENCE {

dot11NoiseHistogramRprtIndex Unsigned32,

dot11NoiseHistogramRprtRqstToken OCTET STRING,

dot11NoiseHistogramRprtIfIndex InterfaceIndex,

dot11NoiseHistogramMeasuringSTAAddr MacAddress,

dot11NoiseHistogramRprtChanNumber INTEGER,

dot11NoiseHistogramRprtRegulatoryClass INTEGER,

dot11NoiseHistogramRprtActualStartTime TSFType,

dot11NoiseHistogramRprtAntennaID INTEGER,

dot11NoiseHistogramRprtMeasurementDuration Unsigned32,

dot11NoiseHistogramRprtRPIDensity0 INTEGER,

dot11NoiseHistogramRprtRPIDensity1 INTEGER,

dot11NoiseHistogramRprtRPIDensity2 INTEGER,

dot11NoiseHistogramRprtRPIDensity3 INTEGER,

dot11NoiseHistogramRprtRPIDensity4 INTEGER,

dot11NoiseHistogramRprtRPIDensity5 INTEGER,

dot11NoiseHistogramRprtRPIDensity6 INTEGER,

dot11NoiseHistogramRprtRPIDensity7 INTEGER }

dot11NoiseHistogramRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Beacon Request elements in dot11NoiseHistogramReportTable,

greater than 0."

::= { dot11NoiseHistogramReportEntry 1 }

dot11NoiseHistogramRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the request token that was specified in the

measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11NoiseHistogramReportEntry 2 }

dot11NoiseHistogramRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Noise Histogram Report has been received

on. "

::= { dot11NoiseHistogramReportEntry 3 }

dot11NoiseHistogramMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Noise Histogram

report."

::= { dot11NoiseHistogramReportEntry 4 }

dot11NoiseHistogramRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which the this measurement report was made."

::= { dot11NoiseHistogramReportEntry 5 }

dot11NoiseHistogramRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER {

twoPointFourGHz(0),

fiveGHz(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band, in which this row of

noise histogram report has been conducted."

REFERENCE

"IEEE 802.11k, Error: Reference source not found"

::= { dot11NoiseHistogramReportEntry 6 }

dot11NoiseHistogramRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11NoiseHistogramReportEntry 7 }

dot11NoiseHistogramRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Antenna ID for this Noise Histogram measurement.

The Antenna ID contains the identifying number for the antenna used for this

measurement.  The valid range for the Antenna ID is 1 through 254. The value 0

indicates that the antenna identifier is unknown.  The value 255

indicates that this measurement was made with multiple antennas. that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction."

::= { dot11NoiseHistogramReportEntry 8 }

dot11NoiseHistogramRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Noise Histogram

Report was measured."

UNITS

"TUs"

::= { dot11NoiseHistogramReportEntry 9 }

dot11NoiseHistogramRprtRPIDensity0 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11 signals

from Power <= -87dBm."

::= { dot11NoiseHistogramReportEntry 10 }

dot11NoiseHistogramRprtRPIDensity1 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11 signals

from -87dBm < Power <= -82dBm."

::= { dot11NoiseHistogramReportEntry 11 }

dot11NoiseHistogramRprtRPIDensity2 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11 signals

from -82dBm < Power <= -77dBm."

::= { dot11NoiseHistogramReportEntry 12 }

dot11NoiseHistogramRprtRPIDensity3 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11 signals

from -77dBm < Power <= -72dBm."

::= { dot11NoiseHistogramReportEntry 13 }

dot11NoiseHistogramRprtRPIDensity4 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11 signals

from -72dBm < Power <= -67dBm."

::= { dot11NoiseHistogramReportEntry 14 }

dot11NoiseHistogramRprtRPIDensity5 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11 signals

from -67dBm < Power <= -62dBm."

::= { dot11NoiseHistogramReportEntry 15 }

dot11NoiseHistogramRprtRPIDensity6 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11 signals

from -62dBm < Power <= -57dBm."

::= { dot11NoiseHistogramReportEntry 16 }

dot11NoiseHistogramRprtRPIDensity7 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured RPI density for non-802.11 signals

from Power -57dBm <= Power."

::= { dot11NoiseHistogramReportEntry 17 }

-- ********************************************************************

-- * End of dot11NoiseHistogramReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11BeaconReport TABLE

-- ********************************************************************

dot11BeaconReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Beacon report that have been received by

the MLME. The report tables shall be maintained as FIFO to preserve

freshness, thus the rows in this table can be deleted for memory constraints

or other implementation constraints determined by the vendor. New rows shall

have different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase RprtIndex

for new reports being written in the table."

::= { dot11RRMReport 3 }

dot11BeaconReportEntry OBJECT-TYPE

SYNTAX Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11BeaconReportTable Indexed by dot11BeaconRprtIndex."

INDEX { dot11BeaconRprtIndex }

::= { dot11BeaconReportTable 1 }

Dot11BeaconReportEntry ::=

SEQUENCE {

dot11BeaconRprtIndex Unsigned32,

dot11BeaconRprtRqstToken OCTET STRING,

dot11BeaconRprtIfIndex InterfaceIndex,

dot11BeaconMeasuringSTAAddr MacAddress,

dot11BeaconRprtChanNumber INTEGER,

dot11BeaconRprtRegulatoryClass INTEGER,

dot11BeaconRprtActualStartTime TSFType,

dot11BeaconRprtMeasurementDuration Unsigned32,

dot11BeaconRprtPhyType INTEGER,

dot11BeaconRprtRCPI INTEGER,

dot11BeaconRprtBSSID MacAddress,

dot11BeaconRprtAntennaID INTEGER,

dot11BeaconRprtParentTSF TSFType,

dot11BeaconRprtTargetTSF TSFType,

dot11BeaconRprtBeaconInterval Unsigned32,

dot11BeaconRprtCFStatus INTEGER,

dot11BeaconRprtPrivacy TruthValue,

dot11BeaconRprtBSSMode INTEGER,

dot11BeaconRprtReceivedElements OCTET STRING }

dot11BeaconRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Beacon Report elements in dot11BeaconReportTable, greater than

0."

::= { dot11BeaconReportEntry 1 }

dot11BeaconRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the request token that was specified in the

measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11BeaconReportEntry 2 }

dot11BeaconRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Beacon Report has been received on."

::= { dot11BeaconReportEntry 3 }

dot11BeaconMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Beacon report."

::= { dot11BeaconReportEntry 4 }

dot11BeaconRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which the this row of report detected beacons

and probe responses."

::= { dot11BeaconReportEntry 5 }

dot11BeaconRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER {

twoPointFourGHz(0),

fiveGHz(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band, in which this row

of beacon report has been conducted."

REFERENCE

"IEEE 802.11k, Error: Reference source not found"

::= { dot11BeaconReportEntry 6 }

dot11BeaconRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11BeaconReportEntry 7 }

dot11BeaconRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Beacon Report was

measured."

UNITS

"TUs"

::= { dot11BeaconReportEntry 8 }

dot11BeaconRprtPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the PHY Type for this row of

Beacon Report."

UNITS

"dot11PHYType"

::= { dot11BeaconReportEntry 9 }

dot11BeaconRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the beacon or probe response

frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type.

"

::= { dot11BeaconReportEntry 10 }

dot11BeaconRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the beacon for this row of

Beacon Report."

::= { dot11BeaconReportEntry 11 }

dot11BeaconRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Antenna ID in the measurement

Report Frame for this row of the Beacon Report.

The Antenna ID contains the identifying number for the antenna used for this

measurement.  The valid range for the Antenna ID is 1 through 254. The value 0

indicates that the antenna identifier is unknown.  The value 255

indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction."

::= { dot11BeaconReportEntry 12 }

dot11BeaconRprtParentTSF OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value of the serving measuring STA’s

TSF value at the time the measuring STA received the beacon or probe

response frame."

::= { dot11BeaconReportEntry 13 }

dot11BeaconRprtTargetTSF OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value contained in the beacon or

probe response received by the measuring STA."

::= { dot11BeaconReportEntry 14 }

dot11BeaconRprtBeaconInterval OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the beacon interval value contained in the

beacon or probe response received by the measuring STA."

UNITS

"TUs"

::= { dot11BeaconReportEntry 15 }

dot11BeaconRprtCFStatus OBJECT-TYPE

SYNTAX INTEGER {

noPc(0),

pcDeliveryOnly(1),

pcDeliveryAndPolling(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the combination of CF pollable, and CF-Poll

Request bits in the capability information contained in this row of Beacon

report, thus indicating the contention-free capabilities of

the STA that this row of beacon report corresponds to."

::= { dot11BeaconReportEntry 16 }

dot11BeaconRprtPrivacy OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Privacy subfield of the capability

information contained in the beacon report element corresponding to this

row."

::= { dot11BeaconReportEntry 17 }

dot11BeaconRprtBSSMode OBJECT-TYPE

SYNTAX INTEGER {

unknown(0),

infrastructure(1),

adhoc(2)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates whether the BSS in the beacon report element

corresponding to this row is in the Ad-Hoc mode or not, based on the

information available from the beacon report corresponding to this row."

::= { dot11BeaconReportEntry 18 }

dot11BeaconRprtReceivedElements OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..100))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains a variable number of information elements. All

fields, except Timestamp, Beacon interval and Capability Information, that

are present in the measured beacon or probe response must be included if

the measured BSSID is different from the STA’s current BSS. The Beacon Report

will hereby enclose information about the 802.11 enhancements supported by the

STA that transmitted the beacon or probe response. All reported TIM elements

shall be truncated to 4 octets."

::= { dot11BeaconReportEntry 19 }

-- ********************************************************************

-- * End of dot11BeaconReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11FrameReport TABLE

-- ********************************************************************

dot11FrameReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Frame reports that have been received by

the MLME. The report tables shall be maintained as FIFO to preserve

freshness, thus the rows in this table can be deleted for memory constraints

or other implementation constraints determined by the vendor. New rows shall

have different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase RprtIndex

for new reports being written in the table."

::= { dot11RRMReport 4 }

dot11FrameReportEntry OBJECT-TYPE

SYNTAX Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FrameReportTable Indexed by dot11FrameRprtIndex."

INDEX { dot11FrameRprtIndex }

::= { dot11FrameReportTable 1 }

Dot11FrameReportEntry ::=

SEQUENCE {

dot11FrameRprtIndex Unsigned32,

dot11FrameRprtIfIndex InterfaceIndex,

dot11FrameRprtRqstToken Unsigned32,

dot11FrameRprtChanNumber INTEGER,

dot11FrameRprtRegulatoryClass INTEGER,

dot11FrameRprtActualMsmtStart TSFType,

dot11FrameRprtMeasurementDuration Unsigned32,

dot11FrameRprtMeasuringSTAAddr MacAddress,

dot11FrameRprtBSSID MacAddress,

dot11FrameRprtRCPI INTEGER,

dot11FrameRprtAntennaID INTEGER,

dot11FrameRprtNumberFrames INTEGER }

dot11FrameRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Frame Request elements in dot11FrameReportTable, greater than 0."

::= { dot11FrameReportEntry 1 }

dot11FrameRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Frame Report has been received on."

::= { dot11FrameReportEntry 2 }

dot11FrameRprtRqstToken OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Frame Request elements in dot11FrameRequestTable that corresponds

to this row of frame report. Since a single frame request can generate

multiple rows in the frame report table, one per BSSID, this

dot11FrameRprtRqstToken indicates which request this particular row

corresponds to. If this row of report is received without a particular

request, this attribute should be 0"

::= { dot11FrameReportEntry 3 }

dot11FrameRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which the this row of report detected frame request

responses."

::= { dot11FrameReportEntry 4 }

dot11FrameRprtRegulatoryClass OBJECT-TYPE

SYNTAX INTEGER {

twoPointFourGHz(0),

fiveGHz(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band, taken from Error: Reference source not found, TGk

Spec, in which this row of frame report has been conducted."

REFERENCE

" "

::= { dot11FrameReportEntry 5 }

dot11FrameRprtActualMsmtStart OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when measurement

started."

::= { dot11FrameReportEntry 6 }

dot11FrameRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Beacon Report

was measured, expressed in TUs."

::= { dot11FrameReportEntry 7 }

dot11FrameRprtMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of STA for this row of Frame report that it has been

received from."

::= { dot11FrameReportEntry 8 }

dot11FrameRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the STA that transmitted this

frame."

::= { dot11FrameReportEntry 9 }

dot11FrameRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" This attribute indicates the received channel power of the beacon or probe response

frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type.

"

::= { dot11FrameReportEntry 10 }

dot11FrameRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the Antenna ID in the measurement

Report Frame for this row of the Beacon Report.

The Antenna ID contains the identifying number for the antenna used for this

measurement.  The valid range for the Antenna ID is 1 through 254. The value 0

indicates that the antenna identifier is unknown.  The value 255

indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction."

::= { dot11FrameReportEntry 11 }

dot11FrameRprtNumberFrames  OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the number of received frames in the

measurement Report Frame for this row of Frame Report."

::= { dot11FrameReportEntry 12 }

-- ********************************************************************

-- * End of dot11FrameReport TABLE

-- ********************************************************************