IEEE P802.11
Wireless LANs

| Removal of Neighbor Report from Association |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Simon Black | Nokia |  | +44 1353 648567 | EXT-simon.3.black @nokia.com |

Abstract

This document contains the normative text changes required on IEEE802.11k/D2.0 to remove the neighbor report from the association exchange. This change was agreed at the May 2005 meeting in Cairns.

Acceptance of this normative text closes LB74 comments: 80, 83, 199, 205, 242, 337, 348, 349, 364, 366, 370, 413, 461, 474, 475, 490, 508, 524, 561, 583, 602, 626, 637, 640, 670, 671, 687, 697, 741, 776, 798, 799, 878, 1043, 1104, 1110, 1144, 1249, 1271, 1324, 1325, 1327, 1328, 1346, 1357.

[Instructions to the editor are given in square brackets using 14pt italic text]

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

[1. Replace 7.2.3.4 and 7.2.3.5 with the following:]

#### 7.2.3.4 Association Request frame format

Change the order 6 information field as follows:

Table 7—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true |

#### 7.2.3.5 Association Response frame format

Insert new row into table 8 as shown below:

Table 8—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | RCPI | The RCPI information element shall only be present if dot11RadioMeasurementEnabled is true. |

[2. Replace 7.3.2 with the following:]

### 7.3.2 Information Elements

Insert Element ID 51-53 into Table 20 and change the Reserved row accordingly:

Table 20—Element IDs

| Information Element | Element ID |
| --- | --- |
| AP Channel Report | 51 |
| Neighbor Report | 52 |
| RCPI | 53 |
| Reserved | 5154-255 |

[3. Delete 7.3.2.12 from the draft]

[4. Change 11.8.2 and the first paragraph of 11.8.3 as follows – double strikethrough used to indicate text to be deleted]

### 11.8.2 Requesting a Neighbor Report

An associated STA requesting a Neighbor Report shall send a Neighbor Report Request frame to its associated AP. A STA that is not yet associated may send an Association Request frame with a Request information element containing the element ID for the Neighbor Report or the Neighbor Report w/ Neighbor TSF Information in order to request a Neighbor Report in the Association Response frame. If the Request element in the Association Request contains the element ID for the Neighbor Report, then the Neighbor Report in the Association Response frame shall not contain the Neighbor TSF Information in the Neighbor Report. If the Request element in the Association Request frame contains the element ID for the Neighbor Report w/ Neighbor TSF Information, then the Neighbor Report returned in the Association Response frame shall also contain the Neighbor TSF Information, if available from the AP.

### 11.8.3 Receiving a Neighbor Report

An AP receiving a Neighbor Report Request shall respond with a Neighbor Report Response frame containing zero or more Neighbor Report elements. If the Neighbor Report is requested using the Request element in an Association Request frame, an AP receiving the request shall respond with the Neighbor Report in the corresponding Association Response frame. If the SSID information element is specified in the corresponding Neighbor Report Request frame, the Neighbor Report elements shall only contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that match the SSID. Otherwise, the Neighbor Report elements shall contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that match the current SSID with which the requesting STA is associated.

References: