IEEE P802.11
Wireless LANs

| Normative Text for Removing BSSID Info Field From Neighbor Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-05-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Martin Lefkowitz | Realnet Solutions Inc. | 3375 Scott Blvd Suite 303 Santa Clara, Ca |  | lefko@realnetsi.com |
| Bernard Aboba | Microsoft | One Microsoft Way, Redmond WA 98008 |  | bernarda@microsoft.com |
| Dan Harkins | Trapeze Networks |  |  | dharkins@trpz.com |
| David Nelson | Enterasys Networks |  |  | dnelson@enterasys.com |
| Malik Audeh | Tropos Networks | 555 Del Rey Ave Sunnyvale, CA 94085 |  | audeh@tropos.com |

Abstract

This document provides normative text that addresses Neighbour Report comments from letter ballot 73.  The instructions to the editior are to be applied to the next revision of the 802.11k draft amendment.  This document is based off 476/r0, accepted at the May 05 session not draft 2.0.  The change bars both address comments that needed to be addressed by the undoing of 476/r0 as well as the re-addressing of comments caused by this undoing.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

s

Add the following subclause to the end of definitions:

3.56. AP Reachablility:  An AP is reachable by a STA if an 802.1X pre-authentication frame sent by the STA to the AP BSSID can be received by the AP.

|  | . |
| --- | --- |

# Frame formats

## 7.1 MAC frame formats

### 7.3.2 Information elements

Change the following subclause with tables and figures included herein as follows:

7.3.2.27 Neighbor Report element

The format of the Neighbor Report element is shown in Figure k23.

|  |  |  | Zero or more, one for each neighbor |
| --- | --- | --- | --- |
|  | Element ID | Length | Neighbor List Entry |
| Octets: | 1 | 1 | variable |

Figure k22—Neighbor Report element format

|  | BSSID | BSSID Information | Channel Number | Channel Band | PHY Options | Neighbor TSF Information (Optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 2 | 1 | 1 | 1 | 4 |

Figure k23—Neighbor list entry format

The Element ID field shall be equal to the Neighbor Report value in Table 20.

The Length field is dependent on the number of Neighbor List Entries representing the neighboring APs being reported. Each entry describes an AP and consists of BSSID, BSSID Information, Channel Number, Regulatory Class, PHY Options, and optionally may include the TSF Information fields. The minimum value of the Length field is 0 (i.e., with no neighbor APs in the Neighbor Report element).

The BSSID is the BSSID of the BSS being reported. The subsequent fields: BSSID Information, Channel Number, Regulatory Class, PHY Options, and optionally TSF Information fields that are pertaining to the corresponding BSSID.

The BSSID Information field may be used to help determine neighbor AP BSS transition candidates. It shall be two octets in length and shall contain the subfields as shown in Figure k24.

|  | B0 B1 | B2 | B3 | B4 B9 | B10 B15 |
| --- | --- | --- | --- | --- | --- |
|  | AP Reachability | RSN | Key Scope | Capabilities | Reserved |
| Bits | 2 | 1 | 1 | 5 | 7 |

Figure k24—BSSID Information field format

The AP Reachability field indicates whether the AP represented by this BSSID is reachable by the STA that requested the Neighbor Report for the exchange of preauthentication frames as described in clause 8.4.6.1. The values are as follows:

Table k12 –Reachability field

| Value | Reachability | Usage |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | Not Reachable | A station sending a preauthentication frame to the BSSID will not receive a response even if the AP represented by the BSSID is capable of preauthentication. |
| 2 | Unknown | The AP is unable to determine if the value Reachable or Not Reachable is to be returned. |
| 3 | Reachable | The station sending a preauthentication frame to the BSSID can receive a response from an AP that is capable of preauthentication. |

• 	The Security bit, if set, indicates that the AP represented by this BSSID supports all of the security capabilities of the AP with which the STA is currently associated. If the bit is not set, it indicates that either the capabilities do not match or the information is not available to the AP at this time.

• 	The Key Scope bit, when set, indicates the AP represented by this BSSID has the same authenticator as the AP sending the report. If this bit is not set, it indicates a distinct authenticator or the information is not available.

• 	The Capabilities Subfield contains selected capability information for the AP represented by this BSSID. The bit fields within this subfield shall have the same meaning and be set equal to the equivalent bits within the Capability Information field being sent in the beacons by the AP being reported. The format of the Capabilities subfield is as follows:

|  | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Spectrum Management | QoS | APSD | Radio Measurement | Delayed Block Ack | Immediate Block Ack |
| Bits | 1 | 1 | 1 | 1 | 1 | 1 |

Figure k25—Capabilities Subfield

• 	Bits 10 – 15 are reserved and shall be set to 0 on transmission and ignored on reception.

Channel Number indicates the current operating channel of the AP represented by the BSSID in this neighbor list entry.

Regulatory Class shall contain an enumerated value from Table k1 specifying the frequency band in which the Current Channel is valid.

The PHY Options field represents the Condensed PHY type and TSF Information Flag as shown in the Figure k26.

|  | B0 B6 | B7 |
| --- | --- | --- |
|  | Condensed PHY Type | Neighbor TSF Information Flag |
| Bits: | 7 | 1 |

Figure k26—PHY Options field

• 	Condensed PHY Type is the lower 7 bits of the PHY Type of the AP represented by this BSSID. The PHY type shall be encoded as defined by dot11PHYType.

• 	TSF Information Flag is a one bit field. When set it indicates that a TSF Information field is present for this Neighbor List Entry. The Neighbor Report may have the TSF Information Flag set in some Neighbor List Entries and not set in others.

The TSF Information is 4 octets long and contains TSF Offset and Beacon Interval subfields.

|  | Neighbor TSF Offset | Neighbor Beacon Interval |
| --- | --- | --- |
| Octets: | 2 | 2 |

Figure k27—TSF Information field format

• 	The TSF Offset subfield is 2 octets long and contains the TSF timer offset, in terms of TUs between the serving AP and a neighbor AP, modulo the neighbor AP’s Beacon Interval and rounded to the nearest TU boundary.

• 	The Beacon Interval field is the beacon interval of the Neighbor AP being reported. This field is defined in clause 7.3.1.3 and illustrated in Figure 26.