IEEE P802.11
Wireless LANs

| Revised 11k PICS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-07-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Simon Black | Nokia |  | +44 1353 648567 | EXT-simon.3.black@nokia.com |

Abstract

This submission contains a revised Annex A (PICS) for 11k based on the baseline in D2.1 modified by the comment resolutions in 05/0678r0. Comment ID numbers 21 and 93 are not addressed, as they require changes to a section in the body of the 11k draft

This submission is based on Annex A in D2.1 and I have taken the liberty of fixing some issues relating to PICS item RRM19 – both of them! Hopefully the meaning of these items has not been affected by my corrections.

Note to the editor: This submission contains a complete Annex A that can be placed into the next draft. All changes present in Annex A in D2.1 are present here also. References in the PICS are relative to D2.1. If section numbering changes in a later draft this PICS may have to be updated.

R1 contains reference and numbering corrections and changes as a result for the removal of medium sense time histogram and hidden station measurements.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition

### A.4.3 IUT configuration

Append this entry to the end of this table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFk | Is Radio Resource Measurement supported? | 7.3.1.4 | O | Yes, No |

Insert this new clause after clause A.4.12:

### A.4.13 Radio Resource Measurement extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRM1 | Are the following Radio Resource Measurement capabilities supported? Radio Measurement Capability | 7.3.1.4 | CFk:M | Yes, No, N/A |
| RRM2 RRM2.1 RRM2.2 RRM2.3 RRM2.4 RRM2.5 RRM2.5.1 RRM2.5.2 RRM2.6 RRM2.6.1 RRM2.6.2 | Action frame protocol for measurements Measurement Request frame Measurement Report frame Link Measurement Request frame Link Measurement Report frame Neighbor Report Request Generate and transmit Neighbor Report Request Receive and process Neighbor Report Request Neighbor Report Response Generate and transmit Neighbor Report Response Receive and process Neighbor Report Response | 7.4 7.4.2.1 7.4.2.2 7.4.2.3 7.4.2.4 7.4.2.5 7.4.2.5 7.4.2.6, 7.3.2.27 7.4.2.6, 7.3.2.27 | CFk:M CFk:M CFk:M CFk:M CFk:M (CF2 AND CFk):M (CF1 AND CFk):M (CF1 AND CFk):M (CF2 AND CFk):M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM3 RRM3.1 RRM3.2 RRM3.3 RRM3.4 RRM3.5 RRM3.6 RRM3.7 RRM3.8 | General protocol for requesting and reporting of measurements Parallel Measurements Enable Measurement Requests Enable Autonomous Report Duration Mandatory Incapable Indication Refused Indication Repeated Measurement Measurement Pause | 7.3.2.21, 7.3.2.22, 11.11, 11.11.7 7.3.2.21, 7.3.2.22 7.3.2.21, 11.11.7 7.3.2.21, 11.11.7 7.3.2.21, 11.11.4 7.3.2.22 7.3.2.22 7.4.2.1, 11.11.7 7.3.2.21.10, 11.11.8.7 | CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/a Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No N/A |
| RRM4 RRM4.1 RRM4.2 RRM4.3 RRM4.4 RRM4.5 RRM4.6 | Beacon Measurement Type Beacon Request Active Scan mode Passive Scan mode Beacon table mode Reporting Conditions Beacon Report | 11.11, 11.11.8.1 7.3.2.21.6 7.3.2.21.6 7.3.2.21.6 7.3.2.21.6 7.3.2.21.6 7.3.2.22.6 | CFk:M CFk:M CFk:M CFk:M CFk:M CFk:O CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| * RRM5 RRM5.1 RRM5.2 | Frame Measurement Type Frame Request Frame Report | 11.11, 11.11.8.2 7.3.2.21.7 7.3.2.22.7 | CFk:O (CFk AND RRM5):M (CFk AND RRM5):M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM6 RRM6.1 RRM6.2 | Channel Load Measurement Type Channel Load Request Channel Load Report | 11.11, 11.11.8.3 7.3.2.21.4 7.3.2.22.4 | CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM7 RRM7.1 RRM7.2 | Noise Histogram Measurement Type Noise Histogram Request Noise Histogram Report | 11.11, 11.11.8.4 7.3.2.21.5 7.3.2.22.5 | CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM8 RRM8.1 RRM8.2 | STA Statistics Measurement Type STA Statistics Request STA Statistics Report | 11.11, 11.11.8.5 7.3.2.21.8 7.3.2.22.8 | CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM9 RRM9.1 RRM9.2 | LCI Measurement Type LCI Request LCI Report | 11.11, 11.11.8.6 7.3.2.21.9 7.3.2.22.9 | CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM10 | QoS Metrics Type | 11.11, 11.11.8.10 | CFk: O | Yes, No, N/A |
| RRM10.1 | QoS Metrics Type Request | 7.3.2.21.11 | CFk AND CF12:M | Yes, No, N/A |
| RRM10.2 | QoS Metrics Type Report | 7.3.2.22.11 | CFk AND CF12:M | Yes, No, N/A |
| RRM11 | AP Channel Report | 7.3.2.9, 7.3.2.26 | (CFk AND CF1):M | Yes, No, N/A |
| RRM12 | Neighbor Report Procedure | 11.12.2, 11.12.3 | CFk:M | Yes, No, N/A |
| RRM13 RRM13.1 RRM13.2 RRM13.3 RRM13.4 | RCPI Measurement RCPI Measurement for DSSS PHY at 2.4GHz RCPI Measurement for OFDM PHY at 5GHz RCPI Measurement for HR DSSS PHY at 2.4GHz RCPI Measurement for Extended Rate PHY at 2.4GHz | 15.4.8.5 17.2.3.5, 17.3.10.6, 17.5.4.3 18.4.8.5 19.9.5.14 | (CFk AND CF4): M (CFk AND CF6): M (CFk AND CF7): M (CFk AND CF9):M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM14 RRM14.1 RRM14.2 | RCPI Measurement during Active Scanning Respond with RCPI information element when requested Measurement of RCPI on Probe Request frames | 11.1.3.2.2 11.1.3.2.2 | (CFk AND CF1 AND CF12):M (CFk AND CF1 AND CF12):O | Yes, No, N/A Yes, No, N/A |
| RRM15 | Country element included in Beacon and Probe Response Frames | 7.2.3.1, 7.2.3.9, 7.3.2.9 | CFk:M | Yes, No, N/A |
| RRM16 | TPC Information in Beacon and Probe Response frames |  |  |  |
| RRM16.1 | Transmit Power Control (TPC) Report element included in Beacon and Probe Reponse frames | 7.2.3.1, 7.2.3.9, 7.3.2.18, 11.13 | CFk:M | Yes, No, N/A |
| RRM16.2 | Power Constraint element included in Beacon and Probe Response frames | 7.2.3.1, 7.2.3.9, 7.3.2.15 | CFk:O | Yes, No, N/A |
| RRM17 | Power Capability elements in Association and Reassociation frames | 7.2.3.4, 7.2.3.5, 11.10.1 | CFk:M | Yes, No, N/A |
| RRM18 RRM18.1 RRM18.2 RRM18.3 | Management Information Base dot11SMTRRMRequest dot11SMTRRMReport dot11SMTRRMConfig | Annex D Annex D Annex D | (CFk AND CF1):M (CFk AND CF1):M (CFk AND CF1):M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM19 | Measurement Pilot Frame | 7.1.3.1.2, 7.2.3.10, 7.3.1.18, 7.3.1.19, 7.3.1.20, 7.3.1.21, 7.3.1.22, 7.3.1.23, 11.9, 11.14 | CFk:O | Yes, No, N/A |

References: