IEEE P802.11
Wireless LANs

| Adjacent Channel Interference Test Methodology |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 1-17-2005 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mark Kobayashi | Broadcom Corporation | 190 Mathilda Place Santa Clara, CA 94086 | 408-543-3318 | mmk@broadcom.com |
| Jason Trachewsky | Broadcom Corporation | 190 Mathilda Place Santa Clara, CA 94086 | 408-543-3319 | jat@broadcom.com |
| Dalton Victor | Broadcom Corporation | 190 Mathilda Place Santa Clara, CA 94086 | 408-922-5824 | dvictor@broadcom.com |

Abstract

This document provides a procedure for testing performance with an Adjacent Channel Interferer (ACI) present following the metrics template document (document #1540r1). The purpose of this document is to submit a metric for consideration by TGT in the format prescribed by the template.  All text presented is to be inserted into the TGT recommended practice document with editorial changes.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1 Adjacent Channel Interference (ACI) Test Methodology

1.1 Introduction and Purpose

The purpose of this test is to determine the maximum ACI at which the DUT can achieve an acceptable PER as described in Std 802.11a-1999 Section 17.3.10.2  and 802.11b-1999 Section 18.4.8.3.  This test is applicable to STAs or Access Points.

The general setup for the test comprises one DUT, 3 or more Wireless Counter Parts (WLCP), an RF shielded environment, a power meter and 2 variable attenuators.  This methodology is designed to help determine the packet error rate (PER) at various levels of ACI.

1.3 Test Configuration

1.3.1 Resource Requirements

The following equipment is required to carry out tests using this methodology:

- An RF shielded environment.

- A traffic generator (reference node) capable of associating with the wireless interface(s) of the DUT and generating traffic at the desired rate.

- 3 or more WLCPs capable of generating traffic on the adjacent channel.

- 2 variable attenuators.

- 1 or more Power Meters

- A controller machine

The following setup is used:

Mux

AP outband

Controller

Power meter

AP inband

STA2   outband

+

STA1 dut   inband

Atten1 inband

Atten2 outband

+

1.3.2 Test Environment

This test can be done using any of the environments called out in the environmental methodology section of .11T

1.3.3 Permissible Error Margins and Reliability of Test

Prior to beginning the test, the test equipment described above shall be calibrated, and all test software verified. The test setup may be monitored during the test to ensure that the test conditions do not change.

The expected error margins for the test results are +/- 1dB.

1.4 Approach

1.4.1 Configuration Parameters

This methodology uses the basic PER test parameters.

1.4.1.1 Baseline Configuration

The baseline DUT setup that should be configured, measured and reported whenever this test is performed is as follows:

- Maximum transmit power setting.

- RTS threshold set to maximum MAC frame size.

- Fragmentation threshold set to maximum MAC frame size.

- MAC QoS and service priority disabled.

- Security settings matching those required by the test conditions (see below).

1.4.1.2 Modifiers

The baseline DUT setup parameters may be modified as follows to enable additional trials to be performed for this test. Only one variation should be tested at a time.

- Transmit power settings: 10%, 25%, 50% and 75% of maximum.

- RTS threshold: 256, 512, 1024, 1518 and 2048 bytes.

- Fragmentation threshold: 256, 512, 1024, 1518 and 2048 bytes.

- MAC QoS and service priority enabled.

- Multiple ACI sources

1.4.2 Test Conditions

The test conditions used while performing this test are as follows for OFDM:

| A | B | = b + 3 | c | = b + 3 + c |
| --- | --- | --- | --- | --- |
| Data Rate (Mbps) | Min. sensitivity (dBm) | signal level at DUT (in-band) (dBm) | Adjacent channel rejection (dB) | Signal level at DUT (adjacent channel) (dBm) |
| 6 | -82 | -79 | 16 | -63 |
| 9 | -81 | -78 | 15 | -63 |
| 12 | -79 | -76 | 13 | -63 |
| 18 | -77 | -74 | 11 | -63 |
| 24 | -74 | -71 | 8 | -63 |
| 36 | -70 | -67 | 4 | -63 |
| 48 | -66 | -63 | 0 | -63 |
| 54 | -65 | -62 | -1 | -63 |

The test conditions for CCK are:

- In-band signal level at DUT for 11Mbps shall be -70dBm.

- ACI signal level for 11Mbps shall be at least -35dBm.

1.4.3 Procedure

1.4.3.1 Calibration

Mux

AP outband

Contro-ller

Power meter

AP inband

STA2   outband

+

STA1 dut   inband

Atten1 inband

Atten2 outband

+

1

2

3

4

  - Use a Network Analyzer to take path loss measurements for the DUT path (1), the ACI path (2), DUT to powermeter (3), and ACI path to powermeter (4)

  - Measurements made on center frequencies of channel of interest

1.4.3.2 Test Procedure

  - Measure power for inband AP and outband AP using a powermeter

  - Adjust attenuators to achieve desired inband AP power to DUT, if possible otherwise abort test run

  - Adjust attenuators to achieve desired outband AP power to DUT, if possible otherwise abort current test point

  - measure the frame error rate (FER)

  - repeat above for each desired ACI test point following guidelines of requirements (Take measurements at a minimum of +/- 5dB around the ACI specification and/or until the FER specified is achieved.)

  - Note each additional point beyond the minimum ACI requirements meeting the Frame Error Conditions is considered the Adjacent Channel Rejection Margin (ACRM)

1.4.4 Reported Results

Reported results should include a plot (and/or table) of PER vs ACI.   Calibration numbers relating to various path losses should also be included.