IEEE P802.11
Wireless LANs

| Adjacent Channel Interference Test Methodology |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 9-17-2005 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mark Kobayashi | Broadcom Corporation | 190 Mathilda Place Santa Clara, CA 94086 | 408-543-3318 | mmk@broadcom.com |
| Jason Trachewsky | Broadcom Corporation | 190 Mathilda Place Santa Clara, CA 94086 | 408-543-3319 | jat@broadcom.com |
| Dalton Victor | Broadcom Corporation | 190 Mathilda Place Santa Clara, CA 94086 | 408-922-5824 | dvictor@broadcom.com |

Abstract

This document provides a procedure for testing performance with an Adjacent Channel Interferer (ACI) present following the metrics template document (document #1540r1). The purpose of this document is to submit a metric for consideration by TGT in the format prescribed by the template.  All text presented is to be inserted into the TGT recommended practice document with editorial changes.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1 Adjacent Channel Interference (ACI) Test Methodology

1.1 Introduction and Purpose

The purpose of this test is to determine the wireless performance of a DUT in the presence of ACI as described in Std 802.11a-1999 Section 17.3.10.2  and 802.11b-1999 Section 18.4.8.3.  This test is applicable to STAs or Access Points.  It is applicable to all usage models.

The general setup for the test comprises one DUT, 3 or more Wireless Counter Parts (WLCP) or interferers, an RF shielded environment, a power meter and 2 variable attenuators.  This methodology is designed to help determine the packet error rate (PER) at various levels of ACI.

1.3 Test Configuration

1.3.1 Resource Requirements

The following equipment is required to carry out tests using this methodology:

- An RF shielded environment.

- A traffic generator (reference node) capable of associating with the wireless interface(s) of the DUT and generating traffic at the desired rate.

- A traffic analyzer (dut node) capable of associating with the wireless interface(s) of the REF and analyzing traffic at the desired rate.

- 3 or more WLCPs or interferers capable of generating ACI.

- 4 variable attenuators.

- 1 or more Power Meters

- A controller machine

- A 4-way MUX

- A Circulator

- 2 directional couplers

The following setup is used:

Test Controller

Control of Attenuators

Control of Traffic Generator and Analyzer

Control of Power Meter

DUT

(client)

Attenuator

Traffic Generator

Traffic

Analyzer

WLCP (Inband)

Power Meter

Attenuator

WLCP (jammer)

WLCP (client jammer)

AWG

Attenuator

Legend:  ______ RF cables     ______ Wired LAN cables      ______ Control Cables

Terminator                  Directional coupler                   Circulator

MUX

Attenuator

1.3.2 Test Environment

This test can be done using any of the environments called out in the environmental methodology section of 802.11T.  The test may have to be modified per the procedure for each environment in order to yield valid results.  Any modifications should be indicated in the test report.

1.3.3 Permissible Error Margins and Reliability of Test

Prior to beginning the test, the test equipment described above shall be calibrated, and all test software verified. The test setup may be monitored during the test to ensure that the test conditions do not change.

The expected error margins for the test results are +/- 1dB.

1.4 Approach

1.4.1 Configuration Parameters

This methodology uses the basic PER test parameters.

1.4.1.1 Baseline Configuration

The baseline DUT setup that should be configured, measured and reported whenever this test is performed is as follows:

- Maximum transmit power setting.

- RTS threshold set to maximum MAC frame size.

- Fragmentation threshold set to maximum MAC frame size.

- MAC QoS and service priority disabled.

- Security settings matching those required by the test conditions (see below).

- Conducted Test Environment

1.4.1.2 Modifiers

The baseline DUT setup parameters may be modified as follows to enable additional trials to be performed for this test. Only one variation should be tested at a time.

- Non-802.11x compliant ACI source(s)

  - Characteristics of ACI should be included in test report such that the interferer can be regenerated during future runs

- Test in which the ACI network has two-way interaction with the DUT’s network

  - Should be done with various vendors for interoperability

- Measure sensitivity with fixed ACI in order to test RX filtering capability of DUT

- Test ACI performance for DUT at various fixed rates

- Test ACI performance for DUT with autorate negotiation

- Test ACI performance in other test environments (OTA, COAT, indoor LOS, etc.)

1.4.2 Test Conditions

The test conditions used while performing this test are as follows for OFDM:

| A | B | = b + 3 | c | = b + 3 + c |
| --- | --- | --- | --- | --- |
| Data Rate (Mbps) | Min. sensitivity (dBm) | signal level at DUT (in-band) (dBm) | Adjacent channel rejection (dB) | Signal level at DUT (adjacent channel) (dBm) |
| 6 | -82 | -79 | 16 | -63 |
| 9 | -81 | -78 | 15 | -63 |
| 12 | -79 | -76 | 13 | -63 |
| 18 | -77 | -74 | 11 | -63 |
| 24 | -74 | -71 | 8 | -63 |
| 36 | -70 | -67 | 4 | -63 |
| 48 | -66 | -63 | 0 | -63 |
| 54 | -65 | -62 | -1 | -63 |

The test conditions for basic ACI test of CCK signals are:

- In-band signal level at DUT for 11Mbps shall be -70dBm.

- ACI signal level for 11Mbps shall be at least -35dBm.

1.4.3 Procedure

1.4.3.1 Calibration

Test Controller

Control of Attenuators

Control of Traffic Generator and Analyzer

Control of Power Meter

DUT

(client)

Attenuator

WLCP (Inband)

Power Meter

Attenuator

WLCP (jammer)

WLCP

(client jammer)

AWG

Legend:  ______ RF cables     ______ Wired LAN cables      ______ Control Cables

Terminator                  Directional coupler                   Circulator

mux

A

PM

B

C

D

E

Traffic Generator

Traffic

Analyzer

Attenuator

Attenuator

- Measure path loss from every point to point A enumerated as B to A, C to A, D to A and E to A

- Measure path loss from every point to powermeter (PM) enumerated as B to PM, C to PM, D to PM and E to PM

- Measure path loss between point B and C with attenuator set to 0

Basic idea is to measure power at point A given input from any source.

1.4.3.2 Test Procedure

  - Select and note the ACI source being tested (see diagram and inputs to mux):

- AWG

- WLCP (client jammer) isolated

- WLCP (client jammer) non-isolated

- WLCP (jammer) non-isolated

  - Measure power for inband AP and outband interferer using a powermeter

  - Adjust attenuators to achieve desired inband AP power to DUT, if possible otherwise abort test run

  - Adjust attenuators to achieve desired outband interferer power to DUT, if possible otherwise abort current test point

  - measure the frame error rate (FER)

  - repeat above for each desired ACI test point following guidelines of requirements (Take measurements at a minimum of +/- 5dB around the ACI specification and/or until the FER specified is achieved.)

  - Note each additional point beyond the minimum ACI requirements meeting the Frame Error Conditions is considered the Adjacent Channel Rejection Margin (ACRM)

1.4.4 Reported Results

Reported results should include a plot (and/or table) of PER vs ACI.   Calibration numbers relating to various path losses should also be included.