IEEE P802.11
Wireless LANs

| Normative Text Changes for LB73 Statistics Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-July-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4327 | joekwak@mindspring.com |

Abstract

This document contains normative text changes to TGk to correct inconsistencies and make other suggested changes to Statistics clauses noted in the comments.  These comment responses are captured in doc 05/0761r0.

This contribution addresses 14 of 14 LB73 comments in the RPI category: #28, 86, 760, 1120, 1296, 1332, 1489, 1490, 1491, 1492, 1503, 1504, 1505, 1535.

.

Modify clauses of TGk draftD2.0 as indicated below in red and blue highlights:

##### 7.3.2.21.10 STA Statistics Request

The Measurement Request field corresponding to a STA Statistics Request is shown in Figure k1.

|  | Randomization Interval | Measurement Duration | Group Identity |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 |

Figure k1—Measurement Request field format for a STA Statistics Request

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TU. See 11.7.3.

The Measurement Duration field shall be set to the duration of the requested measurement in TUs. The Measurement, if accepted, shall be performed over the requested duration without regard to the Duration Mandatory bit in the Measurement Request Mode field. If a measurement over the requested duration is not possible, the request shall be refused. A Measurement Duration of 0 shall be used to request instantaneous values of the requested STA counters. A non-zero value of Measurement Duration indicates a request for the change in value (increases or decreases) in the statistics of the specified statistics group measured over the specified Measurement Duration.

Group Identity indicates the requested statistics group according to Table k1.

Table k1—Group Identity for a STA Statistics Request

| Statistics Group Name | Group Identity |
| --- | --- |
| STA Counters from dot11CountersTable | 0 |
| Parameters from dot11PhyDSSSTable | 1 |
| Parameters from dot11PhyHRDSSSTable | 2 |
| Parameters from dot11PhyOFDMTable | 3 |
| Parameters from dot11PhyTxPowerTable | 4 |
| Parameters from dot11OperationTable | 5 |
| Reserved | 6 – 255 |

##### 7.3.2.22.10 STA Statistics Report

|  | Measurement Duration | Statistics Group Data |
| --- | --- | --- |

Figure k

The STA Statistics Report reports the change in the requested Statistics Group Data measured within a non-zero Measurement Duration. When Measurement Duration is 0, the Statistics Report reports instantaneous values of the requested  Statistics Group Data.

The format of the Measurement Report field of a STA Statistics Report is shown in Figure k+1.

|  | Measurement Duration | Statistics Group Data |
| --- | --- | --- |
| Octets: | 2 | Variable |

Figure k+1—Measurement Report field format for a STA Statistics Report

The Measurement Duration field shall be set to the duration of the measurement in TUs included in the STA Statistics Request.

Statistics Group Data shall contain the requested statistics from the MIB related to the interface on which the request was received according to Table k2. Units used for reporting a statistic or change in statistic shall be the same units used for the statistic in the MIB.  If the requested Statistics Group Data is not defined for the measuring STA, the STA shall return Statistics Group Data with all octets set to X'FF'.

Table k2—Group Identity for a STA Statistics Report

| Group Identity Requested | Statistics Returned |
| --- | --- |
| 0 | dot11CountersGroup for the Interface on which the STA Statistics Request was received (with the exception of WEPUndecryptableCount):: dot11TransmittedFragmentCount (Counter32), dot11MulticastTransmittedFrameCount (Counter32), dot11FailedCount (Counter32), dot11RetryCount (Counter32), dot11MultipleRetryCount (Counter32), dot11FrameDuplicateCount (Counter32), dot11RTSSuccessCount (Counter32), dot11RTSFailureCount (Counter32), dot11ACKFailureCount (Counter32) dot11ReceivedFragmentCount (Counter32), dot11MulticastReceivedFrameCount (Counter32), dot11FCSErrorCount (Counter32), dot11TransmittedFrameCount (Counter32) |
| 1 | dot11PhyDSSSTable Group:: dot11CurrentChannel (INTEGER), dot11CCAModeSupported (INTEGER), dot11CurrentCCAMode (INTEGER), dot11EDThreshold (Integer32). Instantaneous values are reported; Measurement Duration is ignored. |
| 2 | dot11PhyHRDSSSTable Group: dot11ChannelAgilityPresent (TruthValue), dot11ChannelAgilityEnabled (TruthValue), dot11HRCCAModeSupported (INTEGER) Instantaneous values are reported; Measurement Duration is ignored. |
| 3 | dot11PhyOFDMTable Group: dot11CurrentFrequency (INTEGER), dot11TIThreshold (Integer32), Instantaneous values are reported; Measurement Duration is ignored. |
| 4 | dot11PhyTxPowerTable Group: dot11NumberSupportedPowerLevels (INTEGER), dot11TxPowerLevel1 (INTEGER), dot11TxPowerLevel2 (INTEGER), dot11TxPowerLevel3 (INTEGER), dot11TxPowerLevel4 (INTEGER), dot11TxPowerLevel5 (INTEGER), dot11TxPowerLevel6 (INTEGER), dot11TxPowerLevel7 (INTEGER), dot11TxPowerLevel8 (INTEGER), dot11CurrentTxPowerLevel (INTEGER). Instantaneous values are reported; Measurement Duration is ignored. |
| 5 | dot11OperationTable Group: dot11RTSThreshold (INTEGER), dot11ShortRetryLimit (INTEGER), dot11LongRetryLimit (INTEGER), dot11FragmentationThreshold (INTEGER), dot11MaxTransmitMSDULifetime (Unsigned32), dot11MaxReceiveLifetime (Unsigned32). Instantaneous values are reported; Measurement Duration is ignored. |
| 6 – 255 | None |

The format of the Measurement Report field for dot11Counters Group is shown in Figure k+2.

|  | Transmitted Fragment Count | Multicast Transmitted Frame Count | Failed Count | Retry Count | Multiple Retry Count | Frame Duplicate Count | RTS Success Count |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
|  | RTS Failure Count | ACK Failure Count | Received Fragment Count | Multicast Received Frame Count | FCS Error Count | Transmitted Frame Count |  |
| Octets: | 4 | 4 | 4 | 4 | 4 | 4 |  |

Figure k+2—Measurement Report field format for dot11Counters Group

The Fragment MSDU Count field contains dot11TransmittedFragmentCount as defined in Annex D.

The Multicast Transmitted Frame Count field contains dot11MulticastTransmittedFrameCount as defined in Annex D.

The Failed Count field contains dot11FailedCount as defined in Annex D.

The Retry Count field contains X as defined in Annex D.

The Multiple Retry Count field contains dot11RetryCount as defined in Annex D.

The Frame Duplicate Count field contains dot11FrameDuplicateCount as defined in Annex D.

The RTS Success Count field contains dot11RTSSuccessCoun as defined in Annex D.

The RTS Failure Count field contains dot11RTSFailureCount as defined in Annex D.

The ACK Failure Count field contains dot11ACKFailureCount as defined in Annex D.

The Received Fragment Count field contains dot11ReceivedFragmentCount as defined in Annex D.

The Multicast Received Frame Count field contains dot11MulticastReceivedFrameCount as defined in Annex D.

The FCS Error Count field contains dot11FCSErrorCount as defined in Annex D.

The Transmitted Frame Count field contains dot11TransmittedFrameCount as defined in Annex D.

The format of the Measurement Report field for dot11PhyDSSSTable Group is shown in Figure k+3

|  | Current Channel | CCA Mode Supported | Current CCA Mode | ED Threshold |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 |  |

Figure k+3—Measurement Report field format for dot11PhyDSSSTable Group

The Current Channel field contains dot11CurrentChannel as defined in Annex D.

The CCA Mode Supported field contains dot11CCAModeSupported as defined in Annex D.

The Current CCA Mode field contains dot11CurrentCCAMode as defined in Annex D.

The ED Threshold field contains dot11EDThreshold as defined in Annex D.

The format of the Measurement Report field for dot11PhyHRDSSSTable Group is shown in Figure k+4.

|  | Channel Agility Present | ChannelAgility Enabled | HR CCA Mode Supported |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |  |

Figure k+4—Measurement Report field format for dot11PhyHRDSSSTable Group

The Channel Agility Present field contains dot11ChannelAgilityPresent as defined in Annex D, false is indicated by 0, true is indicated by 1.

The Channel Agility Enabled field contains dot11ChannelAgilityEnabled as defined in Annex D, false is indicated by 0, true is indicated by 1.

The HR CCA Mode Supported field contains dot11HRCCAModeSupported as defined in Annex D.

The format of the Measurement Report field for dot11PhyOFDMTable Group is shown in Figure k+5

|  | Current Frequency | TI Threshold |  |
| --- | --- | --- | --- |
| Octets: | 1 | 4 |  |

Figure k+5—Measurement Report field format for dot11PhyOFDMTable Group

The Current Frequency field contains dot11CurrentFrequency as defined in Annex D.

The TI Threshold field contains dot11TIThreshold as defined in Annex D.

The format of the Measurement Report field for dot11PhyTxPowerTable Statistics Group is shown in Figure k+6.

|  | Number Supported Power Levels | Tx Power Level 1 | Tx Power Level 2 | Tx Power Level 3 | Tx Power Level 4 |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |
|  | Tx Power Level 5 | Tx Power Level 6 | Tx Power Level 7 | Tx Power Level 8 | Current Tx Power Level |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure k+6—Measurement Report field format for dot11PhyTxPowerTable Group

The Number Supported Power Levels field contains dot11NumberSupportedPowerLevels as defined in Annex D.

The Tx Power Level 1 field contains dot11TxPowerLevel1 as defined in Annex D.

The Tx Power Level 2 field contains dot11TxPowerLevel2 as defined in Annex D.

The Tx Power Level 3 field contains dot11TxPowerLevel3 as defined in Annex D.

The Tx Power Level 4 field contains dot11TxPowerLevel4 as defined in Annex D.

The Tx Power Level 5 field contains dot11TxPowerLevel5 as defined in Annex D.

The Tx Power Level 6 field contains dot11TxPowerLevel6 as defined in Annex D.

The Tx Power Level 7 field contains dot11TxPowerLevel7 as defined in Annex D.

The Tx Power Level 8 field contains dot11TxPowerLevel8 as defined in Annex D.

The Current Tx Power Level field contains dot11CurrentTxPowerLevel as defined in Annex D.

The format of the Measurement Report field for dot11OperationTable Group is shown in Figure k+7.

|  | RTS Threshold | Short Retry Limit | Long Retry Limit | Fragmentation Threshold | Max Transmit MSDU Lifetime | Max Receive Lifetime |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 | 4 |  |

Figure k+7—Measurement Report field format for dot11OperationTable Group

The RTS Threshold field contains dot11RTSThreshold as defined in Annex D.

The Short Retry Limit field contains dot11ShortRetryLimit as defined in Annex D.

The Long Retry Limit field contains dot11LongRetryLimit as defined in Annex D.

The Fragmentation Threshold field contains dot11FragmentationThresholdas defined in Annex D.

The Max Transmit MSDU Lifetime field contains dot11MaxTransmitMSDULifetime as defined in Annex D.

The Max Receive Lifetime field contains dot11MaxReceiveLifetime as defined in Annex D.