IEEE P802.11
Wireless LANs

| Normative Text Proposal for Load Balancing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-5-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emily H. Qi | Intel Corporation | 2111 N.E 25th Ave. Hillsboro, OR 97124 | 503-264-7799 | Emily.h.qi@intel.com |
| Jesse Walker | Intel Corporation | 2111 N.E 25th Ave. Hillsboro, OR 97124 | 503-712-1849 | jesse.walker@intel.com |
| Marian Rudolf | InterDigital Communications | Montreal, QC | 514-904-6258 | Marian.Rudolf@InterDigital.com |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | joekwak@sbcglobal.net |
| Joe Epstein | Meru Networks | 1309 S Mary Ave, Sunnyvale CA | 408-215-5345 | jepstein@merunetworks.com |
| Feng Li | ZTE Corporation |  | 86-29-88723130 | li.feng8@zte.com.cn |
| Sudheer Matta | Trapeze Networks | 5753 W. Las Positas Blvd, Pleasanton, CA 94588 | 9254742290 | matta@trapezenetworks.com |

- Abstract

This document contains a normative text proposal in support of an AP load balancing protocol to improve wireless network throughput and QoS effectiveness in accordance with REQ2030.

Contents

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

7. Frame formats	3

7.1 MAC frame formats	3

7.3.2.27 Neighbor Report element	3

7.3.2.27.1 TSF Information	3

7.3.2.27.2 Roaming Candidate Preference Sub-element	4

7.3.2.27.3 QBSS Available Admission Capacity sub-elements	4

7.4 Action frame format details	5

7.4.6 Wireless Network Management action details	5

7.4.6.5 Roaming Management Query frame format	5

7.4.6.6 Roaming Management Request frame format	6

7.4.6.7 Roaming Management Response frame format	7

10. Layer management	8

10.3 MLME SAP Interface	8

10.3.x Roaming Management	8

10.3.x.1 MLME-RMQUERY.request	9

10.3.x.1.1 Function	9

10.3.x.1.2 Semantics of the Service Primitive	9

10.3.x.1.3 When Generated	10

10.3.x.1.4 Effect of Receipt	10

10.3.x.2 MLME-RMQUERY.confirm	10

10.3.x.2.1 Function	10

10.3.x.2.2 Semantics of the Service Primitive	10

10.3.x.2.3 When Generated	11

10.3.x.2.4 Effect of Receipt	11

10.3.x.3 MLME-RMQUERY.indication	12

10.3.x.3.1 Function	12

10.3.x.3.2 Semantics of the Service Primitive	12

10.3.x.3.3 When Generated	12

10.3.x.3.4 Effect of Receipt	12

10.3.x+1 Roaming Management Request	12

10.3.x+1.1 MLME-RMREQUEST.request	12

10.3.x+1.1.1 Function	12

10.3.x+1.1.2 Semantics of the Service Primitive	12

10.3.x+1.1.3 When Generated	13

10.3.x+1.1.4 Effect of Receipt	13

10.3.x+1.2 MLME-RMREQUEST.confirm	13

10.3.x+1.2.1 Function	13

10.3.x+1.2.2 Semantics of the Service Primitive	13

10.3.x+1.2.3 When Generated	14

10.3.x+1.2.4 Effect of Receipt	14

10.3.x+1.3 MLME-RMREQUEST.indication	15

10.3.x+1.3.1 Function	15

10.3.x+1.3.2 Semantics of the Service Primitive	15

10.3.x+1.3.3 When Generated	15

10.3.x+1.3.4 Effect of Receipt	15

10.3.x+2 Roaming Management Response	15

10.3.x+2.1 MLME-RMRESPONSE.request	16

10.3.x+2.1.1 Function	16

10.3.x+2.1.2 Semantics of the Service Primitive	16

10.3.x+2.1.3 When Generated	16

10.3.x+2.1.4 Effect of Receipt	16

10.3.x+2.2 MLME-RMRESPONSE.confirm	16

10.3.x+2.2.1 Function	16

10.3.x+2.2.2 Semantics of the Service Primitive	16

10.3.x+2.2.3 When Generated	17

10.3.x+2.2.4 Effect of Receipt	17

10.3.x+2.3 MLME-RMRESPONSE.indication	18

10.3.x+2.3.1 Function	18

10.3.x+2.3.2 Semantics of the Service Primitive	18

10.3.x+2.3.3 When Generated	18

10.3.x+2.3.4 Effect of Receipt	18

11.15.4 Roaming Management for Network Load Balancing	18

11.15.4.1 Roaming Management Query	19

11.15.4.2 Roaming Management Request	19

11.15.4.3 Roaming Management Response	20

A. 4.14 Wireless Network Management extensions	22

# 7. Frame formats

## 7.1 MAC frame formats

#### 7.3.2.27 Neighbor Report element

- Insert new row into table 43B  in 802.11k Draft 4.0 as follows:

Table 43B—Optional Extensions

| Sub-Element ID | Name |
| --- | --- |
| 0 | Reserved |
| 1 | TSF Information |
| 2 | Roaming Candidate Preference |
| 3 | QBSS Available Admission Capacity |
| 11 | QBSS Load |
| 4, 5, 6, 7, 8, 9, 10, 12-255 | Reserved |

- Insert the following paragraph below table 43B in 802.11k draft 4.0:

QBSS Load is optionally present in the Neighbor Report element included in Roaming Management Request frame. The format of QBSS Load element is defined in 7.3.2.28.

Change the following text after Table 43B in 802.11k Draft 4.0 as indicated below:

##### 7.3.2.27.1 TSF Information

The TSF offset Information sub-element is 4 octets long and contains TSF Offset and Beacon Interval subfields.

|  | TSF Offset | Beacon Interval |
| --- | --- | --- |
| Octets: | 2 | 2 |

Figure k1—TSF Information sub-element offset field data field format

The value of the TSF Information sub-element length field is 4.

The TSF Offset subfield is 2 octets long and contains the neighbor AP’s TSF timer offset. This is the time difference, in TU units, between the serving AP and a neighbor AP. This offset is given modulo the neighbor AP’s Beacon Interval and rounded to the nearest TU boundary.

The Beacon Interval subfield is the beacon interval of the Neighbor AP being reported. This sub field is defined in clause 7.3.1.3 and illustrated in Figure 26.

- Insert the following new clause after Clause 7.3.2.27 in 802.11k Draft 4.0 as indicated below:

##### 7.3.2.27.2 Roaming Candidate Preference Sub-element

Roaming Candidate Preference Sub-element is present in the Neighbor Report element included in Roaming Management Request frame. The format of Roaming Candidate Preference sub-element Data field is shown in Figure v+1.

|  | Preference |
| --- | --- |
| Octets: | 1 |

Figure v+1— Roaming Candidate Preference sub-element Data field format

The value of the Roaming Candidate Information sub-element length field is 1.

The Preference field indicates the network preference for roaming to the BSS listed in this Roaming Candidate List Entries. The Preference value is a number ranging from 0 to 255, as defined in Table v+1, indicating an ordering of preferences for the roaming candidates for this STA. A value of 0 indicates that the AP being listed is a "barred" AP.  The values between 1 and 255 provide the indication of order, with 1 indicating the most preferred AP within the given candidate list, increasing numbers representing decreasing preference relative only to entries with lower values of the Preference field, and equal numbers representing equal preference.  Additional details describing Preference are provided in 11.15.4.2.

Table v+1 — Preference field values

| Preference field value | Description |
| --- | --- |
| 0 | Barred AP |
| 1-255 | Relative values used to indicate the preferred ordering of BSSs, with 1 as the highest order of preference. |

##### 7.3.2.27.3 QBSS Available Admission Capacity sub-elements

QBSS Available Admission Capacity sub-element is optionally present in the Neighbor Report element included in Roaming Management Request frame. The format of QBSS Available Admission Capacity sub-element is shown in Figure v+2.

|  | Available Admission Capacity Bitmask | Available Admission Capacity List |
| --- | --- | --- |
| Octets: | 2 | 2 x (total number of non-zero bits in Available Admission Capacity Bitmask) |

Figure v+2— QBSS Available Admission Capacity Sub-element Data field format

The value of the QBSS Available Admission Capacity sub-element Length field is set to 2 + 2 x (total number of non-zero bits in Available Admission Capacity Bitmask).

Available Admission Capacity Bitmask field indicates the UP traffics that have Available Admission Capacity specified in the following Available Admission Capacity List. The format of Available Admission Capacity Bitmask is defined in Table v+2.

Table v+2: Available Admission Capacity Bitmask Definition

| Bit (s) | Description |
| --- | --- |
| 0 | UP 0 Traffic |
| 1 | UP 1 Traffic |
| 2 | UP 2 Traffic |
| 3 | UP 3 Traffic |
| 4 | UP 4 Traffic |
| 5 | UP 5 Traffic |
| 6 | UP 6 Traffic |
| 7 | UP 7 Traffic |
| 8-15 | Reserved |

- The bit is set to 1 to indicate that the Available Admission Capacity for the corresponding traffic type is present in the Available Admission Capacity List field. The bit is set to 0 to indicate that the Available Admission Capacity for the corresponding traffic type is not present in the Available Admission Capacity List field.

The Available Admission Capacity List comprises a sequence of Available Admission Capacity fields corresponding respectively to the non-zero bits in the Available Admission Capacity Bitmask field.

The Available Admission Capacity field is 2 octets long and contains an unsigned integer that specifies the remaining amount of medium time available via explicit admission control for the corresponding UP traffic, in units of 32 microsecond periods per 1 second. The field is helpful for roaming non-AP QSTAs to select a QAP that is likely to accept future admission control requests, but it does not represent a guarantee that the HC will admit these requests.

## 7.4 Action frame format details

### 7.4.6 Wireless Network Management action details

Insert the following rows into Table v7 and change the reserved row accordingly:

Table v7 — Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 4 | Roaming Management Query |
| 5 | Roaming Management Request |
| 6 | Roaming Management Response |
| 7-255 | Reserved |

- Insert the following new clause after Clause 7.4.6.4 in 802.11v Draft 0.01 as indicated below:

#### 7.4.6.5 Roaming Management Query frame format

The Roaming Management Query frame uses the Action frame body format and is transmitted by a STA requesting information in the Roaming Management Request about roaming candidate AP’s. The format of the Roaming Management Query frame body is shown in Figure v+3.

|  | Category | Action | Dialog Token | Roaming Query Reason |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure v+3 — Roaming Management Query frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Roaming Management Query, as specified in Table v7 in 7.4.6.

The Dialog Token field is set to a non-zero value chosen by the STA sending the Roaming Management Query to identify the query/request/response transaction.

The Roaming Query Reason field contains the reason code for a Roaming Management Query as defined in Table v+2.

Table v+2— Roaming Query Code Definitions for a Roaming Management Query

| Roaming Query Reason Code | Roaming query reason description |
| --- | --- |
| 0 | Unspecified |
| 1 | Too low throughput for current service(s) or poor link |
| 2 | Loss of signal in current BSS |
| 3 | Unacceptable Delay/Jitter for current service(s) |
| 4 | Insufficient QoS capacity from current service(s) (TSPEC rejected) |
| 5 | Frequent Transition |
| 6-255 | Reserved |

#### 7.4.6.6 Roaming Management Request frame format

The Roaming Management Request frame uses the Action frame body format and is transmitted by a STA in response to a Roaming Management Query frame, or  autonomously. The format of the Roaming Management Request frame body is shown in Figure v+4.

|  | Category | Action | Dialog Token | Request mode | Disassociation Timer (optional) | Expiry Timer | Roaming Candidate List Entries |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | variable |

Figure v+4 —Roaming Management Request frame body format

The Category field is set equal to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set equal to the value indicating Roaming Management Request frame, as specified in Table v7 in 7.4.6.

The Dialog Token field is set equal to the value in the corresponding Roaming Management Request frame. If the Roaming Management Request frame is not being transmitted in response to a Roaming Management Query frame then the Dialog token is set equal to zero.

The Request mode field (shown in Figure v+5) is a bit field with the following bits defined:

|  | No Recommendation | Abridged |  | Disassociation Imminent |  | Reserved |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit: | 0 | 1 |  | 2 |  | 3 - 7 |  |

Figure v+5— Request Mode field

- No recommendation (bit 0) is used to indicate how the STA should consider the information contained in this frame. No Recommendation is set to “1” to indicate the STA may ignore this Roaming Candidate List. The No Recommendation bit set to "1" may for example be used in cases where the sender does not currently have a recommendation for a preferred candidate list following reception of a Roaming Management Query. No Recommendation set to "0" shall indicate the sender expects the receiving STA to process this frame according to the procedure described in 11.15.4.

- Disassociation Imminent (bit 1) is used to indicate whether the STA will be disassociated from the current AP.  Disassociation Imminent is set to “1” when STA will be disassociated from the current AP. Disassociation Imminent is set to “0” when Roaming Management Request is a “Suggestion” for STA’s roaming.

- Abridged (bit 2) is used to indicate to the recipient of the frame the intended treatment of all BSSIDs not listed in the Roaming Candidate List.  Abridged is set to “1” to indicate that every BSSID under consideration by the receiving STA should be treated as if it were present in the Roaming Candidate List with a Preference value of 0.  Abridged is set to “0” to indicate that the sender of this frame has no recommendation for or against any BSSID not present in the Roaming Candidate List Entries.

- All other bits shall be set to "0" and ignored on reception.

The Disassociation Timer field is set to the number of beacon transmission times (TBTTs) until the serving AP will send a Disassociation frame to this STA. A value of 1 indicates that the serving AP will send a Disassociation frame before the next TBTT. A value of 0 indicates that the serving AP has not determined when it will send a Disassociation frame to this STA.  The Disassociation Timer field is included in the Roaming Management Request frame when the Disassociation Imminent bit is set to "1", and is not included when the Disassociation Imminent bit is set to "0".

The Expiry Timer field is set to the number of beacon transmission times (TBTTs) until this recommendation of this roaming candidate is not valid. A value of 1 indicates the roaming candidate recommendation is valid until the next TBTT. A value of 0 indicates that the expiry timer is not specified for this roaming candidate recommendation.

The Roaming Candidate List Entries field contains one or more Neighbor Report elements described in 7.3.2.27. If the STA has no information in response to the Roaming Management Query frame, the Neighbor Report Elements are omitted. The length of the Roaming Candidate List Entries in a Roaming Management Request frame is limited by the maximum allowed MMPDU size.

#### 7.4.6.7 Roaming Management Response frame format

The Roaming Management Response frame uses the Action frame body format and is optionally transmitted by a STA in response to a Roaming Management Request frame. The format of the Roaming Management Response frame body is shown in Figure v+6

|  | Category | Action | Dialog Token | Status code | Target BSSID (Optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 6 |

Figure v+6— Roaming Management Response frame body format

The Category field is set equal to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set equal to the value indicating Roaming Response, as specified in Table v7 in 7.4.6.

The Dialog Token field is set equal to the value in the corresponding Roaming Management Request frame. The Roaming Management Response frame is only transmitted in response to a Roaming Management Request frame.

The Target BSSID field is the BSSID of the BSS that STA decided to roam to.  This field is set to null if STA decides not to roam.

The Status code field contains the status code in response to a Roaming Management Request as defined in Table v+3. If STA decides to roam to another BSS, then the status code is set to 0. If STA wants to stay with the current BSS, the status code indicates a reject reason.

Table v+3— Status Code Definitions for a Roaming Management Response

| Status Code | Status code description |
| --- | --- |
| 0 | Accept |
| 1 | Reject - Unspecified reject reason. |
| 2 | Reject – Insufficient Beacons heard from all candidates |
| 3 | Reject – Insufficient available capacity from all candidates |
| 4-255 | Reserved |

# 10. Layer management

## 10.3 MLME SAP Interface

Insert the following new clause after Clause 10.3.31 in 802.11v Draft 0.01 as indicated below:

### 10.3.x Roaming Management

Figure v+7 Roaming Management request — accepted

#### 10.3.x.1 MLME-RMQUERY.request

This set of primitives supports the signaling of Roaming Management Query frames between non-AP STAs and an AP.

##### 10.3.x.1.1 Function

This primitive requests transmission of a Roaming Management Query frame to the AP with which the STA is associated.

##### 10.3.x.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-RMQUERY.request	(
Peer MACAddress

DialogToken,
RoamingQueryReason
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Roaming Management Query frame is sent. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify this Roaming Management transaction. |
| RoamingQuery Reason | Integer | 0 – 255 | As defined in 7.4.6.5 |

##### 10.3.x.1.3 When Generated

This primitive is generated by the SME to request that a Roaming Management Query frame be sent to the AP with which the STA is associated to initiate a Roaming Management procedure.

##### 10.3.x.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Roaming Management Query management frame of action type. The STA then attempts to transmit the frame to the AP with which it is associated.

#### 10.3.x.2 MLME-RMQUERY.confirm

##### 10.3.x.2.1 Function

This primitive reports the result of a request to send a Roaming Management Query frame to the initiating protocol entity.

##### 10.3.x.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-RMQUERY.confirm	(
ResultCode,
DialogToken
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-RMQUERY.request. |
| DialogToken | Integer | As defined in the corresponding MLME-RMQUERY. request | The Dialog Token to identify this Roaming Management transaction. |

##### 10.3.x.2.3 When Generated

This primitive is generated by the MLME when the request to transmit a Roaming Management Query frame completes and indicates the results of the request.

##### 10.3.x.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.x.3 MLME-RMQUERY.indication

##### 10.3.x.3.1 Function

This primitive indicates that a Roaming Management Query frame was received from a non-AP STA.

##### 10.3.x.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-RMQUERY.indication	(
PeerMACAddress,
DialogToken,
RoamingQueryReason
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Roaming Management Query frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token identifying this Roaming Management transaction as received in the Roaming Management Query frame. |
| RoamingQuery Reason | Integer | 0 – 255 | The Roaming Query Reason Code in the Roaming Management Query frame that was received. |

##### 10.3.x.3.3 When Generated

This primitive is generated by the MLME when a valid Roaming Management Query frame is received.

##### 10.3.x.3.4 Effect of Receipt

On receipt of this primitive the SME shall operate according to the procedure in 11.15.4.

### 10.3.x+1 Roaming Management Request

This set of primitives supports the signaling of Roaming Management Request frames between an AP and associated non-AP STAs.

#### 10.3.x+1.1 MLME-RMREQUEST.request

##### 10.3.x+1.1.1 Function

This primitive requests transmission of a Roaming Management Request frame to a non-AP STA.

##### 10.3.x+1.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-RMREQUEST.request	(
PeerMACAddress
DialogToken,
RequestMode,
DisassociationTimer,
RoamingCandidateList
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Roaming Management Request frame is sent. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify the Roaming Management transaction. Set to 0 for an autonomous Roaming Management Request frame. |
| RequestMode | Integer | As specified in 7.4.x.x | Contains the Disassociation Imminent and Abridged bits for the Roaming Management Request. |
| Disassociation Timer | Integer | 0 – 255 | Specifies the number of TBTTs until the AP will disassociate the non-AP STA. A value of 0 indicates time of disassociation has not been determined ye and a value of 1 indicates disassociation will occur before the next TBTT. |
| Roaming CandidateList | Set of Neighbor Report Elements | Set of Neighbor Report Elements as defined in the Neighbor Report Element in 7.3.2.27 | Contains the description of candidate roaming APs and their capabilities as described in section 7.3.2.40 |

##### 10.3.x+1.1.3 When Generated

This primitive is generated by the SME to request that a Roaming Management Request frame be sent to an associated non-AP STA. This request is sent either following the reception of a MLME-RMQUERY.indication or may be sent autonomously.

##### 10.3.x+1.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Roaming Management Request management frame of action type. The STA then attempts to transmit this frame to the indicated non-AP STA.

#### 10.3.x+1.2 MLME-RMREQUEST.confirm

##### 10.3.x+1.2.1 Function

This primitive reports the result of a request to send a Roaming Management Request frame to the initiating protocol entity.

##### 10.3.x+1.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-RMREQUEST.confirm	(
ResultCode,
DialogToken
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-RMREQUEST.request. |
| DialogToken | Integer | As defined in the corresponding MLME-RMREQUEST. request | The Dialog Token to identify this Roaming Management transaction. |

##### 10.3.x+1.2.3 When Generated

This primitive is generated by the MLME when the request to transmit a Roaming Management Request frame completes and indicates the results of the request.

##### 10.3.x+1.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.x+1.3 MLME-RMREQUEST.indication

##### 10.3.x+1.3.1 Function

This primitive indicates that a Roaming Management Request frame was received from the AP with which the STA is associated.

##### 10.3.x+1.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-RMREQUEST.indication	(
PeerMACAddress,
DialogToken,
RequestMode,
DisassociationTimer,
RoamingCandidateList
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the MAC entity from which a Roaming Management Request frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identity this Roaming Management transaction as received in the Roaming Management Request frame. |
| RequestMode | Integer | As specified in 7.4.x.x | Contains the Disassociation Imminent and Abridged bits for the Roaming Management Request. |
| Disassociation Timer | Integer | 0 – 255 | Specifies the number of TBTTs until the AP will disassociate the non-AP STA. A value of 0 indicates time of disassociation has not been determined yet and a value of 1 indicates disassociation will occur before the next TBTT. |
| Roaming CandidateList | Set of Neighbor Report Elements | Set of Neighbor Report Elements as defined in the Neighbor Report Element in 7.3.2.40 | Contains the description of candidate roaming APs and their capabilities as described in section 7.3.2.40. |

##### 10.3.x+1.3.3 When Generated

This primitive is generated by the MLME when a valid Roaming Management Request frame is received.

##### 10.3.x+1.3.4 Effect of Receipt

On receipt of this primitive the SME shall operate according to the procedure in 11.15.4.

### 10.3.x+2 Roaming Management Response

This set of primitives supports the signaling of Roaming Management Response frames between non-AP STAs and an AP.

#### 10.3.x+2.1 MLME-RMRESPONSE.request

##### 10.3.x+2.1.1 Function

This primitive requests transmission of a Roaming Management Response frame to the AP with which the STA is associated.

##### 10.3.x+2.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-RMRESPONSE.request	(
Peer MACAddress

DialogToken,
StatusCode,
TargetBSSID
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Roaming Management Query frame is sent. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify the Roaming Management transaction. |
| StatusCode | Integer | 0 – 255 | As defined in 7.4.6.7 |
| TargetBSSID | MACAddress | Any valid individual MAC Address | The BSSID of the BSS that the STA decided to roam to. Field shall be null if STA decided not to roam. |

##### 10.3.x+2.1.3 When Generated

This primitive is generated by the SME to request that a Roaming Management Response frame be sent to the AP with which the STA is associated.

##### 10.3.x+2.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Roaming Management Response management frame of action type. The non-AP STA then attempts to transmit this to the AP with which it is associated.

#### 10.3.x+2.2 MLME-RMRESPONSE.confirm

##### 10.3.x+2.2.1 Function

This primitive reports the result of a request to send a Roaming Management Response frame to the initiating protocol entity.

##### 10.3.x+2.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-RMRESPONSE.confirm	(
ResultCode,
DialogToken
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-RMRESPONSE.request. |
| DialogToken | Integer | As defined in the corresponding MLME-RMRESPONSE. request | The Dialog Token to identify this Roaming Management transaction. |

##### 10.3.x+2.2.3 When Generated

This primitive is generated by the MLME when the request to transmit a Roaming Management Response frame completes and indicates the results of the request.

##### 10.3.x+2.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.x+2.3 MLME-RMRESPONSE.indication

##### 10.3.x+2.3.1 Function

This primitive indicates that a Roaming Management Response frame was received from a non-AP STA.

##### 10.3.x+2.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-RMRESPONSE.indication	(
PeerMACAddress,
DialogToken,
RoamingQueryReason,
StatusCode,
TargetBSSID
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Roaming Management Response frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify this Roaming Management transaction as received in the Roaming Management Response frame. |
| StatusCode | Integer | 0 – 255 | As defined in 7.4.6.7 |
| TargetBSSID | MACAddress | Any valid individual MAC Address | The BSSID of the BSS that the STA indicated to roam to as received in the Roaming Management Response frame. |

##### 10.3.x+2.3.3 When Generated

This primitive is generated by the MLME when a valid Roaming Management Response frame is received.

##### 10.3.x+2.3.4 Effect of Receipt

On receipt of this primitive the SME shall operate according to the procedure in 11.15.4.

- 11.15 Wireless Network Management Procedures

Insert the following new clause after Clause 11.15.3 in 802.11v Draft 0.01 as indicated below:

### 11.15.4 Roaming Management for Network Load Balancing

The Roaming Management Query, Roaming Management Request, Roaming Management Response frames provide a means  and a protocol to exchange the information needed to enable a network to manage BSS loads by influencing STA roaming decisions and by initiating STA transition to selected target BSS(s).

This protocol enables the improved throughput, effective data rate and/or QoS for the aggregate of STAs in a network by shifting (via transition) individual STA traffic loads to more appropriate points of association within the ESS. The mechanisms and procedures for the transition of a STA from one BSS to another are covered elsewhere in this specification.

The provisions in this clause for Roaming Management and network load balancing do not apply in an IBSS.

#### 11.15.4.1 Roaming Management Query

A STA may solicit network information from the serving BSS to assist its decision to move to another BSS.  However, only the AP shall issue to a STA a recommendation for BSS transition or a disassociation imminent using a prioritized list of candidate BSS(s) for the STA's re-association.

If a STA wishes to solicit network information for roaming, the STA may send a Roaming Management Query frame to the serving AP.  The STA shall specify a reason for the roaming query in the Roaming Management Query frame.  The reason is supplied to the AP to inform the AP and other network elements about conditions at the STA that indicate the impending need for transition to a different AP.

##### 11.15.4.2 Roaming Management Request

A Roaming Management Request frame may be sent by the AP in response to a Roaming Management Query frame or may be sent autonomously.

Upon receipt of the Roaming Management Query frame from a STA, the AP shall respond with a Roaming Management Request frame to that STA.

An AP may send a Roaming Management Request frame to a STA at any time.  The Roaming Management Request frame contains dynamic information intended to influence STA roaming decisions.  The network may implement load balancing algorithms by sending Roaming Management Request frames to selected STAs or all STAs in the network.

The AP shall include a Request Mode in each Roaming Management Request frame.  The STA shall treat the receipt of a Roaming Management Request with the Abridged bit set as if all BSSIDs under consideration for roaming were listed in the Roaming Candidate List, but with the Preference value being 0.  The purpose of the Abridged bit is to help reduce the size of a Roaming Candidate List by allowing the AP to omit potentially long lists of BSSIDs about which the AP wishes to express a Preference value of 0. The Request Mode also contains a Disassociation Imminent bit. The Disassociation Imminent bit in the Request Mode indicates, when set, whether the STA will be disassociated from the current AP .

If the Disassociation Imminent bit in the Request Mode indicates that STA will be disassociated from the current AP, the AP shall include Dissociation Timer information in the Roaming Management Request frame.  The Disassociation Timer indicates the time after which the AP will issue a Disassociation frame to this STA.

The AP shall include Roaming Candidate List Entries in the Roaming Management Request frame.  Roaming Candidate List Entries contain one or more Neighbor Report elements describing the network preferences for recommended target AP candidate to this STA.  The AP shall not list APs that have a different SSID than that of the STA. The candidate Preference value is a number ranging from 0 to 255, indicating a relative preference for this roaming candidate and this STA. A value of Preference “0” indicates that the AP being listed is a “barred” AP, and STA shall not attempt to associate to the listed AP, unless the STA is unable to establish effective service with any non-“barred” APs that are compatible with the STA’s choice of SSID and, if still associated with the AP who sent the Request, it has transmitted a Roaming Candidate Response with the reason of “reject”.. The Preference values are used only to establish the relative order of entries within the given list at the given time, and for the given AP. The preference value is only valid before Expiry Timer is expired. Zero or more Load/ Sub-element may be included in the Roaming Candidate List Entry. Load Sub-elements contain additional information describing the BSS load and QBSS Admission Control Capacity for the corresponding UP traffics.

A STA receiving a Roaming Management Request frame containing Roaming Candidate List Entries may use this list of candidates, with their individual network roaming preference values, in order to make roaming decisions consistent with current network management goals for network load balancing.  Upon receiving a Roaming Management Request, the STA shall disregard any information from any previous Roaming Management Request received from the same AP as that of the current Request. The STA shall not use the corresponding Roaming Candidate List Entry information after the indicated expiry time.  The STA may send the serving AP a Roaming Management Query frame at any time to obtain an updated Roaming Candidates.

A STA receiving a Roaming Management Request frame containing a Disassociation Timer value shall attempt to reassociate with some other AP before the indicated disassociation time. If the receiving STA cannot find a suitable AP to associate with, the STA shall send a Roaming Management Response frame containing a Status Code indicating “reject” before the indicated disassociation time.

##### 11.15.4.3 Roaming Management Response

A STA receiving a Roaming Management Request frame may respond with a Roaming Management Response frame.

The Roaming Management Response frame contains a Status Code field describing the STA's response to the Roaming Management Request frame.  If the Roaming Management Response frame contains a Status Code indicating "accept", the STA has decided to leave the current BSS. A STA shall include the result of its roaming decision in the Target BSSID field in the Roaming Management Response frame.

The STA receiving a Roaming Management Request frame shall always reply with a Roaming Management Response frame with a valid non-zero status code indicating rejection in the case it is unable to comply with this Roaming Management Request.

An AP receiving a Roaming Management Response frame containing a Status Code indicating “reject” may consider directing other STAs for load balancing purpose.

Figure v+8 below diagrams a typical use of this procedure for the purpose of AP load balancing.

AP

STA

Roaming Management Query

(optional)

AP may send a Roaming Management Request frame to any STA at any time, or respond to a Roaming Management Query frame

Roaming Management Response

(optional)

Roaming Management Request

Handover to new BSS Initiated

Disassocation

Timer

(optional)

STA considers network load balancing preferences, control level and evaluates current quality of roaming options.  STA selects target BSS for planned handover.

Disassociation

(optional)

STA begins planned handover using TGr protocol or reassociation.

Figure v+8—Roaming Management for Network Load Balancing

Insert the following new rows in clause A.4.14:

### A. 4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Wireless Network Management capabilities supported? |  |  |  |
| RME5 | Roaming Management | 11.15.4 | CFv:M | Yes, No, N/A |
| RME5.1 | Neighbor Report Element | 7.3.2.27 | (CFv AND CF1):M | Yes, No, N/A |
| RME5.2 | Roaming Management Query | 7.4.6.5 | (CFv AND CF1):M | Yes, No, N/A |
| RME5.3 | Roaming Management Request | 7.4.6.6 | (CFv AND CF12):M | Yes, No, N/A |
| RME5.4 | Roaming Management Response | 7.4.6.7 | (CFv AND CF12):M | Yes, No, N/A |