IEEE P802.11
Wireless LANs

| Interference Detection & Signature Matching |  |  |  |
| --- | --- | --- | --- |
| Date: 2006-Feb-17 |  |  |  |
| Author(s): |  |  |  |
| Name | Company | Phone | email |
| Roger Durand | Research In Motion |  |  |
| Floyd Backes | AutoCell Labs | 1.978.264.4884 x225 | fbackes@autocell.com |
| Joe Epstein | Meru Networks | 1.408.215.5343 | jepstein@meru.com |
| Mike Montemurro | Chantry Networks | 1.905.363.6413 | michael.montemurro@siemens.com |
| Larry Stefani | AutoCell Labs | 1.978.264.4884 | lstefani@autocell.com |

Abstract

This document contains a normative text proposal in support of a mechanism to support interference detection and signature matching in accordance with REQ2070 and REQ2071. This document is based largely on P802.11v-D0.1, P802.11k-D3.2, and P802.11-REVma-D5.1.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

7. Frame formats

7.3.2.21 Measurement Request element

Insert a new row into Table 20b as shown below:

| Interference detection request | 10 | Wireless Network Management |
| --- | --- | --- |

Change the following row in Table 20b:

| Reserved | 101-254 | N/A |
| --- | --- | --- |

Change clause 7.3.2.21 as follows:

When the Enable bit is set to 0 the Measurement Request field contains the specification of the measurement request corresponding to the Measurement Type as described in 7.3.2.21.1 through 7.3.2.21.10a. When the Enable bit is set to 1, the Measurement Request field is only present when requesting a triggered QoS Metrics measurement.

Insert the following new clause after the end of clause 7.3.2.21.10:

7.3.2.21.10a Interference Detection Request

The Measurement Request field corresponding to an Interference Detection Request is shown in Figure v1.

|  | Channel Number | Regulatory Class | Randomization Interval | Measurement Duration | Sample Time Width |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 2 | 4 |

Figure v1 - Measurement Request field format for an Interference Detection Request

Channel Number indicates the channel number for which the measurement request applies.  Channel Number is defined within a Regulatory Class as shown in Annex J.

Regulatory Class indicates the frequency band for which the measurement request applies.  Valid values of Regulatory Class are shown in Annex J.

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TU.  See 11.11.3.

Measurement Duration shall be set to the preferred duration of the requested measurement, expressed in TUs.  If the Duration Mandatory bit is set to 1 in the Measurement Request Mode field this shall be interpreted as mandatory measurement duration.  If the Duration Mandatory bit is set to 0 this shall be interpreted as target measurement duration.  See 11.11.4.

Sample Time Width shall be set to the duration for each sample, expressed as an unsigned integer in units of s. The Sample Time Width must be no greater than the Measurement Duration.

- Measurement Report element

Change clause 7.3.2.22 as follows:

The Measurement Report field shall not be present when the Late bit is set to 1, the Incapable bit is set to 1 or the Refused bit is set to 1. Otherwise, it shall contain a measurement report, as described in 7.3.2.22.1 through 7.3.2.22.811.

Insert a new row into Table 20c as shown below:

| Interference detection report | 10 | Wireless Network Management |
| --- | --- | --- |

Change the following row in Table 20c:

| Reserved | 101-255 | N/A |
| --- | --- | --- |

Insert the following new clause after the end of clause 7.3.2.22.10:

        - Interference Detection Report

The format of the Measurement Report field corresponding to an Interference Detection Report is shown in Figure v2.

|  | Channel Number | Regulatory Class | Actual Measurement Start Time | Measurement Duration | Sample Time Width | Map | CW Level | Pulse Level | Pulse Width |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 | 4 | 1 | 1 | 1 | 4 |

Figure v2 - Measurement Report field format for an Interference Detection Report

Channel Number indicates the channel number for which the measurement report applies.  Channel Number is defined within a Regulatory Class as shown in Annex J.

Regulatory Class indicates the frequency band for which the measurement report applies.  Valid values of Regulatory Class are shown in Annex J.

Actual Measurement Start Time shall be set to the value of the measuring STA’s TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Interference Detection Report was measured, expressed in TUs.

Sample Time Width shall be set to the duration for each sample, expressed as an unsigned integer in units of s. The Sample Time Width must be no greater than the Measurement Duration.

The Map field is coded as a bit field, as shown in Figure v3 below, and shall contain the following bits:

- CW bit, which shall be set to 1 when CW interference is detected. Otherwise, the CW bit shall be set to 0.

- Pulse bit, which shall be set to 1 when pulse interference is detected. Otherwise, the Pulse bit shall be set to 0.

The CW Level field shall be set to the detected continuous wave interference level, expressed as a signed integer in units of decibels relative to 1 mW (dBm). This field is valid when the CW bit is set to 1 in the Map field.

The Pulse Level field shall be set to the detected pulse interference level, expressed as a signed integer in units of decibels relative to 1 mW (dBm). This field is valid when the Pulse bit is set to 1 in the Map field.

The Pulse Width field shall be set to the duration of the detected pulse interference, expressed as an unsigned integer in units of s. This field is valid when the Pulse bit is set to 1 in the Map field.

|  | B0 | B1 | B2 | B7 |
| --- | --- | --- | --- | --- |
|  | CW | Pulse | Reserved |  |
| Bits: | 1 | 1 | 6 |  |

Figure v3 - Map field format for an Interference Detection Report

11 MAC sublayer management entity

11.11 Radio Measurement Procedures

11.11.9 Specific measurement usage

Insert the following new clauses after the end of clause 11.11.9.8:

11.11.9.9 Interference Detection Report

If a station accepts an Interference Detection Request it shall respond with an Interference Detection Report frame.

        - Interference Detection and Signature Mapping (informative)

The Interference detection techniques requires a mechanism for quieting the channel and a mechanism for sampling energy in the channel relative to sample period and measurement duration. The interference results can be forwarded for signature mapping. The signature mapping techniques require a mechanism for matching the signature results to known sources of interference for identification. Once a source of interference is characterized and/or matched, appropriate responses or countermeasures may be taken to improve likelihood of successful transmissions.

        - Interference Detection Channel Quieting (informative)

In order for the measurement mechanism to not be contaminated by valid 802.11 energy, the channel must be temporarily quieted during the Measurement Duration specified in the Measurement Request frame of type Interference Detection request. There are several methods to quiet a channel. For example, transmitting a CTS frame, extending the NAV, or transmitting a Quiet element in a Beacon or Probe Response frame. The quiet interval must be large enough to cover the Measurement Duration.

        - Interference Detection Measurement Mechanism (informative)

All known 802.11 radios employ a digitization method known as an analog-to-digital converter which conducts samples of the energy in the channel. This converter normally outputs energy in a voltage format. The interference detection method employs this converter to sample the energy at the converter and to add any amplifier gain in the radio cascade chain, in order to normalize the energy to the antennae connector. The normalized output for each sample is optimally convertible to units of decibels relative to 1 mW (dBm).

        - Interference Detection Sampling Period and Duration (informative)

The Sample Time Width specified in the Measurement Request frame of type Interference Detection request is the period of an individual measurement sample. Ideally, this period should be shorter in time than an IFS time window or the shortest pulse width on the target interference list. For this example we will be using 10 s. Other sample periods may be used with the tradeoff of resolution versus measurement granularity. The duration of the samples is the number of concurrent samples to run during the quiet interval. The maximum number of concurrent samples can be calculated by dividing Measurement Duration by Sample Time Width. Ideally, the Measurement Duration is longer than the worst case interference pulse duration. For example, a 50 Hz single-cycle microwave oven may have high energy pulses as long as 8.6 ms. For pulse measurement of a 2.4 GHz channel, a 10 s sample for 2000 samples would be used to measure for a total duration of 10 s * 2000 or 20 ms.

11.11.9.9.5 Interference Detection Sampling Results (informative)

The sampling results are the accumulated energy in the channel of all transmitters that did not hear the channel quieting command. In a noise-free environment, the energy results are flat, at or close to the white noise floor across time. In a noisy environment, the energy rises and falls with the pulse characteristics of the source of interference across time. This energy waveform is called an amplitude signature. Such pulse wave shapes can be matched to determine the type of interference heard.

11.11.9.9.6 Signature Mapping (informative)

The sampling results when viewed across time result in the amplitude signature of the interfering transmitter. By finding the power inflection point of the rising and falling edge of the wave form, for example 12 dB lower in energy from the peak of the signature, the type of transmitter may be derived through signature mapping. The signature raw results may be used to derive an optimal countermeasure.

Table v1 details known signature results for the 2.4 GHz band dependent on sampling resolution.

| Table v1 - Signature Results for 2.4 GHz Band |  |
| --- | --- |
| Pulse Duration (s) | Probable Interference Type |
| duration < 182 | Transient noise |
| 182 ≤ duration < 428 | Bluetooth |
| 428 < duration < 550 | Bluetooth or short sync pulse from a FHSS base station |
| 550 < duration < 1343 | FHSS phone |
| 1343 < duration < 2685 | Microwave oven on an adjacent channel |
| 2685 < duration < 3661 | Single pulse microwave oven on the adjacent channel or a double pulse microwave oven |
| 3661 < duration < 8541 | Single pulse microwave oven |
| 8541 < duration | Continuous wave or equivalent |

        - Countermeasures (informative)

The signature results can be used to derive a countermeasure to optimize the radio transmission probability of success.  This includes, but is not limited to

- fragmentation of frames

- increasing transmitter power

- synchronizing transmissions between interference pulses

- changing operating channels

        - Interference impact on CQI (informative)

The interference results degrade the usability of the channel relative to the expected noise floor, dB for dB in the presence of CW interference. For all other pulse type interference, there is a percentage dB degradation of the interference pulse width multiplied by the pulse repetition (i.e. % time occupancy of the pulse).

Annex A

A.4 PICS proforma-IEEE Std 802.11, 1999 Edition9

A.4.14 Wireless Network Management extensions

Append these entries at the end of the table:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME5 | Interference Detection | 11.11, 11.11.9.9 | CFv:O | Yes, No, N/A |
| RME5.1 | Interference Detection Request | 7.3.2.21.10a | (CFv AND RME5):M | Yes, No, N/A |
| RME5.2 | Interference Detection Report | 7.3.2.22.11 | (CFv AND RME5):M | Yes, No, N/A |