IEEE P802.11
Wireless LANs

| Dynamic Multi level Power Control |  |  |  |
| --- | --- | --- | --- |
| Date: 2006-May-15 |  |  |  |
| Author(s): |  |  |  |
| Name | Company | Phone | email |
| Floyd Backes | AutoCell Labs | 1.978.264.4884 x225 | fbackes@autocell.com |

Abstract

This document contains a normative text proposal in support of a mechanism for dynamic multi level transmitter power control in accordance with REQ2040 and REQ2041.  This document is based largely on P802.11v-D0.2, P802.11k-D4.0, and P802.11-REVma-D5.2.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

7. Frame formats

7.1 MAC frame formats

7.2 Format of individual frame types

7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Insert new row into Table 8 as shown below:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 29 | Transmit Power Limit | The Transmit Power Limit element may be present if dot11WirelessManagementImplemented is true. |

7.2.3.5 Association Response frame format

Insert new row into table 11 as shown below:

Table 11—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 9 | Transmit Power Limit | The Transmit Power Limit element may be present if dot11WirelessManagementImplemented is true. |

#### 7.2.3.7 Reassociation Response frame format

Insert new row into table 13 as follows:

Table 13—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 9 | Transmit Power Limit | The Transmit Power Limit element may be present if dot11WirelessManagementImplemented is true. |

#### 7.2.3.9 Probe Response frame format

Insert new row into Table 15 as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 27 | Transmit Power Limit | The Transmit Power Limit element may be present if dot11WirelessManagementImplemented is true. |

7.3 Management frame body components

7.3.2  Information Elements

Insert Element ID x into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| Transmit Power Limit | x |
| Reserved | X+1-220 |

NOTE—x will be defined.

Insert the following new clause after clause 7.3.2.43:

7.3.2.44 Transmit Power Limit element

The format of the Transmit Power Limit element is shown in Figure v74.

|  | Element ID | Length | Transmit Power Limit |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure v73a—Transmit Power Limit element format

The Element ID field is equal to the Transmit Power Limit value in Table 26.

The Length field is set to 1.

The Transmit Power Limit field is coded as an unsigned integer in units of decibels.  The use of the Transmit Power Limit element is described in 11.15.6.

7.4 Action frame format details

7.4.6 Wireless Network Management action details

Insert the following row into table v42 and update the reserved field appropriately:

Table v42 — Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| x | Transmit Power Limit Request |
| x+1-255 | Reserved |

NOTE—x will be defined.

#### Insert the following clause after clause 7.4.6.8

#### 7.4.6.9 Transmit Power Limit Request frame format

The Transmit Power Limit Request frame uses the Action frame body format. It is transmitted by a STA requesting a non-AP STA to limit its maximum transmit power. The format of the frame is shown in Figure v82.

|  | Category | Action | Transmit Power Limit |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 |

Figure v82—Transmit Power Limit Request frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating a Transmit Power Limit Request according to table v42 in clause 7.4.6.

The Transmit Power Limit field contains the Transmit Power Limit element as described 7.3.2.44.

10. Layer management

10.3 MLME SAP Interface

Change 10.3.2 as follows:

10.3.2.2 MLME-SCAN.confirm

10.3.2.2.2 Semantics of the service primitive

Insert the following row at the end of the BSSDescription table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Transmit Power Limit | As defined in the Transmit Power Limit element | As defined in the Transmit Power Limit element | Specifies the maximum transmit power to be used by the receiving STA for data frames. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

Change 10.3.6 as follows:

10.3.6.2 MLME-ASSOCIATE.confirm

10.3.6.2.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.confirm		(

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

EDCAParameterSet,

WirelessManagementCapabilities,

Transmit Power Limit,

)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Transmit Power Limit | As defined in the Transmit Power Limit element | As defined in the Transmit Power Limit element | Specifies the maximum transmit power to be used by the receiving STA for data frames. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

10.3.6.4 MLME-ASSOCIATE.response

10.3.6.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.response		(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

WirelessManagementCapabilities,

Transmit Power Limit

)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Transmit Power Limit | As defined in the Transmit Power Limit element | As defined in the Transmit Power Limit element | Specifies the maximum transmit power to be used by the receiving STA for data frames. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

Change 10.3.7 as follows:

10.3.7.2 MLME-REASSOCIATE.confirm

10.3.2.2.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.confirm		(

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

EDCAParameterSet,

WirelessManagementCapabilities,

Transmit Power Limit,

)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Transmit Power Limit | As defined in the Transmit Power Limit element | As defined in the Transmit Power Limit element | Specifies the maximum transmit power to be used by the receiving STA for data frames. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

10.3.7.4 MLME-REASSOCIATE.response

10.3.7.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response		(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

WirelessManagementCapabilities,

Transmit Power Limit,

)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Transmit Power Limit | As defined in the Transmit Power Limit element | As defined in the Transmit Power Limit element | Specifies the maximum transmit power to be used by the receiving STA for data frames. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

11. MAC sublayer management entity

11.15 Wireless Network Management procedures

Insert the following clause after clause 11.15.5:

11.15.6 Transmit Power Limit Request

The Transmit Power Limit is used by a STA to communicate to a non-AP STA the maximum transmit power to be used by the receiving STA for data frames.  The maximum transmit power to be used is defined as the lesser of:

1) the maximum transmit power level specified for the channel in the Country element minus the transmit power management specified for the current channel (from the MIB) in the Transmit Power Limit element

2) the maximum transmit power level specified for the channel in the Country element minus the local power constraint specified for the channel (from the MIB) in the Power Constraint element.

When the Transmit Power Limit element is received in a Radio Management Action frame the value of Transmit Power Limit shall override the value received in a Beacon frame.

Annex A

A.4 PICS proforma-IEEE Std. 802.11, 1999 Edition

A.4.14 Wireless Network Management extensions

Add the following row to the bottom of the table:

| RRM7 | Transmit Power Limit | 11.15.6 | CFv:O | Yes, No, N/A |
| --- | --- | --- | --- | --- |