IEEE P802.11
Wireless LANs

| Normative Text Proposal for Virtual AP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-NOV-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joe Epstein | Meru Networks | 1309 S Mary Ave, Sunnyvale CA | 408-215-5345 | jepstein@merunetworks.com |

- Abstract

This document contains a normative text proposal for Virtual AP support, fulfilling requirement 2060.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

7. Frame formats	3

7.1 MAC frame formats	3

7.2.3.1 Beacon frame format	3

7.2.3.3 Disassociation frame format	3

7.2.3.5 Association Response frame format	3

7.2.3.7 Reassociation Response frame format	3

7.2.3.8 Probe Request frame format	4

7.2.3.9 Probe Response frame format	4

7.2.3.10 Authentication frame format	4

7.2.3.11 Deauthentication frame format	4

7.2.3.12 Action frame format	4

.	5

7.3.2.x Collocated BSS element	5

# 7. Frame formats

## 7.1 MAC frame formats

#### 7.2.3.1 Beacon frame format

- Insert new row into table 6 as follows:

- Table 5—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Collocated BSS Element | The Collocated BSS element may be present if dot11CollocatedBssEnabled is true. |

#### 7.2.3.3 Disassociation frame format

- Insert new row into table 6 as follows:

- Table 6—Disassociation frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Collocated BSS Element | The Collocated BSS element may be present if dot11CollocatedBssEnabled is true. |

#### 7.2.3.5 Association Response frame format

- Insert new row into table 8 as follows:

- Table 8—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Collocated BSS Element | The Collocated BSS element may be present if dot11CollocatedBssEnabled is true. |

#### 7.2.3.7 Reassociation Response frame format

- Insert new row into table 10 as follows:

- Table 10—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Collocated BSS Element | The Collocated BSS element may be present if dot11CollocatedBssEnabled is true. |

#### 7.2.3.8 Probe Request frame format

- Insert new row into table 11 as follows:

- Table 11—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Collocated BSS Element | The Collocated BSS element may be present if dot11CollocatedBssEnabled is true. |

#### 7.2.3.9 Probe Response frame format

- Insert new row into table 12 as follows:

- Table 12—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Collocated BSS Element | The Collocated BSS element may be present if dot11CollocatedBssEnabled is true. |

#### 7.2.3.10 Authentication frame format

- Insert new row into table 13 as follows:

- Table 13—Authentication frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Collocated BSS Element | The Collocated BSS element may be present if dot11CollocatedBssEnabled is true. |

#### 7.2.3.11 Deauthentication frame format

- Insert new row into table 14 as follows:

- Table 14—Deauthentication frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Collocated BSS Element | The Collocated BSS element may be present if dot11CollocatedBssEnabled is true. |

#### 7.2.3.12 Action frame format

- Insert new row into table 15 as follows:

- Table 15—Action frame body

| Order | Information | Notes |
| --- | --- | --- |
| x | Collocated BSS Element | The Collocated BSS element may be present if dot11CollocatedBssEnabled is true. |

#### .

- Insert the following new clauses:

#### 7.3.2.x Collocated BSS element

- The format of the Collocated BSS element is shown in Figure v1.

|  | Element ID | Length | Root BSSID | Radio Number | Antenna Number | Flags |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 | 1 | 1 |

Figure v1 —Collocated BSS element format

The Element ID field is equal to the Collocated BSS value in Table 20.

The value of Length field is 11 octets.

The Root BSSID is the BSSID the AP has designated to track all Collocated BSSs with.  The Root BSSID must be one of the BSSIDs that the AP advertises, that the BSS using that BSSID must advertise this IE.  All frames with this IE that share the same Root BSSID must be transmitted on the same channel and must not differ in Radio Number across advertisements, as a part of normal operation.

The Radio Number is an arbitrary value, from 0 to 255, representing the radio.  APs should advertise different Radio Numbers from different radios on the same AP.  The AP shall not advertise different Radio Numbers for the same Root BSSID, as a part of normal operation.  The value 0 is taken to mean that the frame is coming from the first radio.

The Antenna Number is an arbitrary value, from 0 to 255, representing the antenna from which the frame was transmitted.  The value 255 is taken as unknown.  The value 0 is taken to mean that the frame is transmitted from the same antenna that beacons for the BSS are transmitted.

The Flags field is a bit field with the following bits defined.

|  |  |  | Reserved |  |
| --- | --- | --- | --- | --- |
| Bit: |  |  | 0-7 |  |

Figure v2— Flags field