IEEE P802.11
Wireless LANs

| 802.11 TGr Use of Key Lifetime Updates |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date November 9, 2005 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Nancy Cam-Winget | Cisco Systems | 3625 Cisco Way, San Jose CA 95134 | +1-408-853-0532 | ncamwing@cisco.com |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | Kapil.Sood@intel.com |

Abstract

The document provides the updates to TGr Draft 0.09 to clarify key lifetime usage.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 1. Overview

This submission presents updates to Draft 0.09 to clarify the semantics of the Key Lifetime TIE and when it begins:

- As the key lifetime is specified for a STA to target TAP link, semantically, the lifetime corresponds to the PTK (or PTKSA) lifetime

- The key lifetime commences once the 802.1X is opened.

# 2. Changes to the Draft 0.09

Changes to Section 8.5A.1:

Update the 3rd paragraph as follows:

The lifetime of the keys is bound to the lifetime of the PSK or AAA-key.  For example, the RADIUS protocol communicates the AAA-key’s key lifetime through the Session-Timeout attribute.  It is important to note that any keys derived from the AAA-key (or PSK) cannot have a lifetime that is greater than its parent.

Add the following paragraph after the 3rd paragraph:

The lifetime of a PTKSA shall be no greater than the lifetime of the PMKSA and is specified by the TIE specifying the Key Lifetime in Message 3 of the 4-way handshake during First Contact, or by the (re)association response.  The Key Lifetime defines the PTKSA lifetime in seconds.  The PTKSA lifetime begins when a successful (re)association has been completed and the 802.1X controlled port has been opened.”

Changes to Section 8A.1.2:

Add the following paragraph after the 3rd paragraph:

Upon successful PTKSA establishment, the target TAP provides the PTKSA lifetime in the Key Lifetime TIE in the (re)association response.  The Key Lifetime TIE defines the PTKSA duration in seconds; this timer commences when the 802.1X controlled port is opened.

Add the following sentence  at the end of  Section 8A.2:

Once the 802.1X port is open, the PTK key lifetime timer is initiated to ensure that the life time of the PTKSA is no longer that the value provided in the Message 3 Key Lifetime TIE.

Changes to Section 8A.3:

Update the sentence on page 50, line 61 as follows, and also the penultimate paragraph of 8A.4.1 and 8A.4.2:

In the (Re)association response, the target TAP shall specify the PTKSA lifetime. The Key Lifetime TIE defines the PTKSA duration in seconds; this timer shall commence when the 802.1X controlled port is opened.

Add the following sentence to the last paragraph of the sections 8A.3,  8A.4.1 and 8A.4.2:

In addition, the PTK key lifetime timer shall be initiated to ensure that the life time of the PTKSA is no longer that the value provided in the (Re)association response’s Key Lifetime TIE.