IEEE P802.11
Wireless LANs

| Joint Proposal: High throughput extension to the 802.11 Standard: PHY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-11-8 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sean Coffey | Realtek Semiconductor | 16269 Laguna Canyon Rd, Suite 100 Irvine, CA 92121 USA | +1 415 572 6221 | coffey@realtek-us.com |
| Adrian Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge, CB3 0FD, United Kingdom | +44 1223 763457 | adrian.p.stephens@intel.com |

Abstract

This document contains those elements of the 802.11 TGn Joint Proposal PHY specification that have been fully approved to be part of the Joint Proposal.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

| Additional Author(s): |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Company | Address | Phone | email |
| Bill Abbott | TI |  | 408-543-5352 | w-abbott@ti.com |
| Santosh Abraham | Qualcomm |  | 858-651-6107 | sabraham@qualcomm.com |
| Tomoko Adachi | Toshiba |  | 81 44 549 2283 | tomo.adachi@toshiba.co.jp |
| Dmitry Akhmetov | Intel |  | 7 812 3319430 x2034 | Dmitry.Akhmetov@intel.com |
| Tsuguhide Aoki | Toshiba |  | 81 44 549 2283 | tsuguhide.aoki@toshiba.co.jp |
| Yusuke Asai | NTT |  | +81 46 859 3494 | asai.yusuke@lab.ntt.co.jp |
| Geert Awater | Airgo |  | 650-475-1900 | awater@airgonetworks.com |
| David Bagby | Sanyo |  | 650 637 7741 | david.bagby@ieee.org |
| Gal Basson | Intel |  |  | gal.basson@intel.com |
| Anuj Batra | TI |  |  | batra@ti.com |
| John Benko | France Telecom |  |  | john.benko@francetelecom.com |
| Bjorn Bjerke | Qualcomm |  | 781-276-0912 | bbjerke@qualcomm.com |
| John Chang | Ralink |  | +886-3-5678868 x1500 | john_chang@ralinktech.com.tw |
| Jerry Chang | Realtek |  | 886 3-5780211 x3818 | jerrychang@realtek.com.tw |
| Jerry Chang | Ralink |  | 886-35678868 | jerry_chang@ralinktech.com.tw |
| Jeng-Hong Chen | Winbond |  |  | jhchen2@winbond.com |
| Yi-Ming Chen | Winbond |  | 310-719-2530 x 2555 | ymchen0@winbond.com |
| Stephen Chen | Zydas |  | 886-3-5773309 x655 | stephen@zydas.com.tw |
| Pei-ju Chiang | Realtek |  | 886-3-5780211 x3577 | pjchiang@realtek.com.tw |
| Emily Chou | Realtek |  |  | sychou@realtek.com.tw |
| Keith Chugg | Trellisware |  | +1 858 753 1604 | kchugg@trellisware.com |
| Brian Classon | Motorola |  | +1 (847) 576-5675 | Brian.Classon@motorola.com |
| Gabriella Convertino | ST |  |  | Gabriella.Convertino@st.com |
| Marc de Courville | Motorola |  | +33 1 69352518 | marc.de.courville@motorola.com |
| Rolf De Vegt | Airgo |  | 1 650 475 1913 | rolf@airgonetworks.com |
| Franz Dielacher | Infineon |  | +43 5 1777 6376 | franz.dielacher@infineon.com |
| Yoshiharu Doi | Sanyo |  | 81 584 64 4903 | doi@gf.hm.rd.sanyo.co.jp |
| Takagi Eiji | Airgo |  | 81-3-5796-3719 | eijitakagi@airgonetworks.com |
| Leonid Epstein | Metalink |  | +972 9 960 5442 | leonide@metalinkbb.com |
| Mustafa Eroz | HNS |  | 301 428-5671 | meroz@hns.com |
| Stefan Fechtel | Infineon |  | +49 89 234 24817 | stefan.fechtel@infineon.com |
| Paul Feinberg | Sony |  | (201) 930-6316 | Paul.Feinberg@am.sony.com |
| Guido Frederiks | Airgo |  | 650-475-1900 | guido@airgonetworks.com |
| Takashi Fukagawa | Panasonic |  | 81-3-5460-2725 | fukagawa.takashi@jp.panasonic.com |
| James Gardner | Airgo |  |  | jamesgardner@airgonetworks.com |
| Jeremy Gosteau | Motorola |  | +33 169352564 | jeremy.gosteau@motorola.com |
| Sudheer Grandhi | InterDigital |  | 631-622-4123 | sudheer.grandhi@interdigital.com |
| Paul Gray | Trellisware |  | 858 753-1611 | pgray@trellisware.com |
| Daqing Gu | Mitsubishi |  | 617 621-7543 | dgu@merl.com |
| Emre Gunduzhan | Nortel |  | 703-600-5253 | egunduzh@nortel.com |
| Jin-Meng Ho | TI |  | 214-480-4220 | jinmengho@ti.com |
| Dale Hocevar | TI |  | +1-214-480-4362 | hocevar@ti.com |
| Muhammad Ikram | TI |  | 214-480-3132 | mzi@ti.com |
| Yasuhiko Inoue | NTT |  | 1 650 725 3654 | yinoue@ansl.ntt.co.jp |
| Kaz Ishida | Qualcomm |  | +81-3-5412-8954 | kishida@qualcomm.com |
| Takashi Ishidoshiro | Buffalo Inc. |  | +81(52)619-7752 | doshir@melcoinc.co.jp |
| Lakshmi Iyer | HNS |  |  | liyer@hns.com |
| Eric Jacobson | Intel |  | 480-554-6078 | eric.a.jacobsen@intel.com |
| Yuh-Ren Jauh | Zydas |  | 886 35773309 x652 | yrjauh@zydas.com.tw |
| Taehyun Jeon | ETRI |  | +82-42-860-1707 | thjeon@etri.re.kr |
| VK Jones | Airgo |  | 650-475-1915 | vkjones@airgonetworks.com |
| Naveen Kakani | Nokia |  | 972 894-6024 | naveen.kakani@nokia.com |
| Srinivas Kandala | Airgo |  | 650 475-1977 | srini@airgonetworks.com |
| Assaf Kasher | Intel |  | +972-4-8651547 | assaf.kasher@intel.com |
| Masato Kato | Buffalo Inc. |  | +81(52)619-7752 | mkato@melcoinc.co.jp |
| John Ketchum | Qualcomm |  | 781 276-0915 | johnk@qualcomm.com |
| Pansop Kim | Winbond |  | 310-719-2530 x2504 | pkim@winbond.com |
| Youngsoo Kim | Samsung |  | 82-31-280-9614 | kimyoungsoo@samsung.com |
| Guenter Kleindl | Siemens AG |  | 43 51707 35738 | guenter.kleindl@siemens.com |
| Kiyotaka Kobayashi | Panasonic |  | 81-3-5460-2726 | kobayashi.kiyotaka@jp.panasonic.com |
| Cenk Kose | Conexant |  | 858-200-5309 | cenk_kose@conexant.com |
| Takushi Kunihiro | Sony |  | 81 3 6409 3201 | kuni@wcs.sony.co.jp |
| Edwin Kwon | Samsung |  | 82-31-200-3862 | cy.kwon@samsung.com |
| Patrick Labbe | Motorola |  | 33 1 69 35 25 99 | labbe@crm.mot.com |
| Sok-kyu Lee | ETRI |  | +82-42-860-5919 | sk-lee@etri.re.kr |
| Sheng Lee | Ralink |  | 1-408-725-8070 | shenghlee@ralinktech.com |
| Lin-Nan Lee | HNS |  | 301-428-5685 | llee@hns.com |
| Dongjun (DJ) Lee | Samsung |  | 82-31-280-9579 | djthekid.lee@samsung.com |
| Yuan Li | Infocomm |  | +65 6874 5702 | liyuan@i2r.a-star.edu.sg |
| Kevin Liao | Ralink |  | 886-3-5678-868x1400 | kevin_liao@ralink.com.tw |
| Isaac Lim | Panasonic |  | 65-6550-5493 | isaac.limwl@sg.panasonic.com |
| Alfred Lin | Winbond |  | 310-719-2530 x2535 | halin@winbond.com |
| Albert Liu | Infineon |  | 886-3-5652317 | Albert.Liu@infineon.com |
| Der-Zheng Liu | Realtek |  | 886-3-5780211 x3518 | dzliu@realtek.com.tw |
| Michael Livshitz | Nortel |  | 703-600-5066 | livshitz@nortel.com |
| Peter Loc | Marvell |  | 408-222-9148 | ploc@marvell.com |
| Peter Lojko | Qualcomm |  | 978-270-8366 | c_plojko@qualcomm.com |
| Hui-Ling Lou | Marvell |  | 408-222-9151 | hlou@marvell.com |
| Siti Massimiliano | ST |  | 310 206 6718 | msiti@ee.ucla.edu |
| Adina Matache | Marvell |  | 408-222-9176 | amatache@marvell.com |
| Laurent Mazet | Motorola |  | 33 1 69 35 48 30 | laurent.mazet@crm.mot.com |
| Darren McNamara | Toshiba |  | +44 117 9060762 | Darren.McNamara@toshiba-trel.com |
| Irina Medvedev | Qualcomm |  | 781-276-0903 | irinam@qualcomm.com |
| Arnaud Meylan | Qualcomm |  | 858-845-1343 | ameylan@qualcomm.com |
| Morgan Miki | Sharp |  | +81-743-65-4529 | miki@slab.tnr.sharp.co.jp |
| Seungwook Min | ETRI |  | 82 42 860-1738 | swmin@etri.re.kr |
| Andy Molisch | Mitsubishi |  | 617 621 7558 | molisch@merl.com |
| Mike Moreton | ST |  |  | mike.moreton@st.com |
| Yuichi Morioka | Sony |  | 81 3 6409 3201 | morioka@wcs.sony.co.jp |
| Markus Muck | Motorola |  | 0033 169352573 | markus.muck@motorola.com |
| Aon Mujtaba | Agere |  | 610 712 6616 | mujtaba@agere.com |
| Yukimasa Nagai | Mitsubishi |  | 81-467-41-2355 | yuki-n@isl.melco.co.jp |
| Seigo Nakao | Sanyo |  | 81 584 64 4903 | snakao@gf.hm.rd.sanyo.co.jp |
| Sanjiv Nanda | Qualcomm |  | 858 845-2375 | snanda@qualcomm.com |
| Chiu Ngo | Samsung |  | 408 544 5633 | cngo@sisa.samsung.com |
| Huaning Niu | Samsung |  | 408-544-5897 | huaning.niu@samsung.com |
| Ivan Oakes | ST |  | +44.118.929.8135 | ivan.oakes@st.com |
| Yoshihiro Ohtani | Sharp |  | +81-743-65-4529 | ohtani@slab.tnr.sharp.co.jp |
| Takeshi Onizawa | NTT |  | ++81-46-859-2453 | onizawa@ansl.ntt.co.jp |
| Fabio Osnato | ST |  | +39 039 603.6196 | fabio.osnato@st.com |
| Pratima Pai | ST |  |  | Pratima.Pai@st.com |
| Thomas Pare | Ralink |  | 1- (408) 725-8070 | tpare@ralinktech.com |
| Jean-Noel Patillon | Motorola |  | +33169352522 | jean-noel.patillon@motorola.com |
| Wei-Chung Peng | Winbond |  | 310-719-2530 x 2589 | WCPeng0@winbond.com |
| Eldad Perahia | Intel |  |  | eldad.perahia@intel.com |
| Jim Petranovich | Conexant |  | 858-713-3377 | jim.petranovich@conexant.com |
| Neeraj Poojary | TI |  | 408-543-5305 | npoojary@ti.com |
| Aleksandar Purkovic | TI |  | 301-515-3725 | apurkovic@ti.com |
| Ali Raissinia | Airgo |  | 650-475-1997 | AliRaissinia@AirgoNetworks.com |
| Sthanunathan Ramakrishnan | TI |  | 091 80 25099123 | sthanu@ti.com |
| Sridhar Ramesh | TI |  | 91-80-250-48173 | rsridhar@india.ti.com |
| Jon Rosdahl | Samsung |  | 801-756-1496 | jon.rosdahl@partner.samsung.com |
| Stephanie Rouqette | Motorola |  | 33 1 69 35 48 24 | stephanie.rouquette@motorola.com |
| Mike Rude | Metalink |  | 952 210 0260 | mike@metalinkbb.com |
| Vincenzo Scarpa | ST |  |  | Vincenzo.Scarpa@st.com |
| Michael Seals | Conexant |  | 321-327-6506 | michael.seals@conexant.com |
| Huai-Rong Shao | Samsung |  | 408-544-5552 | hr.shao@samsung.com |
| Steve Shellhammer | Qualcomm |  | (858) 658-1874 | sshellha@qualcomm.com |
| Masaaki Shida | Hitachi |  | +81-042-323-1111 | shida@crl.hitachi.co.jp |
| Takashi Shono | Intel |  | +81-3-5223-9236 | takashi.shono@intel.com |
| Sebastien Simoens | Motorola |  | 33 1 69 35 25 43 | simoens@motorola.com |
| Matt Smith | Atheros |  | 408-773-5252 | msmith@atheros.com |
| Robert Stacey | Intel |  | 503-264-2823 | robert.j.stacey@intel.com |
| Victor Stolpman | Nokia |  | 972 894-6872 | victor.stolpman@nokia.com |
| Sumei Sun | Infocomm |  | +65 6874 5698 | sunsm@i2r.a-star.edu.sg |
| Shravan Surineni | Qualcomm |  | 781 276-0926 | shravans@qualcomm.com |
| Masahiro Takagi | Toshiba |  | 81 44 549 2238 | masahiro3.takagi@toshiba.co.jp |
| Seiichiro Takahashi | Sanyo |  | 408 501-1314 | stakahashi@stec.sanyo.com |
| Daisuke Takeda | Toshiba |  | 81 44 549 2283 | daisuke.takeda@toshiba.co.jp |
| Teik-Kheong Tan | Philips |  | 408-474-5193 | teik-kheong.tan@philips.com |
| Yasuhiro Tanaka | Sanyo |  | 81 584 64 4903 | y_tanaka@gf.hm.rd.sanyo.co.jp |
| Ganesan Thiagarajan | TI |  | +91-80-25099892 | gana@ti.com |
| Eric Tokubo | Symbol |  | (408) 528-2811 | eric.tokubo@symbol.com |
| Solomon Trainin | Intel |  | 972 4 865 5738 | solomon.trainin@intel.com |
| David Tung | Ralink |  | 408-7258070 | dtung@ralinktech.com |
| Stefano Valle | ST |  | +39 0293519255 | stefano.valle@st.com |
| Richard Van Nee | Airgo |  | 650-475-1900 | vannee@airgonetworks.com |
| Nico van Waes | Nokia |  | 972 894-5669 | nico.vanwaes@nokia.com |
| Allert van Zelst | Airgo |  | +31 346 259663 | allertvanzelst@airgonetworks.com |
| Timothy Wakeley | HP |  | 916-785-1619 | tim.wakeley@hp.com |
| Rod Walton | Qualcomm |  | 781 276-0920 | rwalton@qualcomm.com |
| Xiaowen Wang | Agere |  | 610 712 6628 | xiaowenw@agere.com |
| Mark Webster | Conexant |  | 321-327-6526 | mark.webster@conexant.com |
| Menzo Wentink | Conexant |  | +31 30 225 9752 | menzo.wentink@conexant.com |
| Mike Wilhoyte | TI |  |  | wilhoyte@ti.com |
| Ariton Xhafa | TI |  | 214-480-6556 | axhafa@ti.com |
| Tomoya Yamaura | Sony |  | 81 3 6409 3201 | yamaura@wcs.sony.co.jp |
| Eric Yang | InterDigital |  | 610 878-7851 | eric.yang@interdigital.com |
| Wen-Chang Yeh | Zydas |  | +886-3-5773309 x657 | wcyeh@zydas.com.tw |
| Heejung Yu | ETRI |  | +82-42-860-1651 | heejung@etri.re.kr |
| Eldad Zeira | InterDigital |  | 631-622-4134 | eldad.zeira@interdigital.com |
| Ning Zhang | Atheros |  | 408-773-5363 | ning@atheros.com |
| Jin Zhang | Mitsubishi |  | 6176217595 | jzhang@merl.com |

TBDs refer to the immediately preceding text unless noted otherwise.

# 20.1		Introduction

The support of transmitting and receiving modulation & coding schemes 0-15 in 20 MHz channels is mandatory.  Forward error correction (convolutional coding or TBD1) is used with a code rate of 1/2, 2/3, 3/4, or 5/6.  The support of convolutional coding is mandatory.

TBD1:	The type of advanced coding; e.g., LDPC for TGnSync or WWiSE, turbo coding for Mitmot.

### 20.3.2.   PLCP frame format

Figure 1 shows the PPDU format in the Mixed Mode.  The PPDU consists of a legacy preamble, a High Throughput Signal Field (HT-SIG), a High Throughput Short Training Field (HT-STF), High Throughput Long Training Fields (HT-LTFs) and a data section.   The legacy preamble consists of a Legacy Short Training Field (L-STF), a Legacy Long Training Field (L-STF) and a Legacy Signal Field.  The part of the PPDU not including the data section is denoted the Mixed Mode High Throughput preamble.

Figure 1 - Mixed Mode PLCP frame format

The number of HT-LTFs () shall be at least the number of spatial streams.  The number of HT-LTFs will exceed the number of spatial stream in the case of 3 spatial streams and in TBD modes.  The length of each HT-LTF is 4µsec.

The following section defines the sequences to be transmitted in each of parts of the High Throughput Mixed Mode Preamble.

#### 20.3.2.1   Mixed mode preamble

Subsections 20.3.2.1.1 to 20.3.2.1.4 describe the transmission of the legacy training field and the legacy signal field as part of a mixed mode PPDU.  Subsections 20.3.2.1.5 to 20.3.2.1.8 describe the transmission of the high throughput training field and high throughput signal field as part of a mixed mode PPDU.

##### 20.3.2.1.1   Cyclic shifts definition for the legacy preamble

Cyclic shift is used to prevent undesired beamforming when the same signal or similar signals are transmitted through different spatial streams.   The following table specifies the values for the cyclic shift that shall be applied in the legacy short training field, the legacy long training field, and legacy signal field.  It also applies to the HT signal field in a mixed mode PPDU.

| values for the legacy portion of the packet |  |  |  |  |
| --- | --- | --- | --- | --- |
| Number of Tx Chains | cyclic shift for Tx chain 1 | cyclic shift for Tx chain 2 | cyclic shift for Tx chain 3 | cyclic shift for Tx chain 4 |
| 1 | 0ns | - | - | - |
| 2 | 0ns | -200ns | - | - |
| 3 | 0ns | -100ns | -200ns | - |
| 4 | 0ns | -50ns | -100ns | -150ns |

Table 1 - Cyclic shift for legacy portion of the packet

If spatial expansion applies additional cyclic shift to the packet, the total value of cyclic shift shall not exceed the values in Table 1.

#### 20.3.2.1.2   Legacy short training field

The legacy short training OFDM symbol in the 20MHz mode consists of 12 subcarriers which are modulated by elements of the sequence

.

The normalization factor  is the QPSK normalization.

The L-STF on the  transmit chain is

In the case of Mixed Mode PPDU  takes values from Table 1.  The value of is 1 for 20MHz.  The L-STF has a period of 0.8 µs.  The entire short training field includes ten such periods, with a total duration of  = 8 µsec.

##### 20.3.2.1.3   Legacy long training field

In the 20MHz mode, the long training field OFDM symbol is modulated by elements of the sequence:

.

The L-LTF waveform is

where  = 1.6  sec.  The value of is 1 for 20MHz.

##### 20.3.2.1.4   Legacy signal field

The legacy signal field is used to transfer rate and length information.  It has different meaning when used in transmission as specified in sections 17 or 19 and when used in a HT transmission.   When transmitted in a legacy 20MHz mode (as defined in sections 17 or 19), it is transmitted using the same method and meaning as specified in section 17.3.4.

Figure 2 - The signal field

When the transmission is not a legacy transmission the fields in the signal field have different meaning.  The bits in the rate field are [1,1,0,1] – corresponding to a rate of 6Mbps in section 17.3.4 representation.  The value in the length field is given through the TX vector.  This value is used to cause legacy devices to defer transmission for a period corresponding to the length of the rest of the packet.    The value to be transmitted is   where Ndata is the number of 4usec symbols in the data part of the frame.  While using short GI (TBD) Ndata  is equal to the actual number of  symbols in the data part of the frame multiplied by910.NLTF is the number of HT training symbols. The symbol  denotes the lowest integer greater or equal to x.

The length field is transmitted LSB first.  It is permissible to use a larger number than l to reserve time for more than the length of the packet.  The reserved bit shall be set to 0.  The parity field shall have the even parity of bits 0-16.  The signal field shall be encoded, interleaved and mapped, and have pilots inserted following the steps described in sections 17.3.5.5, 17.3.5.6, and 17.3.5.8.  The stream of 48 complex numbers generated by these steps is dk, k=0…47.  The conversion of these into a time domain signal is described in the following table

Table 2 - generation of the signal field

| Modulation Method | Conversion to Time Domain signal |
| --- | --- |
| 20MHz transmission on several transmit chains – iTX’th tx chain |  |

are defined in section 17.3.5.9.

p0 is the first pilot value in the sequence defined in section 17.3.5.9.

##### 20.3.2.1.5   Cyclic shift for the High Throughput preamble

Throughout the high throughput preamble, cyclic shift is applied to prevent unwanted beamforming when similar signals are transmitted on different spatial streams.  The same cyclic shift will be applied to these streams during the transmission of the data portion of the packet.  The values of the cyclic shift to be used during the HT preamble and the data portion of the packet, are specified in Table 20.3.2.1.5.1:

| values for HT portion of the packet |  |  |  |  |
| --- | --- | --- | --- | --- |
| Number of spatial streams | Cyclic shift for Spatial stream 1 | Cyclic shift for spatial stream 2 | Cyclic shift for spatial stream 3 | Cyclic shift for spatial stream 4 |
| 1 | 0ns | - | - | - |
| 2 | 0ns | -400ns | - | - |
| 3 | 0ns | -400ns | -200ns | - |
| 4 | 0ns | -400ns | -200ns | -600ns |

Table 20.3.2.1.5.1 – Cyclic shift values of HT portion of the packet

##### 20.3.2.1.6 High Throughput Signal Field

The HT-SIG field is used to transmit the packet length, modulation and other parameters to the receiver.  The length of the HT-SIG is 8µsec.  The actual data to be transmitted and the exact modulation are TBD.

##### 20.3.2.1.7  HT Short Training Field

The purpose of the HT Short Training Field is to improve AGC training in a multi-transmit and multi-receive system.  The duration of the HT-STF is 4μsec; the frequency sequence used to construct the HT-STF in 20MHz transmission is identical to legacy STF.  The frequency sequence is:

For 20MHz:

The time domain representation of the transmission in the iSSth spatial stream is:

The value of is 1 for 20MHz.  The values for are given in Table .

##### 20.3.2.1.8   HT-LTF long training field

The HT long training field provides means for the receiver to estimate the channel between each spatial stream transmitter and receive chain; the number of training symbols NLTF is equal or greater than the number of spatial streams and is signaled by HT-SIG (TBD) The first symbol HT-LTF1 is defined in 20.3.2.1.8.1 below; the additional HT-LTF symbols HT-LTF2, HT-LTF3 and HT-LTF4 are defined in 20.3.2.1.8.2.

20.3.2.1.8.1 First HT training symbol - HT-LTF1

HT-LTF1 is the first part of the HT training sequence. The following sequence will be transmitted in the case of 20MHz transmission:

Note that this sequence is extension of the legacy LTF where the 4 extra sub-carriers are filled with +1 for negative frequencies and -1 for positive frequencies

The duration of the long training field HT-LTF1 is 4μsec, consisting of a single repetition of the sequence plus GI insertion; in case of multiple spatial streams cyclic shift will be invoked as specified in Table 20.3.2.1.5.1.

The HT-LTF1 for the iss’th spatial stream is

Where is 1 for 20 MHz.  The values for are given in Table .  PHTLTFi,n is used when more than one spatial stream is used.  It is defined in the next section.

#### 20.3.2.1.8.2 Additional HT training symbols HT-LTF2, HT-LTF3, HT-LTF4

The subsequent HT training symbols use the same sequences as the first symbol HT-LTF1; however for some of the spatial streams the sequence polarity is inverted. The polarity pattern is defined by a 4x4 matrix PHTLTF below where PHTLTFi,n represents polarity of the ith spatial stream in the nth HT training symbol.

Training symbols 2..4 include single repetition of the sequence and single GI. When three spatial streams are used, four LTF's shall be transmitted.  The time domain representation of the waveform transmitted in the ith spatial stream during the nth training symbol (2≤n≤4) is:

The value of  is 1 for 20MHz.  The values for are given in Table 20.3.2.1.5.1.

#### 20.3.2.2 RATE-dependent parameters

The rate-dependent parameters for mandatory modes are as given in Table 1 and Table 2.  There is one encoding stream in each mandatory mode.  The rate-dependent parameters for optional modes are given in Tables 003 through TBD (Motion specified modes through Table 008, but not necessarily as a complete list.  Also any remarks on number of encoding streams are bound up in this TBD.)

Table 1—Rate-dependent parameters for mandatory 20 MHz, NSS = 1 (NES = 1) modes

| Modulation & coding scheme index | Data rate (Mbits/s) | Modulation | Code rate (R) | Coded bits per subcarrier per spatial stream (NBPSC ) | Number of data sub-carriers (NSD ) | Number of pilots (NSP) | Coded bits per MIMO-OFDM symbol (NCBPS ) | Data bits per MIMO-OFDM symbol (NDBPS ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 6.5 | BPSK | 1/2 | 1 | 52 | 4 | 52 | 26 |
| 1 | 13 | QPSK | 1/2 | 2 | 52 | 4 | 104 | 52 |
| 2 | 19.5 | QPSK | 3/4 | 2 | 52 | 4 | 104 | 78 |
| 3 | 26 | 16-QAM | 1/2 | 4 | 52 | 4 | 208 | 104 |
| 4 | 39 | 16-QAM | 3/4 | 4 | 52 | 4 | 208 | 156 |
| 5 | 52 | 64-QAM | 2/3 | 6 | 52 | 4 | 312 | 208 |
| 6 | 58.5 | 64-QAM | 3/4 | 6 | 52 | 4 | 312 | 234 |
| 7 | 65 | 64-QAM | 5/6 | 6 | 52 | 4 | 312 | 260 |

Table 2—Rate-dependent parameters for mandatory 20 MHz, NSS = 2 (NES = 1) modes

| Modulation & coding scheme index | Data rate (Mbits/s) | Modulation | Code rate (R) | Coded bits per subcarrier per spatial stream (NBPSC ) | Number of data sub-carriers (NSD ) | Number of pilots (NSP) | Coded bits per MIMO-OFDM symbol (NCBPS ) | Data bits per MIMO-OFDM symbol (NDBPS ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 13 | BPSK | 1/2 | 1 | 52 | 4 | 104 | 52 |
| 9 | 26 | QPSK | 1/2 | 2 | 52 | 4 | 208 | 104 |
| 10 | 39 | QPSK | 3/4 | 2 | 52 | 4 | 208 | 156 |
| 11 | 52 | 16-QAM | 1/2 | 4 | 52 | 4 | 416 | 208 |
| 12 | 78 | 16-QAM | 3/4 | 4 | 52 | 4 | 416 | 312 |
| 13 | 104 | 64-QAM | 2/3 | 6 | 52 | 4 | 624 | 416 |
| 14 | 117 | 64-QAM | 3/4 | 6 | 52 | 4 | 624 | 468 |
| 15 | 130 | 64-QAM | 5/6 | 6 | 52 | 4 | 624 | 520 |

Table 3—Rate-dependent parameters for optional 20 MHz, NSS = 3 (NES = TBD) modes

| Modulation & coding scheme index | Data rate (Mbits/s) | Modulation | Code rate (R) | Coded bits per subcarrier per spatial stream (NBPSC ) | Number of data sub-carriers (NSD ) | Number of pilots (NSP) | Coded bits per MIMO-OFDM symbol (NCBPS ) | Data bits per MIMO-OFDM symbol (NDBPS ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 19.5 | BPSK | 1/2 | 1 | 52 | 4 | 156 | 78 |
| 17 | 39 | QPSK | 1/2 | 2 | 52 | 4 | 312 | 156 |
| 18 | 58.5 | QPSK | 3/4 | 2 | 52 | 4 | 312 | 234 |
| 19 | 78 | 16-QAM | 1/2 | 4 | 52 | 4 | 624 | 312 |
| 20 | 117 | 16-QAM | 3/4 | 4 | 52 | 4 | 624 | 468 |
| 21 | 156 | 64-QAM | 2/3 | 6 | 52 | 4 | 936 | 624 |
| 22 | 175.5 | 64-QAM | 3/4 | 6 | 52 | 4 | 936 | 702 |
| 23 | 195 | 64-QAM | 5/6 | 6 | 52 | 4 | 936 | 780 |

Table 4—Rate-dependent parameters for optional 20 MHz, NSS = 4 (NES = TBD) modes

| Modulation & coding scheme index | Data rate (Mbits/s) | Modulation | Code rate (R) | Coded bits per subcarrier per spatial stream (NBPSC ) | Number of data sub-carriers (NSD ) | Number of pilots (NSP) | Coded bits per MIMO-OFDM symbol (NCBPS ) | Data bits per MIMO-OFDM symbol (NDBPS ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 26 | BPSK | 1/2 | 1 | 52 | 4 | 208 | 104 |
| 25 | 52 | QPSK | 1/2 | 2 | 52 | 4 | 416 | 208 |
| 26 | 78 | QPSK | 3/4 | 2 | 52 | 4 | 416 | 312 |
| 17 | 104 | 16-QAM | 1/2 | 4 | 52 | 4 | 832 | 416 |
| 28 | 156 | 16-QAM | 3/4 | 4 | 52 | 4 | 832 | 624 |
| 29 | 208 | 64-QAM | 2/3 | 6 | 52 | 4 | 1248 | 832 |
| 30 | 234 | 64-QAM | 3/4 | 6 | 52 | 4 | 1248 | 936 |
| 31 | 260 | 64-QAM | 5/6 | 6 | 52 | 4 | 1248 | 1040 |

Table 5—Rate-dependent parameters for optional 40 MHz, NSS = 1 (NES = 1) modes

| Modulation & coding scheme index | Data rate (Mbits/s) | Modulation | Code rate (R) | Coded bits per subcarrier per spatial stream (NBPSC ) | Number of data sub-carriers (NSD ) | Number of pilots (NSP) | Coded bits per MIMO-OFDM symbol (NCBPS ) | Data bits per MIMO-OFDM symbol (NDBPS ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 13.5 | BPSK | 1/2 | 1 | 108 | 6 | 108 | 54 |
| 1 | 27 | QPSK | 1/2 | 2 | 108 | 6 | 216 | 108 |
| 2 | 40.5 | QPSK | 3/4 | 2 | 108 | 6 | 216 | 162 |
| 3 | 54 | 16-QAM | 1/2 | 4 | 108 | 6 | 432 | 216 |
| 4 | 81 | 16-QAM | 3/4 | 4 | 108 | 6 | 432 | 324 |
| 5 | 108 | 64-QAM | 2/3 | 6 | 108 | 6 | 648 | 432 |
| 6 | 121.5 | 64-QAM | 3/4 | 6 | 108 | 6 | 648 | 486 |
| 7 | 135 | 64-QAM | 5/6 | 6 | 108 | 6 | 648 | 540 |

Table 6—Rate-dependent parameters for optional 40 MHz, NSS = 2 (NES = 1) modes

| Modulation & coding scheme index | Data rate (Mbits/s) | Modulation | Code rate (R) | Coded bits per subcarrier per spatial stream (NBPSC ) | Number of data sub-carriers (NSD ) | Number of pilots (NSP) | Coded bits per MIMO-OFDM symbol (NCBPS ) | Data bits per MIMO-OFDM symbol (NDBPS ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 27 | BPSK | 1/2 | 1 | 108 | 6 | 216 | 108 |
| 9 | 54 | QPSK | 1/2 | 2 | 108 | 6 | 432 | 216 |
| 10 | 81 | QPSK | 3/4 | 2 | 108 | 6 | 432 | 324 |
| 11 | 108 | 16-QAM | 1/2 | 4 | 108 | 6 | 864 | 432 |
| 12 | 162 | 16-QAM | 3/4 | 4 | 108 | 6 | 864 | 648 |
| 13 | 216 | 64-QAM | 2/3 | 6 | 108 | 6 | 1296 | 864 |
| 14 | 243 | 64-QAM | 3/4 | 6 | 108 | 6 | 1296 | 972 |
| 15 | 270 | 64-QAM | 5/6 | 6 | 108 | 6 | 1296 | 1080 |

Table 7—Rate-dependent parameters for optional 40 MHz, NSS = 3 (NES = TBD) modes

| Modulation & coding scheme index | Data rate (Mbits/s) | Modulation | Code rate (R) | Coded bits per subcarrier per spatial stream (NBPSC ) | Number of data sub-carriers (NSD ) | Number of pilots (NSP) | Coded bits per MIMO-OFDM symbol (NCBPS ) | Data bits per MIMO-OFDM symbol (NDBPS ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16 | 40.5 | BPSK | 1/2 | 1 | 108 | 6 | 324 | 162 |
| 17 | 81 | QPSK | 1/2 | 2 | 108 | 6 | 648 | 324 |
| 18 | 121.5 | QPSK | 3/4 | 2 | 108 | 6 | 648 | 486 |
| 19 | 162 | 16-QAM | 1/2 | 4 | 108 | 6 | 1296 | 648 |
| 20 | 243 | 16-QAM | 3/4 | 4 | 108 | 6 | 1296 | 972 |
| 21 | 324 | 64-QAM | 2/3 | 6 | 108 | 6 | 1944 | 1296 |
| 22 | 364.5 | 64-QAM | 3/4 | 6 | 108 | 6 | 1944 | 1458 |
| 23 | 4055 | 64-QAM | 5/6 | 6 | 108 | 6 | 1944 | 1620 |

Table 8—Rate-dependent parameters for optional 40 MHz, NSS = 4 (NES = TBD) modes

| Modulation & coding scheme index | Data rate (Mbits/s) | Modulation | Code rate (R) | Coded bits per subcarrier per spatial stream (NBPSC ) | Number of data sub-carriers (NSD ) | Number of pilots (NSP) | Coded bits per MIMO-OFDM symbol (NCBPS ) | Data bits per MIMO-OFDM symbol (NDBPS ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 54 | BPSK | 1/2 | 1 | 108 | 6 | 432 | 216 |
| 25 | 108 | QPSK | 1/2 | 2 | 108 | 6 | 864 | 432 |
| 26 | 162 | QPSK | 3/4 | 2 | 108 | 6 | 864 | 648 |
| 17 | 216 | 16-QAM | 1/2 | 4 | 108 | 6 | 1728 | 864 |
| 28 | 324 | 16-QAM | 3/4 | 4 | 108 | 6 | 1728 | 1296 |
| 29 | 432 | 64-QAM | 2/3 | 6 | 108 | 6 | 2592 | 1728 |
| 30 | 486 | 64-QAM | 3/4 | 6 | 108 | 6 | 2592 | 1944 |
| 31 | 540 | 64-QAM | 5/6 | 6 | 108 | 6 | 2592 | 2160 |

#### 20.3.2.3 Timing related parameters

Table 010 is a list of timing parameters associated with the MIMO-OFDM PLCP.

Table 010—Timing-related parameters

| Parameter | Value (20 MHz modes) | Value (40 MHz modes) | Remarks |
| --- | --- | --- | --- |
| NSD : Number of data subcarriers | 52 | 108, TBD2 |  |
| NSP : Number of pilot subcarriers | 4 | 6 |  |
| NST : Number of subcarriers, total | 56 | 114 | NST = NSD + NSP |
| NOT: Index of outermost populated subcarrier | 28 | TBD8, TBD2 |  |
| NSS: Number of spatial streams | 1,2,3,4 | 1,2,3,4 |  |
| ∆F: Subcarrier frequency spacing | 0.3125 MHz | 0.3125 MHz | ∆F = 20/64 = 40/128 MHz |
| TFFT : IFFT/FFT period | 3.2 µs | 3.2 µs | TFFT = 1/∆F |
| TGI : GI duration | TBD4 | TBD4 |  |
| TGI2 : Legacy training symbol guard duration | 1.6 µs | 1.6 µs |  |
| TSYM : Symbol interval | TBD4 | TBD4 | (TGI + TFFT ) |
| TBD7 |  |  |  |

TBD2: In 40 MHz, the TGnSync 6 Mbps duplicate mode has 96 data tones (11-04-0889r6, Section 11.1.1.5).

TBD4:  WWiSE has 800 ns guard interval.  TGnSync has 800 ns and 400 ns guard intervals.

TBD5:  WWiSE has training depending on number of transmit antennas; TGnSync has training depending on number of spatial streams.  (Hence in either case these are timing-related parameters that need to be included.)

TBD7:  There are possibly other timing-related parameters to be added.

#### TBD8:  The index of the outermost populated carrier in 40 MHz.

#### Mathematical description of signals

The transmitted signal will be described in a complex base band signal notation. The actual transmitted signal is related to the complex signal by the following relation:

where

represents the real part of a complex variable;

denotes the center frequency of the carrier.

The transmitted baseband signal consists of several fields.  The timing boundaries for the various fields are shown in Figure 3.

Figure 3 - PPDU fields boundaries

The time offsets  determines the starting time of the corresponding field.

The signal transmitted on spatial stream  if the packet is in Mixed Mode is

where

,

,

,

,

and

.

Each baseband waveform  is defined via the discrete Fourier transform as

This general representation holds for all fields, which are L-STF, L-LTF, L-SIG, HT-SIG, HT-STF, HT-LTF.  The definition of  is given in section 17.3.2.4.  The frequency domain symbols  define the field, as shall be specified in the following subsections.  We adopt the convention that:

With this convention, the  scale factor in  ensures that the total power of the time domain signal as summed over all transmit chains is 1.  The following table summarizes the various values of:

| #### Field | #### NTone #### Field |  |
| --- | --- | --- |
|  | #### 20 MHz | #### 40 MHz |
| #### L-STF | #### 12 | #### 24 |
| #### L-LTF | #### 52 | #### 104 |
| #### L-SIG | #### 52 | #### 104 |
| #### HT-SIG | #### 52 | #### 104 |
| #### HT-STF | #### 12 | #### 24 |
| #### HT-LTF | #### 56 | #### 114 |

#### Table 20.3.4.3 - number of tones in each field

#### 20.3.5.5 Convolutional encoder

If CODE_TYPE = 0, TBD1 encoding stream shall be coded with a convolutional encoder that conforms to IEEE 802.11a-1999 subclause 17.3.5.5 for code rates of R = 1/2, 2/3, and 3/4. Additionally, with the same notation as in 17.3.5.5, the code rate of R = 5/6 shall be implemented according to the puncturing pattern illustrated in Figure 018.

TBD1: The proposals differ in the handling of high data rates.  WWiSE uses two encoding streams for 40 MHz, 3 or 4 spatial streams, while TGnSync always has one encoding stream.  In the text above, “TBD” may be replaced by “each” if the WWiSE method is chosen, and “the” if the TGnSync method is chosen.  The term “encoding stream” will be defined separately.

Figure 018 – An illustration of the bit stealing and bit insertion procedure for R = 5/6

#### 20.3.5.6 Parser and interleaver

##### 20.3.5.6.1   	Parser

TBD2

##### 20.3.5.6.2	Interleaver

After parsing, all encoded bits shall be interleaved by a block interleaver with a block size equal to the number of bits in a single OFDM symbol of a single spatial stream, NCBPIS. The interleaver is defined by the following series of permutations.

We shall denote by k the index of the coded bits after parsing and before interleaving, k  = 0, 1 … NCBPIS-1.  n = 0, 1 … NSS-1 denotes the index of the spatial streams.

The interleaver for spatial stream n within its block of NCBPIS bits is defined by the following relations.

i = (NCBPIS/IDEPTH) (k mod IDEPTH) + floor (k/IDEPTH)       k = 0,…(NCBPIS)-1

j = s ×floor(i/s) + (i + NCBPIS – floor (IDEPTH × i / NCBPIS)) mod s		s = max(NBPSC/2,1)

jn = (j + NCBPIS - NBPSC Dn) mod NCBPIS.   							(14)

The parameter Dn which denotes the shift in subcarriers for spatial stream n is given by

TBD3 												(15)

The interleaving depth IDEPTH shall conform to the values given in Table 012.

Table 012—Interleaving parameters

TBD4

The deinterleaver for block n, within its block of NCBPIS bits, which performs the inverse relation, is defined by the following series of permutations.

kd = mod (m + NCBPIS + 2sDn) mod NCBPIS,                       m = 0,…,NCBPIS-1

id = s floor(kd / s) + (kd + floor (IDEPTH× kd  / NCBPIS)) mod s

jdn = IDEPTH × id - (NCBPIS - 1) × floor (IDEPTH × id / NCBPIS).											(16)

where s and Dn are as defined for the interleaver.

TBD2:  WWiSE parses successive bits to different spatial streams (11-05-0149r2, Section 20.3.5.6).  TGnSync parses blocks of s = max(Nbpsc/2,1) bits to different spatial streams (11-04-0889r6, Section 11.2.1.8.7).

TBD3:  WWiSE specifies Dn = 5n for all n.  TGnSync specifies Dn in 11-04-0889r6, Tables 32 and 33.

TBD4:  WWiSE specifies Idepth = 6 for each mode except 1 spatial stream in 40 MHz, for which Idepth = 12.  TGnSync defines Idepth (Ncolumn) = 13 for 20 MHz modes and Idepth = 18 for 40 MHz modes.

##### 20.3.5.7.4 LDPCC encoder

For each of the three available codeword blocklengths, the LDPCC supports rate-1/2, rate-2/3, rate-3/4 and rate-5/6 encoding. The LDPCC encoder is systematic, i.e., it encodes an information block of size k, I=(i0,i1,... i(k-1)) into a codeword c of size n, c=(i0,i1,... i(k-1), p0, p1,…, p(n-k-1)), by adding n-k parity bits obtained so that HcT = 0, where H is an (n-k)xn parity-check matrix. The selection of the codeword blocklength (n) is achieved via the LDPC PPDU encoding process described in subclause 20.3.5.7.3.

##### 20.3.5.7.5 Parity check matrices

Each of the parity-check matrix can be partitioned into square subblocks (submatrices) of size Z x Z. These submatrices are either cyclic-permutation matrices or null submatrices. The submatrix types (except for the null submatrix) are illustrated in Figure 021.

The cyclic permutation matrix Pi is obtained from the identity matrix by cyclically shifting the rows to the right by i elements. The matrix P0 is the identity matrix. Figure 021 illustrates examples (for a subblock size of 8 x 8) of cyclic-permutation matrices Pi.

P0=1000010000100001

Figure 021 – Types of subblock matrices.  The matrix Pi is produced by cyclically shifting the rows of the identity matrix to the right by i places.

Table A.1 of Annex A displays the “matrix prototypes” of parity-check matrices for all four encoder rates at blocklength n=648 bits. The integer i denotes the cyclic-permutation matrix Pi, as illustrated in Figure 021. Vacant entries of the table denote null (zero) submatrices.

Table A.2 of Annex A displays the matrix prototypes of parity-check matrices for blocklength n=1296 bits, in the same fashion.

Table A.3 of Annex A displays the matrix prototypes of parity-check matrices for blocklength n=1944 bits, in the same fashion.

#### 20.3.5.10 Space-Time Block Coding (STBC)

This subclause defines a set of robust transmission rates that are applicable when NTX is greater than NSS. These rates are based either on Space-Time Block Coding (STBC) or hybrid STBC / Spatial-Division-Multiplexing (SDM) schemes. The implementation of all modes defined in this subclause is TBD1. When the use of STBC is indicated (reference to HT-SIG definition) a symbol operation shall occur between the modulator and the IFFT (reference to figure with entire TX chain) as defined in this clause, further adhering to all other requirements for the indicated MCS.

Denote the complex modulator symbol transmitted on all data sub-carriers for stream n destined for the (2m-1)th and (2m)th OFDM epoch as  and  respectively where .

Table 20.3.5.10.1 indicates for each combination of logical NTX defined in sub-clause TBD2 and NSS which modulator symbol shall be transmitted during OFDM epoch (2m-1) and (2m) from antenna  respectively and the MCS set with which this STBC coding may be combined.

Table 20.3.5.10.1  STBC modulator symbol to OFDM epoch mapping

| logical NTX | NSS | MCS set | nTX | Modulator symbols transmitted in OFDM epoch (2m-1) | modulator symbols transmitted in OFDM epoch (2m) |
| --- | --- | --- | --- | --- | --- |
| 2 | 1 | 0-7, TBD3 | 1 |  |  |
|  |  |  | 2 |  |  |
| 3 | 2 | 8-15, TBD3 | 1 |  |  |
|  |  |  | 2 |  |  |
|  |  |  | 3 |  |  |
| 4 | 2 | 8-15, TBD3 | 1 |  |  |
|  |  |  | 2 |  |  |
|  |  |  | 3 |  |  |
|  |  |  | 4 |  |  |
|  | 3 | 16-23, TBD3 | 1 |  |  |
|  |  |  | 2 |  |  |
|  |  |  | 3 |  |  |
|  |  |  | 4 |  |  |

Notes:

TBD1: Open decision whether STBC is mandatory or optional at the transmitter and/or receiver.

TBD2: Sub-clause defining the mapping between logical and physical antenna inputs.

TBD3: Open decision whether STBC should be usable with extended MCS set.

# Annex A – LDPC Matrix Definitions

(normative)

Table A.1 
Matrix prototypes of parity-check matrices for blocklength n= 648 bits. 
Subblock size is Z= 27 bits.

(a)
Encoder rate R= 1/2.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -  -  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -  -  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  0  0  -  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  0  0  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  0  0  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  0  0  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  -  -  -  -  0  0  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  0  0  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  -  0  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  -  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  -  -  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  -  -  -  -  -  -  -  -  0

(b)

Encoder rate R= 2/3.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  0  0  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  0  0  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  -  -  0  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  -  -  -  -  0

(c)

Encoder rate R= 3/4.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  0  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  -  -  0

(d)

Encoder rate R= 5/6.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  0

Table A.2 
Matrix prototypes of parity-check matrices for blocklength n= 1296 bits. 
Subblock size is Z= 54 bits.

(a)
Encoder rate R= 1/2.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -  -  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -  -  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  0  0  -  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  0  0  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  0  0  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  0  0  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  -  -  -  -  0  0  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  0  0  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  -  0  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  -  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  -  -  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  -  -  -  -  -  -  -  -  0

(b)

Encoder rate R= 2/3.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  0  0  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  0  0  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  -  -  0  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  -  -  -  -  0

(c)

Encoder rate R= 3/4.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  0  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  -  -  0

(d)

Encoder rate R= 5/6.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  0

Table A.3 
Matrix prototypes of parity-check matrices for blocklength n=1944 bits. 
Subblock size is Z = 81 bits.

(a)
Encoder rate R= 1/2.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -  -  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -  -  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  0  0  -  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  0  0  -  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  0  0  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  0  0  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  -  -  -  -  0  0  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  0  0  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  -  0  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  -  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  -  -  -  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  -  -  -  -  -  -  -  -  0

(b)

Encoder rate R= 2/3.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  0  0  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  0  0  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  -  -  0  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  -  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  -  -  -  -  0

(c)

Encoder rate R= 3/4.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  0  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  -  -  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  -  -  0

(d)

Encoder rate R= 5/6.

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  0  -  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  -  0  0  -

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  0  -  0  0

X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3 X3  1  -  -  0

- The number of non-null entries in each matrix prototype does not exceed 88.  The row weights are constrained to have at most 2 consecutive values, n-1 and n, where n does not exceed the following values for each of the four code rates: rate ½: 8; rate 2/3: 11; rate ¾: 15; rate 5/6: 22.