IEEE P802.11
Wireless LANs

| Virtual AP Proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Thomas Kuehnel | Microsoft Corporation | One Microsoft Way Redmond WA 98052 | 425-706-4351 | tkuehnel@microsoft.com |
| Sudheer Matta | Trapeze Networks | 5753 W. Las Positas Pleasanton, CA 94588 | 925-474-2290 | matta@trapezenetworks.com |
| Partha Narasimhan | Aruba Networks | 1322 Crossman Ave, Sunnyvale, CA | 408-754-3018 | partha@arubanetworks.com |
| Subbu Ponnuswamy | Aruba Networks | 1322 Crossman Ave, Sunnyvale, CA | 408-754-1213 | subbu@arubanetworks.com |
| Dorothy Stanley | Aruba Networks | Warrenville, IL | 630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains substantive text to satisfy the “Virtual AP” TGv objective, #2060. The solution uses a single beacon to efficiently advertise multiple BSSIDs and SSIDs, while optionally inheriting selected attributes of the transmitted BSSID beacon. The Probe Request frame is augmented to probe for multiple SSIDs in a single frame. The Probe Response frame is extended in the same way as the beacon frame, to include the attributes of multiple SSIDs.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 1. Req 2060 – Virtual Access Points

[Req2060] TGv shall support MAC extensions to improve support of Virtual Access Points, in order to conserve air time as well as enhanced integration with radio measurement extensions, such as TGk. One example would be to allow a single AP to advertise multiple SSIDs through a single beacon, where each SSID is mapped to a unique BSSID that may have a separate set of policies. Other alternatives to tie a virtual AP to a physical AP would also be acceptable.

Insert new elements in Table 5 as indicated below:

#### 7.2.3.1 Beacon frame format

- Table 5—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 22 | Multiple BSSID | The Multiple BSSID element is present only within Beacon frames generated by APs which support multiple BSSIDs. |
| 23 | Multiple BSSID-Index | The Multiple BSSID-Index element is present only within Beacon frames generated by APs which support multiple BSSIDs. |
| 24 | Multiple BSSID-TIM | The Multiple BSSID-TIM element is present only within Beacon frames generated by APs which support multiple BSSIDs. |
| 252 | Vendor Specific | One or more vendor specific information elements may appear in the frame. |

#### 7.2.3.8 Probe Request frame format

- Insert a new row into table 11 as follows:

- Table 11—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 5 | Multiple SSID | The Multiple SSID element is present when the STA requests information on multiple SSIDs. |
| 56 | Vendor Specific | One or more vendor specific information elements may appear in the frame. |

#### 7.2.3.9 Probe Response frame format

- Insert a new row into table 12 as follows:

- Table 12—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 21 | Multiple BSSID-PR | The Multiple BSSID-PR element is present only within Probe Response frames generated by APs supporting the Multiple BSSID capability. |
| 221 | Vendor Specific | One or more vendor specific information elements may appear in the frame. |

#### 7.3.2 Information Elements

Insert the following new information elements in clause 7.3.2, renumbering as appropriate:

| Information Element | Element ID |
| --- | --- |
| Multiple BSSID | ANA Assigned |
| Multiple SSID | ANA Assigned. |
| Multiple BSSID-PR | ANA Assigned |
| Multiple BSSID-Index | ANA Assigned |
| Multiple BSSID-TIM | ANA Assigned |

- Insert the following new clauses, renumbering the Vendor Specific Information Element as 7.3.2.28.:

#### 7.3.2.25 Multiple BSSID Element

- The format of the Multiple BSSID element is shown in Figure xx below.

|  | Element ID | Length | Max BSSID indicator | Non-Transmitted BSSID Profile |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure xx —Multiple BSSID element format

The Element ID field shall be xyz (ANA assigned).

The value of Length field shall be the length of the Non-Transmitted BSSID profile (variable) + 1. More than one Multiple BSSD elements may be included in a Beacon frame. The AP must determine the number of Multiple BSSID elements, and shall not fragment a Non-Transmitted BSSID Profile element across two Multiple BSSID elements.

The BSSID Indicator shall be n, where 2n is the maximum number of BSSIDs supported by the AP, including the transmitted BSSID. The actual number of SSIDs supported by the AP is not explicitly signalled. The non-transmitted BSSID(i) value corresponding to the ith  BSSID profile (indicated by the BSSID index)  shall be derived from the transmitting AP’s BSSID (AP_BSSID) as follows:

BSSID(i) = (AP_BSSID modified to set the  n LSBs to zero) | ((n LSBs of AP_BSSID) + i) mod 2n)

The Non-Transmitted BSSID profile includes selected information elements for each non-transmitted BSSID. The Non-Transmitted BSSID profile shall include the SSID element and Multiple BSSID-Index element for each of the supported BSSIDs. An AP is not required to advertise all supported SSIDs in the beacon, as the AP may hide one or more of its supported SSIDs.

As shown in the table xyz, the Non-Transmitted BSSID profile may include the Capability, Supported Rates, DS Parameter Set, CF Parameter Set, Power Constraint, Quiet, TPC Report, Extended Supported Rates, RSN and Vendor Specific information element fields for each non-transmitted BSSID. If any of these elements are not present for a specific non-transmitted BSSID, the corresponding element values are the element values of the transmitted BSSID. The Timestamp, Beacon Interval, FH Parameter Set, IBSS Parameter Set, TIM, Country, FH Parameters, FH Pattern Table, Channel Switch Assignment, IBSS DFS, and ERP Information information element values for each non-transmitted BSSID shall not be included in the Multiple BSSID element, and shall always be the transmitted BSSID element values.

Table xyz – Non-Transmitted-BSSID Profile

| 1 | Timestamp | Not Included |
| --- | --- | --- |
| 2 | Beacon Interval | Not Included |
| 3 | Capability | Optional |
| 4 | Service Set Identifier (SSID) | Included |
| 5 | Supported Rates | Optional |
| 6 | Frequency Hopping (FH Parameter Set | Not Included |
| 7 | DS Parameter Set | Optional |
| 8 | CF Parameter Set | Optional |
| 9 | IBSS Parameter Set | Not Included |
| 10 | Traffic Indication Map (TIM) | Not Included |
| 11 | Country | Not Included |
| 12 | FH Parameters | Not Included |
| 13 | FH Pattern Table | Not Included |
| 14 | Power Constraint | Optional |
| 15 | Channel Switch Announcement | Not Included |
| 16 | Quiet | Optional |
| 17 | IBSS DFS | Not Inlcluded |
| 18 | TPC Report | Optional |
| 19 | ERP Information | Not Included |
| 20 | Extended Supported Rates | Optional |
| 21 | RSN | Optional |
| 22 | Multiple BSSID | Not Included |
| 23 | Multiple BSSID-Index | Included |
| 24 | Vendor Specific | Optional |

The Multiple BSSID-Index field includes the DTIM Count and DTIM Period for the non-transmitted BSSIDs. When the Multiple BSSID-Index field is present, the Partial Virtual Bitmap field in the transmitted beacon shall indicate the presence or absence of traffic to be delivered to all stations supported by the AP.

When the Multiple BSSID element is present in the probe request frame, one or more of the SSID elements may have a NULL SSID or zero IE length to indicate a broadcast probe request.

#### 7.3.2.26 Multiple BSSID-PR Element

- The format of the Multiple BSSID-PR element is shown in Figure xy below.

|  | Element ID | Length | Max BSSID Indicator | Non-Transmitted BSSID Profile |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure xx —Multiple BSSID-PR element format

The Element ID field shall be xyz (ANA assigned).

The value of Length field shall be the length of the Non-Transmitted BSSID profile (variable) + 1. More than one Multiple BSSD-PR elements may be included in a probe response frame. The AP must determine the number of Multiple BSSID-PR elements, and shall not fragment a Non-Transmitted BSSID Profile element across two Multiple BSSID-PR elements.

The BSSID Indicator shall be n, where 2n is the maximum number of BSSIDs supported by the AP, including the transmitted BSSID. The actual number of SSIDs supported by the AP is not explicitly signalled. The non-transmitted BSSID(i) value corresponding to the ith  BSSID profile (indicated by the BSSID index)  shall be derived from the transmitting AP’s BSSID (AP_BSSID) as follows:

BSSID(i) = (AP_BSSID modified to set the  n LSBs to zero) | ((n LSBs of AP_BSSID) + i) mod 2n)

The Non-Transmitted BSSID profile includes selected information elements for each non-transmitted BSSID. The Non-Transmitted BSSID profile shall include the SSID element and Multiple BSSID-Index element for each of the supported BSSIDs. An AP is not required to include all supported SSIDs in the probe response, and the number of SSIDs included in the probe response may depend on local policies and/or the contents of the probe request.

As shown in the table xyz, the Non-Transmitted BSSID profile may include the Capability, Supported Rates, DS Parameter Set, CF Parameter Set, Power Constraint, Quiet, TPC Report, Extended Supported Rates, RSN and Vendor Specific information element fields for each non-transmitted BSSID. If any of these elements are not present for a specific non-transmitted BSSID, the corresponding element values are the element values  of the transmitted  BSSID, The Timestamp, Beacon Interval, FH Parameter Set, IBSS Parameter Set, Country, FH Parameters, FH Pattern Table, Channel Switch Assignment, IBSS DFS, and ERP Information information element values for each non-transmitted BSSID shall not be included in the Multiple BSSID-PR element, and shall always be the transmitted BSSID element values.

Table xyz – Non-Transmitted-BSSID Profile

| 1 | Timestamp | Not Included |
| --- | --- | --- |
| 2 | Beacon Interval | Not Included |
| 3 | Capability | Optional |
| 4 | Service Set Identifier (SSID) | Included |
| 5 | Supported Rates | Optional |
| 6 | Frequency Hopping (FH Parameter Set | Not Included |
| 7 | DS Parameter Set | Optional |
| 8 | CF Parameter Set | Optional |
| 9 | IBSS Parameter Set | Not Included |
| 10 | Country | Not Included |
| 11 | FH Parameters | Not Included |
| 12 | FH Pattern Table | Not Included |
| 13 | Power Constraint | Optional |
| 14 | Channel Switch Announcement | Not Included |
| 15 | Quiet | Optional |
| 16 | IBSS DFS | Not Inlcluded |
| 17 | TPC Report | Optional |
| 18 | ERP Information | Not Included |
| 19 | Extended Supported Rates | Optional |
| 20 | RSN | Optional |
| 21 | Multiple BSSID | Not Included |
| 22 | Multiple BSSID-Index | Included |
| 23 | Vendor Specific | Optional |

#### 7.3.2.27 Multiple SSID Element

- The format of the Multiple SSID element is shown in Figure xyz below.

|  | Element ID | Length | SSID List |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

Figure xyz —Multiple SSID element format

The Element ID field shall be xyz (ANA assigned).

The value of Length field shall be the length of the SSID list (variable) in octets.

The SSID List is a list of the SSID elements for which the STA is requesting information. A probe request may include one or more Multiple SSID elements.

#### 7.3.2.28 Multiple BSSID-Index Element

- The format of the Multiple BSSID-Index element is shown in Figure xz below.

|  | Element ID | Length | BSSID index | DTIM Period | DTIM Count |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure xz —Multiple BSSID-Index element format

The Element ID field shall be xyz (ANA assigned).

The value of Length field shall be the three octets when the Multiple BSSID-Index element is included in the Beacon frame, and shall be one octet when the Multiple BSSID-Index element is included in the Probe Response frame.

The BSSID Index is a value between 1 and 2n-1, which identifies the non-transmitted BSSID.

The DTIM Count is the DTIM count element for the BSSID. This element is not present when the Multiple BSSID-Index element is included in the Probe Response frame.

The DTIM Period is the DTIM period element for the BSSID. This element is not present when the Multiple BSSID-Index element is included in the Probe Response frame.

#### 7.3.2.29 Multiple BSSID-TIM Element

- The format of the Multiple BSSID-TIM element is shown in Figure xzz below.

|  | Element ID | Length | Bitmap Control | Partial Virtual Bitmap |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1-251 |

Figure xz —Multiple BSSID-TIM element format

The Element ID field shall be xyz (ANA assigned).

The value of the Length field indicates the length of the information fields.

The Bitmap Control field is a single octet. Bit 0 of the field is set to 1 in TIM elements with a value of 0 in the DTIM Count field of any multiple BSSID-index element in the beacon, when one or more broadcast or multicast frames are buffered at any BSS corresponding to a non-transmitted BSSID. The remaining 7 bits of the field form the Bitmap Offset.

The traffic-indication virtual bitmap, maintained by the AP that generates a TIM, consists of 2008 bits, and is organized into 251 octets such that the first 2n bits are broadcast/multicast bits for each BSS corresponding to a non-transmitted BSSID. Bit number N (2n < N < 2007) in the bitmap corresponds to bit number (N mod 8) in octet number [N / 8] + M, where the low-order bit of each octet is bit number 0, and the high order bit is bit number 7. 2n is the maximum number of BSSIDs supported.  M = 1, when n is less than 3, M = 2n-3 when n is 3 or greater. Each bit in the traffic-indication virtual bitmap corresponds to traffic buffered for a specific station within any BSS corresponding to a non-transmitted BSSID that is prepared to deliver the traffic at the time the beacon frame is transmitted. The bitmap is shared by all the BSSs corresponding to non-transmitted BSSIDs. Bit number N is 0 if there are no directed frames buffered for the station whose Association ID is N - 2n. If any directed frames for that station are buffered and the BSS is prepared to deliver them, bit number N - 2n in the traffic indication virtual bitmap is 1. A  PC may decline to set bits in the TIM for CF-Pollable stations it does not intend to poll (see 11.2.1.5).

The Partial Virtual Bitmap field consists of octets numbered N1 through N2 of the traffic indication virtual bitmap, where N1 is the largest even number such that bits numbered 2n + 1 through (N1 × 8) - 1 in the bitmap are all 0 and N2 is the smallest number such that bits numbered (N2 + 1) × 8 through 2007 in the bitmap are all 0. In this case, the Bitmap Offset subfield value contains the number [N1/2], and the Length field will be set to (N2 -N1) + 4.

In the event that all bits other than any of the first 2n bits in the virtual bitmap are 0, the Partial Virtual Bitmap field is encoded with 2n + 1 octets with all bits of the last octet equal to 0, and the Bitmap Offset subfield is 2n.

.

Note to editor: Change [] to standard symbols as in Clause 7.3.2.6.

References: