IEEE P802.11
Wireless LANs

| Modifications to the 802.11ma standard, to include 10 MHz and 5 MHz channels in the US regulatory |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ariel Sharon | Motorola | 1301 E. Algonquin Rd. Schaumburg, IL 60196, USA | +1 847-576-3403 | Ariel.sharon@motorola.com |
| Manny Kahana | Motorola | 1301 E. Algonquin Rd. Schaumburg, IL 60196, USA | +1 847-435-2770 | manny.kahana@motorola.com |

Abstract

This document contains all the modifications required to 802.11REVma (IEEE P802.11-REVma™ /D5.0) for the 4.9GHz public safety band in the USA and the use of 10 and 5MHz channels (1/2 and 1/4 clock) in this band for both the US and Japan.

Only affected sections, tables and figures are included.  Text added to the base document is underlined; deletions are noted with a red strikethrough.  Editorial instructions are in bold Italic.

The FCC references are provided at the end of this document.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

10.4.4.2 Semantics of the service primitive

Change the following raw in the table defining the primitive parameter in clause 10.4.4.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DATA_RATE | integer | 2, 3, 4, 5, 6, 9, 11, 12, 18, 22, 24, 27, 36, 44, 48, 54, 66, 72, 96, 108 | Selects among rates: 02 = 1 Mbit/s 03 = 1.5 Mbit/s 04 = 2 Mbit/s 05 = 2.5 Mbit/s* 06 = 3 Mbit/s 09 = 4.5 Mbit/s 11 = 5.5 Mbit/s 12 = 6 Mbit/s 18 = 9 Mbit/s 22 = 11 Mbit/s 24 = 12 Mbit/s 27 = 13.5 Mbit/s 36 = 18 Mbit/s 44 = 22 Mbit/s 48 = 24 Mbit/s 54 = 27 Mbit/s 66 = 33 Mbit/s 72 = 36 Mbit/s 96 = 48 Mbit/s 108 = 54 Mbit/s |

Insert the following note at the end of the table in clause 10.4.4.2:

*Note: The rate is rounded up to the next higher 0.5 Mbit/s value.

17.1 Introduction

Insert the following paragraph at the end of the clause 17.1:

The OFDM system also provides a “quarter-clocked” operation using 5 MHz channel spacing with data communication capabilities of 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mbit/s. The support of transmitting and receiving at data rates of 1.5, 3, and 6 Mbit/s is mandatory when using 5 MHz channel spacing. The quarter-clocked operation quadruples symbol times and clear channel assessment (CCA) times when using 5 MHz channel spacing. The regulatory requirements and information regarding use of this OFDM system in the 4.9 GHz band is in Annex I and Annex J.

17.2.2 TXVECTOR parameters

Change the following row in Table 97 in clause 17.2.2:

Table 97—TXVECTOR parameters

| Parameter | Associate primitive | Value |
| --- | --- | --- |
| DATATRATE | PHY-TXSTART.request (TXVECTOR) | 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/s for 20 MHz channel spacing (Support of 6, 12, and 24 data rates is mandatory.) |
|  |  | 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s for 10 MHz channel spacing (Support of 3, 6, and 12 data rates is mandatory.) |
|  |  | 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mbit/s for 5 MHz channel spacing (Support of 1.5, 3, and 6 Mbit/s is mandatory) |

17.2.2.2 TXVECTOR DATARATE

Insert in clause 17.2.2.2 as shown:

The DATARATE parameter describes the bit rate at which the PLCP shall transmit the PSDU. Its value can be any of the rates defined in Table 97. Data rates of 6, 12, and 24 Mbit/s shall be supported for 20 MHz channel spacing, and data rates of 3, 6, and 12 Mbit/s shall be supported for 10 MHz channel spacing, and data rates of 1.5, 3, and 6 Mbit/s shall be supported for 5 MHz channel spacing; other rates may also be supported.

17.2.3 RXVECTOR parameters

Change the following row in Table 98 in clause 17.2.3:

Table 98—RXVECTOR parameters

| Parameter | Associate primitive | Value |
| --- | --- | --- |
| DATARATE | PHY-RXSTART.request (RXVECTOR) | 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/s for 20 MHz channel spacing (Support of 6, 12, and 24 data rates is mandatory.) |
|  |  | 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s for 10 MHz channel spacing (Support of 3, 6, and 12 data rates is mandatory.) |
|  |  | 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mbit/s for 5 MHz channel spacing (Support of 1.5, 3, and 6 Mbit/s is mandatory) |

17.2.3.3 DATARATE

Insert in clause 17.2.3.3 as shown:

DATARATE shall represent the data rate at which the current PPDU was received. The allowed values of the DATARATE are 6, 9, 12, 18, 24, 36, 48, or 54 Mbit/s for 20 MHz channel spacing, and 3, 4.5, 6, 9, 12, 18, 24, or 27 Mbit/s for 10 MHz channel spacing, and 1.5, 2.25, 3, 4.5, 6, 9, 12, or 13.5 Mbit/s for 5 MHz channel spacing .

17.3.2.2 Modulation-dependent parameters

Change Table 99 in clause 17.3.2.2 as shown:

Table 99—Modulation-dependent parameters

| Modulation | Coding rate (R) | Coded bits per subcarrier (NBPSC) | Coded bits per OFDM symbol (NCBPS) | Data bits per OFDM symbol (NDBPS) | Data rate (Mbit/s) (20 MHz channel spacing) | Data rate (Mbit/s) (10 MHz channel spacing) | Data rate (Mbit/s) (5 MHz channel spacing) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BPSK | 1/2 | 1 | 48 | 24 | 6 | 3 | 1.5 |
| BPSK | 3/4 | 1 | 48 | 36 | 9 | 4.5 | 2.25 |
| QPSK | 1/2 | 2 | 96 | 48 | 12 | 6 | 3 |
| QPSK | 3/4 | 2 | 96 | 72 | 18 | 9 | 4.5 |
| 16-QAM | 1/2 | 4 | 192 | 96 | 24 | 12 | 6 |
| 16-QAM | 3/4 | 4 | 192 | 144 | 36 | 18 | 9 |
| 64-QAM | 2/3 | 6 | 288 | 192 | 48 | 24 | 12 |
| 64-QAM | 3/4 | 6 | 288 | 216 | 54 | 27 | 13.5 |

17.3.2.3 Timing related parameters

Change Table 100 in clause 17.3.2.3 as shown:

Table 100—Timing-related parameters

| Parameter | Value (20 MHz channel spacing) | Value (10 MHz channel spacing) | Value (5 MHz channel spacing) |
| --- | --- | --- | --- |
| NSD: Number of data subcarriers | 48 | 48 | 48 |
| NSP: Number of pilot subcarriers | 4 | 4 | 4 |
| NST: Number of subcarriers, total | 52 (NSD+ NSP) | 52 (NSD+ NSP) | 52 (NSD+ NSP) |
| ΔF: Subcarrier frequency spacing | 0.3125 MHz (=20 MHz/64) | 0.15625 MHz (= 10 MHz/64) | 0.078125 MHz (= 5 MHz/64) |
| TFFT: Inverse Fast Fourier Transform (IFFT) / Fast Fourier Transform (FFT) period | 3.2 μs (1/ΔF) | 6.4 µs (1/ΔF) | 12.8 µs (1/ΔF) |
| TPREAMBLE: PLCP preamble duration | 16 μs (TSHORT + TLONG) | 32 µs (TSHORT+ TLONG) | 64 µs (TSHORT+ TLONG) |
| TSIGNAL: Duration of the SIGNAL BPSK-OFDM symbol | 4.0 μs (TGI+ TFFT) | 8.0 µs (TGI+ TFFT) | 16.0 µs (TGI+ TFFT) |
| TGI: GI duration | 0.8 μs (TFFT/4) | 1.6 µs (TFFT/4) | 3.2 µs (TFFT/4) |
| TGI2: Training symbol GI duration | 1.6 μs (TFFT/2) | 3.2 µs (TFFT/2) | 6.4 µs (TFFT/2) |
| TSYM: Symbol interval | 4 μs (TGI+ TFFT) | 8 µs (TGI+ TFFT) | 16 µs (TGI+ TFFT) |
| TSHORT: Short training sequence duration | 8 μs (10 × TFFT /4) | 16 µs (10 × TFFT/4) | 32 µs (10 × TFFT/4) |
| TLONG: Long training sequence duration | 8 μs (TGI2+ 2 × TFFT) | 16 µs (TGI2+ 2 × TFFT) | 32 µs (TGI2+ 2 × TFFT) |

17.3.2.4 Mathematical conventions in the signal descriptions

Change the third paragraph of clause 17.3.2.4 as shown:

The subframes of which Equation (7) are composed are described in 17.3.3, 17.3.4, and 17.3.5.9. The time offsets tSUBFRAME determine the starting time of the corresponding subframe; tSIGNAL is equal to 16 µs for

20 MHz channel spacing, and 32 µs for 10 MHz channel spacing, and 64 µs for 5 MHz channel spacing, and tDATA is equal to 20 µs for 20 MHz channel spacing, and 40 µs for 10 MHz channel spacing, and 80 µs for 5 MHz channel spacing.

17.3.3 PLCP preamble (SYNC)

Change the first paragraph of clause 17.3.3 as shown:

The PLCP Preamble field is used for synchronization. It consists of 10 short symbols and two long symbols that are shown in Figure 190 and described in this subclause. The timings described in this subclause and shown in Figure 190 are for 20 MHz channel spacing, and are doubled for half-clocked (i.e., 10 MHz) channel spacing, and are quadrupled for quarter-clocked (i.e., 5 MHz) channel spacing.

17.3.4.1 RATE field

Change Table 101 in clause 17.3.4.1 as shown:

Table 101—Contents of the SIGNAL field

| R1–R4 | Rate (Mbit/s) (20 MHz channel spacing) | Rate (Mbit/s) (10 MHz channel spacing) | Rate (Mbit/s) (5 MHz channel spacing) |
| --- | --- | --- | --- |
| 1101 | 6 | 3 | 1.5 |
| 1111 | 9 | 4.5 | 2.25 |
| 0101 | 12 | 6 | 3 |
| 0111 | 18 | 9 | 4.5 |
| 1001 | 24 | 12 | 6 |
| 1011 | 36 | 18 | 9 |
| 0001 | 48 | 24 | 12 |
| 0011 | 54 | 27 | 13.5 |

17.3.8.1 Outline description

Change Table 107 in clause 17.3.8.1 as shown:

Table 107—Major parameters of the OFDM PHY

| Information data rate | 6, 9, 12, 18, 24, 36, 48 and 54 Mbit/s (6, 12 and 24 Mbit/s are mandatory) (20 MHz channel spacing) | 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s (3, 6, and 12 Mbit/s are mandatory) (10 MHz channel spacing) | 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mb/s (1.5, 3, and 6 Mbit/s are mandatory) (5 MHz channel spacing) |
| --- | --- | --- | --- |
| Modulation | BPSK OFDM QPSK OFDM 16-QAM OFDM 64-QAM OFDM | BPSK OFDM QPSK OFDM 16-QAM OFDM 64-QAM OFDM | BPSK OFDM QPSK OFDM 16-QAM OFDM 64-QAM OFDM |
| Error correcting code | K = 7 (64 states) convolutional code | K = 7 (64 states) convolutional code | K=7 (64 states) convolutional code |
| Coding rate | 1/2, 2/3, 3/4 | 1/2, 2/3, 3/4 | 1/2, 2/3, 3/4 |
| Number of subcarriers | 52 | 52 | 52 |
| OFDM symbol duration | 4.0 µs | 8.0 µs | 16.0 µs |
| GI | 0.8 µsa (TGI) | 1.6 µs (TGI) | 3.2 µs |
| Occupied bandwidth | 16.6 MHz | 8.3 MHz | 4.15 MHz |

17.3.8.6 Slot time

Change the first paragraph of clause 17.3.8.6 as shown:

The slot time for the OFDM PHY shall be 9 µs for 20 MHz channel spacing, and shall be 13 µs for 10 MHz channel spacing, and shall be 21 µs for 5 MHz channel spacing.

17.3.9.4 Transmit center frequency tolerance

Change clause 17.3.9.4 as shown:

The transmitted center frequency tolerance shall be ±20 ppm maximum for 20 MHz and 10 MHz channels, and shall be ±10 ppm maximum for 5 MHz channels. The transmit center frequency and the symbol clock frequency shall be derived from the same reference oscillator.

17.3.9.5 Symbol clock frequency tolerance

Change clause 17.3.9.5 as shown:

The symbol clock frequency tolerance shall be ±20 ppm maximum for 20 MHz and 10 MHz channels, and shall be ±10 ppm maximum for 5 MHz channels.. The transmit center frequency and the symbol clock frequency shall be derived from the same reference oscillator.

17.3.10.1 Receiver minimum input sensitivity

Change Table 109 in clause 17.3.10.1 as shown:

Table 109—Receiver performance requirements

| Modulation | Coding rate (R) | Adjacent channel rejection (dB) | Alternate adjacent channel rejection (dB) | Minimum sensitivity (dBm) (20 MHz channel spacing) | Minimum sensitivity (dBm) (10 MHz channel spacing) | Minimum sensitivity (dBm) (5 MHz channel spacing |
| --- | --- | --- | --- | --- | --- | --- |
| BPSK | 1/2 | 16 | 32 | –82 | –85 | -88 |
| BPSK | 3/4 | 15 | 31 | –81 | –84 | -87 |
| QPSK | 1/2 | 13 | 29 | –79 | –82 | -85 |
| QPSK | 3/4 | 11 | 27 | –77 | –80 | -83 |
| 16-QAM | 1/2 | 8 | 24 | –74 | –77 | -80 |
| 16-QAM | 3/4 | 4 | 20 | –70 | –73 | -77 |
| 64-QAM | 2/3 | 0 | 16 | –66 | –69 | -72 |
| 64-QAM | 3/4 | –1 | 15 | –65 | –68 | -71 |

17.3.10.5 CCA sensitivity

Change clause 17.3.10.5 as shown:

The start of a valid OFDM transmission at a receive level equal to or greater than the minimum modulation

and coding rate sensitivity (–82 dBm for 20 MHz channel spacing, and –85 dBm for 10 MHz channel spacing, and -88 dBm for 5 MHz channel spacing) shall cause CCA to indicate busy with a probability > 90% within 4 µs for 20 MHz channel spacing, and 8 µs for 10 MHz channel spacing, and 16 µs for 5 MHz channel spacing. If the preamble portion was missed, the receiver shall hold the CS signal busy for any signal 20 dB above the minimum modulation and coding rate sensitivity (–62 dBm for 20 MHz channel spacing, and –65 dBm for 10 MHz channel spacing, and –68 dBm for 5 MHz channel spacing).

NOTE—CCA detect time is based on finding the short sequences in the preamble, so when TSYM  doubles, so does CCA detect time.

17.4.2 OFDM PHY MIB

Change the following rows in Table 110 in clause 17.4.2:

Table 110—MIB attribute default values/ranges

| Managed object | Default value/range | Operational semantics |
| --- | --- | --- |
| dot11SupportedDataRatesTxTable |  |  |
| dot11 Supported data rates Tx value | 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/s for 20 MHz channel spacing (Mandatory rates: 6, 12, and 24) 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s for 10 MHz channel spacing (Mandatory rates: 3, 6, and 12) 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mb/s for 5 MHz channel spacing (Mandatory rates: 1.5, 3, and 6) | Static |
| dot11SupportedDataRatesRxTable |  |  |
| dot11 Supported data rates Rx value | 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/s for 20 MHz channel spacing (Mandatory rates: 6, 12, and 24) 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s for 10 MHz channel spacing (Mandatory rates: 3, 6, and 12) 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mb/s for 5 MHz channel spacing (Mandatory rates: 1.5, 3, and 6) | Static |

17.4.4 OFDM PHY characteristics

Change Table 111 in clause 17.4.4 as shown:

Table 111—OFDM PHY characteristics

| Characteristics | Value (20 MHz channel spacing) | Value (10 MHz channel spacing) | Value (5 MHz channel spacing) |
| --- | --- | --- | --- |
| aSlotTime | 9 µs | 13 µs | 21 µs |
| aSIFSTime | 16 µs | 32 µs | 64 µs |
| aCCATime | < 4 µs | < 8 µs | <16 µs |
| aPHY-RX-START-Delay | 24 µs | 48 µs | 96 µs |
| aRxTxTurnaroundTime | < 2 µs | < 2 µs | <2 µs |
| aTxPLCPDelay | Implementation dependent | Implementation dependent | Implementation dependent |
| aRxPLCPDelay | Implementation dependent | Implementation dependent | Implementation dependent |
| aRxTxSwitchTime | << 1 µs | << 1 µs | << 1 µs |
| aTxRampOnTime | Implementation dependent | Implementation dependent | Implementation dependent |
| aTxRampOffTime | Implementation dependent | Implementation dependent | Implementation dependent |
| aTxRFDelay | Implementation dependent | Implementation dependent | Implementation dependent |
| aRxRFDelay | Implementation dependent | Implementation dependent | Implementation dependent |
| aAirPropagationTime | << 1 µs | << 1 µs | << 1 µs |
| aMACProcessingDelay | < 2 µs | < 2 µs | < 2 µs |
| aPreambleLength | 16 µs | 32 µs | 64 µs |
| aPLCPHeaderLength | 4 µs | 8 µs | 16 µs |
| aMPDUMaxLength | 4095 | 4095 | 4095 |
| aCWmin | 15 | 15 | 15 |
| aCWmax | 1023 | 1023 | 1023 |

17.5.4.3 PMD_SAP service primitive parameters

Change Table 114 in clause 17.5.4.3 as shown:

Table 114—List of parameters for the PMD primitives

| Parameter | Associate primitive | Value (20 MHz channel spacing) | Value (10 MHz channel spacing) | Value (5 MHz channel spacing) |
| --- | --- | --- | --- | --- |
| TXD_UNIT | PMD_DATA.request | One(1), Zero(0): one OFDM symbol value | One(1),Zero(0): one OFDM symbol value | One(1),Zero(0): one OFDM symbol value |
| RXD_UNIT | PMD_DATA.indicate | One(1), Zero(0): one OFDM symbol value | One(1),Zero(0): one OFDM symbol value | One(1),Zero(0): one OFDM symbol value |
| TXPWR_LEVEL | PMD_TXPWRLVL.request | 1–8 (max of 8 levels) | 1–8 (max of 8 levels) | 1–8 (max of 8 levels) |
| RATE | PMD_RATE.request | 12 Mbit/s (for BPSK) 24 Mbit/s (for QPSK) 48 Mbit/s (for 16-QAM) 72 Mbit/s (for 64-QAM) | 6 Mbit/s (for BPSK) 12 Mbit/s (for QPSK) 24 Mbit/s (for 16-QAM) 36 Mbit/s (for 64-QAM) | 3 Mbit/s (for BPSK) 6 Mbit/s (for QPSK) 12 Mbit/s (for 16-QAM) 18 Mbit/s (for 64-QAM) |
| RSSI | PMD_RSSI.indicate | 0–8 bits of RSSI | 0–8 bits of RSSI | 0–8 bits of RSSI |

Annex D

(normative)

ASN.1 encoding of the MAC and PHY MIB

In “dot11PhyOFDM TABLE” in Annex D, change dot11FrequencyBandsSupported object as shown:

dot11FrequencyBandsSupported OBJECT-TYPE

SYNTAX INTEGER (1..127)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The capability of the OFDM PHY implementation to operate in the

three U-NII bands, in the CEPT Band B, and in the Japanese 5 GHz

4.9 GHz and 5 GHz bands. Coded as an integer value of a three bit field as follows:

bit 0 .. capable of operating in the lower (5.15-5.25 GHz) U-NII band

bit 1 .. capable of operating in the middle (5.25-5.35 GHz) U-NII band

bit 2 .. capable of operating in the upper (5.725-5.825 GHz) U-NII band

bit 3 .. capable of operating in the CEPT Band B (5.47-5.725 GHz)

band

bit 4 .. capable of operating in the lower Japanese (5.15-5.25 GHz) band

bit 5 .. capable of operating in the Japanese 5.0 GHz (5.03-5.091 GHz) band

bit 6 .. capable of operating in the Japanese 4.9 GHz (4.9-5.0 GHz) band

For example, for an implementation capable of operating in the 5.15-5.35 GHz lower and mid bands this attribute would take the value 3."

::= { dot11PhyOFDMEntry 3 }

Annex I

(informative)

Regulatory classes

I.1 External regulatory references

Change the first paragraph of clause I.1 as shown:

This annex and Annex J provide information and requirements extending the OFDM PHY specification in Clause 17 for operation in the USA 5 GHz band and 4.9 public safety band, Europe, and the 4.9 GHz and 5 GHz bands in Japan. The OFDM system provides a WLAN with data payload communication capabilities of 6, 9, 12, 18, 24, 36, 48, and 54 Mbit/s in 20 MHz channels, and 3, 4.5, 6, 9, 12, 18, 24, and 27 Mbit/s in 10 MHz channels, and 1.5, 2.25, 3, 4.5, 6, 9, 12, and 13.5 Mb/s for 5 MHz channels.

Change Table I.1 in clause I.1 as shown:

Table I.1—Regulatory requirement list

| Geographic area | Approval standards | Documents | Approval authority |
| --- | --- | --- | --- |
| Japan | Ministry of Internal Affairs and Com-munications (MIC) | MIC Equipment Ordinance (EO) for Regulating Radio Equipment Articles 7, 49.20, 49.21a | MIC |
| United States | Federal Communications Commission (FCC) | CFR47, Part 15, Sections 15.205, 15.209, and 15.247; and Subpart E, Sections 15.401–15.407, Section 90.210, Section 90.1201-90.1217 | FCC |
| Europe | European Conference of Postal and Telecommunications (CEPT) Administrations and its Electronic Communications Committee (ECC). Also, European Radiocommunications Office, European Telecommunications Standards Institute | ECC DEC (04) 08, ETSI EN301 893 | CEPT |

a Frequency planning for licensed STAs in Japan is performed by the regulatory authority and the licensees, addressing the coexistence among STAs operating with a variety of air propagation times and the coexistence between STAs using

20 MHz channel spacing, and STAs operating with 10 MHz channel spacing, and STAs operating with 5 MHz channel spacing. Note also the CCA mechanism is preserved in licensed operation.

Change Table I.2 in clause I.1 as shown:

Table I.2—Emissions limits sets

| Emissions limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 0 | Not specified | Not specified | Not specified |
| 1 nomadic use | FCC CFR47, Section 15.407 | ETSI EN 301 389-1 | MIC EO Articles 49.20, 49.21 |
| 2 interference-limited areasa | Reserved | Reserved | MIC EO Article 49.21 |
| 3 other interference areasa | Reserved | Reserved | MIC EO Article 49.21 |
| 4 | FCC CFR47, Section 15.247 | Reserved | Reserved |
| 5 public safety | FCC CFR47, Section 90.210 | Reserved | Reserved |
| 6–255 | Reserved | Reserved | Reserved |

Change Table I.3 in clause I.1 as shown:

Table I.3—Behavior limits sets

| Behavior limits set | USA | Europe | Japan |
| --- | --- | --- | --- |
| 0 | Not specified | Not specified | Not specified |
| 1 nomadic use | FCC CFR47, Section 15.407 | ETSI EN 301 389-1 | MIC EO Articles 49.20, 49.21 |
| 2 indoor only use | FCC CFR47, Section 15.407(e) | ETSI EN 301 389-1 | MIC EO Article 49.20 |
| 3 transmit power control | Reserved | ETSI EN 301 389-1 | Reserved |
| 4 dynamic frequency selection | Reserved | ETSI EN 301 389-1 | Reserved |
| 5 Independent basic service set (IBSS) prohibited | Reserved | Reserved | MIC EO Article 49.21 |
| 6 4 ms CSa | Reserved | Reserved | MIC EO Articles 49.20, 49.21 |
| 7 licensed base station | Reserved | Reserved | MIC EO Article 49.21 |
| 8 mobile station | Reserved | Reserved | MIC EO Articles 49.20, 49.21 |
| 9 public safety | FCC CFR47, Section 90.1209 | Reserved | Reserved |
| 10–255 | Reserved | Reserved | Reserved |

I.2.2 Transmit power levels

Insert at the end of the first paragraph of clause I.2.2 as follows:

The maximum allowable output power by regulatory domain for the USA 4.9 GHz public safety band is shown in Table I.8.

Insert at the end of clause I.2.2 Table I.8 as shown:

Table I.8—USA Public Safety transmit power levels by regulatory domain

| Frequency band (GHz) | USA Public Safety (mW) for 20 MHz channels | USA Public Safety (mW) for 10 MHz channels | USA Public Safety (mW) for 5 MHz channels |
| --- | --- | --- | --- |
| 4.94-4.99 Low Power | 100 | 50 | 25 |
| 4.94-4.99 High Power | 2000 | 1000 | 500 |

I.2.3 Transmit spectrum mask

Insert at the end of clause I.2.3 the following paragraphs and figure, as shown:

For operation in the 4.94-4.99 GHz Low Power USA Public Safety band, FCC CFR47, Section 90.210 mask L, the transmitted spectrum shall be as follows:

a. For operation using 20 MHz channel spacing, the transmitted spectrum shall have a 0 dBr (dB relative to the maximum spectral density of the signal) bandwidth not exceeding 18 MHz, -10 dBr at 10 MHz frequency offset, –20 dBr at 11 MHz frequency offset, –28 dBr at 20 MHz frequency offset,  –40 dBr at 30 MHz frequency offset, and -50 dBr above.

b. For operation using 10 MHz channel spacing, the transmitted spectrum shall have a 0 dBr bandwidth not exceeding 9 MHz, -10 dBr at 5 MHz frequency offset, –20 dBr at 5.5 MHz frequency offset, –28 dBr at 10 MHz frequency offset,  –40 dBr at 15 MHz frequency offset,  and -50 dBr above.

c. For operation using 5 MHz channel spacing, the transmitted spectrum shall have a 0 dBr bandwidth not exceeding 4.5 MHz, -10 dBr at 2.5 MHz frequency offset, –20 dBr at 2.75 MHz frequency offset, –28 dBr at 5 MHz frequency offset,  –40 dBr at 7.5 MHz frequency offset, and -50 dBr above.

For operation in the 4.94-4.99 GHz High Power USA Public Safety band, FCC CFR47, Section 90.210 mask M, the transmitted spectrum shall be as follows:

a. For operation using 20 MHz channel spacing, the transmitted spectrum shall have a 0 dBr (dB relative to the maximum spectral density of the signal) bandwidth not exceeding 18 MHz, -26 dBr at 10 MHz frequency offset, –32 dBr at 11 MHz frequency offset, –40 dBr at 20 MHz frequency offset,  –50 dBr at 30 MHz frequency offset and above.

b. For operation using 10 MHz channel spacing, the transmitted spectrum shall have a 0 dBr bandwidth not exceeding 9 MHz, -26 dBr at 5 MHz frequency offset, –32 dBr at 5.5 MHz frequency offset, –40 dBr at 10 MHz frequency offset,  –50 dBr at 15 MHz frequency offset and above.

c. For operation using 5 MHz channel spacing, the transmitted spectrum shall have a 0 dBr bandwidth not exceeding 4.5 MHz, -26 dBr at 2.5 MHz frequency offset, –32 dBr at 2.75 MHz frequency offset, –40 dBr at 5 MHz frequency offset,  –50 dBr at 7.5 MHz frequency offset and above.

Figure I.6 shows the spectral masks for the USA 4.9 GHz public safety band.

Figure I.6—Transmit spectrum masks for the USA 4.9 GHz public safety band

Annex J

(normative)

Country information element and regulatory classes

Change the third line from the end of the text in Annex J as shown:

The regulatory classes specified for 4.9 GHz and 5 GHz operation in the USA are enumerated in Table J.1.

Change Table J.1 of Annex J, as shown:

Table J.1—Regulatory classes for the 4.9 GHz and 5 GHz bands in the USA

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (mW) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 20 | 36, 40, 44, 48 | 40 | 1 | 1, 2 |
| 2 | 5 | 20 | 52, 56, 60, 64 | 200 | 1 | 1 |
| 3 | 5 | 20 | 149, 153, 157, 161 | 800 | 1 | 1 |
| 4 | 5 | 20 | 100, 104, 108, 112, 116, 120, 124, 128, 132, 136, 140 | 200 | 1 | 1 |
| 5 | 5 | 20 | 165 | 1000 | 4 | 1 |
| 6 | 4.9375 | 5 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10 | 25 | 5 | 9 |
| 7 | 4.9375 | 5 | 1, 2, 3, 4, 5, 6, 7, 8, 9, 10 | 500 | 5 | 9 |
| 8 | 4.89 | 10 | 11, 13, 15, 17, 19 | 50 | 5 | 9 |
| 9 | 4.89 | 10 | 11, 13, 15, 17, 19 | 1000 | 5 | 9 |
| 10 | 4.85 | 20 | 21, 25 | 100 | 5 | 9 |
| 11 | 4.85 | 20 | 21, 25 | 2000 | 5 | 9 |
| 12–255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

Change Table J.3 of Annex J, as shown:

Table J.3—Regulatory classes for 4.9 GHz and 5 GHz bands in Japan

| Regulatory class | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel set | Transmit power limit (dBm) | Emissions limits set | Behavior limits set |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 20 | 34, 38, 42, 46 | 22 | 1 | 1, 2, 6 |
| 2 | 5 | 20 | 8, 12, 16 | 24 | 2 | 5, 6, 7 |
| 3 | 5 | 20 | 8, 12, 16 | 24 | 2 | 5, 6, 8 |
| 4 | 5 | 20 | 8, 12, 16 | 24 | 3 | 5, 6, 7 |
| 5 | 5 | 20 | 8, 12, 16 | 24 | 3 | 5, 6, 8 |
| 6 | 5 | 20 | 8, 12, 16 | 22 | 1 | 5, 6, 8 |
| 7 | 4 | 20 | 184, 188, 192, 196 | 24 | 2 | 5, 6, 7 |
| 8 | 4 | 20 | 184, 188, 192, 196 | 24 | 2 | 5, 6, 8 |
| 9 | 4 | 20 | 184, 188, 192, 196 | 24 | 3 | 5, 6, 7 |
| 10 | 4 | 20 | 184, 188, 192, 196 | 24 | 3 | 5, 6, 8 |
| 11 | 4 | 20 | 184, 188, 192, 196 | 22 | 1 | 5, 6, 8 |
| 12 | 5 | 10 | 7, 8, 9, 11 | 24 | 2 | 5, 6, 7 |
| 13 | 5 | 10 | 7, 8, 9, 11 | 24 | 2 | 5, 6, 8 |
| 14 | 5 | 10 | 7, 8, 9, 11 | 24 | 3 | 5, 6, 7 |
| 15 | 5 | 10 | 7, 8, 9, 11 | 24 | 3 | 5, 6, 8 |
| 16 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 24 | 2 | 5, 6, 7 |
| 17 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 24 | 2 | 5, 6, 8 |
| 18 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 24 | 3 | 5, 6, 7 |
| 19 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 24 | 3 | 5, 6, 8 |
| 20 | 4 | 10 | 183, 184, 185, 187, 188, 189 | 17 | 1 | 5, 6, 8 |
| 21 | 5.0025 | 5 | 6, 7, 8, 9, 10, 11 | 24 | 2 | 5, 6, 7 |
| 22 | 5.0025 | 5 | 6, 7, 8, 9, 10, 11 | 24 | 2 | 5, 6, 8 |
| 23 | 5.0025 | 5 | 6, 7, 8, 9, 10, 11 | 24 | 3 | 5, 6, 7 |
| 24 | 5.0025 | 5 | 6, 7, 8, 9, 10, 11 | 24 | 3 | 5, 6, 8 |
| 25 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 | 24 | 2 | 5, 6, 7 |
| 26 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 | 24 | 2 | 5, 6, 8 |
| 27 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 | 24 | 3 | 5, 6, 7 |
| 28 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 | 24 | 3 | 5, 6, 8 |
| 29 | 4.0025 | 5 | 182, 183, 184, 185, 186, 187, 188, 189 | 17 | 1 | 5, 6, 8 |
| 30–255 | Reserved | Reserved | Reserved | Reserved | Reserved | Reserved |

End of Changes

References:

[1]	IEEE P802.11-REVma™ /D5.0 in Sponsor Ballot, November 2005.

[2]	FCC, Title 47 of the Code of Federal Regulations, Parts 2 and 90, and the related

Memorandum Opinion and Order, FCC 04-265 (revised technical rules for the

4.9 GHz band), available at:

http://hraunfoss.fcc.gov/edocs_public/attachmatch/FCC-04-265A1.doc,

Nov. 12, 2004.

[3]	Peter Ecclesine and Yasuhiko Inoue, “Preparation of PAR and 5 Criterion addressing Japan’s 4.9-5GHz bands”, doc: 11-02-480r2-WNG, July 2002.