IEEE P802.11
Wireless LANs

| Pilot Frame Justification Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-NOV-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tim Olson | Cisco Systems | 170 West Tasman Dr. San Jose, CA 95137 | 408-853-4527 | tolson@cisco.com |

Abstract

This document contains normative text changes for letter ballot 78 comments for section 7.1.3.1.2.  This document resolves comments: 120, 142, 161, 201, 278, 386, 820, 835, and 901.

The submission is referenced to draft D3.0.

All new text is identified by underlining and all deleted text is identified by strikethrough.

TGk editor: Insert the following text after the 11.14 heading:

## 11.14 Measurement Pilot frame generation and usage

The Measurement Pilot frame is a compact management frame periodically transmitted by an AP with a relatively small interval as compared to a Beacon Interval.  The Measurement Pilot frame is designed to provide a minimal set of information as compared to a Beacon frame to allow for the required small interval. The purpose of the Measurement Pilot frame is to provide timely information to a STA for the following functions:

- Rapid discovery of a BSS via passive scanning.

- Reliable collection of neighbor measurements via passive scanning.

- Link margin calculation for various purposes such as transition decisions or initial rate selection.

### 11.14.1 Measurement Pilot frame generation by an AP