IEEE P802.11
Wireless LANs

| TGr MIB Proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-11-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Frank Ciotti | Motorola | 7700 W. Parmer Ln PL67 Austin, TX 78726 | 512-971-6034 | frank.ciotti@ motorola.com |
| Nancy Cam-Winget | Cisco Systems | 3625 Cisco Way, San Jose, Ca 95134 | 408-853-0532 | ncamwing@cisco.com |

Abstract

Proposed text to insert the TGr MIB into the 802.11ma MIB, and related changes to TGr draft 0.9.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary of MIB:

- Added new object dot11FastTransitionImplemented to SMT Station Config Table

- Updated Authentication Alogorithm Number to add Fast Transition

- No changes necessary for FT AKM type

- Added dot11FastTransitionTable to dot11smt GROUPS containing 12 objects:

  - Dot11FastTransitionEnabled

  - New object for Mobility Domain Identifier

  - 4 new objects for FT Resource Mechanisms

  - dot11FTResourceRequestTimeout

  - dot11FTPendingResourceRequestLimit

  - 2 new objects for R0/R1 KHIDs

  - dot11FTReassociationDeadline

  - Created new object for RDIE mandatory TSPEC control

Change the notes entry for Mobility Domain IE in Tables 5 (Beacon) and 12 (Probe Response) to:

The Mobility domain IE is only present when dot11FastTransitionEnabled is set to TRUE.  The Mobility Domain Identifier shall be the value of dot11FTMobilityDomainID.

Change all occurrences of:

dot11FastTransitionDeadline

to:

dot11FTReassociationDeadline

Change all occurrences of:

dot11ResourceRequestTimeout

to:

dot11FTResourceRequestTimeout

Change all occurrences of:

dot11PendingResourceRequestLimit

to:

dot11FTPendingResourceRequestLimit

Annex D

(normative)

ASN.1 encoding of the MAC and PHY MIB

In “Major sections” of Annex D, change the dot11smt object identifier list as follows:

-- dot11smt GROUPS

-- dot11StationConfigTable ::= { dot11smt 1 }

-- dot11AuthenticationAlgorithmsTable ::= { dot11smt 2 }

-- dot11WEPDefaultKeysTable ::= { dot11smt 3 }

-- dot11WEPKeyMappingsTable ::= { dot11smt 4 }

-- dot11PrivacyTable ::= { dot11smt 5 }

-- dot11SMTnotification ::= { dot11smt 6 }

-- dot11MultiDomainCapabilityTable ::= { dot11smt 7 }

-- dot11SpectrumManagementTable ::= { dot11smt 8 }

-- dot11RSNAConfigTable ::= { dot11smt 9 }

-- dot11RSNAConfigPairwiseCiphersTable ::= { dot11smt 10 }

-- dot11RSNAConfigAuthenticationSuitesTable ::= { dot11smt 11 }

-- dot11RSNAStatsTable ::= { dot11smt 12 }

-- dot11RegulatoryClassesTable ::= { dot11smt 13 }

-- dot11FastTransitionTable ::= { dot11smt 14 }

In “SMT Station Config Table” in Annex D, change Dot11StationConfigEntry as follows:

dot11StationID MacAddress,

dot11MediumOccupancyLimit INTEGER,

dot11CFPollable TruthValue,

dot11CFPPeriod INTEGER,

dot11CFPMaxDuration INTEGER,

dot11AuthenticationResponseTimeOut Unsigned32,

dot11PrivacyOptionImplemented TruthValue,

dot11PowerManagementMode INTEGER,

dot11DesiredSSID OCTET STRING,

dot11DesiredBSSType INTEGER,

dot11OperationalRateSet OCTET STRING,

dot11BeaconPeriod INTEGER,

dot11DTIMPeriod INTEGER,

dot11AssociationResponseTimeOut Unsigned32,

dot11DisassociateReason INTEGER,

dot11DisassociateStation MacAddress,

dot11DeauthenticateReason INTEGER,

dot11DeauthenticateStation MacAddress,

dot11AuthenticateFailStatus INTEGER,

dot11AuthenticateFailStation MacAddress,

dot11MultiDomainCapabilityImplemented TruthValue,

dot11MultiDomainCapabilityEnabled TruthValue,

dot11CountryString OCTET STRING,

dot11SpectrumManagementImplemented TruthValue,

dot11SpectrumManagementRequired TruthValue,

dot11RSNAOptionImplemented TruthValue,

dot11RSNAPreauthenticationImplemented TruthValue,

dot11RegulatoryClassesImplemented TruthValue,

dot11RegulatoryClassesRequired TruthValue,

dot11FastTransitionImplemented TruthValue}

In “SMT Station Config Table” in Annex D, insert the following attribute after dot11RegulatoryClassesRequired { dot11StationConfigEntry 29 }:

dot11FastTransitionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object indicates if the entity is Fast BSS Transition capable."

::= { dot11StationConfigEntry 30 }

In the “dot11AuthenticationAlgorithms TABLE” of Annex D, change dot11AuthenticationAlgorithmsTable as follows:

dot11AuthenticationAlgorithmsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11AuthenticationAlgorithmsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This (conceptual) table of attributes shall be a set of

all the authentication algorithms supported by the

stations. The following are the default values and the

associated algorithm:

Value = 1: Open System

Value = 2: Shared Key

Value = 3: Fast Transition"

REFERENCE "IEEE Std 802.11, 1999 Edition, 7.3.1.1"

::= { dot11smt 2 }

In the “dot11AuthenticationAlgorithms TABLE” of Annex D, change dot11AuthenticationAlgorithm as follows:

dot11AuthenticationAlgorithm OBJECT-TYPE

SYNTAX INTEGER { openSystem(1), sharedKey(2), fastTransition(3) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute shall be a set of all the authentication

algorithms supported by the STAs. The following are the

default values and the associated algorithm.

Value = 1: Open System

Value = 2: Shared Key"

Value = 3: Fast Transition"

::= { dot11AuthenticationAlgorithmsEntry 2 }

After the “dot11RegulatoryClasses TABLE” section (and before the “MAC Attribute Templates” section) in

Annex D, insert the new section “dot11FastTransitionConfig TABLE” as follows:

-- ********************************************************************

-- * dot11FastTransitionConfig TABLE

-- ********************************************************************

dot11FastTransitionConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FastTransitionConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The table containing Fast BSS Transition configuration objects."

::= { dot11smt 14 }

dot11FastTransitionConfigEntry OBJECT-TYPE

SYNTAX Dot11FastTransitionConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FastTransitionConfigTable."

INDEX { ifIndex }

::= { dot11FastTransitionConfigTable 1 }

Dot11FastTransitionConfigEntry ::=

SEQUENCE {

dot11FastTransitionEnabled              TruthValue,

dot11FTMobilityDomainID                 OCTET STRING,

dot11FTReservationOverAirEnabled        TruthValue,

dot11FTReservationOverDSEnabled         TruthValue,

dot11FTReservationRequired              TruthValue,

dot11FTReservationLimit                 INTEGER,

dot11FTResourceRequestTimeout           Unsigned32,

dot11FTPendingResourceRequestLimit      INTEGER,

dot11FTR0KeyHolderID                    OCTET STRING,

dot11FTR1KeyHolderID                    OCTET STRING,

dot11FTReassociationDeadline            Unsigned32,

dot11FTTspecAllocationRequired          TruthValue }

dot11FastTransitionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is set to TRUE, this shall indicate that Fast

BSS Transition is enabled on this entity.  The entity will

advertise the Fast Transition related Information Elements

in its Beacon and Probe Response frames. This object requires

that dot11FastTransitionImplemented also be set to TRUE."

::= { dot11FastTransitionConfigEntry 1 }

dot11FTMobilityDomainID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(6))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the Mobility Domain identifier of this

entity.”

::= { dot11FastTransitionConfigEntry 2 }

dot11FTReservationOverAirEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is set to TRUE, this shall indicate that Fast

BSS Transition via the over-the-air mechanism as described

in 8A is enabled on this TAP entity."

::= { dot11FastTransitionConfigEntry 3 }

dot11FTReservationOverDSEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is set to TRUE, this shall indicate that Fast

BSS Transition via the over-the-DS mechanism as described

in 8A is enabled on this TAP entity."

::= { dot11FastTransitionConfigEntry 4 }

dot11FTReservationRequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is set to TRUE, this shall indicate that the Fast

BSS Transition Reservation mechanism as described in 8A.5 is

mandatory on this TAP entity.  When this object is set to FALSE,

this shall indicate that the Fast BSS Transition Reservation

mechanism is optional on this TAP entiry."

::= { dot11FastTransitionConfigEntry 5 }

dot11FTReservationLimit OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the maximum number of TAPs

at which a TSTA entity can make concurrent Fast Transition

Reservations.  A value of zero indicates no limit.”

::= { dot11FastTransitionConfigEntry 6 }

dot11FTResourceRequestTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of time units (TUs)

that this TAP entity shall wait for a Resource Response from a

Target TAP to which this TAP sent a corresponding Resource Request."

::= { dot11FastTransitionConfigEntry 7 }

dot11FTPendingResourceRequestLimit OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the maximum number of pending

Resource Requests from a TSTA that this TAP entity shall support.”

::= { dot11FastTransitionConfigEntry 8 }

dot11FTR0KeyHolderID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(16))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the PMK-R0 identifier of this entity.”

::= { dot11FastTransitionConfigEntry 9 }

dot11FTR1KeyHolderID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(16))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the PMK-R1 identifier of this entity.”

::= { dot11FastTransitionConfigEntry 10 }

dot11FTReassociationDeadline OBJECT-TYPE

SYNTAX Unsigned32 (1..4294967295)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of time units (TUs)

that this Target TAP entity shall retain a PTKSA and reserve

any specified resources for a TSTA while waiting for a

reassociation from that TSTA."

::= { dot11FastTransitionConfigEntry 11 }

dot11FTTspecAllocationRequired OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"When this object is set to TRUE, this shall indicate that

one of the TSPECs that follow the RDIE needs to be allocatable/

allocated on the Target TAP for the resource request made by this

TSTA entity to be considered successful.  When this object is set

to FALSE, this shall indicate that the AP should keep processing

the remainder of the request in the RIC even if none of the TSPEC

IEs following the RDIE is allocatable/allocated.”

::= { dot11FastTransitionConfigEntry 12 }

-- ********************************************************************

-- * End of dot11FastTransitionConfig TABLE

-- ********************************************************************

In “Compliance Statements” of Annex D, change the dot11Compliance as follows:

dot11Compliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities

that implement the IEEE 802.11 MIB."

MODULE -- this module

MANDATORY-GROUPS {

dot11SMTbase45,

dot11MACbase, dot11CountersGroup,

dot11SmtAuthenticationAlgorithms,

dot11ResourceTypeID, dot11PhyOperationComplianceGroup }

In “Compliance Statements” of Annex D, change OPTIONAL-GROUPS as follows:

-- OPTIONAL-GROUPS { dot11SMTprivacy, dot11MACStatistics,

--    dot11PhyAntennaComplianceGroup, dot11PhyTxPowerComplianceGroup,

--    dot11PhyRegDomainsSupportGroup,

--    dot11PhyAntennasListGroup, dot11PhyRateGroup,

--    dot11SMTbase3, dot11MultiDomainCapabilityGroup,

--    dot11PhyFHSSComplianceGroup2, dot11RSNAadditions,

--    dot11RegulatoryClassesGroup, dot11FTComplianceGroup }

--

::= { dot11Compliances 1 }

In “Groups - units of conformance” of Annex D, change dot11SMTbase5 as follows:

dot11SMTbase5 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired }

STATUS currentdeprecated

DESCRIPTION

"The SMTbase5 object class provides the necessary support at the

STA to manage the processes in the STA so that the STA may work

cooperatively as a part of an IEEE 802.11 network, when the STA is

capable of multidomain operation. This object group should be

implemented when the multidomain capability option is implemented."

::= { dot11Groups 30 }

In “Groups - units of conformance” in Annex D, insert the following objects after dot11SMTbase5 { dot11Groups 30 }:

dot11FTComplianceGroup OBJECT-GROUP

OBJECTS { dot11FastTransitionEnabled }

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11 MIB

required to manage Fast BSS Transition functionality. Note that

additional objects for managing this functionality are located in

the IEEE 802.11 Fast Transition MIB."

::= { dot11Groups 31 }

dot11SMTbase6 OBJECT-GROUP

OBJECTS { dot11MediumOccupancyLimit,

dot11CFPollable,

dot11CFPPeriod,

dot11CFPMaxDuration,

dot11AuthenticationResponseTimeOut,

dot11PrivacyOptionImplemented,

dot11PowerManagementMode,

dot11DesiredSSID, dot11DesiredBSSType,

dot11OperationalRateSet,

dot11BeaconPeriod, dot11DTIMPeriod,

dot11AssociationResponseTimeOut,

dot11DisassociateReason,

dot11DisassociateStation,

dot11DeauthenticateReason,

dot11DeauthenticateStation,

dot11AuthenticateFailStatus,

dot11AuthenticateFailStation,

dot11MultiDomainCapabilityImplemented,

dot11MultiDomainCapabilityEnabled,

dot11CountryString,

dot11SpectrumManagementImplemented,

dot11SpectrumManagementRequired,

dot11RSNAOptionImplemented,

dot11RegulatoryClassesImplemented,

dot11RegulatoryClassesRequired,

dot11FastTransitionImplemented }

STATUS current

DESCRIPTION

"The SMTbase5 object class provides the necessary support at the

STA to manage the processes in the STA so that the STA may work

cooperatively as a part of an IEEE 802.11 network, when the STA is

capable of multidomain operation. This object group should be

implemented when the multidomain capability option is implemented."

::= { dot11Groups 32 }