IEEE P802.11
Wireless LANs

| Addendum to 1046r0: QoS procedures |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-11-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Rajneesh Kumar | Cisco Systems | 170 West Tasman San Jose, CA 95134 | +1 408 527 6148 | rajneesh@cisco.com |

Abstract

Submission 1046r0 left out the details provided in section 8A.5.5 of draft D0.09. This submission primarily addresses this issue. It also defines a new MLME primitive to complement the text.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

< Append the following after the flowchart figure 121I >

The SME looks at TSPECs and generates MLME.Reservation_local.request that passes down the TSPECs to the MAC. The SME may also send these TSPECs to an external entity such as a back-end QoS module for its consideration; these procedures are beyond the scope of this specification.The MAC shall respond with MLME.Reservation_local.response that will indicate whether the  MAC has accepted the TSPECs or not. The acceptance of a TSPEC by a new target TAP results in resource allocation for a traffic stream at the target TAP.

<Insert this in the draft at appropriate place >

10.3.36.5 MLME-Reservation_local.request

10.3.36.5.1 Function

This primitive is used to enact the security and QoS reservation method with a specified peerMAC entity.

10.3.36.5.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-Reservation_local.request (

peerMACAddress,

Contents of FT Authentication Information Elements

)

10.3.36.5.3 When generated

This primitive is generated by the SME at a Target TAP to request resource reservation from MAC.

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| PeerMacAddress | MAC Address | Any valid invidual MAC address | Specifies the MAC address of the AP that is the intended immediate recipient of the reservation request. |
| Contents of FT Authentication Information Elements | Sequence of Information elements | As defined in 8.5A.8 | The set of information elemnts to be included in Fast Transition Confirm, asdescribed in section 8.5A.8.3. This includes an optional resource reservation request(RIC). |

10.3.36.5.4 Effect of receipt

Upon receipt of this primitive the MAC checks for resource availability and allocates resources as per the request.

10.3.36.6 MLME-Reservation_local.response

10.3.36.6.1 Function

This primitive is used to respond to the reservation request for resources from the SME..

10.3.36.6.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-Reservation_local.indicate (

PeerMACAddress,

Content of FT Authentication Information Elements

)

| Name | Type | Range | Description |
| --- | --- | --- | --- |
| PeerMacAddress | MAC Address | Any valid invidual MAC address | Specifies the MAC address of the AP that is the intended immediate recipient of the reservation request. |
| Contents of FT Authentication Information Elements | Sequence of Information elements | As defined in 8.5A.8 | The set of information elements to be included in Fast Transition Confirm, as described in section 8.5A.8.3. This includes an optional resource reservation request(RIC). |

10.3.36.6.3 When generated

This primitive is generated by MAC in response to the successful reservation_local.

10.3.36.6.4 Effect of receipt

Upon receipt of this primitive, the SME prepares a success or failure response to be sent to the STA via the Current TAP.

<Modify last sentence in 2nd paragraph of 8A.5.6.1 in 1046r0 to>

If present, the TCLAS shall appear after the corresponding TSPEC.

<Remove the last line in 8A.5.6.1>