IEEE P802.11
Wireless LANs

| LB78 Sections 7.3.1.21 and 7.4.5.3/4 Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-NOV-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111NE 25th Ave Hillsboro, OR 97124 | 503-264-0637 | ganesh.venkatesan@intel.com |

Abstract

This document contains normative text changes for letter ballot 78 comments for sections 7.3.1.21, 7.4.5.3 and 7.4.5.4.  This document resolves comments: 366, 367, 425,  743, 796, 797, 1283, 1379, 1380, 1517.

The submission is referenced to draft D3.0 and replaces all editor instructions as well as actual base draft modification.  This text should replace all draft D3.0 text from page 11 lines 2 through 7,  page 44 lines 1 through 27 and page 45 lines 1 through 4.

All new text relative to the base draft is identified in blue color and underlined and all deleted text is identified by strikethrough.

#### 7.3.1.21 Max Transmit Power field

The Max Transmit Power field is a signed integer and is 1 octet in length. When set by an STA, it provides an upper limit, in units of dBm, on the transmit power as measured at the output of the antenna connector to be used by that STA on the current channel. The maximum tolerance for the value reported in Max Transmit Power field shall be ± 5 dBm. The value of the Max Transmit Power field shall be less than or equal to the Max Regulatory Power value for the current channel. The Max Transmit Power field is illustrated in Error: Reference source not found.

#### 7.4.5.3 Link Measurement Request Frame Format

The Link Measurement Request frame uses the Action frame body format and is transmitted by a STA to request another STA to respond with a Link Measurement Report frame to enable measurement of link path loss and estimation of link margin. The format of the frame is shown in Figure k1.

|  | Category | Action | Dialog Token | Transmit Power | Max Transmit Power |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure k1—Link Measurement Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to indicate a Link Measurement Request according to Error: Reference source not found in 7.4.5.

The Dialog Token field shall be set equal to a non-zero value chosen by the STA sending the request to identify the transaction.

The Transmit Power element shall be set to the transmit power used to transmit the frame containing the Link Measurement Request, as described in 7.3.1.22.

The Max Transmit Power element is coded as a signed integer and is 1 octet in length. It provides an shall provide the upper limit, in units of decibels, on the transmit power as measured at the output of the antenna connector to be used by the transmitting STA on the current channel. This element is described in 7.3.1.21.

#### 7.4.5.4 Link Measurement Report Frame Format

The Link Measurement Report frame uses the Action frame body format and is transmitted by a STA in response to a Link Measurement Request frame. The format of the frame is shown in Figure k2.

|  | Category | Action | Dialog Token | TPC Report element | Receive Antenna ID | Transmit Antenna ID | RCPI | RSNI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 | 1 | 1 | 1 | 1 |

Figure k2—Link Measurement Report frame body format

The Category field shall be set to indicate the Radio Measurement category according to Table 19a in 7.3.1.11.

The Action field shall be set to indicate a Link Measurement Report according to Error: Reference source not found in 7.4.5.

The Dialog Token field shall be set equal to the Dialog Token value in the corresponding Link Measurement Request frame.

The TPC Report element shall be set as described in 7.3.2.18.

The Receive Antenna ID field contains the identifying number for the antenna used to receive the corresponding Link Measurement Request frame. Antenna ID is defined in 7.3.2.30.

The Transmit Antenna ID field contains the identifying number for the antenna used to transmit this Link Measurement Report frame. Antenna ID is defined in 7.3.2.30.

RCPI indicates the received channel power of the Beacon, Measurement Pilot or Probe Response frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type, as described in 7.3.2.28.

RSNI indicates the received signal to noise indication for the beacon or probe response frame in dBm, as described in 7.3.2.31.