IEEE P802.11
Wireless LANs

| LB78 Section 7-2 Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-NOV-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tim Olson | Cisco Systems | 170 West Tasman Dr. San Jose, CA 95137 | 408-853-4527 | tolson@cisco.com |

Abstract

This document contains normative text changes for letter ballot 78 comments for section 7.2.  This document resolves comments: 299, 395, 396, 397, 452, 453, 513, 514, 515, 516, 517, 705, 1031, 1032, 1222, 1223, 1224, 1225, 1226, 1227, 1228, 1311, 1391.

The submission is referenced to draft D3.0 and replaces all editor instructions as well as actual base draft modification.  This text should replace all draft D3.0 text from page xx line aa to page yy line bb.

All new text relative to the 802.11 base is identified by underlining and all deleted text is identified by strikethrough.

For reviewing ease any changes from draft D3.0 are highlighted in yellow.  TGk editor please remember to remove the highlighting.

## 7.2 Format of individual frame types

### 7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Change the first paragraph, and table 5 as shown:

The frame body of a management frame of subtype Beacon contains the information shown in Table 5. If the dot11MultiDomainCapabilityEnabled attribute is true, a STA shall include a Country information element in the transmission of Beacon frames. Optionally, the Beacon frame format may also include the information described in either or both of orders 12 and 13. If the information in both orders 12 and 13 are sent, they shall describe the same hopping pattern. Note that the information described in orders 12 and 13 also may be contained in the Probe Response frame.

Table 5—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Country | The Country element shall be present if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 14 | Power Constraint | The Power Constraint element shall be present if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true. |
| 16 | Quiet | Quiet element may be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 18 | TPC Report | TPC Report element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 25 | AP Channel Report | The AP Channel Report element shall be present if dot11RadioMeasurementEnabled is true and there is at least 1 channel to report. |
| 26 | BSS Load | The BSS Load information element shall be present if dot11RadioMeasurementEnabled is true. |
| 27 | Antenna Information | The Antenna Information element shall be present if dot11RadioMeasurementEnabled is true. |
| 2822 | Vendor Specific | One or more vendor specific information elements may appear in this frame. |

#### 7.2.3.4 Association Request frame format

Change the order 6 information field as follows:

Table 7—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |

#### 7.2.3.5 Association Response frame format

Insert a new row into table 8 and update Vendor Specific element order as shown below:

Table 8—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | RCPI | The RCPI information element shall be present if dot11RadioMeasurementEnabled is true. |
| 78 | Vendor Specific | One or more vendor specific information elements may appear in this frame. |

#### 7.2.3.6 Reassociation Request frame format

Change the order 7 information field as follows:

Table 9—Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | Power Capability | The Power Capability element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |

#### 7.2.3.7 Reassociation Response frame format

Insert new row into table 10 and update Vendor Specific element order as follows:

Table 10—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | RCPI | The RCPI information element shall be present if dot11RadioMeasurementEnabled is true. |
| 78 | Vendor Specific | One or more vendor specific information elements may appear in this frame. |

#### 7.2.3.8 Probe Request frame format

Insert new row 5 and update Vendor Specific element order as follows:

Table 11—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 5 | DS Parameter Set | The DS Parameter Set information element shall be present within Probe Request frames generated by STAs using Clause 15, Clause 18, and Clause 19 PHYs with dot11RadioMeasurementEnabled set to true. The DS Parameter Set information element may be present within Probe Request frames generated by STAs using Clause 15, Clause 18, and Clause 19 PHYs with dot11RadioMeasurementEnabled set to false. |
| 56 | Vendor Specific | One or more vendor specific information elements may appear in this frame. |

#### 7.2.3.9 Probe Response frame format

Change the first paragraph as follows:

The frame body of a management frame of subtype Probe Response contains the information shown in Table 12. If the dot11MultiDomainCapabilityEnabled attribute is true, the Probe Response frame contains a Country information element and all information elements identified by the Requested Element IDs of a Request information element. Note that the information returned as a result of a Probe Request frame with a Request information element may include the FH parameters and/or the FH Pattern Table possibly replicating optional elements identified by orders 12 and 13. When a Probe Response frame is returned in response to a Probe Request frame which contained a Request information element, any of the requested elements that are listed in table 12 with a specific ordering shall be included in the Probe Response both in their listed ordered location as specified in table 12 and in the ordered location reserved for the list of requested elements, where the requested elements appear in increasing numerical element ID order.

Change the order 10, 13 and 17 information fields, insert the order 22, 23 and 24 information field and change the starting order of the Requested information elements field in Table 12 as follows:

Table 12—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Country | Included if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 13 | Power Constraint | Shall be included if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true. |
| 16 | Quiet | May be included if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 17 | TPC Report | Shall be included if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 22 | AP Channel Report | Shall be included if dot11RadioMeasurementEnabled is true and there is at least 1 channel to report. |
| 23 | BSS Load | Shall be included if dot11RadioMeasurementEnabled is true. |
| 24 | Antenna Information | Shall be included if dot11RadioMeasurementEnabled is true. |
| 2225-n | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |