IEEE P802.11
Wireless LANs

| Proposed Text for Clause 7.3.2.27 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2001-12-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| bh | Consultant |  | 510-338-1122 | lefko@sbcglobal.net |
| Bernard Aboba | Microsoft |  |  |  |

Abstract

This document is intended to be normative text for clause 7.3.2.27 to be added by the editor.  The intent of the document is to use Microsoft Word’s “track changes” from the tools menu to convey the changes to be made.

The source for the draft amendment was the word version in the group’s directory on the 802.11 web site with a file name of “802.11k-D2.5-redlined.doc”.  An exception to this is the clerical error that occurred in lb73 comment # 355, 575, 1006, 1068, 1273, 1275, 1276, 1277 & 1433.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

#### 7.3.2.27 Neighbor Report element

The format of the Neighbor Report element is shown in Figure k1.

|  |  |  | Zero or more, one for each neighbor |  |
| --- | --- | --- | --- | --- |
|  | Element ID | Length | Neighbor List |  |
| Octets: | 1 | 1 | Variable |  |

Figure k1—Neighbor Report element format

The Element ID field is equal to the Neighbor Report value in Table 20.

The value of Length field is dependent on the number of Neighbor List Entries representing the neighboring APs being reported. Each entry describes an AP and consists of BSSID, BSSID Information, Channel Number, Regulatory Class, PHY Options, and may include the TSF Information field. The minimum value of the Length field is 0 (i.e., with no neighbor APs in the Neighbor Report element).

The Neighbor List comprises of zero or more Neighbor List Entries. See Figure k2.

|  | BSSID | BSSID information | Regulatory Class | Channel Number | PHY Options | TSF Information (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 2 | 1 | 1 | 1 | 4 |

Figure k2—Neighbor list entry format

The BSSID is the BSSID of the BSS being reported. The subsequent fields in the Neighbor List Entry pertain to this BSS.

The BSSID Information field may be used to help determine neighbor BSS transition candidates. It shall be two octets in length and shall contain the subfields as shown in Figure k3.

|  | B0 | B1 | B2 | B3 | B4 | B9 | B10 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AP Reachability |  | Security | Key Scope | Capabilities |  | Reserved |  |
| Bits: | 2 |  | 1 | 1 | 6 |  | 6 |  |

Figure k3—BSSID information field

The AP Reachability field indicates whether the AP identified by this BSSID is reachable by the STA that requested the Neighbor Report.  For example the AP identified by this BSSID is reachable for the exchange of preauthentication frames as described in clause 8.4.6.1. . The values are shown in Table k1.

Table k1—Reachability field

| Value | Reachability | Usage |
| --- | --- | --- |
| 0 | Reserved | Nor Used |
| 1 | Not Reachable | A station sending a preauthentication frame to the BSSID will not receive a response even if the AP represented by the BSSID is capable of preauthentication. |
| 10 | Unknown | The AP is unable to determine if the value Reachable or Not Reachable is to be returned. |
| 11 | Reachable | The station sending a preauthentication frame to the BSSID can receive a response from an AP that is capable of preauthentication. |

The Security bit, if se to 1 it, indicates that the AP identified by this BSSID supports the current security association of the STA . If the bit is set to 0, it indicates that either the APcan only provde less security than the current association, as defined by clause 8, or the information is not available to the AP at this time.

The Key Scope bit, when set to 1, indicates the AP represented by this BSSID has the same authenticator as the AP sending the report. If this bit is se to 0t, it indicates a distinct authenticator or the information is not available.

The Capabilities Subfield contains selected capability information for the AP represented by this BSSID. The bit fields within this subfield shall have the same meaning and be set equal to the equivalent bits within the Capability Information field being sent in the beacons by the AP being reported. The format of the Capabilities subfield is as in Figure k4.

|  | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Spectrum Management | QoS | APSD | Radio Measurement | Delayed Block Ack | Immediate Block Ack |
| Bits: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure k4—Capabilities Subfield

Bits 10 – 15 are reserved and shall be set to 0 on transmission and ignored on reception.

Regulatory Class contains an enumerated value from Error: Reference source not found specifying the frequency band in which the Current Channel is valid.

Channel Number indicates the last known operating channel of the AP represented by the BSSID in this neighbor list entry.

PHY Options field contains the Condensed PHY type and TSF informationFlag as shown in the Figure k5.

|  | B0 B6 | B7 |
| --- | --- | --- |
|  | Condensed PHY Type | TSF information Flag |
| Bits: | 7 | 1 |

Figure k5—PHY Options field

Condensed PHY Type indicates the PHY type of the AP being reported. .It shall have an integer value between 0 and 127 coded according to the value of dot11PHYType

TSF information Flag is a one bit field. When it is set to 1 it indicates that a TSF information field is present for this Neighbor List Entry. When set to 0 it indicates that a TSF information field is not present for this Neighbor List Entry. The Neighbor Report may have the TSF information Flag set in some Neighbor List Entries and not set in others.

The TSF Information Field is 4 octets long and contains TSF Offset and Beacon Interval subfields.

|  | TSF Offset | Beacon Interval |
| --- | --- | --- |
| Octets: | 2 | 2 |

Figure k6—TSF information field format

The TSF Offset subfield is 2 octets long and contains the neighbor AP’s TSF timer offset. This is the time difference, in TU units, between the serving AP and a neighbor AP. This offset is given modulo the neighbor AP’s Beacon Interval and rounded to the nearest TU boundary.

The Beacon Interval field is the beacon interval of the Neighbor AP being reported. This field is defined in clause 7.3.1.3 and illustrated in Figure 26.

#### 7.4.5.5 Neighbor Report Request frame format

The Neighbor Report Request frame uses the Action frame body format and is transmitted by a STA requesting information in the Neighbor Report about neighboring AP’s. The format of the Neighbor Report Request frame body is shown in Figure k7.

|  | Category | Action | Dialog Token | Neighbor Report Request Types | SSID element |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable |

Figure k7—Neighbor Report Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to the value indicating Neighbor Report Request, as specified in Error: Reference source not found in 7.4.5.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the measurement request to identify the request/report transaction.

The Neighbor Report Request Types field shall be one octet in length and shall contain the subfields as shown in Error: Reference source not found.

|  | B0 | B1 B7 |
| --- | --- | --- |
|  | Neighbor TSFinformation Request | Reserved |
| Bits: | 1 | 7 |

Figure k8—Neighbor Report Request Types Subfield

Neighbor TSF information Request – This bit is set to 1 to request information be provided in neighbor list entires if available. When this bit is set to 0 the TSF Information field shall not be included in any neighbor list entries.

All other bits are reserved and shall be set to 0 on transmission and ignored on reception.

The SSID element is defined in 7.3.2.1. It’s inclusion shall indicate a request for a neighbor list for the specified SSID in the SSID Element. The absence of a SSID element indicates neighbor report for the current ESS.

### 11.12.3 Receiving a Neighbor Report

An AP receiving a Neighbor Report Request shall respond with a Neighbor Report Response frame containing zero or more Neighbor Report elements. If SSID elements are specified in the corresponding Neighbor Report Request frame, the Neighbor Report element shall only contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that are members of the current ESS or ESSs identified by the SSID elements contained within the Neighbor Report Request. If the SSID element is omitted the Neighbor Report element shall contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that belong to the same ESS as the requesting STA. If there are no list entries available the AP shall send a Neighbor Report Response with no Neighbor List Entries.

A serving AP shall include a TSF Information field in the Neighbor List Entry only if it is able to guarantee an accumulated error of ±1.5 TU or better on the TSF Offset  subfield