IEEE P802.11
Wireless LANs

| Normative Text for the 802.11k Measurement Frame Proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-01-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Steve Emeott | Motorola, Inc. | 1301 E Algonquin Road, Schaumburg, IL 60196 | 847-576-8268 | Steve.Emeott@motorola.com |
| Tim Wilson | Motorola, Inc. | 1301 E Algonquin Road, Schaumburg, IL 60196 | 847-576-5296 | Tim.Wilson@motorola.com |
| Floyd Simpson | Motorola, Inc. | 8000 W Sunrise Boulevard, Plantation, FL 33322 | 954-723-5269 | Floyd.Simpson@motorola.com |
| Stephen Wang | Motorola, Inc. | 8000 W Sunrise Boulevard, Plantation, FL 33322 | 954-723-8124 | Stephen.Wang@motorola.com |

Abstract

This document presents normative text for the 802.11k Measurement Frame Proposal (doc# 04/1425r0), which was presented in the IEEE 802.11 November 2004 plenary meeting in San Antonio, TX. The term “Measurement Frame” is renamed as “Measurement Pilot Frame” in this document. The proposal in this document addresses comments 466 and 940 of LB#71.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 4. Abbreviations and acronyms

Insert the following new definitions in alphabetical order, renumbering as necessary:

TMPTT		target measurement pilot transmission time

# 7. Frame formats

## 7.1 MAC frame formats

### 7.1.3 Frame fields

#### 7.1.3.1 Frame Control field

##### 7.1.3.1.2 Type and Subtype fields

Change the contents of Table 1 as shown:

| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 0000 | Association request |
| 00 | Management | 0001 | Association response |
| 00 | Management | 0010 | Reassociation request |
| 00 | Management | 0011 | Reassociation response |
| 00 | Management | 0100 | Probe request |
| 00 | Management | 0101 | Probe response |
| 00 | Management | 0110 | Measurement Pilot |
| 00 | Management | 0111 | Reserved |
| 00 | Management | 1000 | Beacon |
| 00 | Management | 1001 | Announcement traffic indication message (ATIM) |
| 00 | Management | 1010 | Disassociation |
| 00 | Management | 1011 | Authentication |
| 00 | Management | 1100 | Deauthentication |
| 00 | Management | 1101 | Action |
| 00 | Management | 11010-1111 | Reserved |
| 01 | Control | 0000-100111 | Reserved |
| 01 | Control | 1000 | Block Acknowledgement Request (BlockAckReq) |
| 01 | Control | 1001 | Block Acknowledgement (BlockAck) |
| 01 | Control | 1010 | Power Save Poll (PS-Poll) |
| 01 | Control | 1011 | Request To Send (RTS) |
| 01 | Control | 1100 | Clear To Send (CTS) |
| 01 | Control | 1101 | Acknowledgement (ACK) |
| 01 | Control | 1110 | Contention-Free (CF)-End |
| 01 | Control | 1111 | CF-End + CF-Ack |
| 10 | Data | 0000 | Data |
| 10 | Data | 0001 | Data + CF-Ack |
| 10 | Data | 0010 | Data + CF-Poll |
| 10 | Data | 0011 | Data + CF-Ack + CF-Poll |
| 10 | Data | 0100 | Null function (no data) |
| 10 | Data | 0101 | CF-Ack (no data) |
| 10 | Data | 0110 | CF-Poll (no data) |
| 10 | Data | 0111 | CF-Ack + CF-Poll (no data) |
| 10 | Data | 1000-1111 | QoS DataReserved |
| 10 | Data | 1001 | QoS Data + CF-Ack |
| 10 | Data | 1010 | QoS Data + CF-Poll |
| 10 | Data | 1011 | QoS Data + CF-Ack + CF-Poll |
| 10 | Data | 1100 | QoS Null (no data) |
| 10 | Data | 1101 | Reserved |
| 10 | Data | 1110 | QoS CF-Poll (no data) |
| 10 | Data | 1111 | QoS CF-Ack + CF-Poll (no data) |
| 11 | Reserved | 0000-1111 | Reserved |

## 7.2 Format of individual frame types

### 7.2.3 Management frames

Insert the following subclause after subclause 7.2.3.9:

#### 7.3.2.10 Measurement Pilot frame format

The frame body of a management frame of subtype Measurement Pilot contains the information shown in Table Tx0.

Table Tx0 – Measurement Pilot frame body

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Timestamp |  |
| 2 | Measurement Pilot Interval |  |
| 3 | Capability Information |  |
| 4 | Country String |  |
| 5 | Max Regulatory Power |  |
| 6 | AP Max Power |  |
| 7 | AP Power Used |  |
| 8 | AP Noise Floor |  |
| 9 | DS Parameter Set | The DS Parameter Set information element is present within Measurement Pilot frames generated by STAs using direct sequence PHYs. |

## 7.3 Management frame body components

### 7.3.1 Fixed fields

Insert the following subclauses with the figures included therein, renumbering as necessary:

#### 7.3.1.18 Country String

The Country String field is 3 octets in length.  The STA shall set this field to the value contained in the dot11CountryString attribute.  The Country String field is illustrated in Figure Fx00.

|  |  | B0 | B23 |
| --- | --- | --- | --- |
|  | Country String |  |  |
| Octets: | 3 |  |  |

Figure Fx00—Country String fixed field

#### 7.3.1.19 Measurement Pilot Interval field

The Measurement Pilot Interval field represents the number of time units (TUs) between target measurement pilot transmission times (TMPTTs). The length of the Measurement Pilot Interval field is 2 octets. The Measurement Pilot Interval field is illustrated in Figure Fx0.

|  |  | B0 | B15 |
| --- | --- | --- | --- |
|  | Measurement Pilot Interval |  |  |
| Octets: | 2 |  |  |

Figure Fx0—Measurement Pilot Interval fixed field

#### 7.3.1.20 Max Regulatory Power field

The Max Regulatory Power field is a signed number and is 1 octet in length. It shall indicate the maximum power, in units of dBm, a STA is allowed by the regulatory authority to transmit on the current channel. The Max Regulatory Power field is illustrated in Figure Fx1.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | Max Regulatory Power |  |  |
| Octets: | 1 |  |  |

Figure Fx1—Max Regulatory Power fixed field

#### 7.3.1.21 Max Transmit Power field

The Max Transmit Power field is a signed number and is 1 octet in length. When set by an STA, it provides an upper limit, in units of dBm, on the transmit power to be used by that STA on the current channel.  The value of the Max Transmit Power field shall be less than or equal to the Max Regulatory Power value for the current channel.   The AP Max Power field is illustrated in Figure Fx2.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | AP Max Power |  |  |
| Octets: | 1 |  |  |

Figure Fx2—AP Max Power fixed field

#### 7.3.1.22 Transmit Power Used field

The Transmit Power Used field is a signed number and is 1 octet in length. It shall be less than or equal to the Max Transmit Power and indicates the actual power used, in units of dBm, by a STA when transmitting the frame containing the Transmit Power Used field.  The AP Power Used field is illustrated in Figure Fx3.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | AP Power Used |  |  |
| Octets: | 1 |  |  |

Figure Fx3—AP Power Used fixed field

#### 7.3.1.23 AP Noise Floor field

The AP Noise Floor field is 1 octet in length. It shall indicate the noise floor value, in units of dBm below 0 dBm and be set to the value of dot11APNoiseFloor. The AP Noise Floor field is illustrated in Figure Fx4.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | AP Noise Floor |  |  |
| Octets: | 1 |  |  |

Figure Fx4—AP Noise Floor fixed field

# 10. Layer management

## 10.3 MAC sublayer management entity (MLME) service access point (SAP) interface

Insert the following subclauses with the tables included therein, renumbering as necessary:

### 10.3.17 Link Margin request

This set of primitives allows STA to retrieve uplink and downlink link margins as computed in 11.5.9.

#### 10.3.17.1 MLME-LINKMARGIN.request

##### 10.3.17.1.1 Function

This set of primitives allows STA to retrieve uplink and downlink link margins as computed in 11.5.9.

##### 10.3.17.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-LINKMARGIN.request 	(

Link Margin Category

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Link Margin Category | Enumeration | ULM or DLM or ALL | Differentiates between uplink link margin and downlink link margin request. Shall only be present if dot11MeasurementPilotEnabled is true. |

##### 10.3.17.1.3 When generated

This primitive is generated by the SME to request that a Link Margin report, which contains link margins computed base on information received in Measurement Pilot frame, be sent to the SME upper layer application.

##### 10.3.17.1.4 Effect of receipt

On receipt of this primitive, the MLME shall construct a Link Margin Report frame and send it back to the SME’s upper layer interface.

#### 10.3.17.2 MLME-LINKMARGIN.report

##### 10.3.17.2.1 Function

This primitive reports the result of the Link Margin request.

##### 10.3.17.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-LINKMARGIN.report 	(

ResultCode,

Link Margin Category,

Uplink Link Margin,

Downlink Link Margin

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, LM INFO UNAVAILABLE | Reports the outcome of a Link Margin request |
| Link Margin Category | Enumeration | ULM or DLM or ALL | Differentiates between uplink link margin and downlink link margin computation. Shall only be present if dot11MeasurementPilotEnabled is true. |
| Uplink Link Margin | Integer | 0 – 255 | Uplink link margin computed based on information received in Measurement Pilot frame. Shall only be present if dot11MeasurementPilotEnabled is true. |
| Downlink Link Margin | Integer | 0 – 255 | Downlink link margin computed based on information received in Measurement Pilot frame. Shall only be present if dot11MeasurementPilotEnabled is true. |

##### 10.3.17.2.3 When generated

This primitive is generated by the MLME when the Link Margin report is ready.

##### 10.3.17.2.4 Effect of receipt

The SME upper layer application receives the Link Margin report it has requested.

# 11. MAC sublayer management entity

Change subclause 11.5 as follows:

## 11.5 TPC procedures

ERC/DEC/(99)23 requires RLANs operating in the 5GHz band to use transmitter power control, involving specification of a regulatory maximum transmit power and a mitigation requirement for each allowed channel, to reduce interference with satellite services. This Standard describes such a mechanism, referred to as Transmit Power Control (TPC).

This clause describes TPC procedures that may be used to satisfy these and similar future regulatory requirements in Europe. The procedures may also satisfy comparable needs in other regulatory domains and other frequency bands and may be useful for other purposes, such as reduction of interference, range control and reduction of power consumption.

STAs shall use the TPC procedures defined in this clause if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. dot11SpectrumManagementRequired shall be set true when regulatory authorities require TPC. It may also be set true in other circumstances. The TPC procedures provide for the:

- Association of STAs with an AP in a BSS based on the STA’s power capability (11.5.1).

- Specification of regulatory and local maximum transmit power levels for the current channel (11.5.2).

- Selection of a transmit power for each transmission in a channel within constraints imposed by regulatory and local requirements (11.5.3).

- Measurement of link path loss and estimation of link margin (11.5.4).

- Adaptation of transmit power based on a range of information, including path loss and link margin estimates (11.5.4).

For the purposes of TPC:

- A STA with dot11SpectrumManagementRequired set equal to true shall not operate in a BSS or IBSS unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Beacons, Measurement Pilot frames, and Probe Responses received from other STAs in the BSS or IBSS, with the following exception.

- A STA may operate when the Spectrum Management bit is set equal to 0 if the STA can determine that it is in a regulatory domain that does not require TPC, or can ensure that it will meet regulatory requirements even if TPC is not employed. Potential methods for determining the regulatory domain include receiving a country indication in the beacon, Measurement Pilot frame, user confirmation, or configuration information within the device. Potential methods to ensure regulations are met even if TPC is not employed include using a transmit power that is below the legal maximum (including any mitigation factor).

- A STA shall set dot11SpectrumManagementRequired equal to true before associating with a BSS or IBSS in which the Spectrum Management bit is set equal to 1 in the Capability Information field in Beacons, Measurement Pilot frames, and Probe Responses received from the BSS or IBSS.

- APs may allow association of devices that do not have the Spectrum Management bit set equal to 1 in the Capability Information field in Association and Reassociation Requests received from the STA to account for the existence of legacy devices that do not support TPC but do meet regulatory requirements.

Change subclause 11.5.2 as follows:

### 11.5.2 Specification of regulatory and local maximum transmit power levels

A STA shall determine a regulatory maximum transmit power for the current channel. The STA shall use the minimum of the following:

- Any regulatory maximum transmit power received in a Country element from the AP in its BSS or another STA in its IBSS and

- Any regulatory maximum transmit power for the channel in the current regulatory domain received in the Max Regulatory Power field from the AP in its BSS or known by the STA from other sources.

A STA shall determine a local maximum transmit power for the current channel. The STA shall use the minimum of the following:

- Any local maximum transmit power received in the combination of a Country element and a Power Constraint element from the AP in its BSS or another STA in its IBSS and

- Any local maximum transmit power for the channel regulatory domain known by the STA from other sources.

Any calculation of the local maximum transmit power for the channel shall ensure the mitigation requirements for the channel in the current regulatory domain can be satisfied. The conservative approach is to set the local maximum transmit power level equal to the regulatory maximum transmit power level minus the mitigation requirement. However, it may be possible to satisfy the mitigation requirement using a higher local maximum transmit power level. A lower local maximum transmit power level may be used for other purposes (e.g., range control, reduction of interference).

The regulatory and local maximum transmit powers may change in a STA during the life of a BSS. However, network stability should be considered when deciding how often or by how much these maximums are changed. The regulatory and local maximum transmit powers shall not change during the life of an IBSS.

An AP in a BSS and a STA in an IBSS shall advertise the regulatory maximum transmit power for the current channel in Beacon frames and Probe Response frames using a Country element. An AP in a BSS and a STA in an IBSS shall advertise the local maximum transmit power for the current channel in Beacon frames and Probe Response frames using the combination of a Country element and a Power Constraint element.

When dot11MeasurementPilotEnabled is set to true, an AP in a BSS shall advertise the regulatory maximum transmit power for the current channel in Measurement Pilot frames using a Max Regulatory Power field.

Where TPC is being used for radio resource measurement without spectrum management, the inclusion of a Power Constraint element in Beaon and Probe Response frames shall be optional.

Insert new subclause 11.9, renumbering as necessary:

## 11.9 Measurement Pilot frame generation and usage

### 11.9.1 Measurement Pilot frame generation by an AP

If dot11MeasurementPilotEnabled is true, an AP shall maintain a Measurement Pilot frame generation function and transmit Measurement Pilot frames according to the aMeasurementPilotPeriod attribute within the AP. This defines a series of TMPTTs exactly aMeasurementPilotPeriod apart. A TMPTT arrives when the AP’s local TSF timer (in s) modulo the Measurement Pilot Frame Interval equals zero. At each TMPTT, the AP shall schedule a Measurement Pilot frame as the next frame for transmission. If the medium is determined by the carrier-sense mechanism (see 9.2.1) to be unavailable, the AP shall delay the actual transmission of a Measurement Pilot frame according to the basic medium access rules specified in Clause 9. In case a TBTT collides with a TMPTT, the AP shall first transmit beacon and then transmit the Measurement Pilot frame.

An AP shall transmit Measurement Pilot frames to the broadcast address. An AP shall not retransmit or buffer Measurement Pilot frames for power save reasons. Once started, the AP shall maintain Measurement Pilot frame transmissions for the life of the BSS.

### 11.9.2 Link margin calculation (informative)

STA may calculate link margin with information received in measurement pilot frames, use it to assess the current link condition, and assist in roaming decision. Link margin may be calculated as follows:

Let RCPIMaxPwr representing the RCPI that would have been received by a STA in a downlink frame if the frame were sent with the Max Transmit Power. Further assuming that STA knows “STA Noise Floor” and “STA Max Transmit Power”.

RCPIMaxPwr = RCPI + (Max Transmit Power – Transmit Power Used)

Downlink Link Margin (DLM) = RCPIMaxPwr – STA Noise Floor

Uplink Link Margin (ULM) = RCPIMaxPwr – (Max Transmit Power – STA Max Transmit Power) – AP Noise Floor