IEEE P802.11
Wireless LANs

| Joint Proposal: FRCC Compliance |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | Eldad.perahia@intel.com |

Abstract

This document contains the FRCC Compliance statement relating to the 802.11 TGn Joint Proposal Technical Specification.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

| Additional Author(s): |  |  |  |  |
| --- | --- | --- | --- | --- |
| Name | Company | Address | Phone | email |
| Bill Abbott | TI |  |  | w-abbott@ti.com |
| Santosh Abraham | Qualcomm |  |  | sabraham@qualcomm.com |
| Tomoko Adachi | Toshiba |  |  | tomo.adachi@toshiba.co.jp |
| Dmitry Akhmetov | Intel |  |  | Dmitry.Akhmetov@intel.com |
| Tsuguhide Aoki | Toshiba |  |  | tsuguhide.aoki@toshiba.co.jp |
| Yusuke Asai | NTT |  |  | asai.yusuke@lab.ntt.co.jp |
| Geert Awater | Airgo |  |  | awater@airgonetworks.com |
| David Bagby | Sanyo |  |  | david.bagby@ieee.org |
| Gal Basson | Intel |  |  | gal.basson@intel.com |
| Anuj Batra | TI |  |  | batra@ti.com |
| John Benko | France Telecom |  |  | john.benko@francetelecom.com |
| Bjorn Bjerke | Qualcomm |  |  | bbjerke@qualcomm.com |
| Jerry Chang | Ralink |  |  | jerry_chang@ralinktech.com.tw |
| Jerry Chang | Realtek |  |  | jerrychang@realtek.com.tw |
| John Chang | Ralink |  |  | john_chang@ralinktech.com.tw |
| Jeng-Hong Chen | Winbond |  |  | jhchen2@winbond.com |
| Stephen Chen | Zydas |  |  | stephen@zydas.com.tw |
| Yi-Ming Chen | Winbond |  |  | ymchen0@winbond.com |
| Pei-ju Chiang | Realtek |  |  | pjchiang@realtek.com.tw |
| Francois Chin | Infocomm |  |  | chinfrancois@i2r.a-star.edu.sg |
| Woon Hau Chin | Infocomm |  |  | whchin@i2r.a-star.edu.sg |
| Emily Chou | Realtek |  |  | sychou@realtek.com.tw |
| Keith Chugg | Trellisware |  |  | kchugg@trellisware.com |
| Brian Classon | Motorola |  |  | Brian.Classon@motorola.com |
| Sean Coffey | Realtek |  |  | coffey@realtek-us.com |
| Gabriella Convertino | ST |  |  | Gabriella.Convertino@st.com |
| Marc de Courville | Motorola |  |  | marc.de.courville@motorola.com |
| Rolf De Vegt | Airgo |  |  | rolf@airgonetworks.com |
| Franz Dielacher | Infineon |  |  | franz.dielacher@infineon.com |
| Yoshiharu Doi | Sanyo |  |  | doi@gf.hm.rd.sanyo.co.jp |
| Takagi Eiji | Airgo |  |  | eijitakagi@airgonetworks.com |
| Leonid Epstein | Metalink |  |  | leonide@metalinkbb.com |
| Mustafa Eroz | HNS |  |  | meroz@hns.com |
| Stefan Fechtel | Infineon |  |  | stefan.fechtel@infineon.com |
| Paul Feinberg | Sony |  |  | Paul.Feinberg@am.sony.com |
| Guido Frederiks | Airgo |  |  | guido@airgonetworks.com |
| Takashi Fukagawa | Panasonic |  |  | fukagawa.takashi@jp.panasonic.com |
| Patrick Fung | Infocomm |  |  | fungpatrick@i2r.a-star.edu.sg |
| Edoardo Gallizio | ST |  |  | edoardo.gallizio@st.com |
| James Gardner | Airgo |  |  | jamesgardner@airgonetworks.com |
| Monisha Ghosh | Philips |  |  | monisha.ghosh@philips.com |
| Jeremy Gosteau | Motorola |  |  | jeremy.gosteau@motorola.com |
| Sudheer Grandhi | InterDigital |  |  | sudheer.grandhi@interdigital.com |
| Paul Gray | Trellisware |  |  | pgray@trellisware.com |
| Daqing Gu | Mitsubishi |  |  | dgu@merl.com |
| Emre Gunduzhan | Nortel |  |  | egunduzh@nortel.com |
| Jin-Meng Ho | TI |  |  | jinmengho@ti.com |
| Dale Hocevar | TI |  |  | hocevar@ti.com |
| Muhammad Ikram | TI |  |  | mzi@ti.com |
| Yasuhiko Inoue | NTT |  |  | yinoue@ansl.ntt.co.jp |
| Kaz Ishida | Qualcomm |  |  | kishida@qualcomm.com |
| Takashi Ishidoshiro | Buffalo Inc. |  |  | doshir@melcoinc.co.jp |
| Lakshmi Iyer | HNS |  |  | liyer@hns.com |
| Eric Jacobson | Intel |  |  | eric.a.jacobsen@intel.com |
| Yuh-Ren Jauh | Zydas |  |  | yrjauh@zydas.com.tw |
| Taehyun Jeon | ETRI |  |  | thjeon@etri.re.kr |
| VK Jones | Airgo |  |  | vkjones@airgonetworks.com |
| Naveen Kakani | Nokia |  |  | naveen.kakani@nokia.com |
| Srinivas Kandala | Airgo |  |  | srini@airgonetworks.com |
| Assaf Kasher | Intel |  |  | assaf.kasher@intel.com |
| Masato Kato | Buffalo Inc. |  |  | mkato@melcoinc.co.jp |
| John Ketchum | Qualcomm |  |  | johnk@qualcomm.com |
| Pansop Kim | Winbond |  |  | pkim@winbond.com |
| Youngsoo Kim | Samsung |  |  | kimyoungsoo@samsung.com |
| Guenter Kleindl | Siemens AG |  |  | guenter.kleindl@siemens.com |
| Kiyotaka Kobayashi | Panasonic |  |  | kobayashi.kiyotaka@jp.panasonic.com |
| Cenk Kose | Conexant |  |  | cenk_kose@conexant.com |
| Takushi Kunihiro | Sony |  |  | kuni@wcs.sony.co.jp |
| Edwin Kwon | Samsung |  |  | cy.kwon@samsung.com |
| Patrick Labbe | Motorola |  |  | patrick.labbe@motorola.com |
| Dongjun (DJ) Lee | Samsung |  |  | djthekid.lee@samsung.com |
| Lin-Nan Lee | HNS |  |  | llee@hns.com |
| Sheng Lee | Ralink |  |  | shenghlee@ralinktech.com |
| Sok-kyu Lee | ETRI |  |  | sk-lee@etri.re.kr |
| Zhongding Lei | Infocomm |  |  | leizd@i2r.a-star.edu.sg |
| Joseph Levy | InterDigital |  |  | joseph.levy.interdigital.com |
| Pen Li | Philips |  |  | pen.li@philips.com |
| Yuan Li | Infocomm |  |  | liyuan@i2r.a-star.edu.sg |
| Ying-Chang Liang | Infocomm |  |  | ycliang@i2r.a-star.edu.sg |
| Kevin Liao | Ralink |  |  | kevin_liao@ralink.com.tw |
| Isaac Lim | Panasonic |  |  | isaac.limwl@sg.panasonic.com |
| Alfred Lin | Winbond |  |  | halin@winbond.com |
| Albert Liu | Infineon |  |  | Albert.Liu@infineon.com |
| Der-Zheng Liu | Realtek |  |  | dzliu@realtek.com.tw |
| Michael Livshitz | Nortel |  |  | livshitz@nortel.com |
| Peter Loc | Marvell |  |  | ploc@marvell.com |
| Peter Lojko | Qualcomm |  |  | c_plojko@qualcomm.com |
| Hui-Ling Lou | Marvell |  |  | hlou@marvell.com |
| Siti Massimiliano | ST |  |  | massimiliano.siti@st.com |
| Adina Matache | Marvell |  |  | amatache@marvell.com |
| Laurent Mazet | Motorola |  |  | laurent.mazet@crm.mot.com |
| Darren McNamara | Toshiba |  |  | Darren.McNamara@toshiba-trel.com |
| Irina Medvedev | Qualcomm |  |  | irinam@qualcomm.com |
| Arnaud Meylan | Qualcomm |  |  | ameylan@qualcomm.com |
| Morgan Miki | Sharp |  |  | miki@slab.tnr.sharp.co.jp |
| Seungwook Min | ETRI |  |  | swmin@etri.re.kr |
| Andy Molisch | Mitsubishi |  |  | molisch@merl.com |
| Mike Moreton | ST |  |  | mike.moreton@st.com |
| Yuichi Morioka | Sony |  |  | morioka@wcs.sony.co.jp |
| Markus Muck | Motorola |  |  | markus.muck@motorola.com |
| Aon Mujtaba | Agere |  |  | Mujtaba@agere.com |
| Yukimasa Nagai | Mitsubishi |  |  | yuki-n@isl.melco.co.jp |
| Seigo Nakao | Sanyo |  |  | snakao@gf.hm.rd.sanyo.co.jp |
| Sanjiv Nanda | Qualcomm |  |  | snanda@qualcomm.com |
| Chiu Ngo | Samsung |  |  | cngo@sisa.samsung.com |
| Huaning Niu | Samsung |  |  | huaning.niu@samsung.com |
| Ivan Oakes | ST |  |  | ivan.oakes@st.com |
| Yoshihiro Ohtani | Sharp |  |  | ohtani@slab.tnr.sharp.co.jp |
| Takeshi Onizawa | NTT |  |  | onizawa@ansl.ntt.co.jp |
| Job Oostveen | Philips |  |  | job.oostveen@philips.com |
| Fabio Osnato | ST |  |  | fabio.osnato@st.com |
| Pratima Pai | ST |  |  | Pratima.Pai@st.com |
| Thomas Pare | Ralink |  |  | tpare@ralinktech.com |
| Jean-Noel Patillon | Motorola |  |  | jean-noel.patillon@motorola.com |
| Wei-Chung Peng | Winbond |  |  | WCPeng0@winbond.com |
| Eldad Perahia | Intel |  |  | eldad.perahia@intel.com |
| Jim Petranovich | Conexant |  |  | jim.petranovich@conexant.com |
| Neeraj Poojary | TI |  |  | npoojary@ti.com |
| Aleksandar Purkovic | TI |  |  | apurkovic@ti.com |
| Ali Raissinia | Airgo |  |  | AliRaissinia@AirgoNetworks.com |
| Sthanunathan Ramakrishnan | TI |  |  | sthanu@ti.com |
| Sridhar Ramesh | TI |  |  | rsridhar@india.ti.com |
| Jon Rosdahl | Samsung |  |  | jon.rosdahl@partner.samsung.com |
| Stephanie Rouquette-Leveil | Motorola |  |  | stephanie.rouquette@motorola.com |
| Mike Rude | Metalink |  |  | mike@metalinkbb.com |
| Vincenzo Scarpa | ST |  |  | Vincenzo.Scarpa@st.com |
| Erik Schylander | Philips |  |  | erik.schylander@philips.com |
| Michael Seals | Conexant |  |  | michael.seals@conexant.com |
| Huai-Rong Shao | Samsung |  |  | hr.shao@samsung.com |
| Steve Shellhammer | Qualcomm |  |  | sshellha@qualcomm.com |
| Masaaki Shida | Hitachi |  |  | shida@crl.hitachi.co.jp |
| Takashi Shono | Intel |  |  | takashi.shono@intel.com |
| Sebastien Simoens | Motorola |  |  | simoens@motorola.com |
| Matt Smith | Atheros |  |  | msmith@atheros.com |
| Robert Stacey | Intel |  |  | robert.j.stacey@intel.com |
| Adrian Stephens | Intel |  |  | adrian.p.stephens@intel.com |
| Victor Stolpman | Nokia |  |  | victor.stolpman@nokia.com |
| Sumei Sun | Infocomm |  |  | sunsm@i2r.a-star.edu.sg |
| Shravan Surineni | Qualcomm |  |  | shravans@qualcomm.com |
| Masahiro Takagi | Toshiba |  |  | masahiro3.takagi@toshiba.co.jp |
| Seiichiro Takahashi | Sanyo |  |  | stakahashi@stec.sanyo.com |
| Daisuke Takeda | Toshiba |  |  | daisuke.takeda@toshiba.co.jp |
| Teik-Kheong Tan | Philips |  |  | teik-kheong.tan@philips.com |
| Yasuhiro Tanaka | Sanyo |  |  | y_tanaka@gf.hm.rd.sanyo.co.jp |
| Stephan ten Brink | Realtek |  |  | stenbrink@realtek-us.com |
| Ganesan Thiagarajan | TI |  |  | gana@ti.com |
| Eric Tokubo | Symbol |  |  | eric.tokubo@symbol.com |
| Solomon Trainin | Intel |  |  | solomon.trainin@intel.com |
| David Tung | Ralink |  |  | dtung@ralinktech.com |
| Stefano Valle | ST |  |  | stefano.valle@st.com |
| Richard Van Nee | Airgo |  |  | vannee@airgonetworks.com |
| Nico van Waes | Nokia |  |  | nico.vanwaes@nokia.com |
| Allert van Zelst | Airgo |  |  | allertvanzelst@airgonetworks.com |
| George Vlantis | ST |  |  | george.vlantis@st.com |
| Timothy Wakeley | HP |  |  | tim.wakeley@hp.com |
| Rod Walton | Qualcomm |  |  | rwalton@qualcomm.com |
| Xiaowen Wang | Agere |  |  | xiaowenw@agere.com |
| Mark Webster | Conexant |  |  | mark.webster@conexant.com |
| Menzo Wentink | Conexant |  |  | menzo.wentink@conexant.com |
| Mike Wilhoyte | TI |  |  | wilhoyte@ti.com |
| Yan Wu | Infocomm |  |  | wuyan@i2r.a-star.edu.sg |
| Ariton Xhafa | TI |  |  | axhafa@ti.com |
| Tomoya Yamaura | Sony |  |  | yamaura@wcs.sony.co.jp |
| Eric Yang | InterDigital |  |  | eric.yang@interdigital.com |
| Wen-Chang Yeh | Zydas |  |  | wcyeh@zydas.com.tw |
| Heejung Yu | ETRI |  |  | heejung@etri.re.kr |
| Eldad Zeira | InterDigital |  |  | eldad.zeira@interdigital.com |
| Jin Zhang | Mitsubishi |  |  | jzhang@merl.com |
| Ning Zhang | Atheros |  |  | ning@atheros.com |

# Contents

Contents	6

1	Introduction	7

1.1	Purpose of document	7

1.2	References	7

2	Additional Disclosures (AD)	7

2.1	Coverage of Additional Disclosures	7

2.2	Additional Disclosures Responses	7

2.2.1	AD1 Reference Submission	7

3	Functional Requirements	8

3.1	Coverage of Functional Requirements	8

3.2	Functional Requirement Responses	8

3.2.1	R5 .11g backwards compatibility	8

3.2.2	FR6 Control of support for legacy STA from .11n AP	8

3.2.3	FR7 .11e QoS support	8

4	Comparison Criteria	9

4.1	Comparison Criteria Coverage	9

4.2	Comparison Criteria Responses	10

4.2.1	CC2 Regulatory Compliance	10

4.2.2	CC11 Backward compatibility	10

4.2.3	CC51 Data Rates	11

4.2.4	CC42 Preamble	13

4.2.5	CC51.5 Channelization	16

4.2.6	CC52 Spectral Mask	16

4.2.7	CC80 Required changes to 802.11 PHY	16

# Introduction

## Purpose of document

The TGn Joint Proposal Technical Specification (comprising [3] and [7]) is submitted for consideration under the 802.11 TGn selection process [4] and is hereby declared complete based on its compliance with the functional requirements defined in [5] and supported by the results disclosed herein in accordance with the TGn comparison criteria defined in [6].

## References

[1] IEEE 802 11-06/0067, Joint Proposal PHY Results

[2] IEEE 802 11-05/1266, TGn Joint Proposal MAC Results

[3] IEEE 802 11-05/1102, Joint Proposal PHY Specification

[4] IEEE 802 11-03/665, TGn Selection Procedure

[5] IEEE 802 11-03/813, TGn Functional Requirements

[6] IEEE 802.11-03/814, TGn Comparison Criteria

[7] IEEE 802 11-05/1095, Joint Proposal MAC Specification

# Additional Disclosures (AD)

## Coverage of Additional Disclosures

| Number | Name | Definition | Disclosure |  |
| --- | --- | --- | --- | --- |
| AD1 | Reference submissions | A list of related IEEE submissions, both documents and presentations. |  | Section 2.2.1 |
| AD2 | TCP Model Parameters | Include a reference to the TCP protocol type (e.g. Reno) and the parameter values associated with that protocol used for all MAC simulations. |  | [2, Section 3] |
| AD3 | MAC simulation methodology | Include a description of the simulation methodology used for MAC simulations, including a description of how the PHY and its impairments are modeled. |  | [2, Section 3] |
| AD4 | MAC simulation occupied channel width | For each MAC simulation, report the total channel width occupied. |  | [2, Section 3] |
| AD5 | Justification of low PLR rates achieved | For each application with a PLR < 10^-4, proposal shall justify that the proposed PLR could be met. |  | [2, Section 3] |

## Additional Disclosures Responses

### AD1 Reference Submission

Below is a complete list of IEEE submissions, both documents and presentations, for the TGn joint proposal.

IEEE 802 11-05/1102, Joint Proposal PHY Specification

- PHY technical specification

IEEE 802 11-05/1095, Joint Proposal MAC Specification

- MAC technical specification

IEEE 802 11-05/1161, Joint Proposal PHY Presentation

- PHY detailed presentation

IEEE 802 11-05/1165, Joint Proposal MAC Presentation

- MAC detailed presentation

IEEE 802 11-06/0046, Joint Proposal FRCC Compliance

- FRCC compliance – this document.

IEEE 802 11-05/1266, Joint Proposal MAC Results

- Summary of MAC simulation results

IEEE 802 11-05/1267, Joint Proposal MAC Simulation Methodology

- MAC simulation methodology used to generate the MAC results

IEEE 802 11-05/1268, Joint Proposal MAC1 Simulation Results

- Detailed results from MAC1 simulator

IEEE 802 11-05/1269, Joint Proposal MAC2 Simulation Results

- Detailed results from MAC2 simulator

IEEE 802 11-05/1270, Joint Proposal MAC3 Simulation Results

- Detailed results from MAC3 simulator

IEEE 802 11-06/0067, TGn Joint Proposal PHY Results

- PHY simulation results

# Functional Requirements

## Coverage of Functional Requirements

| Number | Name | Coverage (Yes/No) | Reference |
| --- | --- | --- | --- |
| R1 | Single Link HT rate supported | Yes | [2, Section 2.5, 2.6] |
| R2 | HT rate supported in 20MHz channel | Yes | [2, Section 2.6] |
| R3 | Supports 5GHz bands | Yes | [3, Section 5] |
| R4 | .11a backwards compatibility | Yes | Section 4.2.2 CC11 |
| R5 | .11g backwards compatibility | Yes | Section 3.2.1 |
| R6 | Control of support for legacy STA from .11n AP | Yes | Section 3.2.2 |
| R7 | .11e QoS support | Yes | Section 3.2.3 |
| R8 | Spectral Efficiency | Yes | [2, Section 2.2] |
| R9 | Compliance to PAR | Yes |  |

## Functional Requirement Responses

### R5 .11g backwards compatibility

An .11n STA is capable of operating as a .11g legacy device if it supports operation in that band, if it finds no HT networks available.  It can send and receive legacy format PPDUs as described in [3, Section 3.1].

When operating in a high throughput “mixed” mode, backwards compatibility is provided at the PHY level [3, Section 3.1] essentially by retaining the legacy short and long training fields and the legacy signal field.

Coexistence is managed within the MAC through the definition of an operating mode [7, section 7.2] and rules [7, Sections 7.3, 7.4 and 6.1.3.2] for operation in each operating mode.

Protection mechanisms are defined using both PHY-level ("L-SIG TXOP protection”") [7, section 6.1.3.2] and MAC-level techniques [7, Sections 6.1.3.1 and 7.3.1.6 ].

When operating in the presence of 802.11b clients, MAC-level techniques are used using 802.11b PPDU formats as described in [7, Section 7.2.1].

### FR6 Control of support for legacy STA from .11n AP

The AP may choose to accept or deny an association from any STA according to its capabilities.  This enables the AP to accept or deny legacy STA according to local policy.  No additional standardization of this feature is considered necessary.

### FR7 .11e QoS support

This is mandatory requirement of a HT STA.

The optional 802.11e block ack feature is enhanced by HT, and is a mandatory requirement.

# Comparison Criteria

## Comparison Criteria Coverage

| Number | Name | Mandatory / optional | Coverage (Yes/No) | Reference |
| --- | --- | --- | --- | --- |
| General |  |  |  |  |
| CC2 | Regulatory compliance | Mandatory | Yes | Section 4.2.1 |
| Marketability |  |  |  |  |
| CC3 | List of goodput results for usage models 1,4 and 6. | Mandatory | Yes | [2 , Section 2.2] |
| CC6 | PHY complexity | Optional | No |  |
| CC7 | MAC processing complexity | Optional | No |  |
| Backward Compatibility and Coexistence with Legacy Devices |  |  |  |  |
| CC11 | Backward compatibility with 802.11-1999 (Rev 2003) and 802.11g | Mandatory | Yes | Section 4.2.2 |
| CC15 | Sharing of medium with legacy devices | Mandatory | Yes | [2, Section 2.2] |
| MAC Related |  |  |  |  |
| Performance Measurements at the MAC SAP |  |  |  |  |
| CC18 | HT Usage Models Supported (non QoS) | Mandatory | Yes | [2, Section 2.2] |
| CC19 | HT Usage Models Supported (QoS) | Mandatory | Yes | [2, Section 2.2] |
| CC20 | BSS Aggregate Goodput at the MAC data SAP | Mandatory | Yes | [2, Section 2.2] |
| CC24 | MAC Efficiency | Mandatory | Yes | [2, Section 2.2] |
| CC27 | Throughput / Range | Mandatory | Yes | [2, Section 2.5] |
| CC28 | Throughput / Range in 20MHz | Mandatory | Yes | [2, Section 2.6] |
| MAC Changes |  |  |  |  |
| CC46 | MAC Compatibility and parameters. | Mandatory | Yes | [2, Section 2.7] |
| CC47 | MAC extensions | Mandatory | Yes | [2, Section 2.8] |
| PHY Related |  |  |  |  |
| PHY Rates and Preambles |  |  |  |  |
| CC51 | Data rates | Mandatory | Yes | Section 4.2.3 |
| CC42 | Preambles | Mandatory | Yes | Section 4.2.4 |
| Channelization |  |  |  |  |
| CC51.5 | Channelization | Mandatory | Yes | Section 4.2.5 |
| CC52 | Spectral Mask | Mandatory | Yes | Section 4.2.6 |
| Spectral Efficiency |  |  |  |  |
| CC58 | HT Spectral Efficiency | Mandatory | Yes | [2, Section 2.2] |
| PHY Performance |  |  |  |  |
| CC59 | AWGN PER performance | Mandatory | Yes | [1, Section 2] |
| CC67 | PER performance in non AWGN channels | Mandatory | Yes (set 1) | [1, Section 3] |
| CC67.2 | Offset Compensation | Mandatory | Yes | [1, Section 3] |
| PHY Changes |  |  |  |  |
| CC80 | Required changes to 802.11 PHY | Mandatory | Yes | Section 4.2.7 |

## Comparison Criteria Responses

### CC2 Regulatory Compliance

There are no known problems with regulatory compliance in USA, Europe, and China.

#### Japan Domain

This section refers the following Japanese Regulations.

RE:  Radio Equipment Regulations

RLE: Radio Law Enforcement Regulations

Known problems with Japan domain are as follows:

- Transmit burst length

According to RE: Article 49-20, 49-21, transmit burst length is defined as follows;

  - 2400-2483.5MHz

There is no definition of transmit burst length.

  - 5170, 5190, 5210, 5230MHz

Transmit burst length should be less than 4 milliseconds.

  - 4900-5000MHz, 5030-5090MHz

Transmit burst length should be less than 4 milliseconds.

An .11n device which obtains a type approval for Japan region should keep a 4ms burst length limitation.

An .11n device is also an 802.11e device, and respects the TXOP duration limits defined by 802.11e. An 802.11e AP operating in Japan can ensure that the TXOP duration limits respect the Japanese limits.  The proposal does not modify the 802.11e default TXOP duration values.

- Occupied Bandwidth

According to RE: Article 6, Annex Table-2, occupied bandwidth is defined as follows;

  - 2400-2483.5MHz

    - Occupied bandwidth of transmitting equipment using FH, combination of DS-SS and FH, or combination of OFDM and FH shall not exceed 83.5MHz.

    - Occupied bandwidth of transmitting equipment except (1) shall not exceed 26MHz.

  - 5170, 5190, 5210, 5230MHz

Occupied bandwidth of transmitting equipment which utilizes 5170, 5190, 5210, and 5230MHz frequency band shall not exceed 18MHz.

  - 4900-5000MHz, 5030-5090MHz

Occupied bandwidth of a 5GHz band Wireless Access System shall be as follows;

for 20MHz system: 19.7MHz

for 10MHz system: 9MHz

for 5MHz system: 4.5MHz

An .11n device that obtains a type approval for the Japan region must keep its occupied bandwidth to 18MHz for the 5GHz band, and 26MHz for the 2.4GHz band.

- MIMO (Multiple-Input Multiple-Output)/Multiple Transmit

At this time, regarding the use of MIMO, there is no explicit definition in RE and RLE.

### CC11 Backward compatibility

Backwards compatibility with legacy .11a and .11g OFDM devices is provided at the PHY level.  The preamble begins with a legacy OFDM short training (STF), long training (LTF), and signal field (L-SIG).  Legacy OFDM stations will be able to perform start-of-packet detection, AGC, frequency estimation, timing offset estimation, channel estimation without any degradation.  Furthermore, legacy OFDM stations will be able to decode the legacy signal field and extract the RATE and LENGTH fields.  The RATE and LENGTH fields in the legacy signal field are “spoofed” such that a legacy OFDM RX can defer for the correct period of time.  Further details are described in [3, Sections 3.1, 3.5] and [7, Section 6.1.3.2].

Backward compatibility is managed within the MAC through the definition of an operating mode [7, Section 7.2.1] and rules [7, Sections 7.3 and 7.4] for operation in each operating mode.  Protection mechanisms are defined using both PHY-level ("spoofing") [7, Section 6.1.3.2] and MAC-level techniques [7, Section 6.1.3.1].

When operating in the presence of 802.11b clients, MAC-level techniques are used using 802.11b PPDU formats as described in [7, Section 7.2.2].

### CC51 Data Rates

#### Basic Modes

The following modes are the basic mode set.

|  |  |  |  | Full GI |  |  |  | Half GI |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 20MHz |  | 40MHz |  | 20MHz |  | 40MHz |  |
| MCS Index | Number of spatial streams | Modulation | Coding rate | Rate (Mbps) | Mandatory/ Optional | Rate (Mbps) | Mandatory/ Optional | Rate (Mbps) | Mandatory/ Optional | Rate (Mbps) | Mandatory/ Optional |
| 0 | 1 | BPSK | 1/2 | 6.50 | Mandatory | 13.5 | Optional | 7.22 | Optional | 15 | Optional |
| 1 | 1 | QPSK | 1/2 | 13.00 | Mandatory | 27.0 | Optional | 14.44 | Optional | 30 | Optional |
| 2 | 1 | QPSK | 3/4 | 19.50 | Mandatory | 40.5 | Optional | 21.67 | Optional | 45 | Optional |
| 3 | 1 | 16-QAM | 1/2 | 26.00 | Mandatory | 54.0 | Optional | 28.89 | Optional | 60 | Optional |
| 4 | 1 | 16-QAM | 3/4 | 39.00 | Mandatory | 81.0 | Optional | 43.33 | Optional | 90 | Optional |
| 5 | 1 | 64-QAM | 2/3 | 52.00 | Mandatory | 108.0 | Optional | 57.78 | Optional | 120 | Optional |
| 6 | 1 | 64-QAM | 3/4 | 58.50 | Mandatory | 121.5 | Optional | 65.00 | Optional | 135 | Optional |
| 7 | 1 | 64-QAM | 5/6 | 65.00 | Mandatory | 135.0 | Optional | 72.22 | Optional | 150.0 | Optional |
| 8 | 2 | BPSK | 1/2 | 13.00 | Mandatory for AP Optional for non-AP STA | 27 | Optional | 14.44 | Optional | 30 | Optional |
| 9 | 2 | QPSK | 1/2 | 26.00 | Mandatory for AP Optional for non-AP STA | 54 | Optional | 28.89 | Optional | 60 | Optional |
| 10 | 2 | QPSK | 3/4 | 39.00 | Mandatory for AP Optional for non-AP STA | 81 | Optional | 43.33 | Optional | 90 | Optional |
| 11 | 2 | 16-QAM | 1/2 | 52.00 | Mandatory for AP Optional for non-AP STA | 108 | Optional | 57.78 | Optional | 120 | Optional |
| 12 | 2 | 16-QAM | 3/4 | 78.00 | Mandatory for AP Optional for non-AP STA | 162 | Optional | 86.67 | Optional | 180 | Optional |
| 13 | 2 | 64-QAM | 2/3 | 104.00 | Mandatory for AP Optional for non-AP STA | 216 | Optional | 115.56 | Optional | 240 | Optional |
| 14 | 2 | 64-QAM | 3/4 | 117.00 | Mandatory for AP Optional for non-AP STA | 243 | Optional | 130.00 | Optional | 270 | Optional |
| 15 | 2 | 64-QAM | 5/6 | 130.00 | Mandatory for AP Optional for non-AP STA | 270.0 | Optional | 144.44 | Optional | 300.0 | Optional |
| 16 | 3 | BPSK | 1/2 | 19.50 | Optional | 40.5 | Optional | 21.67 | Optional | 45 | Optional |
| 17 | 3 | QPSK | 1/2 | 39.00 | Optional | 81 | Optional | 43.33 | Optional | 90 | Optional |
| 18 | 3 | QPSK | 3/4 | 58.50 | Optional | 121.5 | Optional | 65.00 | Optional | 135 | Optional |
| 19 | 3 | 16-QAM | 1/2 | 78.00 | Optional | 162 | Optional | 86.67 | Optional | 180 | Optional |
| 20 | 3 | 16-QAM | 3/4 | 117.00 | Optional | 243 | Optional | 130.00 | Optional | 270 | Optional |
| 21 | 3 | 64-QAM | 2/3 | 156.00 | Optional | 324 | Optional | 173.33 | Optional | 360 | Optional |
| 22 | 3 | 64-QAM | 3/4 | 175.50 | Optional | 364.5 | Optional | 195.00 | Optional | 405 | Optional |
| 23 | 3 | 64-QAM | 5/6 | 195.00 | Optional | 405 | Optional | 216.67 | Optional | 450 | Optional |
| 24 | 4 | BPSK | 1/2 | 26.00 | Optional | 54 | Optional | 28.89 | Optional | 60 | Optional |
| 25 | 4 | QPSK | 1/2 | 52.00 | Optional | 108 | Optional | 57.78 | Optional | 120 | Optional |
| 26 | 4 | QPSK | 3/4 | 78.00 | Optional | 162 | Optional | 86.67 | Optional | 180 | Optional |
| 27 | 4 | 16-QAM | 1/2 | 104.00 | Optional | 216 | Optional | 115.56 | Optional | 240 | Optional |
| 28 | 4 | 16-QAM | 3/4 | 156.00 | Optional | 324 | Optional | 173.33 | Optional | 360 | Optional |
| 29 | 4 | 64-QAM | 2/3 | 208.00 | Optional | 432 | Optional | 231.11 | Optional | 480 | Optional |
| 30 | 4 | 64-QAM | 3/4 | 234.00 | Optional | 486 | Optional | 260.00 | Optional | 540 | Optional |
| 31 | 4 | 64-QAM | 5/6 | 260.00 | Optional | 540 | Optional | 288.89 | Optional | 600 | Optional |
| 32 | 1 | BPSK | 1/2 | N/A | N/A | 6 | Optional | N/A | N/A | 6.67 | Optional |

#### Extended Modes

The following modes are the extended mode set.  All the extended modes are optional.

| MCS Index | Code Rate | Stream Modulations |  |  |  | Full GI |  | Half GI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | M0 | M1 | M2 | M3 | 20MHz | 40MHz | 20MHz | 40MHz |
| 33 | 1/2 | 16-QAM | QPSK | OFF | OFF | 39 | 81 | 43.3 | 90 |
| 34 | 1/2 | 64-QAM | QPSK | OFF | OFF | 52.00 | 108.00 | 57.78 | 120.00 |
| 35 | 1/2 | 64-QAM | 16-QAM | OFF | OFF | 65.00 | 135.00 | 72.22 | 150.00 |
| 36 | 3/4 | 16-QAM | QPSK | OFF | OFF | 58.50 | 121.50 | 65.00 | 135.00 |
| 37 | 3/4 | 64-QAM | QPSK | OFF | OFF | 78.00 | 162.00 | 86.67 | 180.00 |
| 38 | 3/4 | 64-QAM | 16-QAM | OFF | OFF | 97.50 | 202.50 | 108.33 | 225.00 |
| 39 | 1/2 | 16-QAM | QPSK | QPSK | OFF | 52.00 | 108.00 | 57.78 | 120.00 |
| 40 | 1/2 | 16-QAM | 16-QAM | QPSK | OFF | 65.00 | 135.00 | 72.22 | 150.00 |
| 41 | 1/2 | 64-QAM | QPSK | QPSK | OFF | 65.00 | 135.00 | 72.22 | 150.00 |
| 42 | 1/2 | 64-QAM | 16-QAM | QPSK | OFF | 78.00 | 162.00 | 86.67 | 180.00 |
| 43 | 1/2 | 64-QAM | 16-QAM | 16-QAM | OFF | 91.00 | 189.00 | 101.11 | 210.00 |
| 44 | 1/2 | 64-QAM | 64-QAM | QPSK | OFF | 91.00 | 189.00 | 101.11 | 210.00 |
| 45 | 1/2 | 64-QAM | 64-QAM | 16-QAM | OFF | 104.00 | 216.00 | 115.56 | 240.00 |
| 46 | 3/4 | 16-QAM | QPSK | QPSK | OFF | 78.00 | 162.00 | 86.67 | 180.00 |
| 47 | 3/4 | 16-QAM | 16-QAM | QPSK | OFF | 97.50 | 202.50 | 108.33 | 225.00 |
| 48 | 3/4 | 64-QAM | QPSK | QPSK | OFF | 97.50 | 202.50 | 108.33 | 225.00 |
| 49 | 3/4 | 64-QAM | 16-QAM | QPSK | OFF | 117.00 | 243.00 | 130.00 | 270.00 |
| 50 | 3/4 | 64-QAM | 16-QAM | 16-QAM | OFF | 136.50 | 283.50 | 151.67 | 315.00 |
| 51 | 3/4 | 64-QAM | 64-QAM | QPSK | OFF | 136.50 | 283.50 | 151.67 | 315.00 |
| 52 | 3/4 | 64-QAM | 64-QAM | 16-QAM | OFF | 156.00 | 324.00 | 173.33 | 360.00 |
| 53 | 1/2 | 16-QAM | QPSK | QPSK | QPSK | 65.00 | 135.00 | 72.22 | 150.00 |
| 54 | 1/2 | 16-QAM | 16-QAM | QPSK | QPSK | 78.00 | 162.00 | 86.67 | 180.00 |
| 55 | 1/2 | 16-QAM | 16-QAM | 16-QAM | QPSK | 91.00 | 189.00 | 101.11 | 210.00 |
| 56 | 1/2 | 64-QAM | QPSK | QPSK | QPSK | 78.00 | 162.00 | 86.67 | 180.00 |
| 57 | 1/2 | 64-QAM | 16-QAM | QPSK | QPSK | 91.00 | 189.00 | 101.11 | 210.00 |
| 58 | 1/2 | 64-QAM | 16-QAM | 16-QAM | QPSK | 104.00 | 216.00 | 115.56 | 240.00 |
| 59 | 1/2 | 64-QAM | 16-QAM | 16-QAM | 16-QAM | 117.00 | 243.00 | 130.00 | 270.00 |
| 60 | 1/2 | 64-QAM | 64-QAM | QPSK | QPSK | 104.00 | 216.00 | 115.56 | 240.00 |
| 61 | 1/2 | 64-QAM | 64-QAM | 16-QAM | QPSK | 117.00 | 243.00 | 130.00 | 270.00 |
| 62 | 1/2 | 64-QAM | 64-QAM | 16-QAM | 16-QAM | 130.00 | 270.00 | 144.44 | 300.00 |
| 63 | 1/2 | 64-QAM | 64-QAM | 64-QAM | QPSK | 130.00 | 270.00 | 144.44 | 300.00 |
| 64 | 1/2 | 64-QAM | 64-QAM | 64-QAM | 16-QAM | 143.00 | 297.00 | 158.89 | 330.00 |
| 65 | 3/4 | 16-QAM | QPSK | QPSK | QPSK | 97.50 | 202.50 | 108.33 | 225.00 |
| 66 | 3/4 | 16-QAM | 16-QAM | QPSK | QPSK | 117.00 | 243.00 | 130.00 | 270.00 |
| 67 | 3/4 | 16-QAM | 16-QAM | 16-QAM | QPSK | 136.50 | 283.50 | 151.67 | 315.00 |
| 68 | 3/4 | 64-QAM | QPSK | QPSK | QPSK | 117.00 | 243.00 | 130.00 | 270.00 |
| 69 | 3/4 | 64-QAM | 16-QAM | QPSK | QPSK | 136.50 | 283.50 | 151.67 | 315.00 |
| 70 | 3/4 | 64-QAM | 16-QAM | 16-QAM | QPSK | 156.00 | 324.00 | 173.33 | 360.00 |
| 71 | 3/4 | 64-QAM | 16-QAM | 16-QAM | 16-QAM | 175.50 | 364.50 | 195.00 | 405.00 |
| 72 | 3/4 | 64-QAM | 64-QAM | QPSK | QPSK | 156.00 | 324.00 | 173.33 | 360.00 |
| 73 | 3/4 | 64-QAM | 64-QAM | 16-QAM | QPSK | 175.50 | 364.50 | 195.00 | 405.00 |
| 74 | 3/4 | 64-QAM | 64-QAM | 16-QAM | 16-QAM | 195.00 | 405.00 | 216.67 | 450.00 |
| 75 | 3/4 | 64-QAM | 64-QAM | 64-QAM | QPSK | 195.00 | 405.00 | 216.67 | 450.00 |
| 76 | 3/4 | 64-QAM | 64-QAM | 64-QAM | 16-QAM | 214.50 | 445.50 | 238.33 | 495.00 |

### CC42 Preamble

A detailed description of the preamble is given in [3, Sections 3.5, 3.6].

The mixed mode preamble is constructed as follows:

- L-STF: Legacy short training field (8usec)

- L-LTF: Legacy long training field (8usec)

- L-SIG: Legacy signal field (one OFDM symbol, 4usec)

- HT-SIG: High throughput signal field (two OFDM symbols, 8usec)

- HT-STF: High throughput short training field (4usec)

- HT-LTF: High throughput long train fields (4usec per data stream)

The legacy OFDM short training, long training, and signal field ensure that legacy OFDM stations will be able to perform start-of-packet detection, AGC, frequency estimation, timing offset estimation, and channel estimation without any degradation. Furthermore, legacy OFDM stations will be able to decode the legacy signal field and extract the RATE and LENGTH fields.  The RATE and LENGTH fields in the legacy signal field are “spoofed” such that a legacy OFDM RX can defer for the correct period of time. Small cyclic delay is applied for L-STF, L-LTF and L-SIG according to the number of transmission antennas.

In the 40MHz mode, the STF in the lower sub-channel is identical to legacy, whereas the STF in the upper sub-channel is a 90-degree phase rotation of the legacy STF, similarly for LTF. This rotation is performed to keep the PAPR of the STF low.  In a 40MHz transmission, the L-SIG in the upper sub-channel is rotated by 90 degrees, to be consistent with the LTF.

The High Throughput signal field (HT-SIG) contains information needed to correctly demap and decode the HT data fields (i.e. the MIMO-OFDM symbols). It also contains fields for supporting advanced techniques. Same cyclic delay is applied for HT-SIG as L-STF, L-LTF and L-SIG.

To achieve an accurate HT AGC measurement, the HT-STF is transmitted after the HT-SIG. The HT-STF uses 12 tones. In the 40 MHz channel, 24 tones are used in the HT-STF. The purpose of HT-STF is purely for AGC purposes. It is anticipated that the latter portion of the 4s duration would act as a blanking interval to allow the RFIC transients to settle down.

After the AGC is adjusted for HT reception, the system is re-trained for channel estimation, fine frequency offset estimation etc. The number of HT-LTFs is equal to the number of spatial streams except optional sounding mode. The cyclic delay is also applied for HT-STF and HT-LTF according to the number of spatial streams.

The greenfield preamble is constructed with similar fields as mixed mode as follows:

- L-STF: Legacy short training field (8usec)

- HT-LTF: Legacy long training field (8usec)

- HT-SIG: High throughput signal field (two OFDM symbols, 8usec)

- HT-LTF: High throughput long train fields (4usec per additional data stream)

The mean and standard deviation (Std) of the peak to sidelobe ratio (in dB) of the autocorrelation function for the various components of the preamble are given in the table below. These values are same even if cyclic delay is applied.

|  | L-STF |  | L-LTF |  | HT-STF |  | HT-LTF |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mean | Std | Mean | Std | Mean | Std | Mean | Std |
| 20MHz HT Mode | 9.0 | 10.3 | 14.1 | 13.3 | 9.0 | 10.3 | 14.7 | 14.7 |
| 40MHz HT Mode | 12.2 | 10.7 | 17.1 | 14.2 | 12.2 | 10.7 | 17.2 | 16.1 |
| Duplicated Legacy Mode | 12.2 | 10.7 | 17.1 | 14.2 | N/A | N/A | N/A | N/A |
| 40MHz Upper/Lower Mode | 8.1 | 7.6 | 13.3 | 10.1 | 8.1 | 7.6 | 13.6 | 10.4 |

The PAPR values (in dB) for the various components of the preamble are given in the table below.

|  | L-STF | L-LTF | HT-STF | HT-LTF |
| --- | --- | --- | --- | --- |
| 20MHz HT Mode | 2.1 | 3.2 | 2.1 | 3.6 |
| 40MHz HT Mode | 2.1 | 3.2 | 2.1 | 3.4 |
| Duplicated Legacy Mode | 2.1 | 3.2 | N/A | N/A |
| 40MHz Upper/Lower Mode | 2.1 | 3.2 | 2.1 | 3.6 |

Followings are cross correlation properties of STF and LTF when different cyclic delay is applied. However, each HT-LTF is multiplied by the orthogonal sequence and cross correlation value should be zero after receiving whole HT-LTFs.

### CC51.5 Channelization

This proposal supports 10, 20, and 40MHz channelization.

### CC52 Spectral Mask

Transmit spectral mask requirements are given in [3, Section 6] for 20 and 40MHz operation and [3, Section 1] describes compliance with 802.11j for 10MHz operation.

### CC80 Required changes to 802.11 PHY

- Optional 40MHz channel

  - [3, Sections 3, 4]

- MIMO spatial division multiplexing with up to 4 spatial streams

  - [3, Sections 3,4]

- High throughput preambles (mixed mode and green field)

  - [3, Sections 3.6]

- Mandatory 5/6 coding

  - [3, Sections 4.3.2]

- Optional ½ guard interval

  - [3, Sections 3.2, 3.4]

- Optional LDPC advanced coding

  - [3, Sections 4.3.3]

- Optional Transmit beamforming

  - [3, Section 8]

- High throughput preambles for sounding

  - [3, Section 9]

- Optional STBC

  - [3, Section 4.5.1]

- 56 tones

  - [3, Sections 3.4]