IEEE P802.11
Wireless LANs

| Normative Text Proposal for Frame Request/Report enhancements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-NOV-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sudheer Matta | Trapeze Networks | 5753 W. Las Positas Blvd, Pleasanton, CA, 94588 | 925 474 2290 | matta@trapezenetworks.com |
| Manish Tiwari | Trapeze Networks | 5753 W. Las Positas Blvd, Pleasanton, CA, 94588 | 925 474 2278 | manish@trapezenetworks.com |

- Abstract

This document contains a normative text proposal for enabling request of specific frames in Frame Report and modifies the Frame Response appropriately. This also addresses comments in that area.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Contents

7. Frame formats	3

7.3 Management frame body components	3

7.3.2.21 Measurement Request element	3

7.3.2.21.7 Frame Request	3

7.3.2.22.7 Frame Report	4

# 7. Frame formats

## 7.3 Management frame body components

## 7.3.2.21 Measurement Request element

## 7.3.2.21.7 Frame Request

|  | Channel Number | Regulatory Class | Randomization Interval | Measurement Duration | Frame Request Parameter Set | Mac Address (Optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 1 | 2 | 2 | 2 | 6 |

Figure k10—Measurement Request field format for a Frame Request

Channel Number indicates the channel number for which the measurement request applies. Channel

Number is defined within a Regulatory Class as shown in Error! Reference source not found.

(Editor’s note: Please fill the appropriate number).

Regulatory Class indicates the frequency band for which the measurement request applies. Valid values of

Regulatory Class are shown in Error! Reference source not found.

(Editor’s note: Please fill the appropriate number).

Randomization Interval specifies the upper bound of the random delay to be used prior to making the 1

measurement in units of TU. See 11.11.3. 2

The Measurement Duration field shall be set to the preferred duration of the requested measurement, 3

expressed in TUs. If the Duration Mandatory bit is set to 1 in the Measurement Request Mode field this 4

shall be interpreted as a mandatory measurement duration. If the Duration Mandatory bit is set to 0 this 5

shall be interpreted as a target measurement duration. See 11.11.4.

The format of Frame Request Parameter is shown in Figure XX. (Editor’s note: Please fill the appropriate number).

B0                          B1                     B2                  B3                  B4      B5               B8    B9              B15

| Match Type | Match Subtype | Match Mac | Type | Subtype | Reserved |
| --- | --- | --- | --- | --- | --- |

The Match Type bit indicates whether the type fields indicated in the Frame Request Parameter should match for the frames counted for the frame report generation. If the bit is set to 1, only frames that match the type should be counted towards the generation of a frame report.

The Match Subtype bit is only valid when the match type bit is set to 1. This bit indicates whether the subtype field indicated in the Frame Request Parameter should match for the frames counted for the frame report generation. If the bit is set to 1, only frames that match the subtype should be counted towards the generation of a frame report.

The Match Mac address field indicates whether the mac address included in the frame request frame should match for the frames counted for the frame report generation. If the bit is set to 1, only frames that match the mac address included in the frame request should be counted towards the generation of a frame report. When this bit set to 1, the mac address field in the frame request is mandatory.

The type field is used to indicate the type of packets that would be counted towards the frame report generation. The type field is only used when the match type field is set to 1.

The Subtype field is used to indicate the subtype of packets that would be counted towards the frame report generation. The subtype field is only used when the match subtype field is valid and is set to 1.

The reserved field is set to zeros on transmit and should be ignored on receive.

The Mac address field shall be included in the frame request frame, if the match mac address field in the frame request parameter set is set to 1. If this field is included, only frames from this mac address shall be counted towards the frame report generated in response to this frame request.

## 7.3.2.22.7 Frame Report

|  | Channel Number | Regulatory Class | Actual Measurement Start time | Measurement Duration | Frame Report Parameter Set | Frame Report Entry |
| --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 1 | 8 | 2 | 1 | n * 18 |

Figure k10—Measurement Request field format for a Frame Request

Channel Number indicates the channel number for which the measurement request applies. Channel

Number is defined within a Regulatory Class as shown in Error! Reference source not found.

(Editor’s note: Please fill the appropriate number).

Regulatory Class indicates the frequency band for which the measurement request applies. Valid values of

Regulatory Class are shown in Error! Reference source not found.

(Editor’s note: Please fill the appropriate number).

Actual Measurement Start Time shall be set to value of the measuring STA's TSF timer at the time the

measurement started.

Measurement Duration shall be set equal to the duration over which the Frame Report was measured,

expressed in TUs.

The format of Frame Report Parameter is shown in Figure XX. (Editor’s note: Please fill the appropriate number).

B0                      B1                  B2                  B3      B4             B7

| Type Valid | Subtype Valid | Type | Subtype |
| --- | --- | --- | --- |

The Type valid bit indicates whether the type field indicated in the Frame Report Parameter element were used to filter the frames counted for the frame report generation. If the bit is set to 1, only frames that matched the type field were counted towards the generation of this frame report.

The Type field is used to indicate the type of packets that were counted towards the frame report generation. The type field is only used when the type valid field is set to 1.

The Subtype valid bit indicates whether the Subtype field indicated in the Frame Report Parameter element were used to filter the frames counted for the frame report generation. If the bit is set to 1, only frames that matched the Subtype field were counted towards the generation of this frame report.

The Subtype field is used to indicate the subtype of packets that were counted towards the frame report generation. The subtype field is only used when the type/subtype valid field is set to 1.

The format of the Frame Report Entry is shown in Figure k23. 9

|  | Transmit Address | BSSID | Phy Type | Average RCPI | RSNI last | RCPI | Antenna ID | Number of Frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets | 6 | 6 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure k23—Frame Report Entry format

The Transmit Address field contains the Transmit Address (TA) from the frames being reported.

The BSSID field contains the BSSID from the frames being reported.

PHY Type indicates the physical medium type for the frame(s) being reported. Valid entries are coded

according to the value of dot11PHYType.

Average RCPI indicates the average value for the received channel power of all the frames counted for this report.

Average RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

RSNI indicates the received signal to noise indication of the received frame in dBm, as described in

7.3.2.31. This field shall be the RSNI value for the most recently received frame.

Last RCPI indicates the received channel power of the most recently counted frame in this Frame Report

entry. Last RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

The Antenna ID field contains the identifying number for the antenna used to receive the most recently counted frame in this Frame Report entry. Antenna ID is defined in 7.3.2.29.

Number of Data Frames is a count of the frames that passed any matching, specified by the type/subtype and mac address matching fields in the frame request parameter set, with the indicated Transmit Address and BSSID during the measurement duration. The value 255 shall indicate a count of 255 or more.

## 11.11.9.2 Frame Report

If a station accepts a Frame Request it shall respond with a Radio Measurement Report frame containing

one or more Measurement (Frame) Report elements.

The measuring station shall filter the observable traffic during the measurement duration, using the parameters specified in the frame request parameter set, and shall summarize this traffic in one or more Frame Report

elements. Each Frame Report element contains one or more Frame Report Entries.

Each Frame Report Entry contains the Transmit Address, BSSID, Average RCPI, Last RCPI, Antenna ID and Number of

Frames for the counted in the Frame Report Entry. The reported Average RCPI shall be the average of the

RCPI values of all frames counted in the Frame Report Entry. The Last RCPI shall be the RCPI value of

the most recently received frame counted in the Frame Report Entry. The Antenna ID field contains the

identifying number for the antenna used to receive the most recently received frame included in this report entry.

If different antennas are used to receive the frame preamble and the frame body, this Antenna ID shall

contain the identifying number for the antenna used to receive the frame body.