IEEE P802.11
Wireless LANs

| LB78 Annex D Editorial Comment Resolution normative text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-1-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Paul Gray | AirWave Wireless | 1700 S. El Camino RealEl Camino Real, San Mateo CA 94025 | 650-286-6107 | paul@airwave.com |

Abstract

LB78 Annex D normative text resolving Editorial comments form 802.11k D3.0.

This contains a full MIB which the editor should be able to apply in full after accepting all changes and reconnecting 10 broken links.

Corresponding spreadsheet is 11-05-1217-00-000k

Deletions are noted via strikethough and additions are noted via underline

Notes:

Accepts – 213 (610, 1186), 627, 629, 637, 638, 639, 640, 641, 642, 643, 644, 645,  646, 647, 648, 649, 650, 651, 653, 654, 655, 656, 657, 658, 659, 660, 662, 663, 664, 666, 668, 670, 673, 674, 675, 676, 677, 678, 679, 682, 685, 686, 687, 688, 689, 690, 691, 692, 761, 762 (1193), 768, 769 (1199), 772, 1118, 1123, 1184 (1189), 1195, 1200, 1201, 1208

Declines – 628, 630, 631, 632, 634, 635, 665

Counter – 1194 (Chair change to editorial? – covered by Editorial

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Annex D

In the IMPORT Section of Annex D, insert the following import statement to the end behind the ifIndex statement:

InterfaceIndex FROM IF-MIB;

In the Major Section of Annex D, insert the following comment to the end of dot11smt section behind comment dot11RSNAStatsTable:

-- dot11RadioResourceMeasurement ::= { dot11smt 12 }

In the Major Section of Annex D, insert the following comment to the end of dot11mac section behind dot11GroupAddressesTable:

-- dot11PeerStatsTable ::= { dot11mac 4 }

In the TEXTUAL CONVENTION of Annex D, insert the following convention to the end behind the WEPKeytype convention definition:

TSFType ::= OCTET STRING (SIZE (8))

PHYType ::= TEXTUAL-CONVENTION

STATUS current

DESCRIPTION

"The textual convention indicates the frequency band.”

SYNTAX INTEGER

{

twoPointFour(0),

fiveGHz(1)

}

In the dot11StationConfig table of Annex D, change the dot11StationConfigEntry sequence list as

follows:

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID MacAddress,

dot11MediumOccupancyLimit INTEGER,

dot11CFPollable TruthValue,

dot11CFPPeriod INTEGER,

dot11CFPMaxDuration INTEGER,

dot11AuthenticationResponseTimeOut Unsigned32,

dot11PrivacyOptionImplemented TruthValue,

dot11PowerManagementMode INTEGER,

dot11DesiredSSID OCTET STRING,

dot11DesiredBSSType INTEGER,

dot11OperationalRateSet OCTET STRING,

dot11BeaconPeriod INTEGER,

dot11DTIMPeriod INTEGER,

dot11AssociationResponseTimeOut Unsigned32,

dot11DisassociateReason INTEGER,

dot11DisassociateStation MacAddress,

dot11DeauthenticateReason INTEGER,

dot11DeauthenticateStation MacAddress,

dot11AuthenticateFailStatus INTEGER,

dot11AuthenticateFailStation MacAddress,

dot11SpectrumManagementImplemented TruthValue,

dot11SpectrumManagementRequired TruthValue,

dot11MultiDomainCapabilityImplemented TruthValue,

dot11MultiDomainCapabilityEnabled TruthValue,

dot11CountryString OCTET STRING,

dot11RSNAOptionImplemented TruthValue,

dot11RSNAPreauthenticationImplemented TruthValue,

dot11AssociateStation MacAddress,

dot11AssociateID INTEGER,

dot11AssociateFailStation MacAddress,

dot11AssociateFailStatus INTEGER,

dot11ReassociateStation MacAddress,

dot11ReassociateID INTEGER,

dot11ReassociateFailStation MacAddress,

dot11ReassociateFailStatus INTEGER,

dot11RadioMeasurementCapable TruthValue,

dot11RadioMeasurementEnabled TruthValue,

dot11RadioMeasurementProbeDelay INTEGER,

dot11MeasurementPilotEnabled TruthValue,

dot11MeasurementPilotPeriod INTEGER,

dot11MeasurementPilotTransmitPriority INTEGER }

Update the dot11BeaconPeriod element as shown:

dot11BeaconPeriod OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TUs that a station shall use for scheduling Beacon transmissions. This value is transmitted in Beacon, Probe Response, and Measurement Pilot frames."

::= { dot11StationConfigEntry 12 }

Insert the following elements to the end of dot11StationConfigTable element definitions after dot11RSNAPreauthenticationImplemented:

dot11AssociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

association response frame. If no association response

frame has been transmitted, the value of this attribute

shall be 0."

::= { dot11StationConfigEntry 28 }

dot11AssociateID OBJECT-TYPE

SYNTAX INTEGER(0..2007)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the Association ID from the

most recently transmitted association response frame.

If no association response frame has been transmitted,

the value of this attribute shall be 0."

::= { dot11StationConfigEntry 29 }

dot11AssociateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

failed association response frame. If no failed

association response frame has been transmitted,

the value of this attribute shall be 0."

::= { dot11StationConfigEntry 30 }

dot11AssociateFailStatus OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently transmitted Status

Code in a failed association response frame. If no failed

association response frame has been transmitted, the

value of this attribute shall be 0."

::= { dot11StationConfigEntry 31 }

dot11ReassociateStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

reassociation response frame. If no reassociation response

frame has been transmitted, the value of this attribute

shall be 0."

::= { dot11StationConfigEntry 32 }

dot11ReassociateID OBJECT-TYPE

SYNTAX INTEGER(0..2007)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the Association ID from the

most recently transmitted reassociation response frame.

If no reassociation response frame has been transmitted,

the value of this attribute shall be 0."

::= { dot11StationConfigEntry 33 }

dot11ReassociateFailStation OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the MAC address from the

Address 1 field of the most recently transmitted

failed reassociation response frame. If no failed

reassociation response frame has been transmitted,

the value of this attribute shall be 0."

::= { dot11StationConfigEntry 34 }

dot11ReassociateFailStatus OBJECT-TYPE

SYNTAX INTEGER(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the most recently transmitted

Status Code in a failed reassociation response frame.

If no failed reassociation response frame has been

transmitted, the value of this attribute shall be 0."

::= { dot11StationConfigEntry 35 }

dot11RadioMeasurementCapable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting Radio

Measurement. Otherwise it is not capable of performing

Radio Measurement.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry 36 }

dot11RadioMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A STA may use the defined Radio Measurement procedures if

this attribute is TRUE. The default value of this attribute

is FALSE."

::= { dot11StationConfigEntry 37 }

dot11RadioMeasurementProbeDelay OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The value of ProbeDelay to be used when making a beacon

type measurement with measurement mode active."

::= { dot11StationConfigEntry 38 }

dot11MeasurementPilotEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"A STA shall transmit Measurement Pilot frames if this attribute is TRUE.

The default value of this attribute is FALSE."

::= { dot11StationConfigEntry 39 }

dot11MeasurementPilotPeriod OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the number of TUs that a station shall use for

scheduling Measurement Pilot transmissions. This value is transmitted in

Measurement Pilot frames."

::= { dot11StationConfigEntry 40 }

dot11MeasurementPilotTransmitPriority OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the Access Category (AC), at which Measurement Pilot

frames are transmitted."

::= { dot11StationConfigEntry 41 }

In SMT Notification Objects Section of Annex D, insert the following notifications behind dot11AuthenticateFail:

dot11Associate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AssociateStation, dot11AssociateID}

STATUS current

DESCRIPTION

"The associate notification shall be sent when the STA sends

an Association Response frame with a status code equal to ‘successful.’

The value of the notification shall include the MAC address of the

MAC to which the Association Response frame was sent and the

Association ID.

ifIndex - Each 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 4 }

dot11AssociateFailed NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11AssociateFailStatus,

dot11AssociateFailStation }

STATUS current

DESCRIPTION

"The associate failed notification shall be sent when the

STA sends an Association Response frame with a status code

other than ‘successful.’ The value of the notification shall

include the MAC address of the MAC to which the Association

Response frame was sent and the reason for the association failure.

ifIndex - Each 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 5 }

dot11Reassociate NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11ReassociateStation, dot11ReassociateID}

STATUS current

DESCRIPTION

"The reassociate notification shall be sent

when the STA sends an Reassociation Response frame with a

status code equal to ‘successful.’ The value of

the notification shall include the MAC address of the

MAC to which the Reassociation Response frame was sent and the

Reassociation ID.

ifIndex - Each 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 6 }

dot11ReassociateFailed NOTIFICATION-TYPE

OBJECTS { ifIndex, dot11ReassociateFailStatus,

dot11ReassociateStation }

STATUS current

DESCRIPTION

"The reassociate failed notification shall be sent

when the STA sends an Reassociation Response frame with a

status code other than ‘successful.’ The value of

the notification shall include the MAC address of the

MAC to which the Reassociation Response frame was sent and

the reason for the reassociation failure.

ifIndex - Each 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

::= { dot11SMTnotification 0 7 }

In dot11CountersTable change Dot11CountersEntry as follows:

Dot11CountersEntry ::=

SEQUENCE {	dot11TransmittedFragmentCount	Counter32,

dot11MulticastTransmittedFrameCount	Counter32,

dot11FailedCount	Counter32,

dot11RetryCount	Counter32,

dot11MultipleRetryCount	Counter32,

dot11FrameDuplicateCount	Counter32,

dot11RTSSuccessCount	Counter32,

dot11RTSFailureCount	Counter32,

dot11ACKFailureCount	Counter32,

dot11ReceivedFragmentCount	Counter32,

dot11MulticastReceivedFrameCount	Counter32,

dot11FCSErrorCount	Counter32,

dot11TransmittedFrameCount	Counter32,

dot11WEPUndecryptableCount	Counter32,

dot11QoSDiscardedFragmentCount	Counter32,

dot11AssociatedStationCount	Counter32,

dot11QoSCFPollsReceivedCount	Counter32,

dot11QoSCFPollsUnusedCount	Counter32,

dot11QoSCFPollsUnusableCount	Counter32,

dot11QoSCFPollsLostCount	Counter32 }

Insert the following attribute definition to the end of the dot11CountersEntry attributes:

dot11QoSCFPollsLostCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This counter shall increment for each QoS (+)CF-Poll that has been issued by the HC where there was no response from the QSTA indicated in the RA field.”

::= { dot11CountersEntry 20 }

In SMT MIB of Annex D, insert the following Radio Resource Measurement MIB at the after the Robust Security Network Association (RSNA and TSN) MIB

-- ********************************************************************

-- * Radio Resource Measurement

-- ********************************************************************

dot11RadioResourceMeasurement OBJECT IDENTIFIER ::= { dot11smt 13 }

-- **********************************************************************

-- *  dot11RRMRequest and dot11RRMReport Usage

-- *

-- *  The dot11RRMRequest and dot11RRMReport portions of the SMT MIB

-- *  provide access to the Radio Measurement service.  By performing

-- *  SET operations on the various dot11RRMRequest MIB objects,

-- *  radio measurements may be initiated directly on the local STA or

-- *  on any peer station within the same BSS.  Subsequently, by

-- *  performing GET operations on the various dot11RRMReport MIB

-- *  objects the results of the requested measurements may be

-- *  retrieved.

-- *

-- *  In the diagram below, a radio measurement could be initiated

-- *  for STA x by performing a MIB.set operation on the SMT MIB of

-- *  STA x and specifying the MAC address of STA x in

-- *  dot11RRMRqstTargetAdd.  Additionally, it is possible to have STA x

-- *  request a measurement from STA y by performing a MIB.set operation

-- *  on the SMT MIB of STA x and specifying the MAC address of STA y in

-- *  dot11RRMRqstTargetAdd.  In both cases the result of the measurements

-- *  can be retrieved by performing a MIB.get operation on the SMT MIB

-- *  of STA x upon completion of the measurement.

-- *

-- *          MIB.Set                                        MIB.Set

-- *            or                                              or

-- *          MIB.Get                                        MIB.Get

-- *    +--------|---------+                           +--------|---------+

-- *    | SME    |         |                           | SME    |         |

-- *    |       \ /        |                           |       \ /        |

-- *    |   +---------+    |                           |   +---------+    |

-- *    |   | SMT MIB |    |                           |   | SMT MIB |    |

-- *    |   |         |    |                           |   |         |    |

-- *    |   |         |    |                           |   |         |    |

-- *    |   +---------+    |                           |   +---------+    |

-- *    |                  |                           |                  |

-- *    |   / \            |                           |   / \            |

-- *    |    |   MREQUEST  |                           |    |   MREQUEST  |

-- *    +----+-------------+                           +----+-------------+

-- *    |    |   MREPORT   |                           |    |   MREPORT   |

-- *    |   \ /  MEASURE   |     Action Frames         |   \ /  MEASURE   |

-- *    |                  | <--Measurement Request--> |                  |

-- *    |                  | <--Measurement Report---> |                  |

-- *    | MLME             |                           | MLME             |

-- *    +------------------+                           +------------------+

-- *          STA x                                           STA y

-- *

-- *

-- *  Each STA maintains a single dot11RRMRequestTable in the SMT MIB

-- *  used to initiate RM Measurement Requests.  Each dot11RRMRequestEntry

-- *  in the table represents an individual Measurement Request that

-- *  makes up a complete Measurement Request Action frame.

-- *  Multiple Measurement Requests may be concatenated into a single

-- *  Measurement Request Action frame by setting the same

-- *  dot11RRMRqstToken value into multiple dot11RRMRequestEntrys.

-- *

-- *  Each row, dot11RRMRequestEntry, of the dot11RRMRequestTable

-- *  provides read-create access for the initiation of a measurement

-- *  request.  The dot11RRMRequestNextIndex object can be used to

-- *  determine which is the next row available.  Each row corresponding to

-- *  one measurement in the sequence is created with a dot11RRMRqstRowStatus

-- *  set to notInService.  Once the dot11RRMRequestEntry(s) have been

-- *  created for a desired measurement sequence the corresponding

-- *  dot11RRMRqstRowStatus(s) objects are set to active to indicate that

-- *  the SME can trigger the appropriate MLME primitives.  Upon processing

-- *  the request, the SME returns the corresponding dot11RRMRqstRowStatus(s)

-- *  object to notInsService and are now available for additional

-- *  measurement requests.

-- *

-- *  After a radio measurement is complete the SME populates the RRMReport

-- *  objects with the results of the measurement.  Each STA maintains a set

-- *  of RRMReport tables, one for each corresponding measurement type.  The

-- *  results of the entire measurement sequence are spread across the tables

-- *  based on what types of measurements were requested.  Each xxxReportEntry

-- *  within a xxxReportTable contains a xxxRprtRqstToken that corresponds

-- *  to the original dot11RRMRqstToken in the measurement request.  So the

-- *  results of the measurement can be collected by searching the appropriate

-- *  xxxReportTables and retrieve any reports with the matching request

-- *  token.

-- **********************************************************************

-- ********************************************************************

-- * Radio Resource Measurement Requests

-- ********************************************************************

dot11RRMRequest OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 1 }

-- ********************************************************************

-- * dot11RRMRequest TABLE

-- ********************************************************************

dot11RRMRequestNextIndex OBJECT-TYPE

SYNTAX Unsigned32(0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies a hint for the next value of dot11RRMRqstIndex to be used in a

row creation attempt for dot11RRMRequestTable. If no new rows can be created

for some reason, such as memory, processing requirements, etc, the SME shall

set this attribute to 0. It shall update this attribute to a proper value

other than 0 as soon as it is capable of receiving new measurement requests.

The nextIndex is not necessarily sequential nor monotonically increasing."

::= { dot11RRMRequest 1 }

dot11RRMRequestTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RRMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This group contains the current list of requests for RRM

reports to be issued and have been issued until removed. A network manager

adds a RRM request by creating a row with createAndWait row status and then

filling in the request parameters/attributes.

The request becomes active to be issued when the row status

is set to Active. The columnar objects or attributes other than the

rowstatus shall not be written if the rowStatus is Active. The request rows

can be deleted, if commanded by a network manager via changing the value of

dot11RRMRqstRowStatus to Destroy. This may leave orphaned rows if a manager

crashes and forgets which rows are being used by it. One recommended way to

manage orphaned or finished rows is to delete rows if their

dot11RRMRqstRowStatus remains other than Active for longer than a

period (recommend at least 5 minutes, RFC 2579). Or another recommended

way is to delete older rows as needed based on their dot11RRMRqstTimeStamp

values. This can be done by the agent as well as the manager. "

::= { dot11RRMRequest 2 }

dot11RRMRequestEntry OBJECT-TYPE

SYNTAX Dot11RRMRequestEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RRMRequestTable Indexed by dot11RRMRqstIndex."

INDEX { dot11RRMRqstIndex }

::= { dot11RRMRequestTable 1 }

Dot11RRMRequestEntry ::=

SEQUENCE {

dot11RRMRqstIndex Unsigned32,

dot11RRMRqstRowStatus RowStatus,

dot11RRMRqstToken OCTET STRING,

dot11RRMRqstRepetitions INTEGER,

dot11RRMRqstIfIndex InterfaceIndex,

dot11RRMRqstType INTEGER,

dot11RRMRqstTargetAdd MacAddress,

dot11RRMRqstTimeStamp TimeTicks,

dot11RRMRqstChanNumber INTEGER,

dot11RRMRqstRegulatoryClass INTEGER,

dot11RRMRqstRndInterval Unsigned32,

dot11RRMRqstDuration Unsigned32,

dot11RRMRqstParallel TruthValue,

dot11RRMRqstEnable TruthValue,

dot11RRMRqstRequest TruthValue,

dot11RRMRqstReport TruthValue,

dot11RRMRqstDurationMandatory TruthValue,

dot11RRMRqstBeaconRqstMode INTEGER,

dot11RRMRqstBssid MacAddress,

dot11RRMRqstSSID OCTET STRING,

dot11RRMRqstReportingCondition INTEGER,

dot11RRMRqstThresholdOffset INTEGER,

dot11RRMRqstSTAStatRqstGroupID INTEGER,

dot11RRMRqstLCIRqstOctet INTEGER,

dot11RRMRqstPauseTimeUnit INTEGER,

dot11RRMRqstPauseTime INTEGER,

dot11RRMRqstQoSMetricsPeerQSTAAddress MacAddress,

dot11RRMRqstQoSMetricsTrafficIdentifier INTEGER,

dot11RRMRqstQoSMetricsBin0Range INTEGER,

dot11RRMRqstTrigdQoSAverageCondition TruthValue,

dot11RRMRqstTrigdQoSConsecutiveCondition TruthValue,

dot11RRMRqstTrigdQoSDelayCondition TruthValue,

dot11RRMRqstTrigdQoSAverageThreshold INTEGER,

dot11RRMRqstTrigdQoSConsecutiveThreshold INTEGER,

dot11RRMRqstTrigdQoSDelayThresholdRange INTEGER,

dot11RRMRqstTrigdQoSDelayThreshold INTEGER,

dot11RRMRqstTrigdQoSMeasurementCount INTEGER,

dot11RRMRqstTrigdQoSTimeout INTEGER }

dot11RRMRqstIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for RRM Request elements in dot11RRMRequestTable, greater than 0."

::= { dot11RRMRequestEntry 1 }

dot11RRMRqstRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Row Status column of the current row, used for tracking status of

an individual request. When this attribute is set to Active, and Active,

and a measurement request can be unambiguously created based on the

parameters in the row, then the MLME may proceed to issue the request to

its intended targets when appropriate. If not, this attribute may be set

to Not-ready immediately to indicate parametric errors. However, it is

the network manager’s responsibility to correct the error. If the request

is successfully issued to the target STA, then the rowStatus is set to

notInService."

REFERENCE

"IEEE 802.11, Clause 7.3.2.21"

::= { dot11RRMRequestEntry 2 }

dot11RRMRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"A unique string to identify a group of rows to be issued as parallel

or back to back measurements. To guarantee the uniqueness of this token across multiple network managers, it is recommended that this token be

prefixed with the IP address of the network manager creating this row.

This token is not necessarily equivalent to the measurement tokens in

RRM request frames.  If this attribute is an empty string, then this row

of request is independent from other requests."

DEFVAL { "" }

::= { dot11RRMRequestEntry 3 }

dot11RRMRqstRepetitions OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Number of Repetions field contains the requested number of repetitions for all the measurement request elements in this frame.  A value of zero in the Number of Repetitions field indicates measurement request elements are executed once without repetition."

::= { dot11RRMRequestEntry 4 }

dot11RRMRqstIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex for this row of RRM Request to be issued on."

::= { dot11RRMRequestEntry 5 }

dot11RRMRqstType OBJECT-TYPE

SYNTAX INTEGER {

channelLoad(3),

noiseHistogram(4),

beaconRequest(5),

frameRequest(6),

staStatistics(7),

lciRequest(8),

qosMetrics(9),

measurementPause(255)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The measurement type of this RRM request row."

::= { dot11RRMRequestEntry 6 }

dot11RRMRqstTargetAdd OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The MAC address of STA for this row of RRM Request is to be issued to. If

this attribute matches the MAC address of the dot11RRMRqstIfIndex, then

measurement request is for this STA itself to carry out."

::= { dot11RRMRequestEntry 7 }

dot11RRMRqstTimeStamp OBJECT-TYPE

SYNTAX TimeTicks

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The SysUpTime Value the last time when the dot11RRMRqstRowStatus is set to

active or when this row is created the first time. This attribute shall be

set by this STA or AP automatically, not by an SNMP manager."

::= { dot11RRMRequestEntry 8 }

dot11RRMRqstChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The channel number on which this request instructs the target STA to perform measurements and report. This attribute is ignored if dot11RRMRqstType = STA statistics Request, LCI Request, QoS Metrics, or Measurement Pause. However, even in that case, the manager

should set this attribute to the current channel for this interface, so

that the row can be set to active when ready with all attributes specified."

::= { dot11RRMRequestEntry 9 }

dot11RRMRqstRegulatoryClass OBJECT-TYPE

SYNTAX PHYType

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which the receiving STA shall conduct its measurement. This attribute is ignored if dot11RRMRqstType = STA statistics Request, LCI Request, QoS Metrics, or Measurement Pause. However, even in that case, this value should set to the current Regulatory Class for this interface, so that the row can be set to active when ready with all attributes specified."

REFERENCE

"IEEE 802.11, Error: Reference source not found"

::= { dot11RRMRequestEntry 10 }

dot11RRMRqstRndInterval OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute sets the Randomization Interval in the measurement Request Frame for this RRM Request. This attribute is ignored if dot11RRMRqstType = STA statistics Request, LCI Request or Measurement Pause."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 11 }

dot11RRMRqstDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute sets the Measurement Duration in the measurement Request

Element for this Mesurement Request."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 12 }

dot11RRMRqstParallel OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the parallel bit of the request frame that

contains this row of request. Default is false which means the measurement

shall be preformed in sequence. If this value is true, then the

measurement should start at the same time as the measurement described by

the next Measurement Request element in the same Measurement Request frame.

DEFVAL { false }

::= { dot11RRMRequestEntry 13 }

dot11RRMRqstEnable OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the enable bit of the request frame that

contains this row of request. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 14 }

dot11RRMRqstRequest OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the request bit of the request element that

contains this row of request. This attribute indicates whether this STA

shall accept measurement requests from the target STA. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 15 }

dot11RRMRqstReport OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the report bit of the request element that

contains this row of request. This bit controls whether the target STA shall

enable autonomous measurement report, and does not affect directed requests.

Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 16 }

dot11RRMRqstDurationMandatory OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the duration mandatory bit of the request frame that contains this row of request. Default is false."

DEFVAL { false }

::= { dot11RRMRequestEntry 17 }

dot11RRMRqstBeaconRqstMode OBJECT-TYPE

SYNTAX INTEGER {

passive(0),

passivePilot(1),

active(2),

staSelected(3),

beaconTable(4)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"dot11RRMRqstBeaconRqstMode corresponds to the Measurement Mode for

Beacon Request element. This attribute is only valid if the

dot11RRMRqstType is 5, indicating a beacon report. Otherwise this

attribute is ignored."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 18 }

dot11RRMRqstBssid OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

" BSSID indicates the BSSID of the particular AP for which this measurement

is requested. The BSSID shall be set to the broadcast BSSID when the

measurement is to be performed on any AP(s) on the specified channel."

DEFVAL { 'FFFFFFFF'H }

::= { dot11RRMRequestEntry 19 }

dot11RRMRqstSSID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute indicates the SSID for the measurement. The 
SSID shall be set to the wildcard SSID when the measurement 
is to be performed on any ESS/IBSS on the specified channel."

DEFVAL {''H }

::= { dot11RRMRequestEntry 20 }

dot11RRMRqstReportingCondition OBJECT-TYPE

SYNTAX INTEGER {

afterEveryMeasurement(0),

rcpiAboveAbsoluteThreshold(1),

rcpiBelowAbsoluteThreshold(2),

rssiAboveAbsoluteThreshold(3),

rssiBelowAbsoluteThreshold(4),

rcpiAboveOffsetThreshold(5),

rcpiBelowOffsetThreshold(6),

rssiAboveOffsetThreshold(7),

rssiBelowOffsetThreshold(8),

rcpiInBound(9),

rssiInBound(10)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Reporting Condition defines when the measured results are to be

reported to the requesting STA."

REFERENCE

"IEEE 802.11,

Error: Reference source not found"

DEFVAL {0}

::= { dot11RRMRequestEntry 21 }

dot11RRMRqstThresholdOffset OBJECT-TYPE

SYNTAX INTEGER (-127..127)

UNITS "dB"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Threshold/Offset provides either the threshold value or the offset value

to be used for conditional reporting."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 22 }

dot11RRMRqstSTAStatRqstGroupID OBJECT-TYPE

SYNTAX INTEGER {

staCounters(0),

bssLoad(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The attribute corresponds to the group identity if dot11RRMRqstType =

STA statistics Request. This attribute shall be ignored otherwise.

0 = STA Counters from dot11CountersTable

1 = BSS Load as defined in 7.3.2.29"

DEFVAL { 0 }

::= { dot11RRMRequestEntry 23 }

dot11RRMRqstLCIRqstOctet OBJECT-TYPE

SYNTAX INTEGER {

local(0),

remote(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The attribute corresponds to the subject of the LCI measurement request."

DEFVAL { 0 }

::= { dot11RRMRequestEntry 24 }

dot11RRMRqstPauseTimeUnit OBJECT-TYPE

SYNTAX INTEGER {

tu(0),

tu1000(1)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Measurement Period Unit subfield defines the time unit for the

Measurement Interval subfield."

REFERENCE

"IEEE 802.11, Table k5—Time Unit Definitions"

DEFVAL {0}

::= { dot11RRMRequestEntry 25 }

dot11RRMRqstPauseTime OBJECT-TYPE

SYNTAX INTEGER (0..32768)

UNITS "TUs or 1000*TUs, according to dot11RRMRqstPauseTimeUnit"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Pause Time consists of a 15 bit unsigned integer number

representing the time period during which measurements are

suspended or paused. Measurement Pause Requests are used to

provide time delays between the execution times of measurement

request elements in a Measurement Request Frame"

DEFVAL { 0 }

::= { dot11RRMRequestEntry 26 }

dot11RRMRqstQoSMetricsPeerQSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute specifies the peer QSTA address to be measured for a QoS Metrics measurement"

::= { dot11RRMRequestEntry 27 }

dot11RRMRqstQoSMetricsTrafficIdentifier OBJECT-TYPE

SYNTAX INTEGER(0..16)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute specifies the TC, or TS to be measured for a QoS Metrics measurement"

::= { dot11RRMRequestEntry 28 }

dot11RRMRqstQoSMetricsBin0Range OBJECT-TYPE

SYNTAX INTEGER(1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute specifies the delay range for bin 0 of the transmit delay histogram."

::= { dot11RRMRequestEntry 29 }

dot11RRMRqstTrigdQoSAverageCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Corresponds to the Average bit in the Triggered Condition bit-field and is used to request triggered reporting with triggering based on the number of MSDUs discarded averaged over dot11RRMRqstTrigdQoSMEasurementCount reaching dot11RRMRqstTrigdQoSAverageThreshold."

DEFVAL { false }

::= { dot11RRMRequestEntry 30 }

dot11RRMRqstTrigdQoSConsecutiveCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Corresponds to the Consecutive bit in the Triggered Condition bit-field and is used to request triggered reporting with triggering based on the consectutive number of MSDUs discarded reaching dot11RRMRqstTrigdQoSConsecutiveThreshold."

DEFVAL { false }

::= { dot11RRMRequestEntry 31 }

dot11RRMRqstTrigdQoSDelayCondition OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Corresponds to the Delay bit in the Triggered Condition bit-field and is used to request triggered reporting with triggering based on the consectutive number of MSDUs that experience a transmit delay greater than dot11RRMRqstTrigdQoSDelayThresholdRange reaching dot11RRMRqstTrigdQoSDelayThreshold."

DEFVAL { false }

::= { dot11RRMRequestEntry 32 }

dot11RRMRqstTrigdQoSAverageThreshold OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The trigger threshold for triggered QoS Metrics based on average MSDUs discarded. Triggering occurs if the number of MSDUs discarded over the moving average number of transmitted MSDUs in dot11RRMRqstTrigdQoSMeasurementCount reaches this threshold."

DEFVAL { 10 }

::= { dot11RRMRequestEntry 33 }

dot11RRMRqstTrigdQoSConsecutiveThreshold OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The trigger threshold for triggered QoS Metrics based on consecutive MSDUs discarded. Triggering occurs if the consecutive number of MSDUs discarded reaches this threshold."

DEFVAL { 5 }

::= { dot11RRMRequestEntry 34 }

dot11RRMRqstTrigdQoSDelayThresholdRange OBJECT-TYPE

SYNTAX INTEGER (0..3)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The first part of the trigger threshold for triggered QoS Metrics based on consecutive MSDUs delayed. Triggering occurs if the a consecutive number of MSDUs experience a transmit delay greater than or equal to the lower bound of the bin of the Transmit Delay Histogram given by the value of this attribute + 2, e.g. if this attribute is 1 the lower bound of bin 3."

DEFVAL { 1 }

::= { dot11RRMRequestEntry 35 }

dot11RRMRqstTrigdQoSDelayThreshold OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The second part of the trigger threshold for triggered QoS Metrics based on consecutive MSDUs delayed. Triggering occurs if the consecutive number of MSDUs that experience a transmit delay greater than dot11RRMRqstQoSDelayThresholdRange reaches this value."

DEFVAL { 20 }

::= { dot11RRMRequestEntry 36 }

dot11RRMRqstTrigdQoSMeasurementCount OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Contains a number of MSDUs to be used as a moving average count in the average error threshold and in determining the scope of the reported QoS metrics in a triggered measurement report."

DEFVAL { 100 }

::= { dot11RRMRequestEntry 37 }

dot11RRMRqstTrigdQoSTimeout OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Contains a value in units of 100TU during which a measuring STA shall not generate further triggered QoS metrics reports after a trigger condition has been met and a report generated."

DEFVAL { 20 }

::= { dot11RRMRequestEntry 38 }

-- **********************************************************************

-- * End of dot11RRMRequest TABLE

-- **********************************************************************

-- ********************************************************************

-- * Radio Resource Measurement Reports

-- * Report tables contain measurement reports received by this STA or

-- * results of measurements performed by this STA.

-- ********************************************************************

dot11RRMReport OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 2 }

-- ********************************************************************

-- * dot11ChannelLoadReport TABLE

-- ********************************************************************

dot11ChannelLoadReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11ChannelLoadReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Channel Load reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 1 }

dot11ChannelLoadReportEntry OBJECT-TYPE

SYNTAX Dot11ChannelLoadReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11ChannelLoadReportTable Indexed by

dot11ChannelLoadRprtIndex."

INDEX { dot11ChannelLoadRprtIndex }

::= { dot11ChannelLoadReportTable 1 }

Dot11ChannelLoadReportEntry ::=

SEQUENCE {

dot11ChannelLoadRprtIndex Unsigned32,

dot11ChannelLoadRprtRqstToken OCTET STRING,

dot11ChannelLoadRprtIfIndex InterfaceIndex,

dot11ChannelLoadMeasuringSTAAddr MacAddress,

dot11ChannelLoadRprtChanNumber INTEGER,

dot11ChannelLoadRprtRegulatoryClass INTEGER,

dot11ChannelLoadRprtActualStartTime TSFType,

dot11ChannelLoadRprtMeasurementDuration Unsigned32,

dot11ChannelLoadRprtChannelLoad INTEGER,

dot11ChannelLoadRprtMeasurementMode INTEGER }

dot11ChannelLoadRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Channel Load Report elements in

dot11ChannelLoadReportTable, greater than 0."

::= { dot11ChannelLoadReportEntry 1 }

dot11ChannelLoadRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the

Measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be  multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11ChannelLoadReportEntry 2 }

dot11ChannelLoadRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of ChannelLoad Report has been received on."

::= { dot11ChannelLoadReportEntry 3 }

dot11ChannelLoadMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Channel Load report."

::= { dot11ChannelLoadReportEntry 4 }

dot11ChannelLoadRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number for this row of Channel Load Report Table."

::= { dot11ChannelLoadReportEntry 5 }

dot11ChannelLoadRprtRegulatoryClass OBJECT-TYPE

SYNTAX PHYType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row of

ChannelLoad report has been conducted."

REFERENCE

"IEEE 802.11, Error: Reference source not found"

::= { dot11ChannelLoadReportEntry 6 }

dot11ChannelLoadRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11ChannelLoadReportEntry 7 }

dot11ChannelLoadRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the ChannelLoad

Report was measured."

::= { dot11ChannelLoadReportEntry 8 }

dot11ChannelLoadRprtChannelLoad OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "1/255"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Channel Load shall contain the fractional duration over which the

measuring STA determined the channel to be busy during the

measurement duration.”

REFERENCE

"IEEE 802.11, Clause 7.3.2.22.4"

::= { dot11ChannelLoadReportEntry 9 }

dot11ChannelLoadRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 2 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11ChannelLoadReportEntry 10 }

-- ********************************************************************

-- * End of dot11ChannelLoadReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11NoiseHistogramReport TABLE

-- ********************************************************************

dot11NoiseHistogramReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Noise Histogram reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 2 }

dot11NoiseHistogramReportEntry OBJECT-TYPE

SYNTAX Dot11NoiseHistogramReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11NoiseHistogramReportTable

Indexed by dot11NoiseHistogramRprtIndex."

INDEX { dot11NoiseHistogramRprtIndex }

::= { dot11NoiseHistogramReportTable 1 }

Dot11NoiseHistogramReportEntry ::=

SEQUENCE {

dot11NoiseHistogramRprtIndex Unsigned32,

dot11NoiseHistogramRprtRqstToken OCTET STRING,

dot11NoiseHistogramRprtIfIndex InterfaceIndex,

dot11NoiseHistogramMeasuringSTAAddr MacAddress,

dot11NoiseHistogramRprtChanNumber INTEGER,

dot11NoiseHistogramRprtRegulatoryClass INTEGER,

dot11NoiseHistogramRprtActualStartTime TSFType,

dot11NoiseHistogramRprtMeasurementDuration Unsigned32,

dot11NoiseHistogramRprtAntennaID INTEGER,

dot11NoiseHistogramRprtANPI INTEGER,

dot11NoiseHistogramRprtCNPIDensity0 INTEGER,

dot11NoiseHistogramRprtCNPIDensity1 INTEGER,

dot11NoiseHistogramRprtCNPIDensity2 INTEGER,

dot11NoiseHistogramRprtCNPIDensity3 INTEGER,

dot11NoiseHistogramRprtCNPIDensity4 INTEGER,

dot11NoiseHistogramRprtCNPIDensity5 INTEGER,

dot11NoiseHistogramRprtCNPIDensity6 INTEGER,

dot11NoiseHistogramRprtCNPIDensity7 INTEGER,

dot11NoiseHistogramRprtCNPIDensity8 INTEGER,

dot11NoiseHistogramRprtMeasurementMode INTEGER }

dot11NoiseHistogramRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Noise Histogram elements in dot11NoiseHistogramReportTable,

greater than 0."

::= { dot11NoiseHistogramReportEntry 1 }

dot11NoiseHistogramRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the request token that was specified in the

measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11NoiseHistogramReportEntry 2 }

dot11NoiseHistogramRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Noise Histogram Report has been received

on. "

::= { dot11NoiseHistogramReportEntry 3 }

dot11NoiseHistogramMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Noise Histogram

report."

::= { dot11NoiseHistogramReportEntry 4 }

dot11NoiseHistogramRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which this measurement report was made."

::= { dot11NoiseHistogramReportEntry 5 }

dot11NoiseHistogramRprtRegulatoryClass OBJECT-TYPE

SYNTAX

PHYType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row of

noise histogram report has been conducted."

REFERENCE

"IEEE 802.11, Error: Reference source not found"

::= { dot11NoiseHistogramReportEntry 6 }

dot11NoiseHistogramRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11NoiseHistogramReportEntry 7 }

dot11NoiseHistogramRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Noise Histogram

Report was measured."

::= { dot11NoiseHistogramReportEntry 8 }

dot11NoiseHistogramRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Antenna ID contains the identifying number for the antenna used for

this measurement. The value 0 indicates that the antenna identifier is

unknown.  'The value 255 indicates that the measurement was made with

multiple antennas or that the antenna ID is unknown. that the antenna

identifier is unknown. The value 255 indicates that this measurement was

made with multiple antennas. The value 1 is used for a STA with only one

antenna.  STAs with more than one antenna shall assign Antenna IDs to

each antenna as consecutive, ascending numbers. Each  Antenna ID number

represents a unique antenna characterized by a fixed relative position,

a fixed relative direction and a peak gain for that position and

direction."

::= { dot11NoiseHistogramReportEntry 9 }

dot11NoiseHistogramRprtANPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

UNITS "dBm"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the ANPI for this Noise Histogram measurement.  Average Noise Power Indicator (ANPI) value represents

the average noise plus interference power on the measured channel at the antenna connector during the measurement duration To calculate ANPI, the

STA shall measure and use CNPI in the specified channel when NAV is equal

to 0 (when virtual CS mechanism indicates idle channel) except during

frame transmission or reception."

::= { dot11NoiseHistogramReportEntry 10 }

dot11NoiseHistogramRprtCNPIDensity0 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured CNPI density for non-802.11 signals

from Power <= -92dBm."

::= { dot11NoiseHistogramReportEntry 11 }

dot11NoiseHistogramRprtCNPIDensity1 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured CNPI density for non-802.11 signals

from -92dBm < Power <= -87dBm."

::= { dot11NoiseHistogramReportEntry 12 }

dot11NoiseHistogramRprtCNPIDensity2 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured CNPI density for non-802.11 signals

from -87dBm < Power <= -82dBm."

::= { dot11NoiseHistogramReportEntry 13 }

dot11NoiseHistogramRprtCNPIDensity3 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured CNPI density for non-802.11 signals

from -82dBm < Power <= -77dBm."

::= { dot11NoiseHistogramReportEntry 14 }

dot11NoiseHistogramRprtCNPIDensity4 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured CNPI density for non-802.11 signals

from -77dBm < Power <= -72dBm."

::= { dot11NoiseHistogramReportEntry 15 }

dot11NoiseHistogramRprtCNPIDensity5 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured CNPI density for non-802.11 signals

from -72dBm < Power <= -67dBm."

::= { dot11NoiseHistogramReportEntry 16 }

dot11NoiseHistogramRprtCNPIDensity6 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured CNPI density for non-802.11 signals

from -67dBm < Power <= -62dBm."

::= { dot11NoiseHistogramReportEntry 17 }

dot11NoiseHistogramRprtCNPIDensity7 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured CNPI density for non-802.11 signals

from -62dBm < Power <= -57dBm."

::= { dot11NoiseHistogramReportEntry 18 }

dot11NoiseHistogramRprtCNPIDensity8 OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the measured CNPI density for non-802.11 signals

from Power -57dBm <= Power."

::= { dot11NoiseHistogramReportEntry 19 }

dot11NoiseHistogramRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 3 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11NoiseHistogramReportEntry 20 }

-- ********************************************************************

-- * End of dot11NoiseHistogramReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11BeaconReport TABLE

-- ********************************************************************

dot11BeaconReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Beacon reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve

freshness, thus the rows in this table can be deleted for memory constraints

or other implementation constraints determined by the vendor. New rows shall

have different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase RprtIndex

for new reports being written in the table."

::= { dot11RRMReport 3 }

dot11BeaconReportEntry OBJECT-TYPE

SYNTAX Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11BeaconReportTable Indexed by dot11BeaconRprtIndex."

INDEX { dot11BeaconRprtIndex }

::= { dot11BeaconReportTable 1 }

Dot11BeaconReportEntry ::=

SEQUENCE {

dot11BeaconRprtIndex Unsigned32,

dot11BeaconRprtRqstToken OCTET STRING,

dot11BeaconRprtIfIndex InterfaceIndex,

dot11BeaconMeasuringSTAAddr MacAddress,

dot11BeaconRprtChanNumber INTEGER,

dot11BeaconRprtRegulatoryClass INTEGER,

dot11BeaconRprtActualStartTime TSFType,

dot11BeaconRprtMeasurementDuration Unsigned32,

dot11BeaconRprtPhyType INTEGER,

dot11BeaconRprtReportedFrameType INTEGER,

dot11BeaconRprtRCPI INTEGER,

dot11BeaconRprtRSNI INTEGER,

dot11BeaconRprtBSSID MacAddress,

dot11BeaconRprtAntennaID INTEGER,

dot11BeaconRprtParentTSF TSFType,

dot11BeaconRprtReportedFrameBody OCTET STRING,

dot11BeaconRprtMeasurementMode INTEGER }

dot11BeaconRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Beacon Report elements in dot11BeaconReportTable, greater than

0."

::= { dot11BeaconReportEntry 1 }

dot11BeaconRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the request token that was specified in the

measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11BeaconReportEntry 2 }

dot11BeaconRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Beacon Report has been received on."

::= { dot11BeaconReportEntry 3 }

dot11BeaconMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Beacon report."

::= { dot11BeaconReportEntry 4 }

dot11BeaconRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which this row of report detected beacons

and probe responses."

::= { dot11BeaconReportEntry 5 }

dot11BeaconRprtRegulatoryClass OBJECT-TYPE

SYNTAX PHYType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row

of beacon report has been conducted."

REFERENCE

"IEEE 802.11, Error: Reference source not found"

::= { dot11BeaconReportEntry 6 }

dot11BeaconRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11BeaconReportEntry 7 }

dot11BeaconRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Beacon Report was

measured."

::= { dot11BeaconReportEntry 8 }

dot11BeaconRprtPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

UNITS "dot11PHYType"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the PHY Type for this row of

Beacon Report."

::= { dot11BeaconReportEntry 9 }

dot11BeaconRprtReportedFrameType OBJECT-TYPE

SYNTAX INTEGER {

beaconOrProbeResponse(0),

measurementPilot(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the frame type reported in
dot11BeaconRprtReportedFrameBody"

::= { dot11BeaconReportEntry 10 }

dot11BeaconRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the beacon or

probe response frame in dBm, as defined in the RCPI measurement clause

for the indicated PHY Type."

::= { dot11BeaconReportEntry 11 }

dot11BeaconRprtRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received signal to noise ratio of the beacon or probe response frame in dBm. RSNI is the received signal to noise plus interference ratio derived from the measured RCPI for the received frame and from the measured ANPI for the channel used to received the frame. RSNI is calculated by the ratio of  the received signal power (RCPI - ANPI) over the noise plus interference power (ANPI), expressed in db (1/2 db steps), where RSNI = [(ratio(dB) + 10) * 2], for ratios in the range -10dB to +118dB.

"

::= { dot11BeaconReportEntry 12 }

dot11BeaconRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the beacon for this row of

Beacon Report."

::= { dot11BeaconReportEntry 13 }

dot11BeaconRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Antenna ID contains the identifying number for the antenna used

for this measurement. The value 0 indicates that the antenna identifier is unknown.  The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction."

::= { dot11BeaconReportEntry 14 }

dot11BeaconRprtParentTSF OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value of the serving measuring STA’s

TSF value at the time the measuring STA received the beacon or probe

response frame."

::= { dot11BeaconReportEntry 15 }

dot11BeaconRprtReportedFrameBody OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..100))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the fixed fields and information elements from the

frame body of the Beacon, Measurement Pilot or Probe Response frame being received.

All reported TIM elements are truncated to 4 octets."

::= { dot11BeaconReportEntry 16 }

dot11BeaconRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 3 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11BeaconReportEntry 17 }

-- ********************************************************************

-- * End of dot11BeaconReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11FrameReport TABLE

-- ********************************************************************

dot11FrameReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Frame reports that have been received by

the MLME. The report tables shall be maintained as FIFO to preserve

freshness, thus the rows in this table can be deleted for memory constraints

or other implementation constraints determined by the vendor. New rows shall

have different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase RprtIndex

for new reports being written in the table."

::= { dot11RRMReport 4 }

dot11FrameReportEntry OBJECT-TYPE

SYNTAX Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FrameReportTable Indexed by dot11FrameRprtIndex."

INDEX { dot11FrameRprtIndex }

::= { dot11FrameReportTable 1 }

Dot11FrameReportEntry ::=

SEQUENCE {

dot11FrameRprtIndex Unsigned32,

dot11FrameRprtIfIndex InterfaceIndex,

dot11FrameRprtRqstToken Unsigned32,

dot11FrameRprtChanNumber INTEGER,

dot11FrameRprtRegulatoryClass INTEGER,

dot11FrameRprtActualStartTime TSFType,

dot11FrameRprtMeasurementDuration Unsigned32,

dot11FrameRprtMeasuringSTAAddr MacAddress,

dot11FrameRprtBSSID MacAddress,

dot11FrameRprtRCPI INTEGER,

dot11FrameRprtRSNI INTEGER,

dot11FrameRprtAntennaID INTEGER,

dot11FrameRprtNumberFrames INTEGER,

dot11FrameRptMeasurementMode INTEGER }

dot11FrameRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Frame Report elements in dot11FrameReportTable, greater than 0."

::= { dot11FrameReportEntry 1 }

dot11FrameRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Frame Report has been received on."

::= { dot11FrameReportEntry 2 }

dot11FrameRprtRqstToken OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Frame Request elements in dot11FrameRequestTable that corresponds

to this row of frame report. Since a single frame request can generate

multiple rows in the frame report table, one per BSSID, this

dot11FrameRprtRqstToken indicates which request this particular row

corresponds to. If this row of report is received without a particular

request, this attribute should be 0"

::= { dot11FrameReportEntry 3 }

dot11FrameRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which this row of report detected frame request

responses."

::= { dot11FrameReportEntry 4 }

dot11FrameRprtRegulatoryClass OBJECT-TYPE

SYNTAX PHYType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row of

frame report has been conducted."

REFERENCE

"IEEE 802.11, Error: Reference source not found"

::= { dot11FrameReportEntry 5 }

dot11FrameRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when measurement

started."

::= { dot11FrameReportEntry 6 }

dot11FrameRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Frame Report

was measured, expressed in TUs."

::= { dot11FrameReportEntry 7 }

dot11FrameRprtMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of STA for this row of Frame report that it has been

received from."

::= { dot11FrameReportEntry 8 }

dot11FrameRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the STA that transmitted this

frame."

::= { dot11FrameReportEntry 9 }

dot11FrameRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the beacon or probe response frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type."

::= { dot11FrameReportEntry 10 }

dot11FrameRprtRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute indicates the received signal to noise ratio of the receivedframe in dBm. RSNI is the received signal to noise plus interference ratio derived from the RCPI for the received frame and from the most recent ANPI value measured on the channel used to received the frame. RSNI may be calculated by the ratio of  the received signal power (RCPI - ANPI) over the noise plus interference power (ANPI), expressed in db (1/2 db steps), where RSNI = [(ratio(dB) + 10) * 2],  for  ratios in the range -10dB to +118dB.  Other measurement techniques are allowed.”

::= { dot11FrameReportEntry 11 }

dot11FrameRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Antenna ID contains the identifying number for the antenna used for this measurement. The value 0 indicates that the antenna identifier is unknown.  The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction."

::= { dot11FrameReportEntry 12 }

dot11FrameRprtNumberFrames  OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the number of received frames in the

measurement Report Frame for this row of Frame Report."

::= { dot11FrameReportEntry 13 }

dot11FrameRptMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 3 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11FrameReportEntry 14 }

-- ********************************************************************

-- * End of dot11FrameReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11STAStatisticsReport TABLE

-- ********************************************************************

dot11STAStatisticsReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of STA Statistics reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 5 }

dot11STAStatisticsReportEntry OBJECT-TYPE

SYNTAX Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11STAStatisticsReportTable

Indexed by dot11STAStatisticsReportIndex."

INDEX { dot11STAStatisticsReportIndex }

::= { dot11STAStatisticsReportTable 1 }

Dot11STAStatisticsReportEntry ::=

SEQUENCE {

dot11STAStatisticsReportIndex Unsigned32,

dot11STAStatisticsReportToken OCTET STRING,

dot11STAStatisticsIfIndex InterfaceIndex,

dot11STAStatisticsSTAAddress MacAddress,

dot11STAStatisticsMeasurementDuration Unsigned32,

dot11STAStatisticsTransmittedFragmentCount Counter32,

dot11STAStatisticsMulticastTransmittedFrameCount Counter32,

dot11STAStatisticsFailedCount Counter32,

dot11STAStatisticsRetryCount Counter32,

dot11STAStatisticsMultipleRetryCount Counter32,

dot11STAStatisticsFrameDuplicateCount Counter32,

dot11STAStatisticsRTSSuccessCount Counter32,

dot11STAStatisticsRTSFailureCount Counter32,

dot11STAStatisticsACKFailureCount Counter32,

dot11STAStatisticsReceivedFragmentCount Counter32,

dot11STAStatisticsMulticastReceivedFrameCount Counter32,

dot11STAStatisticsFCSErrorCount Counter32,

dot11STAStatisticsTransmittedFrameCount Counter32,

dot11STAStatisticsAPServiceLoad INTEGER,

dot11STAStatisticsAverageAccessDelayBestEffort INTEGER,

dot11STAStatisticsAverageAccessDelayBackGround INTEGER,

dot11STAStatisticsAverageAccessDelayVIdeo INTEGER,

dot11STAStatisticsAverageAccessDelayVOice INTEGER,

dot11STAStatisticsStationCount INTEGER,

dot11STAStatisticsChannelUtilization INTEGER,

dot11STAStatisticsRprtMeasurementMode INTEGER }

dot11STAStatisticsReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for STA Statistics Report elements in dot11STAStatisticsReportTable,

greater than 0."

::= { dot11STAStatisticsReportEntry 1 }

dot11STAStatisticsReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the token that was specified in the measurement

request that generated this measurement report. This should be an exact

match to the original dot11RRMRqstToken attribute. Note that there may be

multiple entries in the table that match this value since a single request

may generate multiple measurement reports."

::= { dot11STAStatisticsReportEntry 2 }

dot11STAStatisticsIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the Interface that this row of STA Statistics Report has been

received on"

::= { dot11STAStatisticsReportEntry 3 }

dot11STAStatisticsSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA that returned this STA Statistics report"

::= { dot11STAStatisticsReportEntry 4 }

dot11STAStatisticsMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the STA Statistics

was measured."

::= { dot11STAStatisticsReportEntry 5 }

dot11STAStatisticsTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11TransmittedFragmentCount returned from the STA in this

STA Statistics Report"

::= { dot11STAStatisticsReportEntry 6 }

dot11STAStatisticsMulticastTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MulticastTransmittedFrameCount returned from the STA in

this STA Statistics Report"

::= { dot11STAStatisticsReportEntry 7 }

dot11STAStatisticsFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FailedCount returned from the STA in this STA Statistics

Report"

::= { dot11STAStatisticsReportEntry 8 }

dot11STAStatisticsRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RetryCount returned from the STA in this STA Statistics

Report"

::= { dot11STAStatisticsReportEntry 9 }

dot11STAStatisticsMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MultipleRetryCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 10 }

dot11STAStatisticsFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FrameDuplicateCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 11 }

dot11STAStatisticsRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RTSSuccessCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 12 }

dot11STAStatisticsRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RTSFailureCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 13 }

dot11STAStatisticsACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11ACKFailureCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 14 }

dot11STAStatisticsReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11ReceivedFragmentCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 15 }

dot11STAStatisticsMulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MulticastReceivedFrameCount returned from the STA in this

STA Statistics Report"

::= { dot11STAStatisticsReportEntry 16 }

dot11STAStatisticsFCSErrorCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FCSErrorCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 17 }

dot11STAStatisticsTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11TransmittedFrameCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 18 }

dot11STAStatisticsAPServiceLoad OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The AP Service Load shall be a scalar indication of the relative level

of service loading at an AP.  A low value shall indicate more available service capacity than a higher value.  The value 0 shall indicate that

this AP is not currently serving any STA. The value 255 shall indicate

that the AP ServiceLoad is not available.  If dot11QoSOptionImplemented

is true: the values between 0 and 254 shall beset equal to the subfield value for the Average Access Delay for the Best Effort(AADBE) within the Access Category Service Load field.  If dot11QoSOptionImplemented is

false: the values between 0 and 254 shall be a logarithmically scaled representation of the average medium access delay for DCF transmitted packets measured from the time the DCF packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start

time.  A value of 1 shall represent a 50 us delay while a value of 253

shall represent a 5.5 ms delay or any delay greater than 5.5 ms. The

value 254 shall indicate that DCF services are currently blocked.  The

AP shall measure and average the medium access delay for all transmit

packets using DCF access mechanism over a continuous thirty second measurement window.  The accuracy for the average medium access delay

shall be +/- 200 usec or better when averaged over at least 200 packets."

REFERENCE

"IEEE 802.11 – Clause 7.3.2.29"

::= { dot11STAStatisticsReportEntry 19 }

dot11STAStatisticsAverageAccessDelayBestEffort OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access Delay Best Effort element shall consist of an Average

Access Delay (AAD) for the Best Effort Access Category. The AAD shall be

a scalar indication of the Average Access Delay at a QAP for EDCF services of the Best Effort Access Category. A low value shall indicate shorter access delay than a higher value.  If the QAP is not currently providing services at the indicated AC, the AAD for this AC shall be set equal to

the AAD of the following AC (located adjacent and to the right) within the Access Category Service field.  The value 0 shall indicate that this QAP

is not currently providing services of the indicated AC or of any higher priority AC.  The values between 0 and 254 shall be a logarithmically

scaled representation of the average medium access delay for transmitted packets in the indicated AC measured from the time the EDCF packet is

ready for transmission (i.e. begins CSMA/CA access) until the actual

packet transmission start time. A value of 1 shall represent a 50 us

delay while a value of 253 shall represent a 5.5 ms delay or any delay greater than 5.5 ms. The value 254 shall indicate that services at the indicated AC are currently blocked or suspended.  The value 255 shall indicate that the AAD is not available."

REFERENCE

"IEEE 802.11 – Clause 7.3.2.29"

::= { dot11STAStatisticsReportEntry 20 }

dot11STAStatisticsAverageAccessDelayBackGround OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access Delay BackGround element shall consist of an

Average Access Delay (AAD) for the Backgound Access Category.  The AAD

shall be a scalar indication of the Average Access Delay at a QAP for

EDCF services of the Background Access Category. A low value shall

indicate shorter access delay than a higher value. If the QAP is not currently providing services at the indicated AC, the AAD for this AC

shall be set equal to the AAD of the following AC (located adjacent and

to the right) within the Access Category Service field.  The value 0

shall indicate that this QAP is not currently providing services of the indicated AC or of any higher priority AC.  The values between 0 and 254 shall be a logarithmically scaled representation of the average medium access delay for transmitted packets in the indicated AC measured from

the time the EDCF packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start time. A value of 1

shall represent a 50 us delay while a value of 253 shall represent a 5.5

ms delay or any delay greater than 5.5 ms. The value 254 shall indicate

that services at the indicated AC are currently blocked or suspended.

The value 255 shall indicate that the AAD is not available."

REFERENCE

"IEEE 802.11 – Clause 7.3.2.29"

::= { dot11STAStatisticsReportEntry 21 }

dot11STAStatisticsAverageAccessDelayVIdeo OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access Delay VIdeo element shall consist of anAverage

Access Delay (AAD) for the Video Access Category.  The AAD shall be

a scalar indication of the Average Access Delay at a QAP for EDCF

services of the Video Access Category. A low value shall indicate

shorter access delay than a higher value. If the QAP is not currently providing services at the indicated AC, the AAD for this AC shall be

set equal to the AAD of the following AC (located adjacent and to the

right) within the Access Category Service field.  The value 0 shall

indicate that this QAP is not currently providing services of the

indicated AC or of any higher priority AC.  The values between 0 and

254 shall be a logarithmically scaled representation of the average

medium access delay for transmitted packets in the indicated AC

measured from the time the EDCF packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start time.

A value of 1 shall represent a 50 us delay while a value of 253 shall represent a 5.5 ms delay or any delay greater than 5.5 ms. The value

254 shall indicate that services at the indicated AC are currently

blocked or suspended.  The value 255 shall indicate that the AAD is

not available."

REFERENCE

"IEEE 802.11 – Clause 7.3.2.29"

::= { dot11STAStatisticsReportEntry 22 }

dot11STAStatisticsAverageAccessDelayVOice OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access Delay VOice element shall consist of an Average

Access Delay (AAD) for the Voice Access Category.  The AAD shall be a

scalar indication of the Average Access Delay at a QAP for EDCF services

of the Voice Access Category. A low value shall indicate shorter access delay than a higher value. If the QAP is not currently providing

services at the indicated AC, the AAD for this AC shall be set equal to

the AAD of the following AC (located adjacent and to the right) within

the Access Category Service field.  The value 0 shall indicate that this

QAP is not currently providing services of the indicated AC or of any

higher priority AC.  The values between 0 and 254 shall be a

logarithmically scaled representation of the average medium access delay

for transmitted packets in the indicated AC measured from the time the

EDCF packet is ready for transmission (i.e. begins CSMA/CA access) until

the actual packet transmission start time. A value of 1 shall represent

a 50 us delay while a value of 253 shall represent a 5.5 ms delay or any delay greater than 5.5 ms. The value 254 shall indicate that services at

the indicated AC are currently blocked or suspended.  The value 255

shall indicate that the AAD is not available."

REFERENCE

"IEEE 802.11 – Clause 7.3.2.29"

::= { dot11STAStatisticsReportEntry 23 }

dot11STAStatisticsStationCount OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Station Count field is an unsigned integer that indicates the total number of STAs currently associated with this BSS."

::= { dot11STAStatisticsReportEntry 24 }

dot11STAStatisticsChannelUtilization OBJECT-TYPE

SYNTAX INTEGER (0..255)

UNITS "1/255"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Channel Utilization field is the percentage of time the AP sensed

the medium busy, as indicated by either the physical or virtual

carrier sense mechanism. This percentage is represented as a moving

average of ((channel busy time/(dot11ChannelUtilizationBeaconIntervals

* dot11BeaconPeriod * 1024)) *255), where ‘channel busy time’ is

defined to be the number of microseconds during which the carrier

sense mechanism, as defined in 9.2.1, has indicated a channel busy indication, and dot11ChannelUtilizationBeaconIntervals represents the

number of consecutive beacon intervals during which the average should

be calculated."

REFERENCE

"IEEE 802.11 – Clause 9.2.1"

::= { dot11STAStatisticsReportEntry 25 }

dot11STAStatisticsRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 2 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11STAStatisticsReportEntry 26 }

-- ********************************************************************

-- * End of dot11STAStatisticsReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11LCIReport TABLE

-- ********************************************************************

dot11LCIReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11LCIReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of LCI reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 6 }

dot11LCIReportEntry OBJECT-TYPE

SYNTAX Dot11LCIReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11LCIReportTable

Indexed by dot11LCIReportIndex."

INDEX { dot11LCIReportIndex }

::= { dot11LCIReportTable 1 }

Dot11LCIReportEntry ::=

SEQUENCE {

dot11LCIReportIndex Unsigned32,

dot11LCIReportToken OCTET STRING,

dot11LCIIfIndex InterfaceIndex,

dot11LCISTAAddress MacAddress,

dot11LCILatitudeResolution INTEGER,

dot11LCILatitudeInteger INTEGER,

dot11LCILatitudeFraction Unsigned32,

dot11LCILongitudeResolution INTEGER,

dot11LCILongitudeInteger INTEGER,

dot11LCILongitudeFraction Unsigned32,

dot11LCIAltitudeType INTEGER,

dot11LCIAltitudeResolution INTEGER,

dot11LCIAltitudeInteger Integer32,

dot11LCIAltitudeFraction Unsigned32,

dot11LCIDatum INTEGER,

dot11LCIRprtMeasurementMode INTEGER }

dot11LCIReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for LCI Report elements in dot11LCIReportTable,

greater than 0."

::= { dot11LCIReportEntry 1 }

dot11LCIReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the token that was specified in the measurement

request that generated this measurement report. This should be an exact

match to the original dot11RRMRqstToken attribute. Note that there may be

multiple entries in the table that match this value since a single request

may generate multiple measurement reports."

::= { dot11LCIReportEntry 2 }

dot11LCIIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the Interface that this row of LCI Report has been

received on"

::= { dot11LCIReportEntry 3 }

dot11LCISTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA that returned this LCI report"

::= { dot11LCIReportEntry 4 }

dot11LCILatitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Latitude."

::= { dot11LCIReportEntry 5 }

dot11LCILatitudeInteger OBJECT-TYPE

SYNTAX INTEGER (0..511)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction.  This field contains the

9 bits of integer portion of Latitude."

::= { dot11LCIReportEntry 6 }

dot11LCILatitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Latitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction.  This field contains the

25 bits of fraction portion of Latitude."

::= { dot11LCIReportEntry 7 }

dot11LCILongitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude resolution is 6 bits indicating the number of valid

bits in the fixed-point value of Longitude."

::= { dot11LCIReportEntry 8 }

dot11LCILongitudeInteger OBJECT-TYPE

SYNTAX INTEGER (0..511)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction.  This field contains the

9 bits of integer portion of Longitude."

::= { dot11LCIReportEntry 9 }

dot11LCILongitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Longitude is a 34 bit fixed point value consisting of 9 bits

of integer and 25 bits of fraction.  This field contains the

25 bits of fraction portion of Longitude."

::= { dot11LCIReportEntry 10 }

dot11LCIAltitudeType OBJECT-TYPE

SYNTAX INTEGER {

meters(1),

floors(2) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude Type is four bits encoding the type of altitude.

Codes defined are:

meters : in 2s-complement fixed-point 22-bit integer part

with 8-bit fraction

floors : in 2s-complement fixed-point 22-bit integer part

with 8-bit fraction"

::= { dot11LCIReportEntry 11 }

dot11LCIAltitudeResolution OBJECT-TYPE

SYNTAX INTEGER (0..63)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude resolution is 6 bits indicating the number of valid

bits in the altitude."

::= { dot11LCIReportEntry 12 }

dot11LCIAltitudeInteger OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Altitude is a 30 bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit integer

Part with 8-bit fraction.  This field contains the fixed-poing

Part of Altitude."

::= { dot11LCIReportEntry 13 }

dot11LCIAltitudeFraction OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

" Altitude is a 30 bit value defined by the Altitude type field.

The field is encoded as a 2s-complement fixed-point 22-bit integer

Part with 8-bit fraction.  This field contains the fraction part

of Altitude."

::= { dot11LCIReportEntry 14 }

dot11LCIDatum OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Datum is an eight-bit value encoding the horizontal and vertical

references used for the coordinates given in this LCI."

::= { dot11LCIReportEntry 15 }

dot11LCIRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 3 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11LCIReportEntry 16 }

-- ********************************************************************

-- * End of dot11LCIReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11QoSMetricsReport TABLE

-- ********************************************************************

dot11QoSMetricsReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11QoSMetricsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Transmit Delay Metrics reports that

have been received by the MLME. The report tables shall be maintained as

FIFO to preserve freshness, thus the rows in this table can be deleted

for memory constraints or other implementation constraints determined by

the vendor. New rows shall have different RprtIndex values than those

deleted within the range limitation of the index. One easy way is to

monotonically increase RprtIndex for new reports being written in the

table."

::= { dot11RRMReport 7 }

dot11QoSMetricsReportEntry OBJECT-TYPE

SYNTAX Dot11QoSMetricsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11QoSMetricsReportTable Indexed by dot11QoSMetricsRprtIndex."

INDEX { dot11QoSMetricsRprtIndex }

::= { dot11QoSMetricsReportTable 1 }

Dot11QoSMetricsReportEntry ::=

SEQUENCE {

dot11QoSMetricsRprtIndex Unsigned32,

dot11QoSMetricsRprtRqstToken OCTET STRING,

dot11QoSMetricsRprtIfIndex InterfaceIndex,

dot11QoSMetricsMeasuringSTAAddr MacAddress,

dot11QoSMetricsRprtActualStartTime TSFType,

dot11QoSMetricsRprtMeasurementDuration Unsigned32,

dot11QoSMetricsRprtPeerSTAAddress MacAddress,

dot11QoSMetricsRprtTID INTEGER,

dot11QoSMetricsRprtAverageQueueDelay Integer32,

dot11QoSMetricsRprtAverageTransmitDelay Integer32,

dot11QoSMetricsRprtTransmittedMSDUCount Integer32,

dot11QoSMetricsRprtMSDUDiscardedCount Integer32,

dot11QoSMetricsRprtMSDUFailedCount Integer32,

dot11QoSMetricsRprtMultipleRetryCount Integer32,

dot11QoSMetricsRprtCFPollsLostCount Integer32,

dot11QoSMetricsRprtBin0Range INTEGER,

dot11QoSMetricsRprtDelayHistogram OCTET STRING,

dot11QoSMetricsRprtMeasurementMode INTEGER }

dot11QoSMetricsRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Transmit Delay Metrics Report elements in

dot11QoSMetricsReportTable, greater than 0."

::= { dot11QoSMetricsReportEntry 1 }

dot11QoSMetricsRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the request token that was specified in the

measurement request that generated this measurement report.  This

should be an exact match to the original dot11RRMRqstToken attribute.

Note that there may be multiple entries in the table that match this

value since a single request may generate multiple measurement

reports."

::= { dot11QoSMetricsReportEntry 2 }

dot11QoSMetricsRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The InterfaceIndex for this row of TransmitQoSMetrics Report has

been received on."

::= { dot11QoSMetricsReportEntry 3 }

dot11QoSMetricsMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Transmit

Delay Metrics report."

::= { dot11QoSMetricsReportEntry 4 }

dot11QoSMetricsRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when

the measurement started."

::= { dot11QoSMetricsReportEntry 5 }

dot11QoSMetricsRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS “TUs”

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Transmit

Delay Metrics Report was measured, expressed in TUs."

::= { dot11QoSMetricsReportEntry 6 }

dot11QoSMetricsRprtPeerSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address present in the Address 1 filed of the measured

data frames for this row of Transmit QoS Metrics report."

::= { dot11QoSMetricsReportEntry 7 }

dot11QoSMetricsRprtTID OBJECT-TYPE

SYNTAX INTEGER(0..16)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Traffic Identifier shall indicate the TC or TS for which traffic is to

be measured. Values 0 through 15 are defined. Values 16-255 are reserved. .”

::= { dot11QoSMetricsReportEntry 8 }

dot11QoSMetricsRprtAverageQueueDelay OBJECT-TYPE

SYNTAX Integer32

UNITS “TUs”

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the average delay of the frames (MSDUs) that are

passed to the MAC during the measurement duration for the specified

destination and the specified Traffic Identifier. Queue Delay shall be

measured from the time the MSDU is passed to the MAC until the transmission

starts and shall be expressed in TUs."

::= { dot11QoSMetricsReportEntry 9 }

dot11QoSMetricsRprtAverageTransmitDelay OBJECT-TYPE

SYNTAX Integer32

UNITS “TUs”

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the average delay of the frames (MSDUs) that are

successfully transmitted during the measurement duration for the

specified destination and the specified Traffic Identifier. Delay shall

be measured from the time the MSDU is passed to the MAC until ACK is

received from the intermediate destination."

::= { dot11QoSMetricsReportEntry 10}

dot11QoSMetricsRprtTransmittedMSDUCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the number of MSDUs to the peer QSTA for the TC, or TS

given by the Traffic Identifier successfully transmitted in

the measurement duration”

::= {dot11QoSMetricsReportEntry 11}

dot11QoSMetricsRprtMSDUDiscardedCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the number of MSDUs to the peer QSTA for the TC, or

TS given by the Traffic Identifier discarded due either to the

number of transmit attempts exceeding dot11ShortRetryLimit or

dot11LongRetryLimit as appropriate, or due to the MSDU lifetime

having been reached”

::= {dot11QoSMetricsReportEntry 12}

dot11QoSMetricsRprtMSDUFailedCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the number of MSDUs to the peer QSTA for the TC, or TS

given by the Traffic Identifier discarded during the measurement

duration due to the number of transmit attempts exceeding

dot11ShortRetryLimit or dot11LongRetryLimit as appropriate.”

::= {dot11QoSMetricsReportEntry 13}

dot11QoSMetricsRprtMultipleRetryCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the number of MSDUs for the TC, or TS given by the

Traffic Identifier that are successfully transmitted after

more than one retransmission attempt.”

::= {dot11QoSMetricsReportEntry 14}

dot11QoSMetricsRprtCFPollsLostCount OBJECT-TYPE

SYNTAX Integer32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the number of QoS (+)CF-Poll frames transmitted to the

peer QSTA where there was no response from the QSTA. ”

::= {dot11QoSMetricsReportEntry 15}

dot11QoSMetricsRprtBin0Range OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“Contains the delay range for Bin 0 of the delay histogram, expressed

in TU.”

::= { dot11QoSMetricsReportEntry 16 }

dot11QoSMetricsRprtDelayHistogram OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (6))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute holds the histogram of delay of the frames (MSDUs)

that are successfully transmitted during the measurement duration

for the specified Traffic Identifier and the specified destination.

Delay shall be measured from the time the MSDU is passed to the MAC

until the ACK is received from the intermediate destination and shall

be expressed in TUs. "

::= { dot11QoSMetricsReportEntry 17 }

dot11QoSMetricsRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 3 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11QoSMetricsReportEntry 18 }

-- ********************************************************************

-- * End of dot11QoSMetricsReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * Radio Resource Measurement Configuration Information

-- ********************************************************************

dot11RRMConfig OBJECT IDENTIFIER ::= { dot11RadioResourceMeasurement 3 }

-- ********************************************************************

-- * dot11APChannelReport TABLE

-- ********************************************************************

dot11APChannelReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11APChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"AP Channel Report information, in tabular form."

::= { dot11RRMConfig 1 }

dot11APChannelReportEntry OBJECT-TYPE

SYNTAX Dot11APChannelReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11APChannelReportTable. Each entry in the table is

indexed by dot11APChannelReportIndex."

INDEX { dot11APChannelReportIndex }

::= { dot11APChannelReportTable 1 }

Dot11APChannelReportEntry ::=

SEQUENCE {

dot11APChannelReportIndex Unsigned32,

dot11APChannelReportIfIndex InterfaceIndex,

dot11APChannelReportRegulatoryClass INTEGER,

dot11APChannelReportChannelList OCTET STRING,

dot11APChannelReportMeasurementMode INTEGER }

dot11APChannelReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for AP channel report entry in dot11APChannelReportTable, greater

than 0."

::= { dot11APChannelReportEntry 1 }

dot11APChannelReportIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex this row of AP channel report entry belongs to."

::= { dot11APChannelReportEntry 2 }

dot11APChannelReportRegulatoryClass OBJECT-TYPE

SYNTAX PHYType

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This corresponds to the frequency band for the channel list

being reported."

REFERENCE

"IEEE 802.11, Error: Reference source not found"

::= { dot11APChannelReportEntry 3 }

dot11APChannelReportChannelList OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(16))

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This attribute corresponds to the specific channels being reported.

Each octet corresponds to a different channel within the specified

Regulatory Class"

::= { dot11APChannelReportEntry 4 }

dot11APChannelReportMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 3 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11APChannelReportEntry 5 }

-- ********************************************************************

-- * End of dot11APChannelReportTable TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11RRMNeighborReport TABLE

-- ********************************************************************

dot11RRMNeighborReportNextIndex OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Identifies the next available index for managing the Neighbor Report table.

If this attribute is 0, it indicates that the Neighbor Report feature is not

configurable via SNMP, or the table is full and new rows cannot be

accepted."

::= { dot11RRMConfig 2 }

dot11RRMNeighborReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11RRMNeighborReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains pertinent information on a collection of BSSID’s that are

candidates to which STA’s can roam. The rows are created using createAndWait

method and fill in the attributes. When the rowStatus is set to active, the

row can be included in Neighbor Report IEs. If there is an error, the

rowStatus shall be set to notReady by SME. Since this table contains all

Neighbor Report IE entries for all interfaces enabled with the Neighbor

Report feature, it is possible to have too many entries for one interface,

while still remaining under the MaxTableSize. In that situation, SME shall

only include Neighbor Report entries with lower

dot11RRMNeighborReportIFIndex up to the maximum possible number of entries

for a particular interface identified by ifIndex.

SME shall set the rowStatus to notInService for those rows that cannot be

included in the Neighbor Report element for that interface."

::= { dot11RRMConfig 3 }

dot11RRMNeighborReportEntry OBJECT-TYPE

SYNTAX Dot11RRMNeighborReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11RRMNeighborReportTable"

INDEX { dot11RRMNeighborReportIndex }

::= { dot11RRMNeighborReportTable 1 }

Dot11RRMNeighborReportEntry ::=

SEQUENCE {

dot11RRMNeighborReportIndex Unsigned32,

dot11RRMNeighborReportIfIndex InterfaceIndex,

dot11RRMNeighborReportBSSID MacAddress,

dot11RRMNeighborReportReachability INTEGER,

dot11RRMNeighborReportSecurity TruthValue,

dot11RRMNeighborReportCapSpectrumMgmt TruthValue,

dot11RRMNeighborReportCapQoS TruthValue,

dot11RRMNeighborReportCapAPSD TruthValue,

dot11RRMNeighborReportCapRRM TruthValue,

dot11RRMNeighborReportCapBlockAck TruthValue,

dot11RRMNeighborReportKeyScope TruthValue,

dot11RRMNeighborReportChannelNumber INTEGER,

dot11RRMNeighborReportRegulatoryClass INTEGER,

dot11RRMNeighborReportPhyType	INTEGER,

dot11RRMNeighborReportNeighborTSFInfo Unsigned32,

dot11RRMNeighborReportTSFOffsetFlag TruthValue,

dot11RRMNeighborReportBeaconInterval Unsigned32,

dot11RRMNeighborReportRowStatus RowStatus,

dot11RRMNeighborReportMeasurementMode INTEGER }

dot11RRMNeighborReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Neighbor Report configuration table in

dot11RRMNeighborReportTable, greater than 0."

::= { dot11RRMNeighborReportEntry 1 }

dot11RRMNeighborReportIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The ifIndex for this row of Neighbor Report entry belongs to."

::= { dot11RRMNeighborReportEntry 2 }

dot11RRMNeighborReportBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The BSSID of the STA to which this row of Neighbor Report applies."

::= { dot11RRMNeighborReportEntry 3 }

dot11RRMNeighborReportReachability OBJECT-TYPE

SYNTAX INTEGER {

notReachable(1),

unknown(2),

reachable(3)

}

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the reachability of the AP represented by

the dot11NeighborReportBSSID."

::= { dot11RRMNeighborReportEntry 4 }

dot11RRMNeighborReportSecurity OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the RSN capability of the AP

represented by dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 5 }

dot11RRMNeighborReportCapSpectrumMgmt OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the spectrum management capability of the AP

represented by dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 6 }

dot11RRMNeighborReportCapQoS OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"Indicates the QoS capability of the AP represented by

dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 7 }

dot11RRMNeighborReportCapAPSD OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the APSD capability of the AP represented by

dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 8 }

dot11RRMNeighborReportCapRRM OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the RRM capability of the AP represented by

dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 9 }

dot11RRMNeighborReportCapBlockAck OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the BlockAck capability of the AP represented

by dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 10 }

dot11RRMNeighborReportKeyScope OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates a shared authenticator identity for the AP represented by dot11NeighborReportBSSID"

::= { dot11RRMNeighborReportEntry 11 }

dot11RRMNeighborReportChannelNumber OBJECT-TYPE

SYNTAX INTEGER (1..255)

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This is the current operating channel of the STA returning the report"

::= { dot11RRMNeighborReportEntry 12 }

dot11RRMNeighborReportRegulatoryClass OBJECT-TYPE

SYNTAX PHYType

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"This corresponds to the frequency band for the channel list being

reported."

REFERENCE

"IEEE 802.11, Error: Reference source not found"

::= { dot11RRMNeighborReportEntry 13 }

dot11RRMNeighborReportPhyType OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The PHY Type of the Neighbor AP Entry."

::= { dot11RRMNeighborReportEntry 14 }

dot11RRMNeighborReportNeighborTSFInfo OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Neighbor TBTT Offset field is as defined in clause 7.3.2.26."

::= { dot11RRMNeighborReportEntry 15 }

dot11RRMNeighborReportTSFOffsetFlag OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Indicates the TSFOffset is included in the Neighbor Report entry.”

::= { dot11RRMNeighborReportEntry 16 }

dot11RRMNeighborReportBeaconInterval OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"The Beacon Interval field is as defined in clause 7.3.2.27."

::= { dot11RRMNeighborReportEntry 17 }

dot11RRMNeighborReportRowStatus OBJECT-TYPE

SYNTAX RowStatus

MAX-ACCESS read-create

STATUS current

DESCRIPTION

"Contains the row status of the Neighbor Report, essentially used for

indicating whether the row has all valid attributes filled in. Then set to

active to be used in Neighbor Report information elements. If any parameter

is invalid, the SME shall set this attribute back to notReady. It is the

responsibility of the manager to correct the parameters."

::= { dot11RRMNeighborReportEntry 18 }

dot11RRMNeighborReporMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 3 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11RRMNeighborReportEntry 19 }

-- ********************************************************************

-- * End of dot11RRMNeighborReport TABLE

-- ********************************************************************

In dot11OperationTable of Annex D, insert the following text to the end of Dot11OperationEntry sequence list:

dot11PeerStatsTableBSSaging Unsigned32,

dot11PeerStatsTableIBSSaging Unsigned32 }

Insert the following elements to the end of dot11OperationTable element definition behind dot11ProductID:

dot11PeerStatsTableBSSaging OBJECT-TYPE

SYNTAX Unsigned32

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The peerStatsTableBSSaging indicates the period of time

which entries for peer STAs that are in the peerStatsTable

because the peer STA was authenticated or associated with the

local STA shall be kept in the table after the peer STA

becomes disassociated. The peer STA entry may be kept in the

peerStatsTable for longer than this time."

DEFVAL { 300 }

::= { dot11OperationEntry 10 }

dot11PeerStatsTableIBSSaging OBJECT-TYPE

SYNTAX Unsigned32

UNITS "seconds"

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The peerStatsTableIBSSaging indicates the period of time

which entries for peer STAs that are in the peerStatsTable

because the peer STA was in an IBSS that the local STA was

joined to, and send a data frame to the local STA, or the

local STA attempted to send any frame to the peer STA shall

be kept in the table after the peer STA last communicated

with the local STA. The peer STA entry may be kept in the

peerStatsTable for longer than this time."

DEFVAL { 300 }

::= { dot11OperationEntry 11 }

In dot11MAC MIB of Annex D, insert the peerStatsTable behind GroupAddressesTable:

--**********************************************************************

--* dot11PeerStatsTable TABLE

--**********************************************************************

dot11PeerStatsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11PeerStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This table contains statistics for peer STAs. The following peer STAs

appear in this table:

1. Authenticated peer STAs must appear in this table. Their

entries must not be removed until at least

dot11PeerStatsTableBSSaging seconds have passed after they are

marked as deauthenticated.

2. Peer STAs joined to the same IBSS as this STA that have

sent any data frame to this STA must appear in this table.

Entries for these peer STAs may be removed when this STA is

no longer joined to the same IBSS, or if dot11PeerStatsTableIBSSaging

seconds have passed since a data frame was exchanged with the peer STA.

3. Other peer STAs may be included in this table."

::= { dot11mac 4 }

dot11PeerStatsEntry OBJECT-TYPE

SYNTAX Dot11PeerStatsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the Dot11PeerStaStatsEntry Table.

Indexed by ifIndex and the MAC address of the remote

STA dot11PeerStaAddress"

INDEX { ifIndex, dot11PeerAddress }

::= { dot11PeerStatsTable 1 }

Dot11PeerStatsEntry ::=

SEQUENCE {

dot11PeerAddress MacAddress,

dot11PeerTransmittedFragmentCount Counter32,

dot11PeerMulticastTransmittedFrameCount Counter32,

dot11PeerFailedCount Counter32,

dot11PeerRetryCount Counter32,

dot11PeerMultipleRetryCount Counter32,

dot11PeerFrameDuplicateCount Counter32,

dot11PeerRTSSuccessCount Counter32,

dot11PeerRTSFailureCount Counter32,

dot11PeerACKFailureCount Counter32,

dot11PeerReceivedFragmentCount Counter32,

dot11PeerMulticastReceivedFrameCount Counter32,

dot11PeerTransmittedFrameCount Counter32,

dot11PeerWEPUndecryptableCount Counter32,

dot11PeerWEPICVErrorCount Counter32,

dot11PeerReceivedOctetCount Counter32,

dot11PeerTransmittedOctetCount Counter32,

dot11PeerReceivedBeaconCount Counter32,

dot11PeerLastReceivedRCPI INTEGER }

dot11PeerAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"MAC address identifying a peer STA."

::= { dot11PeerStatsEntry 1 }

dot11PeerTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for an acknowledged MPDU

with an individual address in the address 1 field or an MPDU

with a multicast address in the address 1 field of type Data

or Management."

::= { dot11PeerStatsEntry 2 }

dot11PeerMulticastTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment only when the multicast bit

is set in the destination MAC address of a successfully

transmitted MSDU. When operating as a non-AP STA in an ESS, where

these frames are directed to the AP, this implies having

received an acknowledgment to all associated MPDUs."

::= { dot11PeerStatsEntry 3 }

dot11PeerFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is not transmitted

successfully due to the number of transmit attempts exceeding

either the dot11ShortRetryLimit or dot11LongRetryLimit."

::= { dot11PeerStatsEntry 4 }

dot11PeerRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted after one or more retransmissions."

::= { dot11PeerStatsEntry 5 }

dot11PeerMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU is successfully

transmitted after more than one retransmission."

::= { dot11PeerStatsEntry 6 }

dot11PeerFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received

that the Sequence Control field indicates is a duplicate."

::= { dot11PeerStatsEntry 7 }

dot11PeerRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is received in

response to an RTS."

::= { dot11PeerStatsEntry 8 }

dot11PeerRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a CTS is not received in

response to an RTS."

::= { dot11PeerStatsEntry 9 }

dot11PeerACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an ACK is not received

when expected."

::= { dot11PeerStatsEntry 10 }

dot11PeerReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for each successfully

received MPDU of type Data or Management."

::= { dot11PeerStatsEntry 11 }

dot11PeerMulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a MSDU is received

with the multicast bit set in the destination MAC address."

::= { dot11PeerStatsEntry 12 }

dot11PeerTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each successfully

transmitted MSDU."

::= { dot11PeerStatsEntry 13 }

dot11PeerWEPUndecryptableCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received with

the WEP subfield of the Frame Control field set to one and the

WEPOn value for the key mapped to the TA's MAC address

indicates that the frame should not have been encrypted

that frame is discarded due to the receiving STA not

implementing the privacy option."

::= { dot11PeerStatsEntry 14 }

dot11PeerWEPICVErrorCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame is received with the

WEP subfield of the Frame Control field set to one and the value

of the ICV as received in the frame does not match the ICV value

that is calculated for the contents of the received frame."

::= { dot11PeerStatsEntry 15 }

dot11PeerReceivedOctetCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Total number of octets of data in MSDUs received from this peer."

::= { dot11PeerStatsEntry 16 }

dot11PeerTransmittedOctetCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Total number of octets of data in unicast MSDUs successfully transmitted

to this peer."

::= { dot11PeerStatsEntry 17 }

dot11PeerReceivedBeaconCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Number of beacon frames received from this peer."

::= { dot11PeerStatsEntry 18 }

dot11PeerLastReceivedRCPI OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The RCPI from the most recently received frame, or 0 if no

frames have been received."

::= { dot11PeerStatsEntry 19 }

-- ********************************************************************

-- * End of dot11PeerStats TABLE

-- ********************************************************************

Insert the following compliance groups as appropriate in Compliance Statements:

dot11RRMCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the

IEEE 802.11 MIB for Measurement Services."

MODULE -- this module

MANDATORY-GROUPS {

dot11SMTRRMRequest,

dot11SMTRRMReport,

dot11SMTRRMConfig

}

-- OPTIONAL-GROUPS { }

::= { dot11Compliances 3 }

Insert the following at the end of the section “Groups – units of conformance”:

dot11SMTRRMRequest OBJECT-GROUP

OBJECTS {   dot11RRMRqstIndex,

dot11RRMRqstRowStatus,

dot11RRMRqstToken,

dot11RRMRqstIfIndex,

dot11RRMRqstType,

dot11RRMRqstTargetAdd,

dot11RRMRqstTimeStamp,

dot11RRMRqstChanNumber,

dot11RRMRqstRegulatoryClass,

dot11RRMRqstRndInterval,

dot11RRMRqstDuration,

dot11RRMRqstParallel,

dot11RRMRqstEnable,

dot11RRMRqstRequest,

dot11RRMRqstReport,

dot11RRMRqstDurationMandatory,

dot11RRMRqstBeaconRqstMode,

dot11RRMRqstBssid,

dot11RRMRqstReportingCondition,

dot11RRMRqstThresholdOffset,

dot11RRMRqstSTAStatRqstGroupID }

STATUS current

DESCRIPTION

"The SMTRRMRequest package is a set of attributes that shall be

present if the STA supports the Radio Measurement service."

::= { dot11Groups 29 }

dot11SMTRRMReport OBJECT-GROUP

OBJECTS {   dot11ChannelLoadRprtIndex,

dot11ChannelLoadRprtRqstToken,

dot11ChannelLoadRprtIfIndex,

dot11ChannelLoadMeasuringSTAAddr,

dot11ChannelLoadRprtChanNumber,

dot11ChannelLoadRprtChanBand,

dot11ChannelLoadRprtActualStartTime,

dot11ChannelLoadRprtMeasurementDuration,

dot11ChannelLoadRprtCcaBusyFraction,

dot11ChannelLoadRprtChannelLoad,

dot11NoiseHistogramRprtIndex,

dot11NoiseHistogramRprtRqstToken,

dot11NoiseHistogramRprtIfIndex,

dot11NoiseHistogramMeasuringSTAAddr,

dot11NoiseHistogramRprtChanNumber,

dot11NoiseHistogramRprtChanBand,

dot11NoiseHistogramRprtActualStartTime,

dot11NoiseHistogramRprtMeasurementDuration,

dot11NoiseHistogramRprtCNPIDensity0,

dot11NoiseHistogramRprtCNPIDensity1,

dot11NoiseHistogramRprtCNPIDensity2,

dot11NoiseHistogramRprtCNPIDensity3,

dot11NoiseHistogramRprtCNPIDensity4,

dot11NoiseHistogramRprtCNPIDensity5,

dot11NoiseHistogramRprtCNPIDensity6,

dot11NoiseHistogramRprtCNPIDensity7,

dot11NoiseHistogramRprtCNPIDensity8,

dot11BeaconRprtIndex,

dot11BeaconRprtRqstToken,

dot11BeaconRprtIfIndex,

dot11BeaconMeasuringSTAAddr,

dot11BeaconRprtChanNumber,

dot11BeaconRprtChanBand,

dot11BeaconRprtActualStartTime,

dot11BeaconRprtMeasurementDuration,

dot11BeaconRprtPhyType,

dot11BeaconRprtRCPI,

dot11BeaconRprtBSSID,

dot11BeaconRprtParentTSF,

dot11BeaconRprtTargetTSF,

dot11BeaconRprtBeaconInterval,

dot11BeaconRprtCFStatus,

dot11BeaconRprtPrivacy,

dot11BeaconRprtBSSMode,

dot11BeaconRprtReceivedElements,

dot11STAStatisticsReportIndex,

dot11STAStatisticsReportToken,

dot11STAStatisticsIfIndex,

dot11STAStatisticsSTAAddress,

dot11STAStatisticsMeasurementDuration,

dot11STAStatisticsTransmittedFragmentCount,

dot11STAStatisticsMulticastTransmittedFrameCount,

dot11STAStatisticsFailedCount,

dot11STAStatisticsRetryCount,

dot11STAStatisticsMultipleRetryCount,

dot11STAStatisticsFrameDuplicateCount,

dot11STAStatisticsRTSSuccessCount,

dot11STAStatisticsRTSFailureCount,

dot11STAStatisticsACKFailureCount,

dot11STAStatisticsReceivedFragmentCount,

dot11STAStatisticsMulticastReceivedFrameCount,

dot11STAStatisticsFCSCount,

dot11STAStatisticsTransmittedFrameCount,

dot11QoSMetricsRprtIndex Unsigned32,

dot11QoSMetricsRprtRqstToken OCTET STRING,

dot11QoSMetricsRprtIfIndex InterfaceIndex,

dot11QoSMetricsMeasuringSTAAddr MacAddress,

dot11QoSMetricsRprtActualStartTime TSFType,

dot11QoSMetricsRprtMeasurementDuration Unsigned32,

dot11QoSMetricsRprtPeerSTAAddress MacAddress,

dot11QoSMetricsRprtTID INTEGER,

dot11QoSMetricsRprtAverageQueueDelay Integer32,

dot11QoSMetricsRprtAverageTransmitDelay Integer32,

dot11QoSMetricsRprtTransmittedMSDUCount Integer32,

dot11QoSMetricsRprtMSDUDiscardedCount Integer32,

dot11QoSMetricsRptMSDUFailedCount Integer32,

dot11QoSMetricsRptMultipleRetryCount Integer32,

dot11QoSMetricsRptCFPollsLostCount Integer32,

dot11QoSMetricsRprtBin0Range INTEGER,

dot11QoSMetricsRprtDelayHistogram OCTET STRING,

dot11QoSMetricsRprtMeasurementMode INTEGER,

dot11QoSMetricsRprtIndex Unsigned32,

dot11QoSMetricsRprtRqstToken OCTET STRING,

dot11QoSMetricsRprtIfIndex InterfaceIndex,

dot11QoSMetricsMeasuringSTAAddr MacAddress,

dot11QoSMetricsRprtActualStartTime TSFType,

dot11QoSMetricsRprtMeasurementDuration Unsigned32,

dot11QoSMetricsRprtPeerSTAAddress MacAddress,

dot11QoSMetricsRprtTID INTEGER,

dot11QoSMetricsRprtAverageQueueDelay Integer32,

dot11QoSMetricsRprtAverageTransmitDelay Integer32,

dot11QoSMetricsRprtTransmittedMSDUCount Integer32,

dot11QoSMetricsRprtMSDUDiscardedCount Integer32,

dot11QoSMetricsRptMSDUFailedCount Integer32,

dot11QoSMetricsRptMultipleRetryCount Integer32,

dot11QoSMetricsRptCFPollsLostCount Integer32,

dot11QoSMetricsRprtBin0Range INTEGER,

dot11QoSMetricsRprtDelayHistogram OCTET STRING,

dot11QoSMetricsRprtMeasurementMode INTEGER }

STATUS current

DESCRIPTION

"The SMTRRMReport package is a set of attributes that shall be

present if the STA supports the Radio Measurement service."

::= { dot11Groups 30 }

dot11SMTRRMConfig OBJECT-GROUP

OBJECTS {   dot11APChannelReportIndex,

dot11APChannelReportIfIndex,

dot11APChannelReportRegulatoryClass,

dot11APChannelReportChannelList,

dot11APChannelReporMeasurementMode

dot11RRMNeighborReportIndex,

dot11RRMNeighborReportIfIndex,

dot11RRMNeighborReportReachability,

dot11RRMNeighborReportSecurity,

dot11RRMNeighborReportCapSpectrumMgmt,

dot11RRMNeighborReportCapQoS,

dot11RRMNeighborReportCapAPSD,

dot11RRMNeighborReportCapRRM,

dot11RRMNeighborReportCapBlockAck,

dot11RRMNeighborReportKeyScope,

dot11RRMNeighborReportChannelNumber,

dot11RRMNeighborReportRegulatoryClass,

dot11RRMNeighborReportPhyOptions,

dot11RRMNeighborReportNeighborTSFInfo,

dot11RRMNeighborReportTSFOffsetFlag,

dot11RRMNeighborReportBeaconInterval,

dot11RRMNeighborReportRowStatus,

dot11RRMNeighborReportMeasurementMode }

STATUS current

DESCRIPTION

"The SMTRRMConfig package is a set of attributes that shall be

present if the STA supports the Radio Measurement service."

::= { dot11Groups 31 }