IEEE P802.11
Wireless LANs

| LB78 Normative Text Changes for Comments Assigned to Joe Kwak |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-JAN-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | joekwak@sbcglobal.net |

Abstract

This document contains normative text changes to TGk draft to incorporate all the "accepted" and "counter" comment resolutions assigned to Kwak, as described in 06/0120rx, where x is the same rev number as the rev number for this document.   Changes are shown with respect to TGk draft D3.0.

# Draft Amendment to STANDARD FOR
Telecommunications and Information Exchange Between Systems -
LAN/MAN Specific Requirements - 

Part 11: Wireless Medium Access Control (MAC) and physical layer (PHY) specifications:

Amendment 7: Radio Resource Measurement

[This amendment is based on IEEE Std 802.11™, 1999 Edition (Reaff 2003), as amended by IEEE Std 802.11a™-1999 (Reaff 2003), 802.11b™-1999 (Reaff 2003), 802.11b™-1999/Cor 1-2001 (Reaff 2003), 802.11d™-2001 (Reaff 2003), 802.11e™-2005, IEEE Std 802.11g™-2003, IEEE Std 802.11h™-2003, IEEE Std 802.11i™-2004 and IEEE Std 802.11j™-2004.]

NOTE—The editing instructions contained in this amendment define how to merge the material contained herein into the existing base standard to form the new comprehensive standard.

The editing instructions are shown in bold italic. Three editing instructions are used: change, delete, and insert. Change is used to make small corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed either by using strikethrough (to remove old material) or underscore (to add new material). Delete removes existing material. Insert adds new material with-out disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Editorial notes will not be carried over into future editions.

# 2 Normative references

Insert the following new definition in alphabetical order, renumbering as necessary:

IETF RFC 3825, Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information, Polk, J., Schnizlein, J., Linsner, M., July 2004

# 3. Definitions

Insert the following new definitions in alphabetical order, renumbering as necessary:

3.95 neighbor AP: Any validated AP that is a potential transition candidate.

3.96 non-serving channel: A channel that is not the operating channel of the BSS of which the STA is a member.

3.97 received channel power indicator (RCPI): An indication of the total channel power (signal, noise, and interference) of a received IEEE 802.11 frame measured at the antenna connector used to receive the frame

3.98 received signal to noise indicator (RSNI): An indication of the signal to noise plus interference ratio of a received IEEE 802.11 frame. RSNI is defined by the ratio of the received signal power (RCPI-ANPI) to the noise plus interference power (ANPI) as measured at the antenna connector used to receive the frame.

3.99 average noise power indicator (ANPI):  An indication of the average noise plus interference power measured on a channel when NAV is equal to 0 (when virtual CS mechanism indicates idle channel) excluding periods of frame transmission or reception.

3.100 serving channel: The operating channel of the BSS of which the STA is a member.

3.101 location configuration information (LCI): As defined in IETF RFC 3825, includes latitude, longitude, and altitude, with resolution indicators for each.

3.102 received power indicator (RPI): An indication of the total channel power (noise and interference) as measured in the current channel at the currently-in-use receiving antenna connector while the STA is neither transmitting nor receiving a frame.

3.103 ap reachablility: An AP is reachable by a STA if an 802.1X pre-authentication frame sent by the STA to the AP BSSID can be received by the AP.

3.104 validated neighbor: an AP that has either been explictedly configured as a Neighbor in the MIB, or learned through a mechanism like the Beacon Report and confirmed through trusted mechanisms such as a secure Inter-Access Point Protocol (IAPP).

3.105 serving AP: The AP which transmits beacons on the serving channel.

3.106 currently in use antenna: The antenna used for a particular noise or frame measurement. For frame measurements of RCPI, the currently in use antenna is the antenna used to measure the reported RCPI for the body of the frame.

# 4. Abbreviations and acronyms

Insert the following new acronym in alphabetical order:

ANPI		average noise power indicator
LCI		location configuration information
TMPTT		target measurement pilot transmission time
RCPI		received channel power indicator
RSNI		received signal to noise indicator
RFC		request for comments

# 7. Frame formats

## 7.1 MAC frame formats

### 7.1.3 Frame fields

#### 7.1.3.1 Frame Control field

##### 7.1.3.1.2 Type and Subtype fields

Change the contents of Table 1 as shown:

| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 00 | Management | 0101 | Probe response |
| 00 | Management | 0110-0111 | ReservedMeasurement Pilot |
| 00 | Management | 0111 | Reserved |
| 00 | Management | 1000 | Beacon |

## 7.2 Format of individual frame types

### 7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Change the first paragraph, and table 5 as shown:

The frame body of a management frame of subtype Beacon contains the information shown in Table 5. If the dot11MultiDomainCapabilityEnabled attribute is true, a STA shall include a Country information element in the transmission of Beacon frames. Optionally, the Beacon frame format may also include the information described in either or both of orders 12 and 13. If the information in both orders 12 and 13 are sent, they shall describe the same hopping pattern. Note that the information described in orders 12 and 13 also may be contained in the Probe Response frame.

Table 5—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 11 | Country | The Country element shall be present if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true |
| 14 | Power Constraint | Power Constraint element shall be present if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true |
| 18 | TPC Report | TPC Report element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true |
| 24 | AP Channel Report | The AP Channel Report element shall be present if dot11RadioMeasurementEnabled is true and there is at least 1 channel to report. |
| 25 | BSS Load | The BSS Load information element shall be present only in a non-QAP and only if dot11RadioMeasurementEnabled true |
| 26 | Antenna Information | The Antenna Information element shall be present if dot11RadioMeasurementEnabled is true. |

#### 7.2.3.4 Association Request frame format

Change the order 6 information field as follows:

Table 7—Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 6 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true |

#### 7.2.3.5 Association Response frame format

Insert a new row into table 8 as shown below:

Table 8—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 8 | RCPI | The RCPI information element shall be present if dot11RadioMeasurementEnabled is true. The RCPI value represents the measured RCPI of the corresponding Association Request frame. |

#### 7.2.3.6 Reassociation Request frame format

Change the order 7 information field as follows:

Table 9—Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 7 | Power Capability | Power Capability element shall be present if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true |

#### 7.2.3.7 Reassociation Response frame format

Insert new row into table 10 as follows:

Table 10—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 5 | RCPI | The RCPI information element shall be present if dot11RadioMeasurementEnabled is true. The RCPI value represents the measured RCPI of the corresponding Reassociation Request frame. |

#### 7.2.3.8 Probe Request frame format

Insert order 4 element into Table 11 as shown below:

Table 11—Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 4 | DS Parameter Set | The DS Parameter Set information element shall be present within Probe Request frames generated by STAs using direct sequence PHYs with dot11RadioMeasurementEnabled set to true. The DS Parameter Set information element may be present within Probe Request frames generated by STAs using direct sequence PHYs with dot11RadioMeasurementEnabled set to false. |

#### 7.2.3.9 Probe Response frame format

Change the first paragraph as follows:

The frame body of a management frame of subtype Probe Response contains the information shown in Table 12. If the dot11MultiDomainCapabilityEnabled attribute is true, the Probe Response frame contains a Country information element and all information elements identified by the Requested Element IDs of a Request information element. Note that the information returned as a result of a Probe Request frame with a Request information element may include the FH parameters and/or the FH Pattern Table possibly replicating optional elements identified by orders 12 and 13. When a probe response frame is returned in response to a probe request frame which contained a Request information element, any of the requested elements which appear as individual items in the ordering list of table 12 shall appear both in their individual ordered location as specified in table 12 and in the ordered location reserved for the list of requested elements, where the requested elements appear in increasing numerical element ID order.

Change the order 10, 13 and 17 information fields, insert the order 22 and 23 information field and change the starting order of the Requested information elements field in Table 12 as follows:

Table 12—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 10 | Country | Included if dot11MultiDomainCapabilityEnabled is true or dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 13 | Power Constraint | Shall be included if dot11SpectrumManagementRequired is true and may be present if dot11RadioMeasurementEnabled is true. |
| 17 | TPC Report | Shall be included if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. |
| 22 | AP Channel Report | Shall be included if dot11RadioMeasurementEnabled is true. |
| 23 | BSS Load | Shall be included only in a non-QAP and only if dot11RadioMeasurementEnabled is true. |
| 24 | Antenna Information | The Antenna Information element shall be present if dot11RadioMeasurementEnabled is true. |
| 2225-n | Requested information elements | Elements requested by the Request information element of the Probe Request frame. |

Insert the following clause after clause 7.2.3.9:

#### 7.2.3.10 Measurement Pilot frame format

The frame body of a management frame of subtype Measurement Pilot contains the information shown in Table k1.

Table k1 – Measurement Pilot frame body

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Timestamp |  |
| 2 | Measurement Pilot Interval |  |
| 3 | Beacon Interval |  |
| 4 | Capability Information |  |
| 5 | RSN Capabilities |  |
| 6 | Country String |  |
| 7 | Max Regulatory Power |  |
| 8 | Max Transmit Power |  |
| 9 | Transmit Power Used |  |
| 10 | Transceiver Noise Floor |  |
| 11 | DS Parameter Set | The DS Parameter Set information element is present within Measurement Pilot frames generated by STAs using direct sequence PHYs. |

## 7.3 Management frame body components

### 7.3.1 Fixed fields

#### 7.3.1.4 Capability Information field

Change the contents of Figure 27 as follows:

| B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ESS | IBSS | CF Pollable | CF Poll Request | Privacy | Short Preamble | PBCC | Channel Agility |
| B8 | B9 | B10 | B11 | B12 | B13 | B14 | B15 |
| Spectrum Mgmt | QoS | Short Slot Time | APSD | Reserved Radio Measurement | DSSS-OFDM | Delayed Block Ack | Immediate Block Ack |

Figure 27—Capability Information fixed field

Insert the following text at the end of 7.3.1.4:

A STA shall set the Radio Measurement subfield in the Capability Information field to 1 when the MIB attribute dot11RadioMeasurementEnabled is true, otherwise it shall be set to 0.

#### 7.3.1.11 Action field

Insert the following new row into table 19a and update the reserved value as shown:

Table 19a—Category values

| Name | Value | See clause |
| --- | --- | --- |
| Radio measurement | 5 | 7.4.5 |
| Reserved | 6-127 | - |

Insert the following new clauses:

#### 7.3.1.18 Country String

The Country String field is 3 octets in length. The STA shall set this field to the value contained in the dot11CountryString attribute. The Country String field is illustrated in Figure k1.

|  |  | B0 | B23 |
| --- | --- | --- | --- |
|  | Country String |  |  |
| Octets: | 3 |  |  |

Figure k1—Country String fixed field

#### 7.3.1.19 Measurement Pilot Interval field

The Measurement Pilot Interval field represents the number of time units (TUs) between target measurement pilot transmission times (TMPTTs). The length of the Measurement Pilot Interval field is 2 octets. The Measurement Pilot Interval field is illustrated in Figure k2.

|  |  | B0 | B15 |
| --- | --- | --- | --- |
|  | Measurement Pilot Interval |  |  |
| Octets: | 2 |  |  |

Figure k2—Measurement Pilot Interval fixed field

#### 7.3.1.20 Max Regulatory Power field

The Max Regulatory Power field is a signed integer and is 1 octet in length. It shall indicate the maximum power, in units of dBm, a STA is allowed by the regulatory authority to transmit on the current channel. The Max Regulatory Power field is illustrated in Figure k3.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | Max Regulatory Power |  |  |
| Octets: | 1 |  |  |

Figure k3—Max Regulatory Power fixed field

#### 7.3.1.21 Max Transmit Power field

The Max Transmit Power field is a signed integer and is 1 octet in length. When set by an STA, it provides an upper limit, in units of dBm, on the transmit power as measured at the output of the antenna connector to be used by that STA on the current channel. The value of the Max Transmit Power field shall be less than or equal to the Max Regulatory Power value for the current channel. The Max Transmit Power field is illustrated in Figure k4.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | Max Transmit Power |  |  |
| Octets: | 1 |  |  |

Figure k4—Max Transmit Power fixed field

#### 7.3.1.22 Transmit Power Used field

The Transmit Power Used field is a signed integer and is 1 octet in length. It shall be less than or equal to the Max Transmit Power and indicates the actual power used as measured at the output of the antenna connector, in units of dBm, by a STA when transmitting the frame containing the Transmit Power Used field. The Transmit Power Used field is illustrated in Figure k5.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | Transmit Power Used |  |  |
| Octets: | 1 |  |  |

Figure k5—Transmit Power Used fixed field

#### 7.3.1.23 Transceiver Noise Floor field

The Transceiver Noise Floor field is a signed integer and is 1 octet in length. It shall indicate the noise floor of the receiver used by the STA transmitting the measurement pilot frame in units of dBm. The transceiver noise floor is referenced to the connector of the currently in-use receiving antenna. The Transceiver Noise Floor field is illustrated in Figure k6.

|  |  | B0 | B7 |
| --- | --- | --- | --- |
|  | Transceiver Noise Floor |  |  |
| Octets: | 1 |  |  |

Figure k6—Transceiver Noise Floor fixed field

### 7.3.2 Information Elements

Insert Element ID 51-53, and TBD(s) into Table 20 and change the Reserved row accordingly:

Table 20—Element IDs

| Information Element | Element ID |
| --- | --- |
| AP Channel Report | 51 |
| Neighbor Report | 52 |
| RCPI | 53 |
| RSNI | TBD |
| BSS Load | TBD |
| Antenna Information | TBD |
| Reserved | 5156-255 |

#### 7.3.2.13 QBSS Load element

Change this clause from TGe amendment as follows:

The QBSS Load element contains information on the current station population and traffic levels in the QBSS.  The element information field is defined in Figure 46.1.

|  | Element ID (11) | Length (9) | Station Count | Channel Utilization | Available Admission Capacity | Access Category Service Load |
| --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 1 | 2 | 1 | 2 | 4 |

Figure 46.1 – QBSS Load element format

The station count field is interpreted as an unsigned integer that indicates the total number of STAs currently associated with this QBSS.

The channel utilization field is defined as the percentage of time, normalised to 255, the QAP sensed the medium busy, as indicated by either the physical or virtual carrier sense mechanism. This percentage is computed using the formula, ((channel busy time/(dot11ChannelUtilizationBeaconIntervals * dot11Beacon Period * 1024)) * 255), where ‘channel busy time’ is defined to be the number of microseconds during which the carrier sense mechanism, as defined in 9.2.1, has indicated a channel busy indication, and the MIB attribute dot11ChannelUtilizationBeaconIntervals represents the number of consecutive beacon intervals during which the channel busy time is measured.  The default value of dot11ChannelUtilizationBeaconIntervals is defined in Annex D.

The Available Admission Capacity field is 2 octets long and contains an unsigned integer that specifies the remaining amount of medium time available via explicit admission control, in units of 32 microsecond periods per 1 second. The field is helpful for roaming non-AP QSTAs to select a QAP that is likely to accept future admission control requests, but it does not represent a guarantee that the HC will admit these requests.

The Access Category (AC) Service Load field shall be included in the BSS Load only if dot11QoSOptionImplemented is true. The AC Service load field is formatted as four subfields as shown in Figure k. The AC Service Load shall be a scalar indication of the average access delay at a QAP for services for each of the indicated Access Categories. A low value shall indicate shorter access delay than a higher value. If the QAP is not currently providing services at the indicated AC, the for this AC shall be set equal to the average access delay of the following AC (located adjacent and to the right) within the Access Category Service field. The value 0 shall indicate that this QAP is not currently providing services of the indicated AC or of any higher priority AC. The values between 0 and 254 shall be a logarithmically scaled representation of the average medium access delay for transmitted packets in the indicated AC measured from the time the EDCF packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start time. The access delay time values shall be scaled as follows:

0:                        Access Delay  <  50 usec

1:     50 usec <=  Access Delay  <  51 usec

2:     51 usec <=  Access Delay  <  52 usec

3:     52 usec <=  Access Delay  <  53 usec

and so on where

n:     50*(10^ ((n-1)*0.081/10))  <=  Access Delay  <  50*(10^ (n*0.081/10))

in which the above range limit values are rounded to nearest integer and where n is the integer value (step) used to indicate the measured Access Delays.

251:     5296 usec <=  Access Delay  <  5396 usec

252:     5396 usec <=  Access Delay  <  5498 usec

253:     5498 usec <=  Access Delay

254:      Service not available (blocked)

255:      Measurement not available

The value 254 shall indicate that services at the indicated AC are currently not available (blocked). The value 255 shall indicate that the AC Service Load is not available. The QAP shall measure and average the medium access delay for all transmit packets of the indicated AC using EDCF access mechanism over a continuous thirty second measurement window. The accuracy for the average medium access delay shall be +/- 200 usec or better when averaged over at least 200 packets.

|  | Average Access Delay for Best Effort (AADBE) | Average Access Delay for Background (AADBG) | Average Access Delay for Video (AADVI) | Average Access Delay for Voice (AADVO) |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure k7—Access Category Service Load subfields

#### 7.3.2.18 TPC Report Element

Change the last paragraph in this clause as follows:

The TPC Report element is included in TPC Report frames, as described in 7.4.1.4; Link Measurement Report frames as described in 7.4.5.4; Beacon frames, as described in 7.2.3.1; and Probe Response frames, as described in 7.2.3.9. The use of TPC Report elements and frames is described in 11.5.4.

#### 7.3.2.21 Measurement Request element

Change clause 7.3.2.21 as follows:

The Measurement Request element contains a request that the receiving STA undertake the specified measurement action. The Measurement Request element is included in spectrum management Measurement Request frames as described in 7.4.1.1, or Radio Measurement Request frames as described in 7.4.5.1. Measurement Types 0, 1 and 2 are defined for spectrum management and shall only be included in spectrum management Measurement Request frames. The use of Measurement Request elements for spectrum management is described in 11.10.6. Measurement Types 3 through 10 and 255 are defined for radio measurement and shall only be included in Radio Measurement Request frames. The use of Measurement Request elements for radio measurement is described in 11.11.

The format of the Measurement Request element is shown in Figure 46g.

|  | Element ID | Length | Measurement Token | Measurement Request Mode (see Figure 46h) | Measurement Type | Measurement Request |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |

Figure 46g—Measurement Request element format

|  | Reserved (0) Parallel | Enable | Request | Report | Duration Mandatory | Reserved (0) |
| --- | --- | --- | --- | --- | --- | --- |
| Bit: | 0 | 1 | 2 | 3 | 4 | 45-7 |

Figure 46h—Measurement Request Mode field

The value of the Length field is variable and depends on the length of the Measurement Request field. The minimum value of the Length field is 3 (based on a minimum length for the Measurement Request field of 0 octets)

The Measurement Token shall be set to a nonzero number that is unique among the Measurement Request elements sent to each destination MAC address for which a corresponding Measurement Report element has not been receivedin a particular Measurement Request frame.

The Measurement Request Mode field (shown in Figure 46h) is a bit field with the following bits defined:

- The Parallel bit (bit 0) is used to request that a more than one measurement is to be started in parallel. Parallel is set to 1 to request that the measurement should start at the same time as the measurement described by the next Measurement Request element in the same Measurement Request frame. Parallel is set to 0 if the measurements are to be performed in sequence. The Parallel bit shall be set to 0 when Enable is set to 1, in the last or only measurement request element in the frame or when the value of the Measurement Type field is 0, 1, or 2 (Spectrum Management measurements). See 11.11.2.

- The Enable bit (bit 1) is used to differentiate between a request to make a measurement and a request to control the measurement requests and autonomous reports generated by the destination STA. Enable is set to 0 when requesting a measurement of the type specified in the Measurement Type field from the destination STA. If Enable is set to 0 Request and Report are reserved and the Measurement Request field contains fields appropriate for the Measurement Type being requested. Enable is set to 1 to request that the destination STA control the sending of measurement requests or autonomous reports of the type indicated in the Measurement Type field to the transmitting STA depending on the values of Request, and Report. If Enable is set to 1 the Measurement Request field is not present. See Table 20a.

- The Request bit (bit 2) is only valid if Enable is set to 1. Request is set to 0 to request that the destination STA not issue measurement Requests of Measurement Type to the transmitting STA. Request is set to 1 to indicate that the transmitting STA may accept measurement requests of Measurement Type from the transmitting STA. See Table 20a.

- The Report bit (bit 3) is only valid if Enable is set to 1. Report is set to 0 to request that the destination STA not issue autonomous measurement reports of Measurement Type to the transmitting STA. Report is set to 1 to indicate that the transmitting STA will accept automnomous measurement reports of Measurement Type from the transmitting STA. See Table 20a.

- Enable bit (bit 1) indicates whether this element is used to request the destination STA to enable or disable the sending of measurement requests and autonomous measurement reports of a specified type to this STA. The Enable bit shall be set to 1 when the Request bit and Report bit are valid. The Enable bit shall be set to 0 when the Request bit and Report bit are invalid.

- Request bit (bit 2) indicates whether the STA receiving the request shall enable or disable measurement requests of the type specified in the Measurement Type field. The Request bit shall be set to 1 when enabling a measurement request. The Request bit shall be set to 0 when disabling a measurement request or when the Request bit is invalid (i.e. when Enable bit is set to 0 or when the Measurement Type field contains a reserved measurement request type value).

- Report bit (bit 3) indicates whether the STA receiving the request shall enable or disable autonomous measurement reports of the type specified in the Measurement Type field. The Report bit shall be set to 1 when enabling an autonomous measurement report. The Report bit shall be set to 0 when disabling an autonomous measurement report or when the Report bit is invalid (i.e. when Enable bit is set to 0 or when the Measurement Type field contains a reserved measurement report type value).

- Duration Mandatory bit (bit 4) indicates whether the measurement duration contained within the Measurement Request should be interpreted as mandatory by the STA receiving the request. A value of 0 shall indicate that the duration requested is a target duration, but the requesting STA shall accept measurement results taken over a shorter duration. A value of 1 shall indicate that the duration requested is a mandatory duration. Duration Mandatory shall be reserved when the value of the Measurement Type field is 0, 1 or 2 (Spectrum Management measurements).

- All other bits are reserved and shall be set to 0 on transmission and ignored on reception.

The use of the Enable, Request and Report bits is also summarized in Table 20a. See 11.10.6 and 11.11.6 for the description of how a STA shall handles requests to enable or disable measurement requests and autonomous reports.

Table 20a—Summary of use of Enable, Request and Report bits

| Bits |  |  | Meaning of bitsMeasurement request meaning |
| --- | --- | --- | --- |
| Enable | Request | Report |  |
| 0 | 0Reserved | 0Reserved | The transmitting STA is requesting that the destination STA make a Measurement of type indicated in the Measurement Type field. When Enable is set to 0, Request and Report bits are reserved and shall be set to 0 on transmission and ignored on reception. When Enable bit set to 0, Request and Report bits are invalid and shall be set to 0 |
| 0 | 0 | 1 | Not allowedReserved |
| 0 | 1 | 0 | Not allowedReserved |
| 0 | 1 | 1 | Not allowedReserved |
| 1 | 0 | 0 | The transmitting STA is requesting that the destination STA sendsit is sent neither measurement requests nor autonomous measurement reports of the types indicated in the Measurement Type field. |
| 1 | 1 | 0 | The transmitting STA is indicating to the destination STA that it will may accept measurement requests and requesting it is not be sent autonomous measurement reports of the types indicated in the Measurement Type field Note: This setting corresponds to the default STA behaviour. |
| 1 | 0 | 1 | The transmitting STA is requesting that the destination STA it not sendbe sent measurement requests and indicating it will accept autonomous measurement reports of the types indicated in the Measurement Type field |
| 1 | 1 | 1 | The transmitting STA is indicating to the destination STA that it will may accept measurement requests and will accept autonomous measurement reports of the type indicated in the Measurement Type field |

The Measurement Type field shall be set to a number that identifies a type of measurement request or a measurement report. Those Measurement Types that have been allocated for measurement requests are shown in Table 20b and measurement reports are shown in Table 20c (in 7.3.2.207.3.2.22).

Table 20b—Measurement Type definitions for measurement requests

| Name | Measurement Type | Measurement Use |
| --- | --- | --- |
| Basic Request | 0 | Spectrum Management |
| Clear Channel Assessment (CCA) request | 1 |  |
| Receive power indication (RPI) histogram request | 2 |  |
| Channel load request | 3 | Radio Resource Measurement |
| Noise histogram request | 4 |  |
| Beacon request | 5 |  |
| Frame request | 6 |  |
| STA statistics request | 7 |  |
| LCI request | 8 |  |
| QoS metrics request | 9 |  |
| Reserved | 310-254 | N/A |
| Measurement Pause request | 255 | Radio Resource Measurement |

The Measurement Request field shall be null when the Enable bit is set to 1 and shall contain the specification of the measurement request, as described in 7.3.2.19.1 through 7.3.2.19.3, when the Enable bit is set to 0.

When the Enable bit is set to 0 the Measurement Request field contains the specification of the measurement request corresponding to the Measurement Type as described in 7.3.2.21.1 through 7.3.2.21.13. When the Enable bit is set to 1, the Measurement Request field is only present when requesting a triggered QoS Metrics measurement.

The Measurement Request element is included in a Measurement Request frame as described in 7.4.1.1. The use of Measurement Request elements and frames is described in 11.10.6.

The Measurement Request element is included in Spectrum Management Measurement Request frames as described in 7.4.1.1, or Radio Resource Measurement Request frames as described in 7.4.5.1. Measurement Types 0, 1 and 2 are defined for spectrum management and shall only be included in Spectrum Management Measurement Request frames. The use of Measurement Request elements for spectrum management is described in 11.10.6. Measurement Types 3 through 10 and 255 are defined for radio measurement and shall only be included in Radio Resource Measurement Request frames. The use of Measurement Request elements for radio measurement is described in 11.11.

Insert the following new clauses after clause 7.3.2.21.3:

##### 7.3.2.21.4 Channel Load Request

The Measurement Request field corresponding to a Channel Load Request is shown in Figure k8.

|  |  | Regulatory Class | Channel Number | Randomization Interval | Measurement Duration |
| --- | --- | --- | --- | --- | --- |
| Octets: |  | 1 | 1 | 2 | 2 |

Figure k8—Measurement Request field format for a Channel Load Request

Channel Number indicates the channel number for which the measurement request applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies. Valid values of Regulatory Class are shown in Error: Reference source not found.

Channel Number indicates the channel number for which the measurement request applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TU. See 11.11.3.

The Measurement Duration field shall be set to the preferred  or mandatory duration of the requested measurement, expressed in TUs.   See 11.11.4.

##### 7.3.2.21.5 Noise Histogram Request

The Measurement Request field corresponding to a Noise Histogram Request is shown in Figure k9.

|  |  | Regulatory Class | Channel Number | Randomization Interval | Measurement Duration |
| --- | --- | --- | --- | --- | --- |
| Octets: |  | 1 | 1 | 2 | 2 |

Figure k9—Measurement Request field format for a Noise Histogram Request

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies.  Valid values of Regulatory Class are shown in Error: Reference source not found.

Channel Number indicates the channel number for which the measurement request applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TU. See 11.11.3.

The Measurement Duration field shall be set to the preferred  or mandatory duration of the requested measurement, expressed in TUs.   See 11.11.4.

##### 7.3.2.21.6 Beacon Request

The Measurement Request field corresponding to a Beacon Request is shown in Figure k10.

|  |  | Regulatory Class | Channel Number |  | Randomization Interval |  | Measurement Duration |  | Measurement Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: |  | 1 | 1 |  | 2 |  | 2 |  | 1 |  |
|  | BSSID |  |  | Reporting Condition |  | Threshhold /Offset (optional) |  | SSID element |  |  |
| Octets: | 6 |  |  | 1 |  | 1 |  | 2-34 |  |  |

Figure k10—Measurement Request field format for a Beacon Request

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies.   Valid values of Regulatory Class are shown in Error: Reference source not found.

Channel Number indicates the channel number for which the measurement request applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found. A Channel Number of 0 indicates a request to make iterative measurements for all supported channels in the Regulatory Class where the measurement is permitted on the channel and the channel is valid for the current regulatory domain. A Channel Number of 255 indicates a request to make iterative measurements for all supported channels listed in the AP Channel Report for the Regulatory Class. The procedures for iterative measurements on multiple channels are described in 11.11.9.1.

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TU. See 11.11.3.

The Measurement Duration field shall be set to the preferred  or mandatory duration of the requested measurement, expressed in TUs.   See 11.11.4.

Measurement Mode indicates the mode to be used for the measurement. The valid measurement modes are listed in Table k2. The procedures for and descriptions of each mode are described in 11.11.9.1.

Table k2—Measurement Mode definitions for Beacon Request element

| Mode | Value |
| --- | --- |
| Passive | 0 |
| Passive Pilot | 1 |
| Active | 2 |
| STA Selected | 3 |
| Beacon Table | 4 |
| Reserved | 5-255 |

The BSSID field indicates the BSSID of the BSS(s) for which a beacon report is requested.  When requesting beacon reports for all BSSs on the channel, the BSSID field contains the broadcast BSSID; otherwise the BSSID field contains a specific BSSID for a single BSS.

The Reporting Condition defines when the measured results are to be reported to the requesting STA. The Reporting Condition values are defined in Table k3. Non-zero values for Reporting Condition may be used only for repeated measurements. Reporting Condition shall be set to 0 for single measurements or when measurement mode is Beacon Table. Reporting Condition shall be limited to the range 0 to 4 inclusive when requesting measurements in an IBSS. The use of Reporting Condition is described in 11.11.9.1.

Table k3—Reporting Condition definitions for Beacon Request element

| Condition for Report to be issued in Repeated Measurement | Reporting Condition |
| --- | --- |
| Report to be issued unconditionally, i.e. after each measurement. | 0 |
| Report to be issued when the measured RCPI level crosses above an absolute threshold. | 1 |
| Report to be issued when the measured RCPI level crosses below an absolute threshold. | 2 |
| Report to be issued when the measured RSNI level crosses above an absolute threshold. | 3 |
| Report to be issued when the measured RSNI level crosses below an absolute threshold. | 4 |
| Report to be issued when the measured RCPI level crosses above a threshold defined by an offset from the serving AP's reference RCPI level. | 5 |
| Report to be issued when the measured RCPI level crosses below a threshold defined by an offset from the serving AP’s reference RCPI level. | 6 |
| Report to be issued when the measured RSNI level crosses above a threshold defined by an offset from the serving AP’s reference RSNI level. | 7 |
| Report to be issued when the measured RSNI level crosses below a threshold defined by an offset from the serving AP’s reference RSNI level. | 8 |
| Report to be issued when the measured RCPI level enters and remains in a range bound by the serving AP’s reference RCPI level and an offset from the serving AP’s reference RCPI level. | 9 |
| Report to be issued when the measured RSNI level enters and remains in a range bound by the serving AP’s reference RSNI level and an offset from the serving AP’s reference RSNI level. | 10 |
| Reserved | 11-255 |

Threshold/Offset provides either the threshold value or the offset value to be used for conditional reporting. Threshold/Offset is not included when the Reporting Condition is 0. Threshold/Offset is always included when  the Reporting Condition is non-zero. For Reporting Conditions 1 to 4, the threshold value is an unsigned 8 bit integer in 1/2 dBm units. For Reporting Conditions 5, 6, and 9, the offset value is an 8 bit 2's complement integer in units of 1/2 dBm. For Reporting Conditions 7, 8 and 10 the offset value is an 8 bit 2's complement integer in units of 1/2 dB.

The SSID element indicates the ESS(s), or IBSS(s) for which a beacon report is requested. When requesting beacon reports for all ESSs on the channel, the SSID field contains the 'wildcard SSID'; otherwise the SSID field contains a specific SSID for a single ESS or IBSS.  The 'wildcard SSID' is defined as a zero length (null) SSID used to represent all possible SSIDs. The SSID element is described in 7.3.2.1. If Passive Pilot Measurement Mode is requested the SSID element shall be the wildcard SSID.

##### 7.3.2.21.7 Frame Request

The Measurement Request field corresponding to a Frame Request is shown Figure k11.

|  | Channel Number | Regulatory Class | Channel Number | Randomization Interval | Measurement Duration |
| --- | --- | --- | --- | --- | --- |
| Octets: |  | 1 | 1 | 2 | 2 |

Figure k11—Measurement Request field format for a Frame Request

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies.   Valid values of Regulatory Class are shown in Error: Reference source not found.

Channel Number indicates the channel number for which the measurement request applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TU. See 11.11.3.

The Measurement Duration field shall be set to the preferred  or mandatory duration of the requested measurement, expressed in TUs.   See 11.11.4.

##### 7.3.2.21.10 STA Statistics Request

The Measurement Request field corresponding to a STA Statistics Request is shown in Figure k12.

|  | Randomization Interval | Measurement Duration | Group Identity |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 |

Figure k12—Measurement Request field format for a STA Statistics Request

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TU. See 11.11.3.

The Measurement Duration field shall be set to the duration of the requested measurement in TUs. The Measurement, if accepted, shall be performed over the requested duration without regard to the Duration Mandatory bit in the Measurement Request Mode field. If a measurement over the requested duration is not possible, the request shall be refused. A Measurement Duration of 0 shall be used to request current values of the requested STA statistics group. A non-zero value of Measurement Duration indicates a request for the change in value (increases or decreases) in the statistics of the specified statistics group measured over the specified Measurement Duration.

Group Identity indicates the requested statistics group according to Table k8

7.3.2.21.11 Location Configuration Indication (LCI) Request

The Measurement Request field corresponding to an LCI request is shown in Figure k13.

|  | Location Subject | Latitude Accuracy | Longitude Accuracy | Altitude Accuracy |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |

Figure k13—LCI Request field format

The Location Subject field of a LCI request is a single octet. See Table k5.

Table k5—Location subject definition

| Value | Location Subject |
| --- | --- |
| 0 | Local |
| 1 | Remote |
| 2-255 | Reserved |

The term ‘Local’ refers to the location of the requesting STA, and ‘Remote’ refers to the location of the reporting STA.

Latitude accuracy is the number of valid bits requested in the fixed-point value of Latitude. Values above 34 (decimal) are undefined and reserved.

Longitude accuracy is the number of valid bits requested in the fixed-point value of Longitude. Values above 34 (decimal) are undefined and reserved.

Altitude accuracy is the number of valid bits requested in the Altitude. Values above 30 (decimal) are undefined and reserved.

##### 7.3.2.21.12 Measurement Pause Request

The Measurement Request field corresponding to a Measurement Pause Request is shown in Figure k14. The Measurement Pause Request cannot be processed in parallel with any other Measurement Request. Parallel measurement request processing, as specified in the Measurement Request Mode field description in section 7.3.2.21, does not apply to the Measurement Pause Request. See 11.11.9.9.

|  | Pause Time |
| --- | --- |
| Octets: | 2 |

Figure k14—Measurement Pause Request field format

The Pause Time field consists of a 16 bit unsigned integer number representing the time period for which measurements are suspended or paused. The time unit for the Pause Time field is 10 TU. Measurement Pause Requests are used to provide time delays between the execution times of measurement request elements in a Measurement Request Frame.

##### 7.3.2.21.13 QoS Metrics Request

The Measurement Request field corresponding to a QoS Metrics Request is shown in Figure k15. A response to a QoS Metrics Request is a QoS Metrics Report.

|  | Randomization Interval | Measurement Duration | Peer QSTA Address | Traffic Identifier | Bin 0 Range |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 6 | 1 | 1 |

Figure k15—Measurement Request field format for a QoS Metrics Request

Randomization Interval shall be set to the desired maximum random delay in the measurement start time, expressed in TUs. The use of Randomization Interval is described in 11.11.3. Randomization Interval is not used and shall be set to 0 when requesting a triggered QoS metrics measurement.

The Measurement Duration shall be set equal to the duration of the requested measurement, expressed in TUs. When setting up a triggered QoS measurement, Measurement Duration is not used and shall be set to 0.

The Peer QSTA Address shall contain the 6 byte MAC address in the Address 1 field of the measured Data frames.

Traffic Identifier shall indicate the TC or TS for which traffic is to be measured. Values 0 through 15 are defined. Values 16-255 are reserved.

Bin 0 Range shall indicate the delay range of the first bin (Bin 0) of the Transmit Delay Histogram, expressed in TUs. It is also used to calculate the delay ranges of the other 5 bins making up the histogram. The delay range for each bin shall increase in a binary exponential fashion as described in 7.3.2.22.13.

The Triggered Reporting field is used to specify measurement trigger thresholds. It is only present if setting up triggered QoS metrics reporting – see 11.11.9.10. The Triggered Reporting field is as shown in Figure k16.

|  | Trigger Condition | Average Error Threshold | Consecutive Error Threshold | Delay Threshold | Measurement Count | Trigger Timeout |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure k16—Triggered Reporting Field

Trigger Condition is a bit-field that specifies reporting triggers when requesting a triggered QoS metrics measurement. The format of Trigger Condition is shown in Figure k17.

|  | B0 | B1 | B2 | B3 | B7 |
| --- | --- | --- | --- | --- | --- |
|  | Average | Consecutive | Delay | Reserved |  |
| Bits: | 1 | 1 | 1 | 5 |  |

Figure k17—Trigger Condition field

- Average is set to 1 to request that a QoS Metrics Report be generated when the number of MSDUs for the TC, or TS given by the Traffic Identifier that are discarded over the moving average number of transmitted MSDUs specified in Measurement Count is equal to the value given in Average Error Threshold. MSDUs discarded due to the number of transmit attempts exceeding dot11ShortRetryLimit or dot11LongRetryLimit, or due to the MSDU lifetime having been reached shall be counted.

- Consecutive is set to 1 to request that a QoS Metrics Report be generated when the number of MSDUs for the TC, or TS given by the Traffic Identifier that are discarded in succession is equal to the value given in Consecutive Error Threshold. MSDUs discarded due to the number of transmit attempts exceeding the appropriate retry limit, or due to the MSDU lifetime having been reached shall be counted.

- Delay is set to 1 to request that a QoS Metrics Report be generated when the number of consecutive MSDUs for the TC, or TS given by the Traffic Identifier that experience a transmit delay greater than or equal to the lower bound of the bin of the Transmit Delay Histogram specified by the value in Delayed MSDU Range equals the value given in Delayed MSDU Count.

The Average Error Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the Average trigger condition.

The Consecutive Error Threshold field contains a value representing the number of MSDUs to be used as the threshold value for the Consecutive trigger condition.

The Delay Threshold field contains two subfields as shown in Figure k18.

|  | B0 | B1 | B2 | B7 |
| --- | --- | --- | --- | --- |
|  | Delayed MSDU Range |  | Delayed MSDU Count |  |
| Bits: | 2 |  | 6 |  |

Figure k18—Delay Threshold subfield

Delayed MSDU Range contains a value representing the MSDU transmit delay at or above which an MSDU will be counted towards the Delayed MSDU Count threshold. Delayed MSDU Range is encoded as a value representing the lower bound of a bin in the Transmit Delay Histogram as shown in Table k6.

Table k6—Delayed MSDU Range Definitions

| Delayed MSDU Range | Condition |
| --- | --- |
| 0 | Transmit Delay ≥ Lower Bound of Bin 2 |
| 1 | Transmit Delay ≥ Lower Bound of Bin 3 |
| 2 | Transmit Delay ≥ Lower Bound of Bin 4 |
| 3 | Transmit Delay ≥ Lower Bound of Bin 5 |

Delayed MSDU Count contains a value representing the number of MSDUs to be used as the threshold value for the Delay trigger condition.

The Measurement Count field contains a number of MSDUs. This value is used in the Average Error Threshold and in place of measurement duration in determining the scope of the reported results when a report is triggered – see 11.11.9.10.

The Trigger Timeout field contains a value in units of 100TU during which a measuring STA shall not generate further triggered QoS metrics reports after a trigger condition has been met. See 11.11.9.10.

#### 7.3.2.22 Measurement Report element

Change clause 7.3.2.22 as follows:

The Measurement Report element contains a measurement report. The format of the Measurement Report element is shown in Figure 13.

|  | Element ID | Length | Measurement Token | Measurement Report Mode | Measurement Type | Measurement Report |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |

Figure 13—Measurement Report element format

|  | Late | Incapable | Refused | Reserved |
| --- | --- | --- | --- | --- |
| Bit: | 0 | 1 | 2 | 3-7 |

Figure 14—Measurement Report Mode field

The value of the Length field is variable and depends on the length of the Measurement Report field. The minimum value of the Length field is 3.

The Measurement Token field shall be set to the Measurement Token in the corresponding Measurement Request element. If the Measurement Report element is being sent autonomously then the Measurement Token shall be set to 0.

The Measurement Report Mode field (shown in Figure 14) is used to indicate the reason for a failed measurement request. The Measurement Report Mode is a bit field with the following bits defined:

- Late bit (bit 0) indicates whether this STA is unable to carry out a measurement request because it received the request after the requested measurement time. The Late bit shall be set equal to 1 to indicate the request was too late. The Late bit shall be set to 0 to indicate the request was received in time for the measurement to be executed. The Late bit only applies to spectrum management measurement and shall be set to 0 in all measurement report elements for radio resource measurement types (see Table 20c).

- Incapable bit (bit 1) indicates whether this STA is incapable of generating a report of the type specified in the Measurement Type field that was previously requested by the destination STA of this Measurement Report element. The Incapable bit shall be set to 1 to indicate the STA is incapable. The Incapable bit shall be set to 0 to indicate the STA is capable or the report is autonomous.

- Refused bit (bit 2) indicates whether this STA is refusing to generate a report of the type specified in the Measurement Type field that was previously requested by the destination STA of this Measurement Report element. The Refused bit shall be set to 1 to indicate the STA is refusing. The Refused bit shall be set to 0 to indicate the STA is not refusing or the report is autonomous.

- All other bits are reserved and shall be set to 0 on transmission and ignored on reception.

Not more than one bit shall be set within a Measurement Report Mode field. All bits within the Measurement Mode field shall be set to 0 if the results of a successful measurement request, or an autonomous measurement are being reported.

The Measurement Type field shall be set to a number that identifies the measurement report. Those Measurement Types that have been allocated are shown in Table 20c.

The Measurement Report field shall be null not be present when the Late bit is set to 1, the Incapable bit is set to 1 or the Refused bit is set to 1. Otherwise, it shall contain the specification of thea measurement report, as described in 7.3.2.20.17.3.2.22.1 through 7.3.2.20.37.3.2.22.10.

Table 20c—Measurement Type definitions for measurement reports

| Name | Measurement Type | Measurement Use |
| --- | --- | --- |
| Basic report | 0 | Spectrum Management |
| Clear Channel Assessment (CCA) report | 1 |  |
| Receive power indication (RPI) histogram report | 2 |  |
| Channel load report | 3 | Radio Resource Measurement |
| Noise histogram report | 4 |  |
| Beacon report | 5 |  |
| Frame report | 6 |  |
| STA statistics report | 7 |  |
| LCI report | 8 |  |
| QoS metrics request | 9 |  |
| Reserved | 310-255 | N/A |

The Measurement Report element is included in a Measurement Report frame as described in 7.4.1.2. The use of Measurement Report elements and frames is described in 11.6.6.

The Measurement Report element is included in Spectrum Management Measurement Report frames as described in 7.4.1.2, or Radio Resource Measurement Report frames as described in 7.4.5.2. Measurement Types 0, 1 and 2 are used for spectrum management and shall only be included in spectrum management Measurement Report frames. All other Measurement Types are used for radio resource measurement and shall only be included in Radio Resource Measurement Report frames. The use of Measurement Report elements and frames for spectrum management is described in 11.6.6. The use of Measurement Report elements and frames for radio measurement is described in 11.7.

Insert the following new clauses after clause 7.3.2.22.3:

##### 7.3.2.22.4 Channel Load Report

The format of the Measurement Report field corresponding to a Channel Load Report is shown in Figure k19.

|  | Channel Number | Regulatory Class | Channel Number | Actual Measurement Start Time | Measurement Duration | Channel Load |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: |  | 1 | 1 | 8 | 2 | 1 |

Figure k19—Measurement Report field format for a Channel Load Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies.   Valid values of Regulatory Class are shown in Error: Reference source not found.

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Channel Load Report was measured, expressed in TUs.

Channel Load shall contain the proportion of measurement duration for which the measuring STA determined the channel to be busy. Procedure for Channel Load measurement and definition of channel load values are found in 11.11.9.3.

##### 7.3.2.22.5 Noise Histogram Report

The format of the Measurement Report field of a Noise Histogram Report is shown in Figure k20.

|  |  |  |  | Regulatory Class |  | Channel Number |  | Actual Measurement Start Time |  |  | Measurement Duration |  |  | Antenna ID |  | ANPI |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: |  |  |  | 1 |  | 1 |  | 8 |  |  | 2 |  |  | 1 |  | 1 |  |
|  |  | RPI 0 Density | RPI 1 Density |  | RPI 2 Density |  | RPI 3 Density |  | RPI4 Density | RPI 5 Density |  | RPI 6 Density | RPI 7 Density |  | RPI 8 Density |  |  |
| Octets: |  | 1 | 1 |  | 1 |  | 1 |  | 1 | 1 |  | 1 | 1 |  | 1 |  |  |

Figure k20—Measurement Report field format for a Noise Histogram Report

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies.   Valid values of Regulatory Class are shown in Error: Reference source not found.

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Noise Histogram Report was measured, expressed in TUs.

Antenna ID shall be set to the identifying number for the antenna(s) used for this measurement. Antenna ID is defined in 7.3.2.30.

ANPI shall be set to the average noise plus interference power value measured during the indicated Measurement Duration while the indicated channel is idle as described in 11.11.9.4.

The Noise Histogram Report shall contain the RPI densities, as defined in 11.11.9.4, observed in the channel for the nine RPI levels defined in Table k7.

Table k7—RPI Definitions for a Noise Histogram Report

| RPI Level | RPI Measured Power (dBm) |
| --- | --- |
| 0 | RPI  -92 |
| 1 | -92 < RPI -87 |
| 2 | -87  RPI -82 |
| 3 | -82  RPI -77 |
| 4 | -77 RPI -72 |
| 5 | -72 RPI -67 |
| 6 | -67 RPI -62 |
| 7 | -62 RPI -57 |
| 8 | -57 RPI |

##### 7.3.2.22.6 Beacon Report

The format of the Measurement Report field corresponding to a Beacon Report is shown in Figure k21.

|  |  |  | Regulatory Class |  |  | Channel Number |  |  | Actual Measurement Start Time |  |  |  | Measurement Duration |  |  | Reported Frame Information |  | RCPI |  | RSNI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: |  |  | 1 |  |  | 1 |  |  | 8 |  |  |  | 2 |  |  | 1 |  | 1 |  | 1 |
|  | BSSID |  |  | Antenna ID | Parent TSF |  |  | Reported Frame Body |  |  |  |  |  |  |  |  |  |  |  |  |
| Octets: | 6 |  |  | 1 | 4 |  |  | variable |  |  |  |  |  |  |  |  |  |  |  |  |

Figure k21—Measurement Report field format for a Beacon Report

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies.   Valid values of Regulatory Class are shown in Error: Reference source not found.

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Beacon Report was measured, expressed in TUs.

The Reported Frame Information field contains two subfields as shown in Figure k22.

|  | B0 B6 | B7 |
| --- | --- | --- |
|  | Condensed PHY Type | Reported Frame Type |
| Bits: | 7 | 1 |

Figure k22—Reported Frame Information Field

Condensed PHY Type indicates the physical medium type on which the Beacon, Measurement Pilot or Probe Response frame being reported was received. It shall have an integer value between 0 and 127 coded according to the value of dot11PHYType.

Reported Frame Type indicates the type of frame reported. A value of 0 indicates a Beacon, or Probe Response frame; a value of 1 indicates a Measurement Pilot frame.

RCPI indicates the received channel power of the Beacon, Measurement Pilot or Probe Response frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type.

RSNI indicates the received signal to noise indication for the beacon or probe response frame in dB, as described in 7.3.2.31.

The BSSID field contains the BSSID from the Beacon, Measurement Pilot or Probe Response frame being reported.

The Antenna ID field contains the identifying number for the antenna(s) used for this measurement. If different antennas are used to receive the beacon preamble and the beacon frame body, this Antenna ID shall contain the identifying number for the antenna used to receive the beacon frame body. Antenna ID is defined in 7.3.2.30.

The Parent TSF field shall contain the lower 4-octets of the measuring STA's TSF timer value at the start of reception of the first octet of the timestamp field of the reported Beacon, Measurement Pilot, or Probe Response frame.

The Reported Frame Body field contains the frame body of the reported Beacon, Measurement Pilot, or Probe Response frame. All fixed fields and information elements are included in the order they appeared in the reported frame. TIM elements shall be truncated such that only the first 4 octets of the element are reported. If the Reported Frame Body would cause the Measurement Report element to exceed the maximum information element size then the Reported Frame Body shall be truncated so that the last information element in the Reported Frame Body field shall be a complete information element.

##### 7.3.2.22.7 Frame Report

The format of the Measurement Report field corresponding to a Frame Report is shown in Figure k23.

|  |  |  |  |  |  | Zero or more entries |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Regulatory Class | Channel Number | Actual Measurement Start Time | Measurement Duration | Frame Report Entry |
| Octets: |  | 1 | 1 | 8 | 2 | n x 16 |

Figure k23—Measurement Report field format for a Frame Report

Regulatory Class indicates the channel set for which the measurement request applies. Regulatory Class and Channel Number together specify the channel frequency and spacing for which the measurement request applies.   Valid values of Regulatory Class are shown in Error: Reference source not found.

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Actual Measurement Start Time shall be set to value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Frame Report was measured, expressed in TUs.

The format of the Frame Report Entry is shown in Figure k24.

|  | Transmit Address | BSSID | PHY Type | Average RCPI | RSNI | Last RCPI | Antenna ID | Number of Unicast Data Frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 6 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure k24—Frame Report Entry format

The Transmit Address field contains the Transmit Address (TA) from the frames being reported.

The BSSID field contains the BSSID from the frames being reported.

PHY Type indicates the physical medium type for the frame(s) being reported. Valid entries are coded according to the value of dot11PHYType.

Average RCPI indicates the average value for the received channel power of all the frames received and counted in this Frame Report Entry. Average RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

RSNI indicates the received signal to noise indication of the received frame in dB, as described in 7.3.2.31. This field shall be the RSNI value for the most recently received frame.

Last RCPI indicates the received channel power of the most recently measured frame in this Frame Report entry. Last RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

The Antenna ID field contains the identifying number for the antenna(s) used to receive the most recently measured frame in this Frame Report entry. Antenna ID is defined in 7.3.2.30.

Number of Unicast Data Frames is a count of the unicast data and management frames received with the indicated Transmit Address and BSSID during the measurement duration. The value 255 shall indicate a count of 255 or more.

##### 7.3.2.22.10 STA Statistics Report

The format of the Measurement Report field of a STA Statistics Report is shown in Figure k25.

|  | Measurement Duration | Statistics Group Data |
| --- | --- | --- |
| Octets: | 2 | Variable |

Figure k25—Measurement Report field format for a STA Statistics Report

The STA Statistics Report reports the change in the requested Statistics Group Data values measured within the Measurement Duration. When the Measurement Duration is set to 0 the current values of the requested Statistics Group Data shall be reported, rather than the change.

The Measurement Duration shall be set equal to the duration over which the change in Statistics Group Data was measured and reported, expressed in TUs. A Measurement Duration value of 0 indicates a report of the current values of the StatisticsGroup Data.

Statistics Group Data shall contain the requested statistics from the MIB related to the interface on which the request was received according to Table k8. Units used for reporting a statistic or change in statistic shall be the same units used for the statistic in the MIB. When Measurement Duration value is non-zero, the reported data values shall be 2's complement integers representing both positive and negative changes in the statistics data..

Table k8—Group Identity for a STA Statistics Report

| Group Identity Requested | Statistics Returned |
| --- | --- |
| 0 | dot11Counters Group for the Interface on which the STA Statistics Request was received (with the exception of WEPUndecryptableCount and those counters listed in Group 1): dot11TransmittedFragmentCount (Counter32), dot11MulticastTransmittedFrameCount (Counter32), dot11FailedCount (Counter32), dot11ReceivedFragmentCount (Counter32), dot11MulticastReceivedFrameCount (Counter32), dot11FCSErrorCount (Counter32), dot11TransmittedFrameCount (Counter32) |
| 1 | dot11MACStatistics Group for the Interface on which the STA Statistics Request was received: dot11RetryCount (Counter32), dot11MultipleRetryCount (Counter32), dot11FrameDuplicateCount (Counter32), dot11RTSSuccessCount (Counter32), dot11RTSFailureCount (Counter32), dot11ACKFailureCount (Counter32) |
| 2 | dot11BSS Load Group (only avaialbe at an AP): dot11STAStatisticsAPServiceLoad (INTEGER), dot11STAStatisticsAverageAccessDelayBestEffort (INTEGER), dot11STAStatisticsAverageAccessDelayBackGround (INTEGER), dot11STAStatisticsAverageAccessDelayVIdeo (INTEGER), dot11STAStatisticsAverageAccessDelayVoice (INTEGER), dot11STAStatisticsStationCount (INTEGER), dot11STAStatisticsChannelUtilization (INTEGER) |
| 2 – 255 | None |

The format of the Measurement Report field for dot11Counters Group is shown in Figure k26.

|  | Dot11Transmitted FragmentCount | dot11Multicast TransmittedFrame Count | dot11FailedCount |  |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 |  |  |
|  |  |  |  |  | dot11Received FragmentCount |
| Octets: |  |  |  |  | 4 |
|  | dot11Multicast ReceivedFrame Count | dot11FCSErrorCount | dot11Transmitted FrameCount |  |  |
| Octets: | 4 | 4 | 4 |  |  |

Figure k26—Measurement Report field format for dot11Counters Group

The format of the Measurement Report field for dot11MACStatistics  is shown in Figure k27

|  | dot11RetryCount | dot11MultipleRetry Count | Dot11FrameDuplicateCount | dot11RTSSuccess Count | dot11RTSFailure Count |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 |
|  | dot11ACKFailure Count |  |  |  |  |
| Octets: | 4 |  |  |  |  |

Figure k27—Measurement Report field format for dot11MACStatistics Group

The format of the Measurement Report field for dot11BSSLoad Group is shown in Figure k28.

|  | dot11STAStatisticsAPServiceLoad | dot11STAStatisticsAverageAccess DelayBestEffort | dot11STAStatisticsAverageAccess DelayBackGround | dot11STAStatisticsAverageAccess DelayVIdeo | dot11STAStatisticsAverageAccess DelayVOice |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |
|  | dot11STAStatisticsStationCount | dot11STAStatisticsChannelUtilization |  |  |  |
| Octets: | 2 | 1 |  |  |  |

Figure k28—Measurement Report field format for dot11BSSLoad Group

##### 7.3.2.22.11 Location Configuration Information Report element

A Location Configuration Information report includes latitude, longitude and altitude information. The LCI report format is shown in Figure k29. This format is based on that described in IETF RFC 3825, “Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information” and the length shall be 16 octets. Fields within the LCI report shall be as defined in RFC3825 section 2.1.

Figure k29—Location Configuration Information

An LCI report with Latitude resolution, Longitude resolution and Altitude resolution set to zero shall indicate that the requested location cannot be reported with the requested accuracy.

The Datum value shall be 1 (WGS 84), unless another datum is required for operation in the regulatory domain.

If the Altitude Type is 2 (Floors of Altitude), the value reported shall be as required for operation in the regulatory domain.

##### 7.3.2.22.13 QoS Metrics Report

The format of the Measurement Report field corresponding to a QoS Metrics Report is shown in Figure k30.

|  | Actual Measurement Start Time | Measurement Duration | Peer QSTA Address | Traffic Identifier | Reporting Reason |
| --- | --- | --- | --- | --- | --- |
| Octets: | 8 | 2 | 6 | 1 | 1 |

|  | Transmitted MSDU Count | MSDU Discarded Count | MSDU Failed Count | MSDU Multiple Retry Count | QoS CFPolls Lost Count | Average Queue Delay | Average Transmit Delay |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 | 4 | 4 |

|  | Bin 0 Range | Bin 0 Count | Bin 1 Count | Bin 2 Count | Bin 3 Count | Bin 4 Count | Bin 5 Count |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 4 | 4 | 4 | 4 | 4 | 4 |

Figure k30—Measurement Report field format for Transmit Delay Metric Report

Actual Measurement Start Time shall be set equal to the TSF at the time at which the measurement started, or for a triggered QoS metrics report the TSF value at the reporting QSTA when the trigger condition was met.

Measurement Duration shall be set equal to the duration over which the QoS Metrics Report was measured, expressed in TUs. For a triggered QoS Metrics Report, metrics are reported over a number of transmitted MSDUs rather than a duration, hence Measurement Duration shall be set to 0 – see 11.11.9.10.

The Peer QSTA Address shall contain the 6 byte MAC address in the Address 1 field of the measured Data frames.

Traffic Identifier shall indicate the TC or TS for which traffic is to be measured. Values 0 through 15 are defined. Values 16-255 are reserved.

The Reporting Reason field is a bit field indicating the reason that the measuring QSTA sent the Transmit QoS metrics report. The Reporting Reason field is shown in Figure k31.

|  | B0 | B1 | B2 | B3 | B7 |
| --- | --- | --- | --- | --- | --- |
|  | Average Trigger | Consecutive Trigger | Delay Trigger | Reserved |  |
| Bits: | 1 | 1 | 1 | 5 |  |

Figure k31—Reporting Reason field

- The Average Trigger bit set to 1 indicates that the Transmit QoS Metrics Report was generated as a triggered report due to the Average Error trigger.

- The Consecutive Trigger bit set to 1 indicates that the Transmit QoS Metrics Report was generated as a triggered report due to the Consecutive Error trigger.

- The Delay Trigger bit set to 1 indicates that the Transmit QoS Metrics Report was generated as a triggered report due to the Delay Error trigger.

In a requested Transmit QoS Metrics Report, all bit fields in the Reporting Reason field are set to 0. More than one bit field in the Reporting Reason field may be set to 1 if more than one trigger condition was met.

The Transmitted MSDU Count, MSDU Failed Count, MSDU Discarded Count, MSDU Multiple Retry Count, QoS CFPolls Lost Count, Average Queue Delay, Average Transmit Delay, and delay histogram fields relate to transmissions to the QSTA given in the Peer QSTA Address field. Metrics shall be reported over the measurement duration, or for triggered QoS metrics, over the measurement count.

The Transmitted MSDU Count field contains the number of MSDUs for the TC, or TS given by the Traffic Identifier successfully transmitted.

The MSDU Discarded Count field contains the number of MSDUs for the TC, or TS given by the Traffic Identifier discarded due either to the number of transmit attempts exceeding dot11ShortRetryLimit or dot11LongRetryLimit as appropriate, or due to the MSDU lifetime having been reached.

The MSDU Failed Count field contains the number of MSDUs for the TC, or TS given by the Traffic Identifier discarded due to the number of transmit attempts exceeding dot11ShortRetryLimit or dot11LongRetryLimit as appropriate.

The MSDU Multiple Retry Count field contains the number of MSDUs for the TC, or TS given by the Traffic Identifier that are successfully transmitted after more than one retransmission attempt.

The QoS CFPolls Lost Count field contains the number of QoS (+)CF-Poll frames transmitted where there was no response from the QSTA. QoS CFPolls Lost Count shall only be returned if the reporting QSTA is a QAP and the Traffic Identifier is for TS. If unused QoS CFPolls Lost count shall be set to 0.

Average Queue Delay shall be the average queuing delay of the frames (MSDUs) that are passed to the MAC for the indicated Peer QSTA Address and the indicated Traffic Identifier. Queue Delay shall be measured from the time the MSDU is passed to the MAC until the point at which the first, or only fragment is ready for transmission and shall be expressed in TUs.

Average Transmit Delay shall be the average delay of the frames (MSDUs) that are successfully transmitted for the indicated Peer QSTA Address and the indicated Traffic Identifier. Delay shall be measured from the time the MSDU is passed to the MAC until the point at which the entire MSDU has been successfully transmitted, including receipt of the final ACK from the peer QSTA if the QoSAck service class is being used. Average Transmit delay shall be expressed in TUs.

Bin 0 Range shall indicate the delay range of the first bin (Bin 0) of the Transmit Delay Histogram, expressed in TUs. It is also used to calculate the delay ranges of the other 5 bins making up the histogram. The delay range for each bin shall increase in a binary exponential fashion as follows:

B0 duration:   	 0   Delay <B0, 		for i =0;

Bi duration:  	2i−1*B0  Delay < 2i*B0     	for 1< i <5

If Bin 0 Range is 10ms, the bin durations should be defined in Table k9.

Table k9—Delay Definitions for a Transmit QoS Report

| Bin | Measured Delay (TUs) |
| --- | --- |
| 0 | Delay <10 |
| 1 | 10  Delay < 20 |
| 2 | 20  Delay <40 |
| 3 | 40  Delay <80 |
| 4 | 80  Delay <160 |
| 5 | Delay  160 |

To compute the value reported in Bin i, Bi, 0 ≤ i < 5, of the Transmit Delay Histogram, the STA initializes all bin counts to zero. For each MSDU successfully transmitted, the measured Transmit Delay determines which bin count is to be incremented. If the measured delay has a duration time t within Bin i, then the frame count in Bin i is increased by one, up to a ceiling value of 232-1. Transmit Delay shall be measured from the time the MSDU is passed to the MAC until the point at which the entire MSDU has been successfully transmitted, including receipt of the final ACK from the peer QSTA if the QoSAck service class is being used. During the QoS Metrics Measurement, a histogram is generated that represents the distribution of Transmit Delay.

Insert the following new clauses adjusting the clause numbers as necessary:

#### 7.3.2.26 AP Channel Report element

The AP Channel Report element contains a list of channels where a STA is likely to find an AP. The format of the AP Channel Report element is shown in Figure k32.

|  | Element ID | Length | Regulatory Class | Channel List |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure k32—AP Channel Report element format

The Element ID field shall be equal to the AP Channel Report value in Table 20.

The Length field is dependent on the number of channels reported in the Channel List. The minimum value of the length field is 1 (based on a minimum length for the channel list field of 0 octets).

Regulatory Class shall contain an enumerated value from Error: Reference source not found, specifying the regulatory clasd in which the Channel List is valid. An AP Channel Report shall only report channels for a single frequency band. Multiple AP Channel Report elements may be used to report channels in more than one frequency band.

The Channel List shall contain a variable number of octets, where each octet describes a single channel number. Channel numbering shall be dependent on Regulatory Class according to Error: Reference source not found.

The AP Channel Report contents shall be derived from dot11APChannelReportTable. An AP Channel report shall only include channels that are valid for the regulatory domain in which the AP transmitting the element is operating and consistent with the Country element in the frame in which it appears.

#### 7.3.2.27 Neighbor Report element

The format of the Neighbor Report element is shown in Figure k33.

|  |  |  | Zero or more, one for each neighbor |  |
| --- | --- | --- | --- | --- |
|  | Element ID | Length | Neighbor List |  |
| Octets: | 1 | 1 | Variable |  |

Figure k33—Neighbor Report element format

The Element ID field is equal to the Neighbor Report value in Table 20.

The value of Length field is dependent on the number of Neighbor List Entries representing the neighboring APs being reported. Each entry describes an AP and consists of BSSID, BSSID Information, Channel Number, Regulatory Class, PHY Options, and optionally may include the TSF offset fields. The minimum value of the Length field is 0 (i.e., with no neighbor APs in the Neighbor Report element).

The Neighbor List comprises of zero or more Neighbor List Entries. See Figure k34.

|  | BSSID | BSSID information | Channel Number | Regulatory Class | PHY Options | TSF offset (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 2 | 1 | 1 | 1 | 4 |

Figure k34—Neighbor list entry format

The BSSID is the BSSID of the BSS being reported. The subsequent fields in the Neighbor List Entry pertain to this BSS.

The BSSID Information field may be used to help determine neighbor BSS transition candidates. It shall be two octets in length and shall contain the subfields as shown in Figure k35.

|  | B0 | B1 | B2 | B3 | B4 | B9 | B10 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AP Reachability |  | Security | Key Scope | Capabilities |  | Reserved |  |
| Bits: | 2 |  | 1 | 1 | 6 |  | 6 |  |

Figure k35—BSSID information field

The AP Reachability field indicates whether the AP identified by this BSSID is reachable by the STA that requested the Neighbor Report for the exchange of preauthentication frames as described in clause 8.4.6.1. The values are shown in Table k10.

Table k10—Reachability field

| Value | Reachability | Usage |
| --- | --- | --- |
| 0 | Reserved | n/a |
| 1 | Not Reachable | A station sending a preauthentication frame to the BSSID will not receive a response even if the AP represented by the BSSID is capable of preauthentication. |
| 2 | Unknown | The AP is unable to determine if the value Reachable or Not Reachable is to be returned. |
| 3 | Reachable | The station sending a preauthentication frame to the BSSID can receive a response from an AP that is capable of preauthentication. |

The Security bit, if set, indicates that the AP identified by this BSSID supports all of the security capabilities of the AP with which the STA is currently associated. If the bit is not set, it indicates that either the capabilities do not match or the information is not available to the AP at this time.

The Key Scope bit, when set, indicates the AP represented by this BSSID has the same authenticator as the AP sending the report. If this bit is not set, it indicates a distinct authenticator or the information is not available.

The Capabilities Subfield contains selected capability information for the AP represented by this BSSID. The bit fields within this subfield shall have the same meaning and be set equal to the equivalent bits within the Capability Information field being sent in the beacons by the AP being reported. The format of the Capabilities subfield is as in Figure k36.

|  | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Spectrum Management | QoS | APSD | Radio Mesaurement | Delayed Block Ack | Immediate Block Ack |
| Bits: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure k36—Capabilities Subfield

Bits 10 – 15 are reserved and shall be set to 0 on transmission and ignored on reception.

Channel Number indicates the current operating channel of the AP represented by the BSSID in this neighbor list entry.

Regulatory Class contains an enumerated value from Error: Reference source not found specifying the frequency band in which the Current Channel is valid.

The PHY Options field contains the Condensed PHY type and TSF offset Flag as shown in the Figure k37.

|  | B0 B6 | B7 |
| --- | --- | --- |
|  | Condensed PHY Type | TSF offset Flag |
| Bits: | 7 | 1 |

Figure k37—PHY Options field

Condensed PHY Type indicates the PHY type of the AP being reported. It shall have an integer value between 0 and 127 as defined by dot11PHYType.

TSF offset Flag is a one bit field. When it is set to 1 it indicates that a TSF offset field is present for this Neighbor List Entry. When set to 0 it indicates that a TSF offset field is not present for this Neighbor List Entry. The Neighbor Report may have the TSF offset Flag set in some Neighbor List Entries and not set in others.

The TSF offset is 4 octets long and contains TSF Offset and Beacon Interval subfields.

|  | TSF Offset | Beacon Interval |
| --- | --- | --- |
| Octets: | 2 | 2 |

Figure k38—TSF offset field format

The TSF Offset subfield is 2 octets long and contains the neighbor AP’s TSF timer offset. This is the time difference, in TU units, between the serving AP and a neighbor AP. This offset is given modulo the neighbor AP’s Beacon Interval and rounded to the nearest TU boundary.

The Beacon Interval field is the beacon interval of the Neighbor AP being reported. This field is defined in clause 7.3.1.3 and illustrated in Figure 26.

#### 7.3.2.28 RCPI element

The RCPI element contains a RCPI value.

|  | Element ID | Length | RCPI |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure k39—RCPI element format

The length field shall be set to 1.

The RCPI field shall contain the RCPI value as defined in the RCPI measurement clause for the PHY type at the measuring STA. The RCPI element is used in the active scan procedure as described in 11.1.3.2.2. The RCPI Information element is also used in the Association and Reassociation Response frame to indicate the received power level of the corresponding Association or Reassociation Request frame.

#### 7.3.2.29 BSS Load element

The BSS Load element contains information on the current service level in the BSS. The element information field is defined in  Figure k.

|  | Element ID | Length | AP Service Load |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |  |  |  |

Figure k40—BSS Load element format

The Length field shall be set to 1.

The AP Service Load shall be a scalar indication of the relative level of service loading at an AP. A low value shall indicate more available service capacity than a higher value. The value 0 shall indicate that this AP is not currently serving any STA. The value 255 shall indicate that the AP Service Load is not available. The values between 0 and 254 shall be a logarithmically scaled representation of the average medium access delay for DCF transmitted packets measured from the time the DCF packet is ready for transmission (i.e. begins CSMA/CA access) until the actual packet transmission start time. The access delay time values shall be scaled as follows:

0:                        Access Delay  <  50 usec

1:     50 usec <=  Access Delay  <  51 usec

2:     51 usec <=  Access Delay  <  52 usec

3:     52 usec <=  Access Delay  <  53 usec

and so on where

n:     50*(10^ ((n-1)*0.081/10))  <=  Access Delay  <  50*(10^ (n*0.081/10))

in which the calculated range limit values are rounded to nearest integer and where n is the integer value (step) used to indicate the measured Access Delays.

251:     5296 usec <=  Access Delay  <  5396 usec

252:     5396 usec <=  Access Delay  <  5498 usec

253:     5498 usec <=  Access Delay

254:      Service not available (blocked)

255:      Measurement not available

A value of 1 shall represent a 50 us delay while a value of 253 shall represent a 5.5 ms delay or any delay greater than 5.5 ms. The value 254 shall indicate that DCF services are currently not available (blocked). The AP shall measure and average the medium access delay for all transmit packets using DCF access mechanism over a continuous thirty second measurement window. The accuracy for the average medium access delay shall be +/- 200 usec or better when averaged over at least 200 packets.

.

Figure k

7.3.2.30 Antenna Information element

The Antenna Information element contains the Antenna ID field as shown in Figure k42.

|  | Element ID | Length | Antenna ID |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |  |

Figure k42—Antenna Information element format

The length field shall be set to 1.

The Antenna ID field contains the identifying number for the relevant antenna. When included in a Beacon or Probe Response, the Antenna ID identifies the antenna used to transmit the Beacon or Probe Repsone frame. When included in a measurement report, the Antenna ID iidentifies the antenna(s) used for the reported measurement. The valid range for the Antenna ID is 1 through 254. The value 0 shall indicate that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna. STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending integers starting with 1. Each Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a fixed peak gain for that position and direction.

#### 7.3.2.31 RSNI element

The RSNI element contains an RSNI value, as shown in Figure k43.

|  | Element ID | Length | RSNI |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure k43—RSNI element format

The length field shall be set to 1.

The RSNI field contains the RSNI value for the received frame as measured by the reporting STA at the receiving antenna connector. RSNI is the received signal to noise plus interference ratio derived from the measured RCPI for the received frame and from the measured ANPI for the channel used to receive the frame. RSNI is expressed in dB in 1/2 dB units (steps). RSNI is calculated by the ratio of the received signal power (RCPI - ANPI) to the noise plus interference power (ANPI), where RSNI = [(ratio(dB) + 10) * 2], for ratios in the range -10dB to +118dB.

## 7.4 Action frame format details

Insert the following new clauses after the end of clause 7.4.4:

### 7.4.5 Radio Measurement action details

Several Action frame formats are defined for Radio Measurement purposes. An Action field, in the octet field immediately after the Category field, differentiates the formats. The Action field values associated with each frame format are defined in Table k11.

Table k11 — Radio Measurement Action field values

| Action field value | Description |
| --- | --- |
| 0 | Radio Measurement Request |
| 1 | Radio Measurement Report |
| 2 | Link Measurement Request |
| 3 | Link Measurement Report |
| 4 | Neighbor Report Request |
| 5 | Neighbor Report Response |
| 6-255 | Reserved |

#### 7.4.5.1 Radio Measurement Request frame format

The Radio Measurement Request frame uses the Action frame body format. It is transmitted by a STA requesting another STA to make one or more measurements one or more channels. The format of the frame is shown in Figure k44.

|  | Category | Action | Dialog Token | Number of Repetitions | Measurement Request Elements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | variable |

Figure k44—Radio Measurement Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to indicate a Measurement Request according to Table k11 in 7.4.5.

The Dialog Token field shall be set equal to a non-zero value chosen by the STA sending the radio measurement request to identify the request/report transaction.

The Number of Repetitions field contains the requested number of repetitions for all the Measurement Request elements in this frame. A value of zero in the Number of Repetitions field indicates Measurement Request elements are executed once without repetition.

The Measurement Request Elements field shall contain zero or more of the Measurement Request elements described in 7.3.2.21. The number and length of the Measurement Request elements in a Measurement Request frame is limited by the maximum allowed MMPDU size.

#### 7.4.5.2 Radio Measurement Report frame format

The Measurement Report frame uses the Action frame body format. It is transmitted by a STA in response to a Measurement Request frame or by a STA providing a triggered autonomous measurement report. The format of the frame is shown in Figure k45.

|  | Category | Action | Dialog Token | Measurement Report Elements |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure k45—Measurement Report frame body format

The Category field shall be set to indicate the Radio Measurement category according to Table 19a.

The Action field shall be set to indicate a Measurement Report according to Table k11.

The Dialog Token field shall be set equal to the value in any corresponding Measurement Request frame. If the Radio Measurement Report frame is not being transmitted in response to a Radio Measurement Request frame then the Dialog token shall be set equal to zero.

The Measurement Report Elements field shall contain one or more Measurement Report elements described in 7.3.2.22. The number and length of the Measurement Report elements in a Radio Measurement Report frame is limited by the maximum allowed MMPDU size.

#### 7.4.5.3 Link Measurement Request Frame Format

The Link Measurement Request frame uses the Action frame body format and is transmitted by a STA to request another STA to respond with a Link Measurement Report frame to enable measurement of link path loss and estimation of link margin. The format of the frame is shown in Figure k46.

|  | Category | Action | Dialog Token | Transmit Power | Max Transmit Power |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure k46—Link Measurement Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to indicate a Link Measurement Request according to Table k11 in 7.4.5.

The Dialog Token field shall be set equal to a non-zero value chosen by the STA sending the request to identify the transaction.

The Transmit Power element shall be set to the transmit power used to transmit the frame containing the Link Measurement Request. The element is coded as a signed integer in units of decibels relative to 1 mW. The maximum tolerance for the transmit power value reported in the Transmit Power element shall be ± 5 dB by the STA transmitting the Link Measurement Request frame.

The Max Transmit Power element is coded as a signed integer and is 1 octet in length. It provides an upper limit, in units of decibels, on the transmit power as measured at the output of the antenna connector to be used by the transmitting STA on the current channel.

#### 7.4.5.4 Link Measurement Report Frame Format

The Link Measurement Report frame uses the Action frame body format and is transmitted by a STA in response to a Link Measurement Request frame. The format of the frame is shown in Figure k47.

|  | Category | Action | Dialog Token | TPC Report element | Receive Antenna ID | Transmit Antenna ID |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 | 1 | 1 |

Figure k47—Link Measurement Report frame body format

The Category field shall be set to indicate the Radio Measurement category according to Table 19a in 7.3.1.11.

The Action field shall be set to indicate a Link Measurement Report according to Table k11 in 7.4.5.

The Dialog Token field shall be set equal to the Dialog Token value in the corresponding Link Measurement Request frame.

The TPC Report element shall be set as described 7.3.2.18.

The Receive Antenna ID field contains the identifying number for the antenna used to receive the corresponding Link Measurement Request frame. Antenna ID is defined in 7.3.2.29.

The Transmit Antenna ID field contains the identifying number for the antenna used to transmit this Link Measurement Report frame. Antenna ID is defined in 7.3.2.29.

#### 7.4.5.5 Neighbor Report Request frame format

The Neighbor Report Request frame uses the Action frame body format and is transmitted by a STA requesting information in the Neighbor Report about neighboring AP’s. The format of the Neighbor Report Request frame body is shown in Figure k48.

|  | Category | Action | Dialog Token | Neighbor Report Request Types | SSID element |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable |

Figure k48—Neighbor Report Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to the value indicating Neighbor Report Request, as specified in Table k11 in 7.4.5.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the measurement request to identify the request/report transaction.

The Neighbor Report Request Types field shall be one octet in length and shall contain the subfields as shown in Error: Reference source not found.

|  | B0 | B1 B7 |
| --- | --- | --- |
|  | Neighbor TSF offset Request | Reserved |
| Bits: | 1 | 7 |

Figure k49—Neighbor Report Request Types Subfield

Neighbor TSF offset Request – This bit is set to 1 to request TSF offset information be provided in neighbor list entires if available. When this bit is set to 0 the TSF Info field shall not be included in any neighbor list entries.

All other bits are reserved and shall be set to 0 on transmission and ignored on reception.

The SSID element is defined in 7.3.2.1. It’s inclusion shall indicate a request for a neighbor list for the specified SSID in the SSID Element. The absence of a SSID element indicates neighbor report for the current ESS.

#### 7.4.5.6 Neighbor Report Response frame format

The Neighbor Report Response frame uses the Action frame body format and is transmitted by a STA in response to a Neighbor Report Request frame. The format of the Neighbor Report Response frame body is shown in Figure k50.

|  | Category | Action | Dialog Token | Neighbor Report Elements |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure k50—Neighbor Report Response frame body format

The Category field shall be set equal to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set equal to the value indicating Neighbor Report Response, as specified in Table k11 in 7.4.5.

The Dialog Token field shall be set equal to the value in the corresponding Neighbor Report Request frame. If the Neighbor Report Response frame is not being transmitted in response to a Neighbor Report Request frame then the Dialog token shall be set equal to zero.

The Neighbor Report Elements field shall contain the Neighbor Report elements described in 7.3.2.27. If the STA has no information in response to the Neighbor Report Request, the Neighbor Report elements shall be omitted. The number and length of the Neighbor Report Elements in a Neighbor Report frame is limited by the maximum allowed MMPDU size.

# 11 MLME

## 11.1 Synchronization

### 11.1.3 Acquiring synchronization, scanning

#### 11.1.3.2 Active scanning

##### 11.1.3.2.1 Sending a probe response

Change the first paragraph as shown below:

STAs, subject to criteria below, receiving Probe Request frames shall respond with a probe response only if the SSID in the probe request is the broadcast SSID or matches the specific SSID of the STA. If the DS Parameter Set information element is present in the probe request, a STA where dot11RadioMeasurementEnabled is true shall respond only if the channel number from the DS Parameter Set element matches the channel in use by the STA. If the DS Parameter Set information element is present in the probe request, a STA where dot11RadioMeasurementEnabled is false may respond only if the channel number from the DS Parameter Set element matches the channel in use by the STA. Probe Response frames shall be sent as directed frames to the address of the STA that generated the probe request. The probe response shall be sent using normal frame transmission rules. An AP shall respond to all probe requests meeting the above criteria. In an IBSS, the STA that generated the last beacon shall be the STA that responds to a probe request.

Insert the following paragraph at the end of clause 11.1.3.2.1:

When a probe response frame is returned in response to a probe request frame which contains Requested information elements, any of the requested elements which appear as individual items in the ordering list of Table 12 shall appear both in their individual ordered location as specified in Table 12 and in the ordered location reserved for the list of requested elements, where the requested elements appear in increasing numerical element ID order.

##### 11.1.3.2.2 Active scanning procedure

Insert the following text at the end of this clause:

If dot11RadioMeasurementEnabled is true and a Probe Request with a Request Information element which contains the RCPI element ID is received then a RCPI element shall be included in the Probe Response. If a RCPI element is received in a Probe Response frame, the RCPI value shall be included in the RCPIMeasurement parameter of the BSSDescription in the MLME-SCAN.confirm.

If dot11RadioMeasurementEnabled is true and if the Request Information element of the Probe Request includes the RCPI element ID, an AP shall include a RCPI element in the Probe Response. An AP may measure RCPI on the received Probe Request frame and include the result in the RCPI element of the Probe Response. If no measurement result is available the RCPI value shall be set to indicate that a measurement is not available.

Change clause 11.9 as follows:

## 11.9 TPC procedures

ERC/DEC/(99)23 requires RLANs operating in the 5GHz band to use transmitter power control, involving specification of a regulatory maximum transmit power and a mitigation requirement for each allowed channel, to reduce interference with satellite services. This Standard describes such a mechanism, referred to as Transmit Power Control (TPC).

This clause describes TPC procedures that may be used to satisfy these and similar future regulatory requirements in Europe. The procedures may also satisfy comparable needs in other regulatory domains and other frequency bands and may be useful for other purposes, such as reduction of interference, range control and reduction of power consumption.

STAs shall use the TPC procedures defined in this clause if dot11SpectrumManagementRequired is true or dot11RadioMeasurementEnabled is true. dot11SpectrumManagementRequired shall be set true when regulatory authorities require TPC. It may also be set true in other circumstances. The TPC procedures provide for the:

- Association of STAs with an AP in a BSS based on the STA’s power capability (11.9.1).

- Specification of regulatory and local maximum transmit power levels for the current channel (11.9.2).

- Selection of a transmit power for each transmission in a channel within constraints imposed by regulatory and local requirements (11.9.3).

- Measurement of link path loss and estimation of link margin (11.9.4).

- Adaptation of transmit power based on a range of information, including path loss and link margin estimates (11.9.4).

For the purposes of TPC:

- A STA with dot11SpectrumManagementRequired set equal to true shall not operate in a BSS or IBSS unless the Spectrum Management bit is set equal to 1 in the Capability Information field in Beacons, Measurement Pilot frames, and Probe Responses received from other STAs in the BSS or IBSS, with the following exception.

- A STA may operate when the Spectrum Management bit is set equal to 0 if the STA can determine that it is in a regulatory domain that does not require TPC, or can ensure that it will meet regulatory requirements even if TPC is not employed. Potential methods for determining the regulatory domain include receiving a country indication in the beacon, Measurement Pilot frame, user confirmation, or configuration information within the device. Potential methods to ensure regulations are met even if TPC is not employed include using a transmit power that is below the legal maximum (including any mitigation factor).

- A STA shall set dot11SpectrumManagementRequired equal to true before associating with a BSS or IBSS in which the Spectrum Management bit is set equal to 1 in the Capability Information field in Beacons, Measurement Pilot frames, and Probe Responses received from the BSS or IBSS.

- APs may allow association of devices that do not have the Spectrum Management bit set equal to 1 in the Capability Information field in Association and Reassociation Requests received from the STA to account for the existence of legacy devices that do not support TPC but do meet regulatory requirements.

Change clause 11.9.2 as follows:

### 11.9.2 Specification of regulatory and local maximum transmit power levels

A STA shall determine a regulatory maximum transmit power for the current channel. The STA shall use the minimum of the following:

- Any regulatory maximum transmit power received in a Country element from the AP in its BSS or another STA in its IBSS and

- Any regulatory maximum transmit power for the channel in the current regulatory domain received in the Max Regulatory Power field from the AP in its BSS or known by the STA from other sources.

A STA shall determine a local maximum transmit power for the current channel. The STA shall use the minimum of the following:

- Any local maximum transmit power received in the combination of a Country element and a Power Constraint element from the AP in its BSS or another STA in its IBSS and

- Any local maximum transmit power for the channel regulatory domain known by the STA from other sources.

Any calculation of the local maximum transmit power for the channel shall ensure the mitigation requirements for the channel in the current regulatory domain can be satisfied. The conservative approach is to set the local maximum transmit power level equal to the regulatory maximum transmit power level minus the mitigation requirement. However, it may be possible to satisfy the mitigation requirement using a higher local maximum transmit power level. A lower local maximum transmit power level may be used for other purposes (e.g., range control, reduction of interference).

The regulatory and local maximum transmit powers may change in a STA during the life of a BSS. However, network stability should be considered when deciding how often or by how much these maximums are changed. The regulatory and local maximum transmit powers shall not change during the life of an IBSS.

An AP in a BSS and a STA in an IBSS shall advertise the regulatory maximum transmit power for the current channel in Beacon frames and Probe Response frames using a Country element. An AP in a BSS and a STA in an IBSS shall advertise the local maximum transmit power for the current channel in Beacon frames and Probe Response frames using the combination of a Country element and a Power Constraint element.

When dot11MeasurementPilotEnabled is set to true, an AP in a BSS shall advertise the regulatory maximum transmit power for the current channel in Measurement Pilot frames using a Max Regulatory Power field.

Where TPC is being used for radio resource measurement without spectrum management, the inclusion of a Power Constraint element in Beacon and Probe Response frames shall be optional.

Insert the following new clauses after clause 11.10:

## 11.11 Radio Measurement Procedures

This clause describes the radio measurements and the procedures for requesting and reporting radio measurements between STAs.

### 11.11.1 Dedicated versus concurrent measurements

Measurements on non-serving channels are dedicated measurements, requiring the measuring STA to interrupt its data services on the serving channel, switch channels and make measurements.

Measurements on the serving channel may be executed by the STA as concurrent measurements and may not require the STA to interrupt its data services.

### 11.11.2 Station responsibility for measuring non-serving channel

All stations are responsible for maintaining data services and an association or membership with the BSS or IBSS respectively, on the serving channel while performing measurements on non-serving channels.

A STA shall determine the time between successive non-serving channel measurements. This time may be a fixed length, or it may be determined by the STA using application-specific, or other knowledge.

### 11.11.3 Measurement start time

A Radio Measurement Request frame may contain a single Measurement Request element or a sequence of Measurement Request elements. A STA that accepts the first, or only measurement request within a Radio Measurement Request frame shall start the measurement as soon as practical after receiving the request. Subsequent measurement requests in the Radio Measurement Request frame that are accepted shall start as soon as practical after processing the previous request in the frame.

The Radio Measurement category permits a randomization interval to be specified for measurement start times. This avoids the traffic storms that could arise with synchronized broadcast and multicast measurements. Prior to making each measurement in the requested sequence, the STA shall calculate a random delay distributed uniformly in the range 0 to the randomization interval specified in the measurement request. The STA shall not start the measurement until this delay has expired. A Randomization Interval of 0 in a measurement request shall mean that no random delay is to be used.

NOTE—It is important that designers recognize the need for statistical independence among the pseudo random number streams among STAs.

A number of repetitions may be specified in the Radio Measurement Request frame. In this case, the measurements in the frame are repeated as detailed further in clause 11.11.7. Each time a measurement is repeated, the STA shall recalculate the random delay as described above.

### 11.11.4 Measurement Duration

If the Duration Mandatory bit is set to 1 in the Measurement Request mode field of a measurement request, the requested STA, if it accepts the request, shall perform the measurement over the Measurement Duration specified in the request. If the STA is unable to commit to making the measurement over the requested duration it shall refuse the request by sending a measurement report with the refused bit set in the Measurement Report Mode field. The measurement duration in the measurement report shall be equal to the requested measurement duration.

If the Duration Mandatory bit is set to 0 in the Measurement Request mode field of a measurement request, the requested STA, if it accepts the request, shall attempt a measurement using the requested duration as a target measurement duration, and may report results with an actual measurement duration less than the requested duration. The duration over which the measurement was made shall be included in the measurement duration field of the measurement report.

Each separate measurement within the Radio Measurement Request frame shall be performed over a continuous measurement duration time period. In Measurement Request frames, the requested Measurement Duration value shall not be set set to 0 except for Beacon Request with Measurement Mode set to Beacon Table mode and Statistics Request measurements.

### 11.11.5 Station responsibility for conducting measurements

A Radio Measurement-capable STA shall decode and interpret each Radio Measurement Request frame that it receives and shall assess the contents against it’s capabilities and the impact on its own performance. A measurement request can be refused by the receiving STA by sending a Radio Measurement Report with the refused bit set in the Measurement Report Mode field if its execution would significantly degrade the station's performance. The reasons for refusing a measurement request are outside the scope of this standard but may include reduced quality of service, unacceptable power consumption, measurement scheduling conflicts, or other significant factors.

In assessing the performance impact of each measurement request element, a STA may use application-specific knowledge or other knowledge to limit the time it spends away from the serving channel. In doing so, the STA may either reject any Measurement Request element with a mandatory measurement duration exceeding the maximum allowed off-serving channel time, or measure for a reduced duration.

Radio measurement requests and reports are specific transactions localized to a BSS. A STA shall cancel all in-process radio measurements and shall delete all pending, unprocessed radio measurement requests upon receipt of a Disassociation message or upon association or reassociation with a BSSID different from its most recent association.

### 11.11.6 Requesting and reporting of measurements

A STA may measure one or more channels itself or a STA may request peer STAs in the same BSS to measure one or more channels on its behalf.

When requesting other STAs to measure one or more channels, a STA shall use a Radio Measurement Request frame containing one or more Measurement Request elements. The measurement request may be sent to an individual or group destination address. The permitted measurement requests are shown in Table k12.

Table k12—Allowed measurement requests

| Service Set | Source of Request | Destination of Request | Receiver Address of Radio Measurement Request Frame |
| --- | --- | --- | --- |
| Infrastructure BSS | AP | STA | Individual or group |
|  | STA | AP | Individual only |
|  | STA | STA | Individual only for DLS within a QBSS, otherwise not allowed |
| IBSS | STA | STA | Individual or group |

The source and destination of a measurement request shall both be a member of the same infrastructure BSS or a member of the same IBSS. Measurement requests with an individual Receiver Address shall only be sent to STAs that have indicated Radio Measurement capability.

The set of requested measurements received in the most recently received Radio Measurement Request frame of highest precedence is active at a STA. The precedence order for measurement requests shall be as follows (highest precedence first):

- Measurement requests received in individually addressed Radio Measurement Request frames

- Measurement requests received in received in Multicast-group addressed Radio Measurement Request frames

- Measurement requests received in received in Broadcast addressed Radio Measurement Request frames

The Measurement Request elements are processed in sequence by default, with certain Measurement Request elements processed in parallel according to the parallel bit field setting – see 7.3.2.21. If measurement resources are available, the STA processes each element by setting up and making the specified measurement.

The Measurement Request elements within a Radio Measurement Request frame may specify multiple measurement types across multiple channels.

A STA may receive another Radio Measurement Request frame while the measurements requested in a previous Radio Measurement Request frame are pending or in progress. In this case, the set of measurement requests in the new frame supersedes any previous requests received in a Radio Measurement Request frame of the same or lower precedence. The measuring STA shall report the results of any completed measurements and terminate any pending or in-progess measurements. Results from a terminated in-progess measurement may be valid and reported if Duration Mandatory was not set to 1 in the corresponding request. It is permissible for the superceding Radio Measurement Request frame to contain no new measurement requests. If a station receives a Radio Measurement Request frame with lower precedence than the currently active Radio Measurement Request frame, the station shall discard the measurement requests in the new Radio Measurement Request frame. Measurement Request elements that have the Enable bit set to 1 shall be processed in all received Radio Measurement Request frames regardless of these precedence rules.

If a STA receives a spectrum management Measurement Request with Measurement Type equal to 0 (Basic Request), this shall always take priority over any pending or in-progress radio measurements.

A STA that issues a radio measurement request to another STA to perform a measurement on the serving channel may continue to transmit MPDUs and MMPDUs to that STA while the measurement is being processed.

A STA that issues a radio measurement request to another STA to perform a measurement on a non-serving channel is not required to take any special action to suspend traffic to that STA. All stations shall maintain state information such that data services and association or membership with the BSS or IBSS respectively can continue when returning from a non-serving channel measurement.

The result of each measurement requested in a Measurement Request element shall be reported in one or more Measurement Report elements of type corresponding to the request. Each Measurement Report element returned shall have the same Measurement Token as in the corresponding Measurement Request element. The results of each measurement should be returned without undue delay to the requesting STA.

Measurement Report elements shall be returned to the requesting STA in one or more Radio Measurement Report frames. Each Radio Measurement Report frame shall contain the same Dialog Token field value as the corresponding Radio Measurement Request frame.

When a STA is permanently unable to make a requested measurement the STA shall respond to such a measurement request received within a unicast Radio Measurement Request frame with a measurement report indicating that it is incapable of completing the measurement request. A STA shall not respond to requests received in broadcast and multicast frames in this manner. Examples of when an incapable response is appropriate are:

—	The requested measurement type is not supported.

—	The measuring STA cannot support requested parallel measurements due to the requests relating to different channels.

A STA that receives a response with an incapable indication shall not make the same request to the responding STA. This is logically the same as the responding STA using the Enable and Request bits in a measurement request to indicate that it will not accept measurement requests of a certain type. A STA that has indicated an incapable response to a requesting STA may discard further requests of the same type from that STA.

A STA may refuse to make any requested measurement. A STA refusing a measurement request within a unicast Radio Measurement Request frame shall respond with a measurement report indicating that it is refusing the measurement request. A STA shall not respond to measurement requests received in broadcast or multicast Radio Mesaurement Request frames in this manner.

Note—Since measurements on non-serving channels could potentially degrade a station’s performance, non-serving channel measurements should be requested sparingly and for short durations. Since measurements on the serving channel execute concurrently with normal traffic processing, serving channel measurements may be requested more frequently and for longer durations.

### 11.11.7 Repeated measurement request frames

Radio Measurement Request frames contain a field specifying the number of repetitions for the Radio Measurement Request frame. If the Radio Measurement Request frame includes a non-zero value for the Number of Repetitions, the STA shall iterate (repeat) the processing of all the Measurement Request elements in the frame as specified by the value in the Number of Repetitions field. When completing the processing of the last Measurement Request element in the frame, the STA shall begin processing of the first Measurement Request element in the frame to repeat the frame.

Measurement results shall be reported for each repetition of a repeated measurement request subject to any conditional reporting requirement.

STAs responding with incapable or refused indications to measurement requests within a Radio Measurement Request frame with a non-zero value for Number of Repetitions shall only respond once.

### 11.11.8 Triggered Autonomous Reporting

Autonomous reporting is defined for Spectrum Management measurements supporting DFS – see 11.6.6. It allows a STA to report the results of measurements to a peer STA for which there was no explicit measurement request. In this case, the transmission of autonomous reports shall be entirely the decision of the STA at which such reporting has been enabled. An example of this use would be to report a change in conditions at the STA observed as a result of background measurement, e.g. the presence of a radar signal.

In radio measurement, all autonomous reporting shall be subject to trigger conditions set by the enabling STA that determine when measurement reports are issued. This is termed triggered autonomous reporting and provides a method for reporting during continuous background measurement. An example of the use of triggered autonomous measurement is for reporting problem conditions in continuous, non-invasive statistical monitoring.

Triggered autonomous reporting is defined only for the Transmit QoS Metrics measurement type – see 11.11.9.10.

A STA indicates that it wishes to accept triggered autonomous reports by sending a Measurement Request element with the Enable and Report bits set to 1 – see 7.3.2.21. The type of measurement is indicated in the Measurement Type field. Trigger conditions that determine when measurement reports are to be generated shall be specified in the Measurement Request field. A Measurement Request element that is being used to control triggered autonomous reporting shall be sent within a Radio Measurement Request frame. Measurement Request elements being used to request measurements may also appear in the same Measurement Request Frame. The Measurement Request frame may be sent to a group receiver address to enable triggered autonomous reports at more than one STA.

A STA shall not send autonomous reports for radio measurement types without trigger conditions having been set. If a Measurement Request element is received with the Enable and Report bits set to 1 without trigger conditions in the Measurement Request field then that Measurement Request element shall be ignored.

If a request to enable triggered autonomous reporting is sent to an individual address and the receipient STA does not support measurements of the type indicated, a Measurement Report element shall be returned to the requesing STA with the Incapable bit set. A STA may also refuse to enable triggered autonomous reporting. In this case a Measurement Report element shall be returned to the requesting STA with the refused bit set. Such responses shall not be issued if the request to enable triggered autonomous reporting was sent to a group address.

A STA receiving a request to enable triggered autonomous reporting from another STA may send reports of the appropriate type, addressed to the individual address of the STA that sent the enable request. Autonomous reports shall only be sent to the individual addresses of STAs from which a valid enable request has been received and shall only be issued when the trigger conditions have been met.

A STA may update the trigger conditions set for triggered autonomous reports by issuing a new Measurement Request element with the Enable and Report bits both set to 1, the Measurement Type field set to the appropriate type and the Measurement Request field indicating the new trigger conditions. A STA disables all triggered autonomous measurement reports by sending a Measurement Request element with the Enable bit set to 1 and the Report bit set to 0 – see 7.2.3.21.

A STA in an infrastructure BSS shall cease all triggered autonomous reporting if it disassociates, or re-associates to a different BSS. A STA in an independent BSS shall cease all triggered autonomous reporting if it leaves the BSS.

Triggered autonomous reporting and requested measurements are independent: a STA may request measurements from another STA even if it has enabled triggered autonomous reporting from that STA. All Measurement Request elements received in Radio Measurement Request frames that have the Enable bit set shall be processed without regard for the measurement precedence rules for requested measurements in 11.7.6.

### 11.11.9 Specific measurement usage

#### 11.11.9.1 Beacon Report

If a STA accepts a Beacon Request it shall respond with a Radio Measurement Report frame containing Beacon Measurement Reports for all observed BSSs matching the BSSID and SSID in the Beacon Measurement Request. The RCPI in the Beacon Report indicates the power level of the received Beacon, Measurement Pilot or Probe Response frame. RCPI in a Beacon Report element may be used with other Beacon Report elements to determine the most suitable AP target for BSS transitions. For repeated measurements (when the Measurement Request frame contains a non-zero value for the Number of Repetitions field), the transmission of the Beacon Report element may be conditional on the measured RCPI or RSNI value. When the Measurement Request frame contains a 0 value for the Number of Repetitions field, the Reporting Condition field in all Beacon Requests in that frame shall be set to 0.  Table k3 lists the reporting conditions that are based on the measured RCPI orIRSNI levels.

If the Measurement Mode in the measurement request is Passive or Passive Pilot, the measuring STA shall perform the following procedure (or equivalent procedure) on the requested channel:

- Set a measurement duration timer.

- At the end of the measurement duration, process all received Beacon, or Probe Response management frames with the requested SSID and BSSID to compile the measurement report. If no Beacons or Probe Responses with the requested SSID and BSSID were received in the measurement duration, and if the requested Measurement Mode was Passive Pilot, process all Measurement Pilot Frames with the requested BSSID to compile the measurement report. Otherwise, compile an empty Beacon measurement report.

If the Measurement Mode in the measurement request is Active, the measuring STA shall perform the following procedure (or equivalent procedure) on the requested channel:

- If the channel is not the serving channel, wait for dot11RadioProbeDelay , or until a PHY-RXSTART.indication has been received.

- Using the basic access protocol in 9.2.5.1, send a Probe Request management frame to the broadcast destination address (DA). The BSSID field in the Probe Request shall be set to the BSSID field in the measurement request. The SSID element in the Probe Request shall be set to the SSID element in the measurement request.

- Set a measurement duration timer.

- At the end of the measurement duration, process all received Probe Response and Beacon management frames with the requested SSID and BSSID to compile the measurement report. Otherwise, compile an empty Beacon measurement report.

If the Measurement Mode is STA Selected, the selection of measurement mode shall be made by the measuring STA. STA selected measurement mode may be requested when a STA requests a Beacon measurement and will accept measurement reports in Passive, Passive Pilot or Active modes. If the measuring STA supports Passive Pilot mode and the measurement request contained the wildcard SSID, the measuring STA shall select between Passive, Passive Pilot, or Active mode. Otherwise the selection shall be between Passive and Active mode. The choice of mode is outside the scope of this amendment.

When more than one Beacon, or Probe Response from a BSS is received in the measurement duration, the contents of the Beacon Report shall be based on the latest received. If only Measurement Pilot frames were received in the measurement duration and the Measurement Mode was Passive Pilot, the contents of the Beacon Report shall be based on the latest Measurement Pilot frame received.

If the BSSID field in the Measurement Request contains a broadcast BSSID, all observed BSSs with the requested SSID shall be reported. If the SSID element in the Measurement Request contains the wildcard SSID, all observed BSSs shall be reported. In Active mode, Probe Response frames shall be evaluated regardless of whether the Probe Response frame was triggered by the measuring STA's Probe Request.

On accepting a Beacon measurement request with Channel Number set to 0 a STA shall conduct iterative measurements on all supported channels in the specified Regulatory Class where the measurement is permitted on the channel and the channel is valid for the current regulatory domain. For these iterative beacon measurements, the measurement duration applies to the measurement on each channel. Measurements shall be made using the specified Measurement Duration with the time between each consecutive measurement as defined in 11.11.2. Iterative measurements shall cease when all supported channels have been measured. Note that while the STA is processing a Beacon measurement request for interative channel measurements, the STA may not begin processing the next measurement request in the measurement request frame.

On accepting a Beacon measurement request with Channel Number set to 255 a STA shall conduct iterative measurements on all supported channels listed in the AP Channel Report where the measurement is permitted on the channel and the channel is valid for the current regulatory domain. For iterative beacon measurements, the measurement duration applies to the measurement on each channel. Measurements shall be made using the specified Measurement Duration with the time between each consecutive measurement as defined in 11.11.2. Iterative measurements shall cease when all supported channels have been measured. A Beacon measurement request with Channel Number set to 255 shall only specify a specific BSSID (broadcast BSSID not allowed). If an AP Channel Report for the specific BSSID is not available in the STA, the STA shall iteratively conduct measurements on all supported channels in the specified Regulatory Class that are valid for the current regulatory domain. Note that while the STA is processing a Beacon measurement request for interative channel measurements, the STA may not begin processing the next measurement request in the measurement request frame.

If the Measurement Mode in the measurement request is Beacon Table, the measuring STA shall return a Beacon Report containing the current contents of any stored beacon information for any supported channel with the requested SSID and BSSID without performing additional measurements. The receiving STA shall ignore the channel and measurement duration specified in the Beacon Request when Beacon Table mode is selected. The beacon information accumulated may be the result of any operation that caused the STA to acquire these results. If the STA has no beacon information available then the STA may either refuse the request or send an empty Beacon Report.

For repeated measurements, the Beacon Request element may specify a reporting condition that determines when the measuring STA is to send a Beacon Report element for a measured Beacon, Measurement Pilot or Probe Response frame with the requested BSSID. When the requested Reporting Condition value is non-zero, the STA shall create and transmit a Beacon Report element for that measured frame only if the condition indicated in Table k3 is true. Otherwise, a Beacon Report element is not created for that measured frame. For reporting conditions 5-10, the serving AP's reference RCPI leveI and the serving AP's reference RSNI leveI referred to in Table k3 arng average values of the RCPI oSIRSNI of the 10 most recent Beacon frames received from the measuring STA's serving AP. The serving AP's reference RCPI levPI and the serving AP's reference RSNI levSI are so averaged to provide a more accurate and stable indication of the signal level from the serving AP. For reporting conditions 5-10, the STA shall use the serving AP's reference RCPI levPI or reference RSNI levSI (with offset, if any) to test the measured RCPI oI RSNI to determine whether to create and send a Beacon Report element for this measured Beacon, Measurement Pilot or Probe Response frame. Figure k51 diagrams the beacon RCPI measurements for reporting conditions 5 and 6.

Figure k51—Reporting Condition for RCPI Crossing Level with Offset

#### 11.11.9.2 Frame Report

If a station accepts a Frame Request it shall respond with a Radio Measurement Report frame containing one or more Measurement (Frame) Report elements. The measuring station shall receive all observable traffic during the measurement duration and shall summarize this traffic in one or more Frame Report elements. Each Frame Report element contains one or more Frame Report Entries. The measuring station shall count the number of unicast data and management frames received from one transmit address during the measurement duration and shall summarize this traffic in a Frame Report Entry. Each Frame Report Entry contains the Transmit Address, BSSID, Average RCPI, Last RCPI, Antenna ID and Number of Frames for the counted in the Frame Report Entry. The reported Average RCPI shall be the average of the RCPI values of all frames counted in the Frame Report Entry. The Last RCPI shall be the RCPI value of the most recently received frame counted in the Frame Report Entry. The Antenna ID field contains the identifying number for the antenna used to receive the most recently received frame included in this report. If different antennas are used to receive the frame preamble and the frame body, this Antenna ID shall contain the identifying number for the antenna used to receive the frame body.

#### 11.11.9.3 Channel Load Report

If a station accepts a Channel Load Request it shall respond with a Radio Measurement Report frame containing one Measurement (Channel Load) Report element. The Channel Load value is defined as Integer(256*[channel busy time (microseconds)]/(1024 * [measurement duration (TU)])). Channel busy time shall be the time during which either the physical carrier sense or the virtual carrier sense (NAV) or both indicates that the channel is busy. See 9.2.1  for description of carrier sense mechanism.

#### 11.11.9.4 Noise Histogram Report

If a station accepts a Noise Histogram Request it shall respond with a Radio Measurement Report frame containing one Measurement (Noise Histogram) Report element. The Noise Histogram Report shall contain the RPI densities observed in the channel for the nine RPI levels defined in Table k7.

To compute the RPI densities, the STA shall measure the RPI in the specified channel as a function of time over the measurement duration when NAV is equal to 0 (when virtual CS mechanism indicates idle channel) except during frame transmission or reception. The time resolution of the RPI measurements shall be in microseconds. The RPI densities are then computed for each of the nine possible RPI values using Integer (256 * ([Duration receiving at RPI value (microseconds)] / (1024 * [Measurement Duration (TU)] – [NAVBUSY (microseconds)]))). NAVBUSY is the total time in microseconds that NAV is non-zero during the Measurement Duration. The sum of the RPI densities will be approximately 256.

Average Noise Power Indicator (ANPI) value represents the average noise plus interference power on the measured channel at the antenna connector during the measurement duration. The STA may use Noise Histogram RPI density values to calculate ANPI.  The Noise Histogram Report RPI densities may be used to calculate an average noise power for the channel during the measurement duration.  This caculated average RPI power value may be reported as the value for ANPI. Other equivalent or superior methods to measure ANPI may also be used. ANPI power is defined in dBm using the same units and accuracy as defined for RCPI.

#### 11.11.9.7 STA Statistics Report

If a station accepts a STA Statistics Request it shall respond with a Radio Measurement Report frame including one STA Statistics Report element. If the Requested Measurement Duration value is 0, the STA shall report the current values for the requested Statistics Group Data. If the Requested Measurement Duration value is greater than 0, the STA Statistics Report reports the change in the requested Statistics Group Data measured within that non-zero Measurement Duration. The reported change in data value shall be the value of the data at the end of the actual Measurement Duration minus the value of the data at the beginning of the actual Measurement Duration.

#### 11.11.9.8 Location Configuration Information Report

A STA receiving an LCI request shall respond with a Radio Measurement Report frame including one Location Configuration Information element (LCI Report).

An LCI request may indicate a location request for the local STA or the remote STA by setting the LCI request Location Subject octet to indicate a Local or Remote request respectively. For a Local Request, the reporting STA shall send a LCI Report that indicates the location of the requesting STA. For a Remote Request, the reporting STA shall send a LCI Report that indicates the location of the reporting STA.

NOTE—Local LCI Measurement Request is used by requesting STA to obtain its own location by asking "Where am I?". Remote LCI Measurement Request is used by requesting STA to obtain location of reporting STA by asking "Where are you?".

If the STA receiving an LCI request lacks the means to report the requested location to the requested accuracy, then the LCI Report shall have Latitude resolution, Longitude resolution and Altitude resolution set to zero. If the STA has no information about the physical location of the ‘Local’ requestor, then it shall set the Incapable bit in the Measurement Report Mode field. The method by which the physical location information in the LCI Report is generated is not specified, to allow the accuracy of the reported location to be ‘best effort.’

NOTE—The physical location information of the STA may be set by administrative means. A STA receiving an LCI Report with the Incapable bit set can make an LCI request for the location of the ‘Remote’ STA to obtain the reporting STA’s physical location.

#### 11.11.9.9 Measurement Pause

A Measurement Pause is used within a Measurement Request Frame to provide a time delay between the processing of two other Measurement Request Elements within the sequence of Measurement Request Elements in that Frame. If a STA accepts a Measurement Pause Request it shall delay processing of the next measurement request in the Measurement Request Frame. If the Measurement Pause Request is the last Request Element in a repeated Measurement Request Frame, the STA shall delay processing the first Request Element in the Measurement Request Frame for the next repeat.. This delay shall be no less than the Pause delay specified in the Maeasurement Pause Request.  A Measurement Pause shall not be sent as the only Request Element in a Measurement Request Frame. A Measurement Pause shall not be included as the last Request Element in a Measurement Request Frame which has the Number of Repetitions field set to 0. There is no measurement report associated with a Measurement Pause Request.

#### 11.11.9.10 QoS Metrics Report

A QSTA receiving a QoS Metrics Request shall respond with a Radio Measurement Report frame containing one Measurement (QoS Metrics) Report element. If the traffic stream (TS) that is corresponding to the Traffic Identifier is deleted, either by a DELTS Action Frame or by disassociation, the STA shall cease sending Radio Measurement Reports.

The QoS Metrics measurement shall be made on traffic that is transmitted from the measuring QSTA to the peer QSTA and TC or TS indicated in the request. The Peer QSTA Address may be the MAC address of the QSTA from which the Measurement Request was sent, or the MAC address of another QSTA within the QBSS. This enables a QAP to query QoS Metrics for DLS links. A QAP shall refuse measurement requests for traffic to other QSTAs in the BSS.

The requesting and reporting STAs must be QSTAs. A non-QSTA receiving a QoS Metrics Measurement Request shall reject the request with indication of “incapable”.

A QSTA may request that a QoS metrics report be sent when MSDU discard, or delay metrics for a specified TC, or TS at a measuring QSTA reach a defined threshold. This is termed a triggered QoS metrics measurement and shall be requested by setting the Enable and Report bits to 1 within a Measurement Request Element containing the QoS Metrics Measurement Type. The Measurement Request field shall contain a QoS Metrics Request with the trigger conditions specified in the Triggered Reporting field. One or more trigger conditions may be set with specified thresholds. See 11.11.9.

A triggered QoS metrics request shall not be sent to a QAP. A QAP that receives a triggered QoS metrics request shall not respond. The number of simultaneous triggered QoS metrics measurements supported at non-AP QSTA is outside the scope of the standard.

A non-AP QSTA accepting a triggered QoS measurement shall measure the requested TC, or TS. If a trigger condition occurs, the measuring non-AP QSTA shall send a QoS metrics measurement report to the requesting QSTA. The measuring non AP-QSTA shall not send further triggered QoS reports until the Trigger Timeout period specified in the request has expired, or new trigger conditions have been requested. Measurement of QoS Metrics shall continue during the reporting timeout period.

If a non-AP QSTA receives a requested QoS metrics measurement for a TC, or TS that is already being measured using a triggered QoS metrics measurement, the triggered QoS measurement shall be suspended for the duration of the requested QoS measurement. When triggered measurement resumes the QoS metrics shall be reset.

QoS metrics reported in a triggered QoS metrics report shall be the values accumulated over the number of transmitted MSDUs prior to the trigger event given in the Measurement Count field of the QoS metrics measurement request that established the trigger condition. It is possible that a consecutive or delay trigger event occurs after acceptance of a triggered QoS metrics measurement but before the number of MSDUs in Measurement Count have been transmitted. In this case the report shall be the values accumulated since measurement started. The measurement count value appears in the Transmitted MSDU Count field of a triggered QoS metrics measurement report. Measurement duration shall not be used in triggered QoS measurement and the Measurement Duration field in both the Measurement Request and any Measurement Report shall be set to 0.

The Measurement Start Time field of a triggered QoS metrics report shall contain the value of the QSTA TSF timer at the time the trigger condition occurred to an accuracy of ±1TU.

Once accepted by a measuring non-AP QSTA, a triggered QoS measurement continues to be active until:

- The relevant TS is deleted

- The measuring non-AP QSTA disassociates or successfully reassociates, or

- The measurement is terminated by the requesting QSTA.

All triggered QoS measurements shall be terminated at a measuring non-AP QSTA by receiving a triggered QoS metrics measurement request with the Enable bit set to 1 and the Report bit set to 0. A QoS metrics measurement request with no trigger conditions shall terminate a triggered QoS measurement for the TC, or TS specified in the request. A QSTA requesting a triggered QoS measurement may update the trigger conditions by sending a triggered QoS metrics measurement request specifying the new trigger conditions.

## 11.12 Usage of the Neighbor Report

A Neighbor Report is sent by an AP and it contains information on known neighbor AP’s. A Neighbor Report may not be exhaustive either by choice, or due to the fact that there may be neighbor APs not known to the AP. The Neighbor Report contents shall be derived from the MIB table dot11RRMNeighborReportTable. The mechanism by which the contents of this table are determined is outside the scope of this amendment, but it may include information from measurement reports received from the STA’s within the BSS, information obtained via a management interface, or the DS.

### 11.12.1 Purpose of a Neighbor Report

The purpose of the Neighbor Report is to enable the STA to optimize aspects of neighbor BSS transition and ESS operation. A Neighbor Report element contains information on APs which the STA may use as candidates for a BSS transition. A Neighbor Report element shall only contain entries for validated neighbor APs that are members of ESSs requested in the Neighbor Report Request.

Since the information in the Neighbor Report may be stale, it should be considered advisory; information obtained by the report recipient through a scan or other sources may also be considered, possibly overriding information in the Neighbor Report. For example, where information contained within a Neighbor Report is contradicted by information in the Beacon/Probe Response, the Beacon/Probe Response information should take precedence; similarly, where information is available within a standardized security handshake (for example the 4-way handshake as defined in clause 8.5.3.), it may be considered.

### 11.12.2 Requesting a Neighbor Report

An associated STA requesting a Neighbor Report shall send a Neighbor Report Request frame to its associated AP. An AP accepting a Neighbor Report Request shall respond with a Neighbor Report Response frame. If there are no list entries available the AP shall send a Neighbor Report Response with no Neighbor List Entries.

### 11.12.3 Receiving a Neighbor Report

An AP receiving a Neighbor Report Request shall respond with a Neighbor Report Response frame containing zero or more Neighbor Report elements. If SSID elements are specified in the corresponding Neighbor Report Request frame, the Neighbor Report element shall only contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that are members of the current ESS or ESSs identified by the SSID elements contained within the Neighbor Report Request. If the SSID element is omitted the Neighbor Report element shall contain information from the MIB table dot11RRMNeighborReportTable concerning neighbor APs that belong to the same ESS as the requesting STA. If there are no list entries available the AP shall send a Neighbor Report Response with no Neighbor List Entries.

A serving AP shall include a TSF Offset field in the Neighbor List Entry only if it is able to guarantee an accumulated error of ±1.5 TU or better on the TSF Offset subfield.

NOTE—The error budget (±1.5 TU) can be broken down as follows:

Delays by the measuring STA in transmitting the first bit of the Beacon Report after receiving the last bit of a neighbor AP’s Beacon or Probe Response (±0.5 TU).

Error caused by rounding to the nearest TU boundary when converting Neighbor TSF Offset from microseconds to TUs (±0.5 TU).

Delays by the serving AP between reception of the last bit of the Beacon Report and transmission of the first bit of the Neighbor Report (±0.5 TU).

## 11.13 Link Measurement

A STA may use a Link Measurement Request frame to request another STA to respond with a Link Measurement Report frame containing link margin and transmit power information. A STA receiving a Link Measurement Request frame shall respond with a Link Measurement Report frame containing the power used to transmit the response in the Transmit Power field and the estimated link margin in the Link Margin field.

An AP in a BSS or a STA in an IBSS shall include a TPC Report element with the Link Margin field set equal to zero and containing transmit power information in the Transmit Power field in any Beacon or Probe Response it transmits.

## 11.14 Measurement Pilot frame generation and usage

### 11.14.1 Measurement Pilot frame generation by an AP

If dot11MeasurementPilotEnabled is true, an AP shall maintain a Measurement Pilot frame generation function and transmit Measurement Pilot frames according to the dot11MeasurementPilotPeriod attribute within the AP. This defines a series of TMPTTs exactly dot11MeasurementPilotPeriod apart. A TMPTT arrives when the AP’s local TSF timer (in s) modulo the Measurement Pilot Frame Interval equals zero. At each TMPTT, the AP shall schedule a Measurement Pilot frame as the next frame for transmission unless the TMPTT collides with a TBTT, in which case the AP shall not generate the Measurement Pilot frame. AP shall schedule and transmit Measurement Pilot frames based on the AC specified in the dot11MeasurementPilotTransmitPriority attribute.

In case the medium is determined by the carrier-sense mechanism (see 9.2.1) to be unavailable, the AP shall delay the actual transmission of a Measurement Pilot frame according to the basic medium access rules specified in Clause 9 for a maximum period of one dot11MeasurementPilotPeriod and drop the Measurement Pilot frame at the next TMPTT.

An AP shall transmit Measurement Pilot frames to the broadcast address. An AP shall not retransmit or buffer Measurement Pilot frames for power save reasons. Once started, the AP shall maintain Measurement Pilot frame transmissions for the life of the BSS.

### 11.14.2 Link margin calculation (informative)

STA may calculate link margin with information received in measurement pilot frames, use it to assess the current link condition, and assist in roaming decision. Link margin may be calculated as follows:

Let RCPIMaxPwr representing the RCPI that would have been received by a STA in a downlink frame if the frame were sent with the Max Transmit Power. Further assuming that STA knows “STA Noise Floor” and “STA Max Transmit Power”.

RCPIMaxPwr = RCPI + (Max Transmit Power – Transmit Power Used)

Downlink Link Margin (DLM) = RCPIMaxPwr – STA Noise Floor

Uplink Link Margin (ULM) = RCPIMaxPwr – (Max Transmit Power – STA Max Transmit Power) – Transceiver Noise Floor

# 12 PHY service specification

## 12.3 Detailed PHY service specifications

### 12.3.4 Basic service and options

#### 12.3.4.3 PHY-SAP service primitives parameters

Change row 4 and insert 2 new rows into Table 29 as shown:

Table 29—PHY-SAP service primitive parameters

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| RXVECTOR | PHY-RXSTART.indication PHY-RXEND.indication | A set of parameters |
| RPI-STATE | PHY-CCARESET.request PHY-CCARESET.confirm | RPI-ON, RPI-OFF |
| RPI-REPORT | PHY-CCA.indication PHY-CCARESET.confirm | A set of RPI values for the preceding time interval |

### 12.3.5 PHY-SAP detailed service specification

#### 12.3.5.8 PHY-CCARESET.request

Change the following clauses as shown:

##### 12.3.5.8.1 Function

This primitive is a request by the MAC sublayer to the local PHY entity to reset the CCA state machine and to turn RPI reporting on and off by means of the RPI-STATE parameter.

##### 12.3.5.8.2 Semantics of the service primitive

The semantics of the primitives are as follows:

PHY-CCARESET.request(RPI-STATE)

This primitive has no parameters.

The RPI-STATE parameter shall be present if dot11RadioMeasurementEnabled is true. The RPI-STATE parameter can be one of two values: RPI-ON or RPI-OFF. The parameter value is RPI-ON when the MAC sublayer is requesting the PHY entity to report RPI values when the PHY is neither receiving nor trans mitting an MPDU. RPI-ON turns on RPI reporting in the PHY entity. RPI-OFF turns off RPI reporting in the PHY entity.

##### 12.3.5.8.3 When generated

This primitive is generated by the MAC sublayer for the local PHY entity at the end of a NAV timer. This request can be used by some PHY implementations that may synchronize antenna diversity with slot timings.

##### 12.3.5.8.4 Effect of receipt

The effect of receipt of this primitive by the PHY entity is to reset the PLCP CS/CCA timers to the state appropriate for the end of a received frame. If RPI-STATE parameter is set to RPI-ON, the PHY entity collects RPI values when it is not transmitting or receiving and provides those values to the MAC sublayer using the RPI-REPORT parameter.

#### 12.3.5.9 PHY-CCARESET.confirm

Change the following clauses as shown:

##### 12.3.5.9.1 Function

This primitive is issued by the PHY to the local MAC entity to confirm that the PHY has reset the CCA state Machine and to provide observed RPI values when RPI reporting is turned on.

##### 12.3.5.9.2 Semantics of the service primitive

The semantics of the primitives are as follows:

PHY-CCARESET.requestconfirm(RPI-STATE, RPI-REPORT)

This primitive has no parameters.

The RPI-STATE parameter shall be present if dot11RadioMeasurementEnabled is true. The RPI-STATE parameter can be one of two values: RPI-ON or RPI-OFF. The RPI-STATE value shall be set to the value of RPI-STATE received by the PHY entity in the most recent PHY-CCARESET.request.

The RPI-REPORT parameter shall be present if dot11RadioMeasurementEnabled is true and if RPI reporting was turned on prior to the receipt of the latest PHY-CCARESET.request. The RPI-REPORT parameter provides a set of RPI values for a time interval. The set of RPI values are recent values observed by the PHY entity since the generation of the most recent PHY-TXEND.confirm, PHY-RXEND.indication, PHY-CCARESET.confirm, or PHY_CCA.indication, whichever occurred latest.

#### 12.3.5.10 PHY-CCA.indication

Change the following clauses as shown:

##### 12.3.5.10.1 Function

This primitive is an indication by the PHY to the local MAC entity of the current state of the medium and to provide observed RPI values when RPI reporting is turned on.

##### 12.3.5.10.2 Semantics of the service primitive

The primitive provides the following parameter:

PHY-CCA.indication (STATE, RPI-REPORT)

The STATE parameter can be one of two values: BUSY or IDLE. The parameter value is BUSY if the channel assessment by the PHY determines that the channel is not available. Otherwise, the value of the parameter is IDLE.

The RPI-REPORT parameter shall be present if dot11RadioMeasurementEnabled is true and if RPI reporting has been turned on by the RPI-STATE parameter. The RPI-REPORT parameter provides a set of RPI values for a time interval. The set of RPI values may be used by the MAC sublayer for Radio Measurement purposes. The set of RPI values are recent values observed by the PHY entity since the generation of the most recent PHY-TXEND.confirm, PHY-RXEND.indication, PHY-CCARESET.confirm, or PHY_CCA.indication, whichever occurred latest.

#### 12.3.5.11 PHY-RXSTART.indication

##### 12.3.5.11.2 Semantics of the service primitive

Change the third paragraph as shown:

The RXVECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt of a valid PLCP Header or upon receipt of the last PSDU data bit in the received frame. This vector may contain both MAC and MAC management parameters. The required parameters are listed in 12.3.4.4.

#### 12.3.5.12 PHY-RXEND.indication

##### 12.3.5.12.2 Semantics of the service primitive

Change the first two paragraphs as shown:

The primitive provides the following parameters:

PHY-RXEND.indication (RXERROR, RXVECTOR)

Insert the following text after the third paragraph:

The RXVECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt of a valid PLCP Header or upon receipt of the last PSDU data bit in the received frame. RXVECTOR is an included parameter only when dot11RadioMeasurementEnabled is true. This vector may contain both MAC and MAC management parameters. The required parameters are listed in 12.3.4.4.

# 15. DSSS PHY specification for the 2.4 GHz band designated for ISM applications

## 15.2 DSSS PLCP sublayer

### 15.2.7 Receive PLCP

Change the second and fifth paragraph of 15.2.7 as shown:

The receive PLCP is shown in Figure 97.

In order to receive data, PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive state. Further, through station management via the PLME, the PHY is set to the appropriate channel and the CCA method is chosen. Other receive parameters such as RSSI, RCPI, signal quality (SQ), and indicated DATARATE may be accessed via the PHY-SAP.

Upon receiving the transmitted energy, according to the selected CCA mode, the PMD_ED shall be enabled (according to 15.4.8.4) as the RSSI reaches the ED_THRESHOLD and/or PMD_CS shall be enabled after code lock is established. These conditions are used to indicate activity to the MAC via PHY-CCA.indicate according to 15.4.8.4. PHY-CCA.indicate(BUSY) shall be issued for energy detection (ED) and/or code lock prior to correct reception of the PLCP frame. The PMD primitives PMD_SQ and PMD_RSSI are issued to update the RSSI and SQ parameters reported to the MAC.

After PHY-CCA.indicate is issued, the PHY entity shall begin searching for the SFD field. Once the SFD field is detected, CRC-16 processing shall be initiated and the PLCP IEEE 802.11® SIGNAL, IEEE 802.11® SERVICE and LENGTH fields are received. The CRC-16 FCS shall be processed. If the CRC-16 FCS check fails, the PHY receiver shall return to the RX IDLE state as depicted in Figure 98. Should the status of CCA return to the IDLE state during reception prior to completion of the full PLCP processing, the PHY receiver shall return to the RX IDLE state.

If the PLCP Header reception is successful (and the SIGNAL field is completely recognizable and supported), a PHY-RXSTART.indicate(RXVECTOR) shall be issued. The RXVECTOR associated with this primitive includes the SIGNAL field, the SERVICE field, the MPDU length in octets (calculated from the LENGTH field in microseconds), the antenna used for receive (RX_ANTENNA), RSSI, RCPI, and SQ.

Change Figure 97 as follows:

Figure 97 — Receive PLCP

## 15.4 DSSS PMD sublayer

### 15.4.4 Basic service and options

#### 15.4.4.2 PMD_SAP peer-to-peer service primitive parameters

Insert a new row into Table 66 as follows:

Table 66—DSSS PMD_SAP peer-to-peer service primitives

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| RCPI | RXVECTOR | 0-255- RCPI is present only when dot11RadioMeasurementEnabled is true. |

#### 15.4.4.3 PMD_SAP sublayer-to-sublayer service primitives

Insert new row into Table 67 as follows:

Table 67—PMD_SAP sublayer-to-sublayer service primitives

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_RCPI |  | X | -- | -- |

#### 15.4.4.4 PMD_SAP service primitive parameters

Insert new row into Table 68 as follows:

Table 68—List of parameters for the PMD primitives

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| RCPI | PMD-RCPI.indicate | 0-255 |

### 15.4.5 PMD_SAP detailed service specification

Insert the following clause (15.4.5.16) after 15.4.5.15:

#### 15.4.5.16 PMD_RCPI.indicate

##### 15.4.5.16.1 Function

This optional primitive, generated by the PMD sublayer, provides the received channel power indicator to the PLCP and MAC.

##### 15.4.5.16.2 Semantics of the service primitive

The primitive shall provide the following parameter:

PMD_RCPI.indicate(RCPI).

The RCPI shall be a measure of the channel power received by the DSSS PHY. RCPI indications are supported as defined in 15.4.8.5.

##### 15.4.5.16.3 When generated

This primitive shall be generated by the PMD when the DSSS PHY is in the receive state. It shall be continuously available to the PLCP, which, in turn, provides the parameter to the MAC entity.

##### 15.4.5.16.4 Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The RCPI may be used in conjunction with RSNI to measure input signal quality.

### 15.4.8 PMD receiver specifications

Insert the following clause after 15.4.8.4:

#### 15.4.8.5 Received Channel Power Indicator (RCPI) Measurement

The RCPI indicator is a measure of the received RF power in the selected channel for a received frame. This parameter shall be a measure by the PHY sublayer of the received RF power in the channel measured over the entire received frame or by other equivalent means which meet the specified accuracy. RCPI shall be a monotonically increasing, logarithmic function of the received power level defined in dBm. The allowed values for the Received Channel Power Indicator (RCPI) parameter shall be an 8 bit value in the range from 0 through 220, with indicated values rounded to the nearest 0.5 dB as follows:

0:     Power <  -110 dBm

1:     Power =  -109.5 dBm

2:     Power =  -109.0 dBm

and so on where

RCPI = int{(Power in dBm +110)*2} for 0dbm > Power > -110dBm

220: Power >  -0 dBm

221-254:      reserved

255: Measurement not available

Accuracy for each measurement shall be +/- 5dB (95% confidence interval) within the specified dynamic range of the receiver. The measurement shall assume a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1.

# 17. Orthogonal frequency division multiplexing (OFDM) PHY specification for the 5 GHz band

## 17.2 OFDM PHY specific service parameter list

### 17.2.3 RXVECTOR parameters

Insert new row into Table 83 as follows:

Table 83—RXVECTOR parameters

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| RCPI | PHY-RXSTART.indicate (RXVECTOR) PHY-RXEND.indicate (RXVECTOR) | 0 to 255. RCPI is present only when dot11RadioMeasurementEnabled is true. |

Insert the following new clause after Clause 17.2.3.4:

#### 17.2.3.5 RXVECTOR RCPI

The allowed values for the RCPI parameter are in the range from 0 through 255, as defined in 17.3.10.6. This parameter is a measure by the PHY of the received channel power. RCPI indications of 8 bits are supported. RCPI shall be measured over the entire received frame.

## 17.3 OFDM PLCP sublayer

### 17.3.10 PMD receiver specifications

Insert the following new clause after Clause 17.3.10.5:

#### 17.3.10.6 Received Channel Power Indicator (RCPI) Measurement

The RCPI indicator is a measure of the received RF power in the selected channel for a received frame. This parameter shall be a measure by the PHY sublayer of the received RF power in the channel measured over the entire received frame or by other equivalent means which meet the specified accuracy. RCPI shall be a monotonically increasing, logarithmic function of the received power level defined in dBm. The allowed values for the Received Channel Power Indicator (RCPI) parameter shall be an 8 bit value in the range from 0 through 220, with indicated values rounded to the nearest 0.5 dB as follows:

0:     Power <  -110 dBm

1:     Power =  -109.5 dBm

2:     Power =  -109.0 dBm

and so on where

RCPI = int{(Power in dBm +110)*2} for  0dbm > Power > -110dBm

220: Power >  -0 dBm

221-254:      reserved

255: Measurement not available

Accuracy for each measurement shall be +/- 5dB (95% confidence interval) within the specified dynamic range of the receiver. The measurement shall assume a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1.

17.3.12 Receive PLCP

Change the first paragraph of 17.3.12 as follows:

The receive PLCP is shown in Figure 128. In order to receive data, PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive state. Further, through station management (via the PLME) the PHY is set to the appropriate frequency. Other receive parameters, such as RSSI, RCPI, and indicated DATARATE, may be accessed via the PHY-SAP.

Change Figure 128 to insert new parameter as follows:

Figure 128—Receive PLCP

## 17.5 OFDM PMD sublayer

### 17.5.4 Basic service and options

#### 17.5.4.2 PMD_SAP sublayer-to-sublayer service primitives

Insert new row into Table 101 as follows:

Table 101—PMD_SAP sublayer-to-sublayer service primitives

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_RCPI | -- | X | -- | -- |

#### 17.5.4.3 PMD_SAP service primitive parameters

Insert new row into Table 102 as follows:

Table 102—List of parameters for the PMD primitives

| Parameter | Associated primitive | Value |
| --- | --- | --- |
| RCPI | PMD_RCPI.indicate | 0-255 |

### 17.5.5 PMD_SAP detailed service specification

Insert the following clause (17.5.5.8) after 17.5.5.7:

#### 17.5.5.8 PMD_RCPI.indicate

##### 17.5.5.8.1 Function

This primitive, generated by the PMD sublayer, provides the received channel power indicator to the PLCP and MAC entity.

##### 17.5.5.8.2 Semantics of the service primitive

The primitive shall provide the following parameter:

PMD_RCPI.indicate(RCPI).

The RCPI shall be a measure of the channel power received by the OFDM PHY. RCPI indications of 8 bits are supported, as defined in 17.3.10.6.

##### 17.5.5.8.3 When generated

This primitive shall be generated by the PMD when the OFDM PHY is in the receive state. It shall be continuously available to the PLCP, which, in turn, provides the parameter to the MAC entity.

##### 17.5.5.8.4 Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The RCPI may be used in conjunction with RSSI to measure input signal quality.

# 18. High Rate direct sequence spread spectrum (HR/DSSS) PHY specification

## 18.2 High Rate PLCP sublayer

### 18.2.6 Receive PLCP

Change the third paragraph of 18.2.6 as shown:

The receive PLCP is shown in Figure 139 In order to receive data, the PHY-TXSTART.request shall be disabled so that the PHY entity is in the receive state. Further, through station management via the PLME, the PHY shall be set to the appropriate channel and the CCA method chosen. Other receive parameters, such as RSSI, RCPI, SQ, and indicated DATARATE, may be accessed via the PHY-SAP.

Change the fourth paragraph of 18.2.6 as shown:

Upon receiving the transmitted energy, according to the selected CCA mode, the PMD_ED shall be enabled (according to 18.4.8.4) as the RSSI reaches the ED_THRESHOLD, and/or PMD_CS shall be enabled after code lock is established. These conditions are used to indicate activity to the MAC via PHY-CCA.indicate, according to 18.4.8.4. PHY-CCA.indicate(BUSY) shall be issued for ED and/or code lock prior to correct reception of the PLCP header. The PMD primitives, PMD_SQ, and PMD_RSSI, and PMD_RCPI, are issued to update the SQ, RSSI and SQRCPI parameters reported to the MAC.

Change the list following the sixth paragraph of 18.2.6 as shown:

- The SIGNAL field;

- The SERVICE field;

- The PSDU length in octets (calculated from the LENGTH field in microseconds and the DATARATE in Mbit/s, in accordance with the formula in 18.2.3.5);

- RXPREAMBLE_TYPE (which is an enumerated type taking on values SHORTPREAMBLE or LONGPREAMBLE);

- The antenna used for receive (RX_ANTENNA)ANT_STATE (the antenna used for receive), RSSI, RCPI, and SQ.

Change Figure 139 to insert new parameter as follows:

Figure 139—Receive PLCP

## 18.3 High Rate PLME

### 18.3.5 Vector descriptions

Insert new rows at the end of Table 108 as follows:

Table 108—Parameter vectors

| Parameter | Associated vector | Value |
| --- | --- | --- |
| ANT_STATE | RXVECTOR | 1 to 256 |
| RSSI | RXVECTOR | 0-8 bits of RSSI |
| RCPI | RXVECTOR | 0-255. RCPI is present only when dot11RadioMeasurementEnabled is true. |
| SQ | RXVECTOR | 0-8 bits of SQ |

## 18.4 High Rate PMD sublayer

### 18.4.4 Basic service and options

#### 18.4.4.2 PMD_SAP sublayer-to-sublayer service primitives

Insert new row into Table 110 as follows:

Table 110—PMD_SAP sublayer-to-sublayer service primitives

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_RCPI | -- | X | -- | -- |

### 18.4.5 PMD_SAP detailed service specification

Insert the following clause (18.4.5.16) at the end of 18.4.5:

#### 18.4.5.16 PMD_RCPI.indicate

##### 18.4.5.16.1 Function

This optional primitive, generated by the PMD sublayer, provides the received channel power indicator to the PLCP and MAC.

##### 18.4.5.16.2 Semantics of the service primitive

The primitive shall provide the following parameter:

| Parameter | Associated primitive | Value | Description |
| --- | --- | --- | --- |
| RCPI | PMD_RCPI.indicate | 0-255 | The RCPI is a measure of the received power by the High Rate PHY as defined in 18.4.8.5. |

##### 18.4.5.16.3 When generated

This primitive shall be generated by the PMD when the High Rate PHY is in the receive state when dot11RadioMeasurementEnabled is true. It is continuously available to the PLCP, which, in turn, provides the parameter to the MAC entity.

##### 18.4.5.16.4 Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The RCPI may be used in conjunction with RSNI to measure input signal quality.

### 18.4.8 PMD receiver specifications

Insert the following clause (18.4.8.5) at the end of 18.4.8:

#### 18.4.8.5 Received Channel Power Indicator (RCPI) Measurement

The RCPI indicator is a measure of the received RF power in the selected channel for a received frame. This parameter shall be a measure by the PHY sublayer of the received RF power in the channel measured over the entire received frame or by other equivalent means which meet the specified accuracy. RCPI shall be a monotonically increasing, logarithmic function of the received power level defined in dBm. The allowed values for the Received Channel Power Indicator (RCPI) parameter shall be an 8 bit value in the range from 0 through 220, with indicated values rounded to the nearest 0.5 dB as follows:

0:     Power <  -110 dBm

1:     Power =  -109.5 dBm

2:     Power =  -109.0 dBm

and so on where

RCPI   =  int{(Power in dBm +110)*2} for  0dbm > Power > -110dBm

220: Power >  -0 dBm

221-254:      reserved

255: Measurement not available

Accuracy for each measurement shall be +/- 5dB (95% confidence interval) within the specified dynamic range of the receiver. The measurement shall assume a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1.

# 19. Extended Rate PHY specification

## 19.2 PHY specific service parameter list

Insert new row into Table 123B as follows:

Table 123B—RXVECTOR parameters

| Parameter | Value |
| --- | --- |
| RCPI | The RCPI is a measure of the received channel power and is included when dot11RadioMeasurementEnabled is true. The 8-bit RCPI value is described in 17.2.3.5 and 18.4.5.16. |

## 19.9 Extended Rate PMD sublayer

### 19.9.4 Basic service and options

#### 19.9.4.2 PMD_SAP sublayer-to-sublayer service primitives

Insert new row into Table 123I as follows:

Table 123I—PMD_SAP sublayer-to-sublayer service primitives

| Primitive | Request | Indicate | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_RCPI | -- | X | -- | -- |

#### 19.9.4.3 PMD_SAP service primitive parameters

Insert new row into Table 123J as follows:

Table 123J—List of parameters for the PMD primitives

| Parameter | Associated primitive | Value | Description |
| --- | --- | --- | --- |
| RCPI | PMD_RCPI.indicate | 0-255 | The RCPI is a measure of the received channel power. See 19.9.5.14. |

### 19.9.5 PMD_SAP detailed service specification

Insert the following clause (19.9.5.14) after 19.9.5.13:

#### 19.9.5.14 PMD_RCPI.indicate

This primitive is the same as that defined in 17.5.5.8 and 18.4.5.16, including the parameter RCPI. This primitive is used for radio measurement purposes and to aid in link optimization algorithms such as roaming decisions.

# Annex A

## A.4 PICS proforma–IEEE Std 802.11, 1999 Edition9

### A.4.3 IUT configuration

Append this entry to the end of this table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CFk | Is Radio Resource Measurement supported? | 7.3.1.4 | O | Yes, No |

Insert this new clause after clause A.4.12:

### A.4.13 Radio Resource Measurement extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RRM1 | Are the following Radio Resource Measurement capabilities supported? Radio Measurement Capability | 7.3.1.4 | CFk:M | Yes, No, N/A |
| RRM2 RRM2.1 RRM2.2 RRM2.3 RRM2.4 RRM2.5 RRM2.5.1 RRM2.5.2 RRM2.6 RRM2.6.1 RRM2.6.2 | Action frame protocol for measurements Radio Measurement Request frame Radio Measurement Report frame Link Measurement Request frame Link Measurement Report frame Neighbor Report Request Generate and transmit Neighbor Report Request Receive and process Neighbor Report Request Neighbor Report Response Generate and transmit Neighbor Report Response Receive and process Neighbor Report Response | 7.4 7.4.5.1 7.4.5.2 7.4.5.3 7.4.5.4 7.4.5.5 7.4.5.5 7.4.5.6, 7.3.2.27 7.4.5.6, 7.3.2.27 | CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM3 RRM3.1 RRM3.2 RRM3.3 RRM3.4 RRM3.5 RRM3.6 RRM3.7 RRM3.8 | General protocol for requesting and reporting of measurements Parallel Measurements Enable Measurement Requests Enable Autonomous Report Duration Mandatory Incapable Indication Refused Indication Repeated Measurement Measurement Pause | 7.3.2.21, 7.3.2.22, 11.11, 11.11.7 7.3.2.21, 7.3.2.22 7.3.2.21, 11.11.7 7.3.2.21, 11.11.7 7.3.2.21, 11.11.4 7.3.2.22 7.3.2.22 7.4.5.1, 11.11.7 7.3.2.21.12 | CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM4 RRM4.1 RRM4.2 RRM4.3 RRM4.4 RRM4.5 RRM4.6 RRM4.7 RRM4.8 | Beacon Measurement Type Beacon Request Passive Measurement mode Passive Pilot Measurement mode Active Measurement mode STA Selected mode Beacon table mode Reporting Conditions Beacon Report | 11.11, 11.11.9.1 7.3.2.21.6 7.3.2.21.6 11.11.9.1 7.3.2.21.6 11.11.9.1 7.3.2.21.6 11.11.9.1 7.3.2.21.6 11.11.9.1 7.3.2.21.6 11.11.9.1 7.3.2.21.6 7.3.2.22.6 | CFk:M CFk:M CFk:M CFk:O CFk:M CFk:O CFk:M CFk:O CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| * RRM5 RRM5.1 RRM5.2 | Frame Measurement Type Frame Request Frame Report | 11.11, 11.11.9.2 7.3.2.21.7 7.3.2.22.7 | CFk:O (CFk AND RRM5):M (CFk AND RRM5):M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM6 RRM6.1 RRM6.2 | Channel Load Measurement Type Channel Load Request Channel Load Report | 11.11, 11.11.9.3 7.3.2.21.4 7.3.2.22.4 | CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| * RRM7 RRM7.1 RRM7.2 | Noise Histogram Measurement Type Noise Histogram Request Noise Histogram Report | 11.11, 11.11.9.4 7.3.2.21.5 7.3.2.22.5 | CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM8 RRM8.1 RRM8.2 | STA Statistics Measurement Type STA Statistics Request STA Statistics Report | 11.11, 11.11.9.7 7.3.2.21.10 7.3.2.22.10 | CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM9 RRM9.1 RRM9.2 | LCI Measurement Type LCI Request LCI Report | 11.11, 11.11.9.8 7.3.2.21.11 7.3.2.22.11 | CFk:M CFk:M CFk:M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM10 | QoS Metrics Type | 11.11, 11.11.9.10 | CFk:O | Yes, No, N/A |
| RRM10.1 | QoS Metrics Type Request | 7.3.2.21.13 | CFk AND CF12:M | Yes, No, N/A |
| RRM10.2 | QoS Metrics Type Report | 7.3.2.22.13 | CFk AND CF12:M | Yes, No, N/A |
| RRM10.3 | Triggered QoS Metrics Report | 7.3.2.21.13 | CFk AND CF12:O | Yes, No, N/A |
| RRM11 | AP Channel Report | 7.3.2.9, 7.3.2.26 | (CFk AND CF1):M | Yes, No, N/A |
| RRM12 | Neighbor Report Procedure | 11.12.2, 11.12.3 | CFk:M | Yes, No, N/A |
| RRM13 | RCPI Measurement RCPI Measurement for DSSS PHY at 2.4GHz RCPI Measurement for OFDM PHY at 5GHz RCPI Measurement for HR DSSS PHY at 2.4GHz RCPI Measurement for Extended Rate PHY at 2.4Ghz | 15.4.8.5 17.2.3.5, 17.3.10.6, 17.5.4.3 18.4.8.5 19.9.5.14 | (CFk AND CF4): M (CFk AND CF6): M (CFk AND CF7): M (CFk AND CF9):M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM14 RRM14.1 RRM14.2 | RCPI Measurement during Active Scanning Respond with RCPI information element when requested Measurement of RCPI on Probe Request frames | 11.1.3.2.2 11.1.3.2.2 | (CFk AND CF12 AND CF1):M (CFk AND CF12 AND CF1):O | Yes, No, N/A Yes, No, N/A |
| RRM15 | RSNI Measurement | 7.3.2.31 | CFk:M | Yes, No, N/A |
| RRM16 | TPC Information in Beacon and Probe Response frames |  |  |  |
| RRM16.1 | Country and Transmit Power Control (TPC) Report elements included in Beacon and Probe Reponse frames | 7.2.3.1, 7.2.3.9, 7.3.2.9, 7.3.2.18, 11.13 | CFk:M | Yes, No, N/A |
| RRM16.2 | Power Constraint element included in Beacon and Probe Response frames | 7.2.3.1, 7.2.3.9, 7.3.2.15 | CFk:O | Yes, No, N/A |
| RRM17 | Power Capability elements in Association and Reassociation frames | 7.2.3.4, 7.2.3.5, 11.10.1 | CFk:M | Yes, No, N/A |
| RRM18 RRM18.1 RRM18.2 RRM18.3 | Management Information Base dot11SMTRRMRequest dot11SMTRRMReport dot11SMTRRMConfig | Annex D Annex D Annex D | (CFk AND CF1):M (CFk AND CF1):M (CFk AND CF1):M | Yes, No, N/A Yes, No, N/A Yes, No, N/A |
| RRM19 | Measurement Pilot Frame | 7.1.3.1.2, 7.2.3.10, 7.3.1.18, 7.3.1.19, 7.3.1.20, 7.3.1.21, 7.3.1.22, 7.3.1.23, 10.3.17, 11.9, 11.14 | CFk:O | Yes, No, N/A |
| RRM20 | BSS Load elements included in Beacon and Probe Reponse frames | 7.2.3.1, 7.2.3.9, 7.3.2.28 | CFk:M | Yes, No, N/A |
| RRM21 | Antenna Information elements included in Beacon and Probe Response frames | 7.2.3.1, 7.2.3.9, 7.3.2.30 | CFk:M | Yes, No, N/A |

# Annex D

-- ********************************************************************

-- * dot11BeaconReport TABLE

-- ********************************************************************

dot11BeaconReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Beacon reports that have been received by the MLME. The report tables shall be maintained as FIFO to preserve

freshness, thus the rows in this table can be deleted for memory constraints

or other implementation constraints determined by the vendor. New rows shall

have different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase RprtIndex

for new reports being written in the table."

::= { dot11RRMReport 3 }

dot11BeaconReportEntry OBJECT-TYPE

SYNTAX Dot11BeaconReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11BeaconReportTable Indexed by dot11BeaconRprtIndex."

INDEX { dot11BeaconRprtIndex }

::= { dot11BeaconReportTable 1 }

Dot11BeaconReportEntry ::=

SEQUENCE {

dot11BeaconRprtIndex Unsigned32,

dot11BeaconRprtRqstToken OCTET STRING,

dot11BeaconRprtIfIndex InterfaceIndex,

dot11BeaconMeasuringSTAAddr MacAddress,

dot11BeaconRprtChanNumber INTEGER,

dot11BeaconRprtRegulatoryClass INTEGER,

dot11BeaconRprtActualStartTime TSFType,

dot11BeaconRprtMeasurementDuration Unsigned32,

dot11BeaconRprtPhyType INTEGER,

dot11BeaconRprtReportedFrameType INTEGER,

dot11BeaconRprtRCPI INTEGER,

dot11BeaconRprtRSNI INTEGER,

dot11BeaconRprtBSSID MacAddress,

dot11BeaconRprtAntennaID INTEGER,

dot11BeaconRprtParentTSF TSFType,

dot11BeaconRprtReportedFrameBody OCTET STRING,

dot11BeaconRptMeasurementMode INTEGER }

dot11BeaconRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Beacon Report elements in dot11BeaconReportTable, greater than

0."

::= { dot11BeaconReportEntry 1 }

dot11BeaconRprtRqstToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the request token that was specified in the

measurement request that generated this measurement report. This should be

an exact match to the original dot11RRMRqstToken attribute. Note that there

may be multiple entries in the table that match this value since a single

request may generate multiple measurement reports."

::= { dot11BeaconReportEntry 2 }

dot11BeaconRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Beacon Report has been received on."

::= { dot11BeaconReportEntry 3 }

dot11BeaconMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the measuring STA for this row of Beacon report."

::= { dot11BeaconReportEntry 4 }

dot11BeaconRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which this row of report detected beacons

and probe responses."

::= { dot11BeaconReportEntry 5 }

dot11BeaconRprtRegulatoryClass OBJECT-TYPE

SYNTAX PHYType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row

of beacon report has been conducted."

REFERENCE

"IEEE 802.11, Error: Reference source not found"

::= { dot11BeaconReportEntry 6 }

dot11BeaconRprtActualStartTime OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when the

measurement started."

::= { dot11BeaconReportEntry 7 }

dot11BeaconRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNITS "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Beacon Report was

measured."

::= { dot11BeaconReportEntry 8 }

dot11BeaconRprtPhyType OBJECT-TYPE

SYNTAX INTEGER {

fhss(1),

dsss(2),

irbaseband(3),

ofdm(4),

hrdsss(5),

erp(6)

}

UNITS "dot11PHYType"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the PHY Type for this row of

Beacon Report."

::= { dot11BeaconReportEntry 9 }

dot11BeaconRprtReportedFrameType OBJECT-TYPE

SYNTAX INTEGER {

beaconOrProbeResponse(0),

measurementPilot(1)

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the frame type reported in
dot11BeaconRprtReportedFrameBody"

::= { dot11BeaconReportEntry 10 }

dot11BeaconRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the beacon or

probe response frame in dBm, as defined in the RCPI measurement clause

for the indicated PHY Type."

::= { dot11BeaconReportEntry 11 }

dot11BeaconRprtRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received signal to noise ratio of the beacon or probe response frame in dB. RSNI is the received signal to noise plus interference ratio derived from the measured RCPI for the received frame and from the measured ANPI for the channel used to receive the frame. RSNI is calculated by the ratio of  the received signal power (RCPI - ANPI) over the noise plus interference power (ANPI), expressed in dB (1/2 dB steps), where RSNI = [(ratio(dB) + 10) * 2], for ratios in the range -10dB to +118dB.

"

::= { dot11BeaconReportEntry 12 }

dot11BeaconRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the beacon for this row of

Beacon Report."

::= { dot11BeaconReportEntry 13 }

dot11BeaconRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Antenna ID contains the identifying number for the antenna used

for this measurement. The value 0 indicates that the antenna identifier is unknown.  The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction."

::= { dot11BeaconReportEntry 14 }

dot11BeaconRprtParentTSF OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value of the serving measuring STA’s

TSF value at the time the measuring STA received the beacon or probe

response frame."

::= { dot11BeaconReportEntry 15 }

dot11BeaconRprtReportedFrameBody OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..100))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the fixed fields and information elements from the

frame body of the Beacon, Measurement Pilot or Probe Response frame being received.

All reported TIM elements are truncated to 4 octets."

::= { dot11BeaconReportEntry 16 }

dot11BeaconRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 3 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11BeaconReportEntry 17 }

-- ********************************************************************

-- * End of dot11BeaconReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11FrameReport TABLE

-- ********************************************************************

dot11FrameReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of Frame reports that have been received by

the MLME. The report tables shall be maintained as FIFO to preserve

freshness, thus the rows in this table can be deleted for memory constraints

or other implementation constraints determined by the vendor. New rows shall

have different RprtIndex values than those deleted within the range

limitation of the index. One easy way is to monotonically increase RprtIndex

for new reports being written in the table."

::= { dot11RRMReport 4 }

dot11FrameReportEntry OBJECT-TYPE

SYNTAX Dot11FrameReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11FrameReportTable Indexed by dot11FrameRprtIndex."

INDEX { dot11FrameRprtIndex }

::= { dot11FrameReportTable 1 }

Dot11FrameReportEntry ::=

SEQUENCE {

dot11FrameRprtIndex Unsigned32,

dot11FrameRprtIfIndex InterfaceIndex,

dot11FrameRprtRqstToken Unsigned32,

dot11FrameRprtChanNumber INTEGER,

dot11FrameRprtRegulatoryClass INTEGER,

dot11FrameRprtActualMsmtStart TSFType,

dot11FrameRprtMeasurementDuration Unsigned32,

dot11FrameRprtMeasuringSTAAddr MacAddress,

dot11FrameRprtBSSID MacAddress,

dot11FrameRprtRCPI INTEGER,

dot11FrameRprtRSNI INTEGER,

dot11FrameRprtAntennaID INTEGER,

dot11FrameRprtNumberFrames INTEGER,

dot11FrameRptMeasurementMode INTEGER }

dot11FrameRprtIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for Frame Report elements in dot11FrameReportTable, greater than 0."

::= { dot11FrameReportEntry 1 }

dot11FrameRprtIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The ifIndex for this row of Frame Report has been received on."

::= { dot11FrameReportEntry 2 }

dot11FrameRprtRqstToken OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Index for Frame Request elements in dot11FrameRequestTable that corresponds

to this row of frame report. Since a single frame request can generate

multiple rows in the frame report table, one per BSSID, this

dot11FrameRprtRqstToken indicates which request this particular row

corresponds to. If this row of report is received without a particular

request, this attribute should be 0"

::= { dot11FrameReportEntry 3 }

dot11FrameRprtChanNumber OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The channel number on which this row of report detected frame request

responses."

::= { dot11FrameReportEntry 4 }

dot11FrameRprtRegulatoryClass OBJECT-TYPE

SYNTAX PHYType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Regulatory Class indicates the frequency band in which this row of

frame report has been conducted."

REFERENCE

"IEEE 802.11, Error: Reference source not found"

::= { dot11FrameReportEntry 5 }

dot11FrameRprtActualMsmtStart OBJECT-TYPE

SYNTAX TSFType

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the TSF value at the time when measurement

started."

::= { dot11FrameReportEntry 6 }

dot11FrameRprtMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the Frame Report

was measured, expressed in TUs."

::= { dot11FrameReportEntry 7 }

dot11FrameRprtMeasuringSTAAddr OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of STA for this row of Frame report that it has been

received from."

::= { dot11FrameReportEntry 8 }

dot11FrameRprtBSSID OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the BSSID of the STA that transmitted this

frame."

::= { dot11FrameReportEntry 9 }

dot11FrameRprtRCPI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute indicates the received channel power of the beacon or probe response frame in dBm, as defined in the RCPI measurement clause for the indicated PHY Type."

::= { dot11FrameReportEntry 10 }

dot11FrameRprtRSNI OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

This attribute indicates the received signal to noise ratio of the receivedframe in dBm. RSNI is the received signal to noise plus interference ratio derived from the RCPI for the received frame and from the most recent ANPI value measured on the channel used to receive the frame. RSNI may be calculated by the ratio of  the received signal power (RCPI - ANPI) over the noise plus interference power (ANPI), expressed in dB (1/2 dB steps), where RSNI = [(ratio(dB) + 10) * 2],  for  ratios in the range -10dB to +118dB.  Other measurement techniques are allowed.

"

::= { dot11FrameReportEntry 11 }

dot11FrameRprtAntennaID OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Antenna ID contains the identifying number for the antenna used for this measurement. The value 0 indicates that the antenna identifier is unknown.  The value 255 indicates that this measurement was made with multiple antennas. The value 1 is used for a STA with only one antenna.  STAs with more than one antenna shall assign Antenna IDs to each antenna as consecutive, ascending numbers. Each  Antenna ID number represents a unique antenna characterized by a fixed relative position, a fixed relative direction and a peak gain for that position and direction."

::= { dot11FrameReportEntry 12 }

dot11FrameRprtNumberFrames  OBJECT-TYPE

SYNTAX INTEGER(0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the number of received frames in the

measurement Report Frame for this row of Frame Report."

::= { dot11FrameReportEntry 13 }

dot11FrameRptMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 3 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11FrameReportEntry 14 }

-- ********************************************************************

-- * End of dot11FrameReport TABLE

-- ********************************************************************

-- ********************************************************************

-- * dot11STAStatisticsReport TABLE

-- ********************************************************************

dot11STAStatisticsReportTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Group contains the current list of STA Statistics reports that have been

received by the MLME. The report tables shall be maintained as FIFO to

preserve freshness, thus the rows in this table can be deleted for memory

constraints or other implementation constraints determined by the vendor.

New rows shall have different RprtIndex values than those deleted within the

range limitation of the index. One easy way is to monotonically increase

RprtIndex for new reports being written in the table."

::= { dot11RRMReport 7 }

dot11STAStatisticsReportEntry OBJECT-TYPE

SYNTAX Dot11STAStatisticsReportEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11STAStatisticsReportTable

Indexed by dot11STAStatisticsReportIndex."

INDEX { dot11STAStatisticsReportIndex }

::= { dot11STAStatisticsReportTable 1 }

Dot11STAStatisticsReportEntry ::=

SEQUENCE {

dot11STAStatisticsReportIndex Unsigned32,

dot11STAStatisticsReportToken OCTET STRING,

dot11STAStatisticsIfIndex InterfaceIndex,

dot11STAStatisticsSTAAddress MacAddress,

dot11STAStatisticsMeasurementDuration Unsigned32,

dot11STAStatisticsTransmittedFragmentCount Counter32,

dot11STAStatisticsMulticastTransmittedFrameCount Counter32,

dot11STAStatisticsFailedCount Counter32,

dot11STAStatisticsRetryCount Counter32,

dot11STAStatisticsMultipleRetryCount Counter32,

dot11STAStatisticsFrameDuplicateCount Counter32,

dot11STAStatisticsRTSSuccessCount Counter32,

dot11STAStatisticsRTSFailureCount Counter32,

dot11STAStatisticsACKFailureCount Counter32,

dot11STAStatisticsReceivedFragmentCount Counter32,

dot11STAStatisticsMulticastReceivedFrameCount Counter32,

dot11STAStatisticsFCSCount Counter32,

dot11STAStatisticsTransmittedFrameCount Counter32,

dot11STAStatisticsAPServiceLoad INTEGER,

dot11STAStatisticsAverageAccessDelayBestEffort INTEGER,

dot11STAStatisticsAverageAccessDelayBackGround INTEGER,

dot11STAStatisticsAverageAccessDelayVIdeo INTEGER,

dot11STAStatisticsAverageAccessDelayVoice INTEGER,

dot11STAStatisticsStationCount INTEGER,

dot11STAStatisticsChannelUtilization INTEGER,

dot11STAStatisticsRptMeasurementMode INTEGER }

dot11STAStatisticsReportIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"Index for STA Statistics Report elements in dot11STAStatisticsReportTable,

greater than 0."

::= { dot11STAStatisticsReportEntry 1 }

dot11STAStatisticsReportToken OBJECT-TYPE

SYNTAX OCTET STRING

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute holds the token that was specified in the measurement

request that generated this measurement report. This should be an exact

match to the original dot11RRMRqstToken attribute. Note that there may be

multiple entries in the table that match this value since a single request

may generate multiple measurement reports."

::= { dot11STAStatisticsReportEntry 2 }

dot11STAStatisticsIfIndex OBJECT-TYPE

SYNTAX InterfaceIndex

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"Identifies the Interface that this row of STA Statistics Report has been

received on"

::= { dot11STAStatisticsReportEntry 3 }

dot11STAStatisticsSTAAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The MAC address of the STA that returned this STA Statistics report"

::= { dot11STAStatisticsReportEntry 4 }

dot11STAStatisticsMeasurementDuration OBJECT-TYPE

SYNTAX Unsigned32

UNIT "TUs"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute corresponds to the duration over which the STA Statistics

was measured."

::= { dot11STAStatisticsReportEntry 5 }

dot11STAStatisticsTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11TransmittedFragmentCount returned from the STA in this

STA Statistics Report"

::= { dot11STAStatisticsReportEntry 6 }

dot11STAStatisticsMulticastTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MulticastTransmittedFrameCount returned from the STA in

this STA Statistics Report"

::= { dot11STAStatisticsReportEntry 7 }

dot11STAStatisticsFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FailedCount returned from the STA in this STA Statistics

Report"

::= { dot11STAStatisticsReportEntry 8 }

dot11STAStatisticsRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RetryCount returned from the STA in this STA Statistics

Report"

::= { dot11STAStatisticsReportEntry 9 }

dot11STAStatisticsMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MultipleRetryCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 10 }

dot11STAStatisticsFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FrameDuplicateCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 11 }

dot11STAStatisticsRTSSuccessCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RTSSuccessCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 12 }

dot11STAStatisticsRTSFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11RTSFailureCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 13 }

dot11STAStatisticsACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11ACKFailureCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 14 }

dot11STAStatisticsReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11ReceivedFragmentCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 15 }

dot11STAStatisticsMulticastReceivedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11MulticastReceivedFrameCount returned from the STA in this

STA Statistics Report"

::= { dot11STAStatisticsReportEntry 16 }

dot11STAStatisticsFCSCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11FCSErrorCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 17 }

dot11STAStatisticsTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The value of dot11TransmittedFrameCount returned from the STA in this STA

Statistics Report"

::= { dot11STAStatisticsReportEntry 18 }

dot11STAStatisticsAPServiceLoad OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The AP Service Load shall be a scalar indication of the relative level of service loading at an AP."

REFERENCE

"IEEE 802.11 – Clause 7.3.2.29"

::= { dot11STAStatisticsReportEntry 19 }

dot11STAStatisticsAverageAccessDelayBestEffort OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access DelayBestEffort element shall consist of an an average access delay  for the Best Effort Access Category."

REFERENCE

"IEEE 802.11 – Clause 7.3.2.29"

::= { dot11STAStatisticsReportEntry 20 }

dot11STAStatisticsAverageAccessDelayBackGround OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access DelayBackGround element shall consist of an an average access delay  for the Backgound Access Category."

REFERENCE

"IEEE 802.11 – Clause 7.3.2.29"

::= { dot11STAStatisticsReportEntry 21 }

dot11STAStatisticsAverageAccessDelayVIdeo OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access DelayVIdeo element shall consist of an an average access delay  for the Video Access Category."

REFERENCE

"IEEE 802.11 – Clause 7.3.2.29"

::= { dot11STAStatisticsReportEntry 22 }

dot11STAStatisticsAverageAccessDelayVoice OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Average Access DelayVOice element shall consist of an an average access delay  for the Voice Access Category."

REFERENCE

"IEEE 802.11 – Clause 7.3.2.29"

::= { dot11STAStatisticsReportEntry 23 }

dot11STAStatisticsStationCount OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Station Count field is an unsigned integer that indicates the total number of STAs currently associated with this BSS."

::= { dot11STAStatisticsReportEntry 24 }

dot11STAStatisticsChannelUtilization OBJECT-TYPE

SYNTAX INTEGER (0..255)

UNITS "1/255"

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"The Channel Utilization field indicates the fraction of time the AP sensed the medium busy."

REFERENCE

"IEEE 802.11 – Clause 9.2.1"

::= { dot11STAStatisticsReportEntry 25 }

dot11STAStatisticsRprtMeasurementMode OBJECT-TYPE

SYNTAX INTEGER {

lateBit(0),

incapableBit(1),

refusedBit(2),

}

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"MeasurementMode corresponds to the Measurement Report Mode field in

the Measurement Report Element and is used to indicate the reason for

a failed measurement request. 0 indicates the STA is unable to carry

out a measurement request because it received the request after the

requested measurement time. 1 indicates this STA is incapable of

generating the report. 3 indicates his STA is refusing to generate

the report."

DEFVAL { 0 }

::= { dot11STAStatisticsReportEntry 26 }

-- ********************************************************************

-- * End of dot11STAStatisticsReport TABLE

-- ********************************************************************