RECOMMENDATION  ITU-R  M.1450-2*,**

Characteristics of broadband radio local area networks

(Questions ITU-R 212/8 and ITU-R 142/9)

(2000-2002-2003)

The ITU Radiocommunication Assembly,

considering

a)	that broadband radio local area networks (RLANs) are widely used for nomadic computer equipment for a variety of broadband applications;

b)	that broadband RLAN standards are compatible with current wired LAN standards;

c)	that it is desirable to establish guidelines for broadband RLANs in various frequency bands;

d)	that broadband RLANs should be implemented with careful consideration to compatibility with other radio applications;

e)	that the above guidelines should not limit the effectiveness of broadband RLANs but be used to enhance their application,

notings

a)	that for other information on RLANs refer to Recommendation ITU-R F.1244.

recommends

1	that for guidance on existing broadband RLAN standards, Table 2 can be referred to;

2	that details for modulation schemes for broadband RLANs

for

IEEE 802.11 and its amendments are available from GetIEEE site http://standards.ieee.org/getieee802/

[HIPERLAN 2 details]

3	-

NOTE 1 – Acronyms and terminology used in this Recommendation are given in Table 1. [Need to check for current –relevance]

TABLE  1

Acronyms and terms used in this Recommendation

| AP | Access point |
| --- | --- |
| BRAN | Broadband radio networks |
| CSMA/CA | Carrier sensing multiple access with collision avoidance |
| DFS | Dynamic frequency selection |
| EIRP | Effective Isotropic Radiated Power |
| ETSI | European Telecommunications Standards Institute |
| HIPERLAN2 | High Performance LAN 2 |
| HiSWAN |  |
| HSWA |  |
| IEEE | Institute of Electrical and Electronics Engineers |
| LAN | Local Area Network |

TABLE  1 (end)

| MMAC |  |
| --- | --- |
| OFDM | Orthogonal frequency division multiplexing |
| PSD | Power Spectral Density |
| RF | Radio frequency |
| RLAN | Radio Local Area Network |
| TPC | Transmit power control |
| Access method | Scheme used to provide multiple access to a channel |
| Bit rate | The rate of transfer of bit information from one network device to another |
| Channelization | Bandwidth of each channel and number of channels that can be contained in the RF bandwidth allocation |
| Frequency band | Nominal operating spectrum of application |
| Modulation | The method used to put digital information on an RF carrier |
| Tx power | (Transmitter power) – RF power in Watts produced by the transmitter |

RLAN operations:

RLANs are  the extension to wired LANs utilizing radio as the connective media.

RLANs utilize CSMA/CA, a listen before talk, protocol in the transfer of data via radio links.  All stations listen for a clear channel prior to initiation of any transmission. There is a minimum interval when all stations must listen to the channel prior to attempting to transmit data.  And, clients associated to an AP will randomly transfer their data packets as the channel is available.

DFS is an interference mitigation technique that [senses energy] and moves the network to a different channel to avoid incumbents in the spectrum.  ***(Need a less objectionable way of stating this.)***

TPC is  an power aggregation tool?????

TABLE 2

RLAN characteristics for frequency sharing

| Characteristics | IEEE 802.11b | IEEE 802.11a | IEEE 802.11g | ETSI BRAN HIPERLAN 2 | MMAC HSWA HiSWAN a |
| --- | --- | --- | --- | --- | --- |
| General: |  |  |  | Should be reviewed | Should be reviewed |
| Frequency Band | 2 400-2 483.5 MHz | 4900-5000 MHz 5 150-5 250 MHz 5 250-5 350 MHz 5470-5725 MHz 5 725-5 825 MHz Is there any RLAN in 5725- 5850MHz | 2 400-2 483.5 MHz | 5 150-5 350 and 5 470-5 725 MHz(3) | 5 150 to 5 250 MHz(3), (6) |
| Emission type | ? | ? | ? |  |  |
| Channelization | 5 MHz | 20 MHz | 5 MHz | 20 MHz | 20 MHz |
| Antenna: | Various | Various | Various |  |  |
| Gain (approximate) | 0-6 dBi Omni | 0-6 dBi Omni | 0-6 dBi Omni |  |  |
| Radiation Pattern | Omni, Directional | Omni, Directional | Omni, Directional |  |  |
| Occupied Bandwidth | 802.11b mask | OFDM mask | OFDM mask | OFDM mask | OFDM mask |
| Transmitter: |  |  |  |  |  |
| Tx Power [Some administrations regulate Tx Power vs. EIRP] [This may be dropped at final editing] | 1 000 mW, PSD 8 dBm/3 KHz | 5.150-5250MHz Tx 50 mW PSD 4 dBm/MHz 5 250-5 350 MHz 5470-5725 MHz Tx 200 mW [footnote that some administration have higher power] PSD 11 dBm/MHz 5725-5825 MHz Tx 1000 mW PSD 17 dBm/MHz 5725-5850 MHz 1 000 mW PSD 8 dBm/3 KHz | 1 000 mW, PSD 8 dBm/3 KHz | 5 150-5 350 MHz: 200 mW maximum mean e.i.r.p. and use of transmitter power control. 5 470-5 725 MHz: 1 W maximum mean e.i.r.p. and use of transmitter power control | 5 150 to 5 250 MHz 10 mW/MHz e.i.r.p(6) |
| EIRP | 1 000 mW e.i.r.p (US) 100 mW e.i.r.p.( CEPT) 10 mW/MHz e.i.r.p. density(6) | 5 150 to 5 250 MHz 10 mW/MHz 200 mW e.i.r.p. in 20 MHz channel Indoor Only 5 250-5 350 MHz 200 mW e.i.r.p. [some Admin -1000 mW] 5470-5725 MHz 1000 mW w DFS [250 mW 5 725-5 825 MHz 4000 mW e.i.r.p.(7) | 1 000 mW e.i.r.p.(4) 100 mW e.i.r.p.(5) 10 mW/MHz e.i.r.p. density(6) | 5 150-5 350 MHz: 200 mW maximum mean e.i.r.p. and use of transmitter power control. 5 470-5 725 MHz: 1000 mW maximum mean e.i.r.p. and use of transmitter power control | 5 150 to 5 250 MHz 10 mW/MHz e.i.r.p(6) |
| Interference Mitigation | CSMA/CA | DFS/TPC CSMA/CA | CSMA/CA | DFS/TPC CSMA/CA | CSMA/CA |
| Receiver: |  |  |  |  |  |
| Sensitivity | Listed in standard | Listed in standard | Listed in standard |  |  |

Comment: Need to coordinate with ETSI/BRAN on updating HIPERLAN, and MMAC for HiSWAN  -- ***** Need to consider  adding columns for  .11j  and something on .11n future requirements ---  *****

Comment:  ** There are three figures included in this document:

Figure 1 is the original spectrum mask for OFDM

Figure 2 is the .11b mask

Figure 3 is a combined mask set showing .11a/b/g spectrums

Request feedback on which mask/masks that should be included in this document.

Add Note that this  spectrum mask is for systems 802.11a, .11g,  HIPERLAN2, and HiSWAN

Figure 2.  802.11b Transmit Spectrum Mask

Figure 3  Spectrum masks for IEEE 802.11a, b, g

*	This Recommendation was jointly developed by Radiocommunication Study Groups 8 and 9, and future revisions should be undertaken jointly.

*	*	This Recommendation should be brought to the attention of Telecommunication Standardization Study Group 17, and Radiocommunication Study Groups 3 and 4.