IEEE P802.11
Wireless LANs

| MIC Calculation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

The procedures for calculation of the MIC when contained in an EAPKIE are too complex.  This is a proposal to significantly simplify them.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Current text in D1.0 explains how to calculate a MIC for an EAPKIE, including various portions of the MAC header.  Unfortunately, when the EAPKIE is included in an Action Frame and transported to the target AP via the RRB, the MAC header is stripped and not available at the target AP for MIC verification.

Current text:

Key MIC. This field is 16 octets in length when the Key Descriptor Version subfield is 1 or 2. When the EAPOL-Key is transported as an IEEE 802.11 data frame, theThe EAPOL-Key MIC is a MIC of the EAPOL-Key frames, from and including the EAPOL protocol version field to and including the Key Data field, calculated with the Key MIC field set to 0. Otherwise, when the EAPOL-Key is encapsulated as an EAPKIE in a management frame, the MIC protects most of the IEEE 802.11 MAC header as well as information elements provided in the management frame commencing with the MIC IE Count IE through and including the EAPKIE. Parts of the IEEE 802.11 MAC header are unprotected and muted prior to computation of the MIC, as follows:

- Frame Control field: the Retry (bit 11), Power management (bit 12) and More Data (bit 13) subfields shall be muted to 0 and the Protected subfield (bit 14) shall be set to 1.

- Duration field: this field shall be muted to 0.

- Sequence control field: this field shall be muted to 0.

The text in 8.5A.8.3 and 8.5A.8.4 is as follows:

The MIC protects parts of the 802.11 header and information elements commencing with the MIC Extent IE and continuing through the EAPKIE. The MIC shall be constructed according to the procedures given in clause 8.5.2. The MIC shall be calculated using the KCK, by algorithm dependent on the value of Key Descriptor Version.

Text in 7.3.2.46 is as follows:

When the EAPKIE is preceded by a MIC Extent, then the MIC is calculated concatenating the IEEE 802.11 MAC header (with some fields muted) followed by the MIC Extent IE up to and including the end of the EAPKIE; see Clause 8.5.2 for full details of the MIC construction.

Revised text:

Key MIC. This field is 16 octets in length when the Key Descriptor Version subfield is 1 or 2. When the EAPOL-Key is transported as an IEEE 802.11 data frame, theThe EAPOL-Key MIC is a MIC of the EAPOL-Key frames, from and including the EAPOL protocol version field to and including the Key Data field, calculated with the Key MIC field set to 0. Otherwise, when the EAPOL-Key is encapsulated as an EAPKIE in a management frame, the EAPOL-Key MIC is a MIC of the concatenation of:

- STA MAC address (6 octets)

- Target AP MAC address (6 octets)

- Transaction sequence number (1 octet)

- Sequence of Information Elements contained in the frame, starting with the MIC Extent IE, up to and including the EAPKIE

The MIC shall be calculated with the Key MIC field of the EAPKIE set to 0.

Revised text in 8.5A.8.3 and 8.5A.8.4 is as follows:

The MIC shall be constructed according to the procedures given in clause 8.5.2, with the Transaction Sequence number being set to 3 (in 8.5A.8.3, or “set to 4” in 8.5A.8.4). The MIC shall be calculated using the KCK, by algorithm dependent on the value of Key Descriptor Version.

Revised text in 7.3.2.46 is as follows:

When the EAPKIE is preceded by a MIC Extent, then the MIC is calculated by the procedures given in clause 8.5.2.