IEEE P802.11
Wireless LANs

| RRB MLME Interface |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

This submission contains the MLME interface for the RRB.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

103.32 MLME SAP interface for remote requests

10.3.32.1 MLME-REMOTE_REQUEST.indication

10.3.32.1.1 Function

This primitive is used by the MAC to indicate to the RRB the receiption of a Management Frame of Subtype Action and Category “Fast BSS Transition”.

10.3.32.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-REMOTE_REQUEST.indication	(

PeerMACAddress,

Contents of FT Action Frame

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMACAddress | MAC Address | Any valid individual MAC address | Specifies the MAC address of the STA that issued the Action Frame |
| Contents of FT Action Frame | Sequence of octets | As defined in 7.4.6 | The Action Frame received from the STA. |

10.3.32.1.3 When generated

This primitive is generated by the MAC as a result of the receipt of an Action Frame of Category Fast BSS Transition from a specific peer MAC entity.

10.3.32.1.4 Effect of receipt

Upon receipt of this primitive, the RRB forwards the Action Frame to the Target AP identified in the Action Frame.

10.3.32.2 MLME-REMOTE_REQUEST.response

10.3.32.2.1 Function

This primitive is used by the RRB to request the MAC to send a Management Frame of Subtype Action and Category “Fast BSS Transition”.

10.3.32.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-REMOTE_REQUEST.response	(

PeerMACAddress,

Contents of FT Action Frame

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMACAddress | MAC Address | Any valid individual MAC address | Specifies the MAC address of the STA that is the destination the Action Frame |
| Contents of FT Action Frame | Sequence of octets | As defined in 7.4.6 | The Action Frame to send to the STA. |

10.3.32.2.3 When generated

This primitive is generated by the RRB to send an Action Frame of Category Fast BSS Transition to a specific peer MAC entity.

10.3.32.2.4 Effect of receipt

Upon receipt of this primitive, the MAC forwards the Action Frame to the STA identified in the Action Frame.