IEEE P802.11
Wireless LANs

| 802.11k Frame Report Request comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sudheer Matta | Trapeze Networks | 5753 W. Las Positas Blvd, Pleasanton, CA 94588 | 925 474 2290 | matta@trapezenetworks.com |

Abstract

802.11k LB78 letter ballot comment resolutions.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

##### 7.3.2.21.7 Frame Request

The Measurement Request field corresponding to a Frame Request is shown in Figure k1.

|  | Regulatory Class | Channel Number | Randomization Interval | Measurement Duration |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 2 |

Figure k1—Measurement Request field format for a Frame Request

Regulatory Class indicates the frequency band for which the measurement request applies. Valid values of Regulatory Class are shown in Error: Reference source not found.

Channel Number indicates the channel number for which the measurement request applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Randomization Interval specifies the upper bound of the random delay to be used prior to making the measurement in units of TU. See 11.11.3.

The Measurement Duration field shall be set to the preferred duration of the requested measurement, expressed in TUs. If the Duration Mandatory bit is set to 1 in the Measurement Request Mode field this shall be interpreted as a mandatory measurement duration. If the Duration Mandatory bit is set to 0 this shall be interpreted as a target measurement duration. See 11.11.4.

##### 7.3.2.22.7 Frame Report

The format of the Measurement Report field corresponding to a Frame Report is shown in Figure k2.

|  |  |  |  |  | Zero or more entries |
| --- | --- | --- | --- | --- | --- |
|  | Regulatory Class | Channel Number | Actual Measurement Start Time | Measurement Duration | Frame Report Entry |
| Octets: | 1 | 1 | 8 | 2 | n x 18 |

Figure k2—Measurement Report field format for a Frame Report

Regulatory Class indicates the frequency band for which the measurement report applies. Valid values of Regulatory Class are shown in Error: Reference source not found.

Channel Number indicates the channel number for which the measurement report applies. Channel Number is defined within a Regulatory Class as shown in Error: Reference source not found.

Actual Measurement Start Time shall be set to the value of the measuring STA's TSF timer at the time the measurement started.

Measurement Duration shall be set equal to the duration over which the Frame Report was measured, expressed in TUs.

The format of the Frame Report Entry is shown in Figure k3.

|  | Transmit Address | BSSID | PHY Type | Average RCPI | RSNI | Last RCPI | Antenna ID | Number of Frames |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 6 | 6 | 1 | 1 | 1 | 1 | 1 | 1 |

Figure k3—Frame Report Entry format

The Transmit Address field contains the Transmitter Address (TA) from the frames being reported.

The BSSID field contains the BSSID from the frames being reported.

PHY Type indicates the physical medium type for the frame(s) being reported. Valid entries are coded according to the value of dot11PHYType.

Average RCPI indicates the average value for the received channel power of all the frames received and counted in this Frame Report Entry. If more than 255 frames are received during this measurement duration, the Average RCPI indicates the average value for the most recent 255 frames. Average RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

RSNI indicates the received signal to noise indication of the most recently received frame in dB, as described in 7.3.2.31.

Last RCPI indicates the received channel power of the most recently measured frame in this Frame Report entry. Last RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

The Antenna ID field contains the identifying number for the antenna used to receive the most recently measured frame in this Frame Report entry. If different antennas are used to receive the preamble and the frame body, this Antenna ID shall contain the identifying number for the antenna used to receive the frame body

Antenna ID is defined in 7.3.2.29.

Number of Frames is a count of the unicast data and management frames received with the indicated Transmit Address and BSSID during the measurement duration. The value 255 shall indicate a count of 255 or more.

#### 11.11.9.2 Frame Report

If a station accepts a Frame Request it shall respond with a Radio Measurement Report frame containing one or more Measurement (Frame) Report elements. The measuring station shall receive all observable traffic during the measurement duration and shall summarize this traffic in one or more Frame Report elements. Each Frame Report element contains one or more Frame Report Entries. The measuring station shall count the number of unicast and multicast data and management frames received from one transmit address during the measurement duration and shall summarize this traffic in a Frame Report Entry. Each Frame Report Entry contains the Transmit Address, BSSID, Average RCPI, Last RCPI, Antenna ID and Number of Frames for the counted in the Frame Report Entry. The reported Average RCPI shall be the average of the RCPI values of all frames counted in the Frame Report Entry. The Last RCPI shall be the RCPI value of the most recently received frame counted in the Frame Report Entry. The Antenna ID field contains the identifying number for the antenna used to receive the most recently received frame included in this report. If different antennas are used to receive the frame preamble and the frame body, this Antenna ID shall contain the identifying number for the antenna used to receive the frame body.