IEEE P802.11
Wireless LANs

| Proposal for an Extended Capabilities Information Element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bob O'Hara | Cisco Systems | 3625 Cisco Way San Jose, CA | +1 408 853 5513 | Bob.ohara@cisco.com |

Abstract

This is a proposal for an information element to carry capability information, after the Capability Information Field (CIF) is consumed.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This submission provides instructions to the editor to add an information element to carry extended capabilities.

Insert the following into Table 22:

| Extended Capabilities | TBD |
| --- | --- |

Insert the following clause after 7.3.2.26:

7.3.2.27 Extended Capabilities

The Extended Capabilities information element carries information about the capabilities of an 802.11 STA, once the Capability Information Field (CIF) is consumed.   The format of this information element is shown in Figure REV-1.

|  | Element ID | Length | Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | n |

Figure REV-1 – Extended Capabilities element format

The Element Identifier of this information element is TBD.

The Value of the Length field is equal to the number of octets in the Capabilities field.

The Capabilities field is a bit field indicating the capabilities being advertised by the STA transmitting the information element.  There are no capabilities defined for this field in this revision of the standard.