IEEE P802.11
Wireless LANs

| Addressing Beacon Bloat Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date February 23, 2006 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Kapil Sood | Intel Corp. | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | Kapil.Sood@intel.com |
| Frank Ciotti | Motorola | 7700 W. Parmer Ln Austin, TX 78726 | 512-996-5753 | frank.ciotti@motorola.com |
| Bill Marshall | AT&T | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

The document provides the text updates to TGr draft D1.1, as proposed in the document 11-06-0214-00-000r-beacon-bloat-proposal.  The text addresses the Beacon Bloat comments to provide mechanisms that reduce the size of beacon and probe advertisements for Fast BSS Transitions.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Updates based on TGr Draft D1.1

Section 7.2.3.9, page 17 line 43

Delete order 26 Fast BSS Transition information, in Table 12

Section 7.2.3.4, page 14 line 16, order 9, Table 7

Replace: A Fast BSS Transition IE is present in an Association Request only when dot11FastBSSTransitionEnabled is true and the frame is being sent to an AP that advertised an FTIE in its Beacon or Probe Response (i.e., AP also last dot11FastBSSTransitionEnabled set to true)

With: A Fast BSS Transition IE is present in an Association Request only when dot11FastBSSTransitionEnabled is true and dot11RSNAAuthenticationSuiteSelected is 3 or 4 (i.e., part of a Fast BSS Transition in an RSN).

Section 7.2.3.5, page 15 line 8, order 9, Table 8

Replace: A Fast BSS Transition IE is present in an Association Response only when dot11FastBSSTransitionEnabled is true, and the Association Request frame contained an FTIE (i.e. either a First Contact exchange, or that is part of a Fast BSS Transition).

With: A Fast BSS Transition IE is present in an Association Response only when dot11FastBSSTransitionEnabled is true, dot11RSNAEnabled is true, and the response is to an Association Request frame that contained an MDIE (i.e., part of a Fast BSS Transition in an RSN).

Section 7.2.3.6, page 15 line 58, order 10, Table 9

Replace: A Fast BSS Transition IE is present in a Reassociation Request only when dot11FastBSSTransitionEnabled is true and the frame is being sent to an AP that advertised an FTIE in its Beacon or Probe Response (i.e., AP also last dot11FastBSSTransitionEnabled set to true)

With: A Fast BSS Transition IE is present in a Reassociation Request only when dot11FastBSSTransitionEnabled is true and dot11RSNAAuthenticationSuiteSelected is 3 or 4 (i.e., part of a Fast BSS Transition in an RSN).

Section 7.2.3.7, page 16 line 48, order 9, Table 10

Replace: A Fast BSS Transition IE is present in a Reassociation Response only when dot11FastBSSTransitionEnabled is true, and the Reassociation Request frame contained an FTIE (i.e. either a First Contact exchange, or that is part of a Fast BSS Transition).

With: A Fast BSS Transition IE is present in an Association Response only when dot11FastBSSTransitionEnabled is true, dot11RSNAEnabled is true, and the response is to a Reassociation Request frame that contained an MDIE (i.e., part of a Fast BSS Transition in an RSN).

Section 8.5A.8.2, page 44 line 26

Replace: "The target AP shall also echo the key holder IDs..."

With: “The target AP shall also provide the key holder IDs..."

Section 8.5A.8.3, page 46 line 5

Replace: “The RSN IE shall be bitwise identical to the RSN IE presented in the Fast BSS Transition request (see clause 8.5A.8.1); in particular it shall be set as follows:

- Version shall be set to 1

- PMKID count shall be set to 1

- PMKID field shall contain the PMKR1Name”

With: “The RSN IE shall be set as follows:

- Version shall be set to 1

- PMKID count shall be set to 1

- PMKID field shall contain the PMKR1Name

- All other fields shall be as specified in clause 7.3.2.25 and 8.4.3.”

Section 7.3.2.42, Page 22, line 20

Replace:

The Mobility Domain information element contains a Mobility Domain Identifier. The Access Point uses the

Mobility Domain Information Element to advertise that it is included in the group of APs that constitute the

Mobility Domain. The format for this information element is given in Figure 80AC.

The Length field shall be set to 6.

The Mobility Domain Identifier is a 48-bit value.

With:

The Mobility Domain information element contains the Mobility Domain Identifier, Fast BSS Transition Capability, and the Resource Policy. The Access Point uses the Mobility Domain Information Element to advertise that it is included in the group of APs that constitute the Mobility Domain, to advertise its support for Fast BSS Transition capability, and to advertrise its resource policy information.  The format for this information element is given in Figure 80AC.

The Length field shall be set to 7.

The Mobility Domain Identifier is a 48-bit value.

The Fast BSS Transition Capability and Resource Policy field is 1 octet.

When sent by an AP, the Fast BSS Transition Capability and Resource Policy value is defined in Figure 80AE. When sent by a STA to an AP, the Fast BSS Transition Capability and Resource Policy value is identical to the Policy value advertised by that AP.

Insert Figure 80AE here from Section 7.3.2.43.  Remove Figure 80AE from Section 7.3.2.43

Replace Title of Figure 80AE from “Figure 80AE—Fast BSS transition resource mechanism value”

With: “Figure 80AE—Fast BSS Transition Capability and Resource Policy value”

Fast BSS Transition Capability is set if the AP supports Fast BSS Transition

Reservation over air is set if the AP recommends Reservation be done over-the-air.

Reservation over DS is set if the AP recommends Reservation be done over-the-DS.

Reserve Option is set if AP requires a reservation if a RIC request will appear in a (re)association, and clear

if the STA’s use of reservation is optional

Reservation Limit is set to the maximum number of APs at which a STA can make concurrent reservations.

Value of zero indicates no limit.

The various combinations of mechanisms supported by the AP are shown in Table 31J:

Insert Table 31J  here from Section 7.3.2.43.  Remove Table 31J from Section 7.3.2.43

The STA can use information from the MDIE to determine the transition mechanisms supported and recommended

by the AP. The choice of executing any specific transition mechanism is outside the scope of this standard.

Replace Figure 80AE in Section 7.3.2.42

With: the following

Fast BSS Transition Capability

Reserve Option

Reservation Over DS

Reservation Limit

Reservation Over Air

Bit:       B0                       B1                                 B2                     B3                                             B4-7

Section 7.3.2.42, Page 22, line 20

Replace: Figure 80AC

With:

Element ID

Length

Mobility Domain

Identifier

Fast BSS Transition  Capability and Resource

Policy

Octets:    1                 1                          6                           1

Section 7.3.2.43, Page 22, line 47

Replace:

The Fast BSS Transition Information Element is defined to include network infrastructure resource reservation

and security policy information. An AP can advertise the FTIE by including the FTIE in a Beacon frame

and Probe Response frame. This information element is defined in Figure 80AD.

With:

The Fast BSS Transition Information Element (FTIE) is defined to include key holder identity information.  This information element is defined in Figure 80AD.

Section 7.3.2.43, Page 23, line 15

Replace: Figure 80AD

With:

Element ID

Length

R0KH-ID

R1KH-ID

(Optional)

# Octets:      1                      1                        16                                 16

The Length field shall be set to 16 for FTIE without the R1KH-ID field

The Length field shall be set to 32 for FTIE wih the R1KH-ID field

Section 7.3.2.43, Page 24, line 26

Replace: “When sent by an AP, R0KH-ID indicates the primary (or default) AP Authenticator, and is the value of the MIB variable dot11FTR0KeyHolderID. When sent by a STA, R0KH-ID indicates the PMK-R0 holder with

which the STA negotiated its current security association.

R1KH-ID indicates the PMK-R1 Key Holder that will be utilized by the AP for key derivation.”

With: “When sent by a STA, R0KH-ID indicates the PMK-R0 holder with which the STA negotiated its current security association.  When sent by AP, R0KH-ID shall be the same as the value received from the STA.  When sent by the AP, the R1KH-ID indicates identity of the key holder of the AP.  When sent by the STA, R1KH-ID shall be same as the value received from the AP.

R0KH-ID indicates the PMK-R0 Key Holder that will be utilized by the AP for PMK-R1 key procurement.  R1KH-ID indicates the AP’s key holder identity which will be used by the STA for deriving the PMK-R1 keys.”

Section 7.3.2.43, Page 24, Remove line 33-36

Section 7.2.3.1, delete order 29 “Fast BSS Transition Information”

Section 7.2.3.4, Page 14, Table 7, Order 9 and 10, and all other places in 7.2 where the same text appears:

Replace: “advertised an FTIE in its Beacon…”

With: “advertised its Fast BSS Transition Capability in the MDIE in its Beacon…”

Section 8.5A.4, Page 39, Line 37

Replace: “Each AP shall advertise its R0KH-ID in the FTIE”

With: “Each AP shall deliver its R0KH-ID to the STA in the FTIE of the (Re)Association Response message during First Contact .”

Section 8.5A.5, Page 40, Line 10

Insert at the end of paragraph: “Each AP shall deliver its R1KH-ID to the STA in the FTIE”

Section 8.5A.8.1, Page 43, Line 30

Replace: “The STA shall include the Key Holder IDs (R0KH-ID and R1KH-ID in the FTIE), and the Key Name (PMKR1Name in the RSNIE) that it is using to generate the PTK. By including the key names and their respective key holders the target AP can use this information to ensure”

With: “The STA shall include the R0KH-ID in the FTIE.  The target AP can use the R0KH-ID and the STA MAC address to ensure”

Section 8.5A.8.1, Page 43

Remove line 61: “—  R1KH-ID shall be the value advertised by the target AP in its Beacons and Probe Responses.”

Remove line 57: “— Resource mechanisms shall be the value advertised by the target AP in its Beacons and Probe Responses.”

Replace Line 64: “The MDIE shall contain the Mobility Domain Identifier obtained from the target AP.”

With: “The MDIE shall contain the Mobility Domain Identifier, Fast BSS Transition Capability, and Resource Policy obtained from the target AP.  The Target AP advertises the MDIE in beacons and probe responses.”

Section 8.5A.8.1, Page 44

Change line 5-6 “PMKID count shall be 1” and “PMKID field shall contain the PMKR1Name…..”

With “PMKID count shall be 0 (i.e., there are no PMKID fields)”

Section 8.5A.8.2, Page 44

Replace Line 64: “The contents of the Fast BSS Transition Information Element (FTIE) shall be identical to the contents of the FTIE advertised by the AP in Beacons and Probe Responses, and shall be set as follows: “

With: “The contents of the Fast BSS Transition Information Element (FTIE) shall be set as follows:”

Section 8.5A.8.2, Page 45

Replace Line 1-8: “— Resource mechanisms shall indicate the capabilities of the target AP.

- Key holder IDs (R0KH-ID, and R1KH-ID) shall be identical to the key holder identification provided by the STA in the first message.

The MDIE shall contain the Mobility Domain Identifier. This shall be the same as the MDIE advertised by the AP in Beacons and Probe Responses.”

With: “ - R0KH-ID shall be identical to the R0 Key Holder Identifier provided by the STA in the first message:

- R1KH-ID shall be set to the R1 Key Holder Identifier of the AP

The MDIE shall contain the Mobility Domain Identifier, Fast BSS Transition Capability, and the Resource Policy. This shall be the same as the MDIE advertised by the AP in Beacons and Probe Responses.”

Section 8.5A.8.3, Page 45

Replace Line 64: “The Fast BSS Transition Information Element (FTIE) shall be identical to the FTIE presented in the first message of this sequence.”

With: “The Fast BSS Transition Information Element (FTIE) shall be identical to the FTIE presented in the second message of this sequence.”

Section 8.5A.8.3, Page 46, Line 1

Replace: “The MDIE shall contain the Mobility Domain Identifier. This shall be identical to the MDIE contained in the first message of this sequence.”

With: “The MDIE shall contain the Mobility Domain Identifier, Fast BSS Transition Capability, and the Resource Policy. This shall be identical to the MDIE contained in the first message of this sequence.”

Section 8.5A.8.4, Page 47, Lines 16-19

Replace: “The MDIE shall contain the Mobility Domain Identifier. This shall be identical to the MDIE contained in the second message of this sequence.”

With: “The MDIE shall contain the Mobility Domain Identifier, Fast BSS Transition Capability, and the Resource Policy. This shall be identical to the MDIE contained in the second message of this sequence.”

Section 8.A.1.1, Page 49, Lines 41-55

Replace Lines 41-55: “Fast BSS Transition capability is advertised in the Beacon and Probe Response frames by inclusion of the FTIE and MDIE. The Mobility Domain Identifier shall be the value of dot11FTMobilityDomainID. The reservation limit advertised in the FTIE shall be the value of dot11FTReservationLimit. Note: the mobility domain identifier IE may be present without FTIE since it may be used in other mecha-nisms beyond the Fast Transition mechanisms.”

With: “Fast BSS Transition capability is advertised in the Beacon and Probe Response frames by setting the Fast BSS Capability bit in the MDIE.  The MDIE is advertised in the Beacons and Probe Responses to indicate the MD-ID, Fast BSS Transition Capability, and the Resource Policy.  When the MDIE is used without Fast BSS Transition Capability (Fast BSS Transition Capability bit set to zsero), then all other bits of the Fast BSS Capability and Resource Policy field shall be ignored.

The Mobility Domain Identifier shall be the value of dot11FTMobilityDomainID. The reservation limit advertised in the MDIE shall be the value of dot11FTReservationLimit. ”

Section 8.A.2, Page53, Lines57

Replace: “FTIE and MDIE in the (re)association request,”

With: “MDIE in the (re)association request,”

Section 8.A.2, Page54, Lines50-54

Replace: “Upon successful IEEE 802.11 open authentication, STA shall send a (re)association request to the AP and shall include the FTIE and MDIE. The contents of the FTIE and MDIE shall be the values advertised by the AP in its Beacons or Probe Responses. The presence of the FTIE and MDIE indicates to the AP a desire to enable future Fast BSS Transitions.”

With: “Upon successful IEEE 802.11 open authentication, STA shall send a (re)association request to the AP and shall include the MDIE. The contents of the MDIE shall be the values advertised by the AP in its Beacons or Probe Responses. The presence of the Fast BSS Transition Capability bit set to one in Fast BSS Transition Capability and Resource Policy field indicates to the AP a desire to enable future Fast BSS Transitions.”

Section 8.A.2, Page54, Lines57-65

Replace: “The (re)association response from the AP shall contain a FTIE and MDIE, with contents as presented in Beacons and Probe Responses. The AP shall include its security capabilities in the RSN Information Element. The PMKID list field shall be empty.

STA->AP: (Re)association Request (FTIESTA, MDIESTA, RSNIESTA)

AP->STA: (Re)association Response (FTIEAP, MDIEAP, RSNIEAP).”

With: “The (re)association response from the AP shall contain the MDIE with contents as presented in Beacons and Probe Responses.  The FTIE in the (re)association response shall include the key holder (R0KH-ID) of the AP.  The AP shall include its security capabilities in the RSN Information Element. The PMKID list field shall be empty.

STA->AP: (Re)association Request (MDIESTA, RSNIESTA)

AP->STA: (Re)association Response (FTIEAP, MDIEAP, RSNIEAP)”

Section 8.A.2, Page55, Lines 13

Insert: “- R0KH-ID”

Section 8.A.2, Page55, Lines 41

Replace: “the STA’s (re)association request.”

With: “the AP’s (re)association response”

Section 8.A.2, Page55, Lines 43-50

Replace: “Message 3: the AP shall include the PMKR1Name in the PMKID field of the RSNIEAP. The AP shall also include the FTIEAP, MDIEAP and PTK key lifetime in the TIE[KeyLifetime]. The FTIEAP, and MDIEAP shall be the same as that advertised by the AP’s Beacon and Probe Responses. The PMKR1Name shall be as calculated by the R1KH according to the procedures of 8.5A.5; in order for future Fast BSS Transitions to be successful it needs to be the same as the PMKR1Name in Message #2.”

With: “Message 3: the AP shall include the PMKR1Name in the PMKID field of the RSNIEAP. The AP shall also include the FTIEAP, MDIEAP, and PTK key lifetime in the TIE[KeyLifetime]. The FTIEAP, and MDIEAP shall be the same as in (re)association response.  The PMKR1Name shall be as calculated by the R1KH according to the procedures of 8.5A.5; in order for future Fast BSS Transitions to be successful it needs to be the same as the PMKR1Name in Message #2”

Section 8.A.3.3, Page57, Lines 22, 27

Replace: “FTIE”

With: “MDIE”

Section 8.A.4, Page58, Lines 1, 3

Replace: “FTIE”

With: “MDIE”

Section 8A.4, page 58 line 4

Replace: “If neither Reservation-over-air (bit 0) nor Reservation-over-DS (bit 1) is set to one, then the STA shall not send a reservation request to the AP. If reservation is marked as mandatory, i.e., Reserve Option bit 2 is set to one, then the STA shall not include a RIC in its (re)association request without first performing a reservation. If Reservation-over-air (bit 0) is clear, then the 	STA shall not issue an over-the-air reservation request. If Reservation-over-DS (bit 1) is clear, then the STA shall not issue an over-the-DS reservation request."

With: “"If neither Reservation-over-air nor Reservation-over-DS is set to one, then the STA shall not send a reservation request to the AP. If reservation is marked as mandatory, i.e., Reserve Option is set to one, then the STA shall not include a RIC in its (re)association request without first performing a reservation. If Reservation-over-air is clear, then the STA shall not issue an over-the-air reservation request. If Reservation-over-DS is clear, then the STA shall not issue an over-the-DS reservation request."

Figures 121F

Remove: FTIESTA from (Re)association Request

Table 35A, Order 4

Replace: Mobility Domain Identifier

With: Mobility Domain Information

Tables 32P, 32Q, 32R, 32S, and everywhere it appears in the “Information” column of tables in 7.2

Replace: “Mobility Domain Identifier”

With:  “Mobility Domain Information”