IEEE P802.11
Wireless LANs

| Updates to TGr Clauses 5.4.8.1 and 8A.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-31 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | Chantry Networks | 1900 Minnesota Ct, Suite 125, Mississauga, ON. CANADA L5N 3C9 | +1-905-363-6413 | michael.montemurro@siemens.com |
| Bill Marshall | TGr Editor | 180 Park Ave, Florham Park, NJ 07932 | +1-973-360-8718 | wtm@att.research.com |

Abstract

This document updates the text in Clauses 5.4.8.1 and 8A.1 to address letter ballot comments ballot. The submission is based on IEEE 802.11r Draft 1.1.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Note: Updates to this clause address comments: 390, 391,923, 926, 955, 1155.

### 5.4.8.1 General concepts

Replace the text in clause 5.4.8.1 with the following:

BSS transition services are invoked by a STA in the process of leaving its association with a current AP, and establishing a new association with a target AP. A STA passes through three stages in the process of transition:

- Discovery - the STA locates and decides to which AP it will attempt a transition;

- Authentication and (optional) resource reservation - the STA authenticates with the target AP, and may reserve resources that it needs at the target AP to maintain active sessions.

- Reassociation - the STA leaves its association with the current AP and establishes an association with the target AP. The STA may reserve the resources that it requires at (re)association.

Comment #390 changed to

Accepted in principle. Text changed to "A STA passes through three stages in the process of transition:"

Comment #391 changed to:

Accepted in principle. Step 2 replaced with "Authentication and (optional) resource reservation - the STA authenticates with the target AP, and may reserve resources that it needs at the target AP to maintain active sessions."  Step 3 augmented with "The STA may reserve the resources that it requires at (re)association."

Comment #926 changed to:

Accepted in principle. "Resource establishment" is not correct. Changed to "Authentication and (optional) resource reservation"

Comment #923 and #955 changed to:

Accepted in principle. "Transition" is not the correct term to use for this stage. Changed to "Reassociation"

Comment #1155 changed to:

Accepted in principle. "Transition" is not the correct term to use for this stage. Changed to "Reassociation" both here and as title of clause 5.4.5.1.3.

### Note: Updates to this clause address comments 742, 482, 1116, 818, 484, 246, 1248, 334, 483, 729, 594

### 8A.1 Overview of transition mechanisms

Replace the text in clause 8A.1 with the following:

Fast BSS-Transition services reduce the amount of time data connectivity is lost between the STA and the DS during a BSS transition. Fast BSS-Transition services only apply to the connection state between a STA and Access Points within the same ESS.

The Fast BSS Transition mechanism requires additional information to be exchanged between the STA and AP, which occurs during the first (re)association between the STA and AP. This initial exchange is referred to as the First Contact. Subsequent (re)associations to other APs within the same Mobility Domain may make use of the FT mechanisms defined herein to minimize the loss of connectivity to the DS.

After (re)association with the target AP the STA may issue a deauthentication message to the previous AP

but is not required to do so. Irrespective of such disassociation message the STA shall enter State 1 with

respect to the previous AP and delete any corresponding security association.

Two Fast BSS Transition mechanisms are defined:

- Fast BSS Transition Base Mechanism: this mechanism is executed when a STA needs to make a transition to a target AP and does not require a reservation prior to its transition.

- Fast BSS Transition Reservation Mechanism: this mechanism is executed when a STA needs assurances that the required security and resources be available prior to a transition.

APs advertise both capabilities and policies for supporting the Fast BSS Transition mechanisms.

NOTE-- Throughout this clause, the notation Authentication-Request refers to an Authentication frame with the Authentication Transaction Sequence number set to 1; Authentication-Response refers to an Authentication frame with the Authentication Transaction Sequence number set to 2; Authentication-Confirm refers to an Authentication frame with the Authentication Transaction Sequence number set to 3; Authentication-Ack or Authentication-Acknowledgement refers to an Authentication frame with the Authentication Transaction Sequence number set to 4. The first parameter to the above four messages is the Authentication Algorithm, such as "Open System Authentication Algorithm" (or shortened to "Open"), or "Fast BSS Transition Authentication Algorithm" (or shortened to "FT").The notation EAPKIE(EAPOL-Key(-)) refers to an EAPOL-Key frame, with notation as defined in clause 8.5.2.2, encapsulated in an EAPKIE, as described in clause 7.3.2.46.

Comment #742 changed to:

Accepted. First three paragraphs of 8A.1 replaced with:

Fast BSS-Transition services reduce the amount of time data connectivity is lost between the STA and the DS during a BSS transition. Fast BSS-Transition services only apply to the connection state between a STA and Access Points within the same ESS.

Introduction (in frontmatter) modified to read:

This amendment describes mechanisms that reduce the amount of time data connectivity is lost between the STA and the DS during a BSS transition. The STA determines when to transition and which AP to transition to based on a number of factors, some of which may be out of scope of this standard.

The following summarizes the typical behavior of the STA and AP when a transition occurs without Fast BSS Transition services:

 The STAs uses scanning or neighbor reports to discover APs available for transition.

 The STA chooses a target AP and performs an IEEE 802.11 authentication exchange with that AP; typically this will be an “open auth” exchange. During this time, the STA may still exchange data with the DS through its current AP.

 The STA sends a (re)association message to establish a connection at the target AP.

 The STA and the AP then generate and confirm matching temporal keys based on PSK or an IEEE 802.1X authentication (which could be through an earlier pre-authentication and key caching)

 The STA and AP install the key and start to exchange data with the DS.

 For a QSTA connected to a QAP, the STA may then issue a QoS admission control request

The Fast BSS Transition mechanism allows a STA to establish security and QoS state at the target AP prior to or during (re)association, avoiding delays in connecting to the DS after transition.

The Fast BSS Transition mechanism requires information to be exchanged during the first (re)association between the STA and AP beyond that which is exchanged during a non-Fast BSS Transition (re)association. This initial exchange is referred to as the “First Contact.” Subsequent (re)associations to other APs within the same Mobility Domain may make use of the FT mechanisms defined herein.

Comment #482 changed to:

Rejected. It may or may not be secure data transmission. It may or may not include QoS. Neither is required in order to perform Fast BSS Transitions.

Comment #1116 changed to:

Accepted in principle. Most of this clause has been deleted.

Comment #818, #484, #246, #1248, #334, #483, #729, #594 changed to:

Accepted in principle, but text deleted by comment #742