IEEE P802.11
Wireless LANs

| Updates to TGr Clause 5.4.8.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-01-31 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | Chantry Networks | 1900 Minnesota Ct, Suite 125, Mississauga, ON. CANADA L5N 3C9 | +1-905-363-6413 | michael.montemurro@siemens.com |

Abstract

This document updates the text in Clauses 5.4.8.2 to address letter ballot comments. The submission is based on IEEE 802.11r Draft 1.1.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Note: Updates to this clause address comments:

### 5.4.8.2 Fast BSS-transition architecture description

Delete the text in red beginning with “Note”.

Replace two paragraphs, after the text in red beginning with “Note”, with the following text:

A representative Fast BSS Transition topology is given in Figure 7A. It consists of a number of APs, a Mobility Domain Controller (MDC), and an Authentication Server. The remote request broker (RRB) resides in each AP to provide a transaction-based mechanism for relaying remote request frames.

All AP’s are configured to be part of a single Mobility Domain. Fast BSS-transition is only possible between AP’s in the same Mobility Domain. AP’s that advertise the same mobility domain identifier all provide access to the R0 Key Holder, and are reachable over the DS. The STA and the target AP will use the Mobility Domain identifier and the PMK-R0 to derive the remainder of the Fast BSS-transition key hierarchy.

Within an RSN, the Mobility Domain Controller authorizes AP 1, AP 2, and AP 3 to exchange key material and context. The R0 and R1 Key Holders use the Mobility Domain Controller to establish a security association for the exchange of PMK-R1 key material and context.

In an  RSN, the STA establishes a First Contact security association with AP 1 using IEEE 802.1X. The STA uses the Fast BSS-Transition AKM to derive the PMK-R0, PMK-R1, and PTK keys for its association to AP1.

After the STA has successfully established an association with the DS through AP1, it may also establish one or more resources (for example, active QoS streams) as necessary.

In this example, the STA has two potential transition targets, AP2 and AP3. The STA decides through scanning, neighbor reports and other means that the best candidate for making a transition is AP2.

The STA and AP 2 will need the PMK-R1 to establish the PTK. The STA would pre-compute the PMK-R1 during the authentication or action frame exchange. The R0 Key Holder would either proactively generate the PMK-R1 and push it to the R1 Key Holder on AP 2, or generate the PMK-R1 on a request from the R1 Key Holder on AP 2.

The STA can reserve resources at AP 2 to maintain its current service level at (re)association time. There are two mechanisms for the STA to issue resource requests to candidate APs, “over-the-air” and “over-the-DS”. In the first mechanism, “over-the-air”, the STA communicates directly with the candidate AP. This may require the STA to go to a different radio channel. In the second mechanism, “over-the-DS”, the STA communicates with the candidate AP through its current AP. In the latter case, the STA uses the Remote Request Broker (RRB) function of the current AP to relay the requests to the RRB function of the target AP, where they are processed. Any advance resource reservation is held by the AP 2 until the STA completes a (re)association, or the (re)association deadline expires.

The resource reservation and security association with AP2 are completed with the (re)association frame exchange. Fast BSS-Transition is completed by a successful (re)association.

Replace Figure 7A with the following:

Figure 1 Representative fast BSS-transition topology

Comment #57, #63 changed to:

Accepted.

Comment #420, #609, #677, #816, #965, #1241, #10 changed to:

Accepted in principle. All mention of the Policy Management Server deleted.

Comment #75, #958 changed to:

Accepted. All mention of Policy Management Server deleted

Comment #928 changed to:

Accepted. Two paragraphs page 7 lines 42-56 changed to:

A representative Fast BSS Transition topology is given in Figure 7A. It consists of a number of APs, a Mobility Domain Controller (MDC), and an Authentication Server. The remote request broker (RRB) resides in each AP to provide a transaction-based mechanism for relaying remote request frames.

All AP’s are configured to be part of a single Mobility Domain. Fast BSS-transition is only possible between AP’s in the same Mobility Domain. AP’s that advertise the same mobility domain identifier all provide access to the R0 Key Holder, and are reachable over the DS. The STA and the target AP will use the Mobility Domain identifier and the PMK-R0 to derive the remainder of the Fast BSS-transition key hierarchy.

Within an RSN, the Mobility Domain Controller authorizes AP 1, AP 2, and AP 3 to exchange key material and context. The R0 and R1 Key Holders use the Mobility Domain Controller to establish a security association for the exchange of PMK-R1 key material and context.

In an RSN, the STA establishes a First Contact security association with AP 1 using IEEE 802.1X. The STA uses the Fast BSS-Transition AKM to derive the PMK-R0, PMK-R1, and PTK keys for its association to AP1.

After the STA has successfully established an association with the DS through AP1, it may also establish one or more resources (for example, active QoS streams) as necessary.

In this example, the STA has two potential transition targets, AP2 and AP3. The STA decides through scanning, neighbor reports and other means that the best candidate for making a transition is AP2.

The STA and AP 2 will need the PMK-R1 to establish the PTK. The STA would pre-compute the PMK-R1 during the authentication or action frame exchange. The R0 Key Holder would either proactively generate the PMK-R1 and push it to the R1 Key Holder on AP 2, or generate the PMK-R1 on a request from the R1 Key Holder on AP 2.

The STA can reserve resources at AP 2 to maintain its current service level at (re)association time. There are two mechanisms for the STA to issue resource requests to candidate APs, “over-the-air” and “over-the-DS”. In the first mechanism, “over-the-air”, the STA communicates directly with the candidate AP. This may require the STA to go to a different radio channel. In the second mechanism, “over-the-DS”, the STA communicates with the candidate AP through its current AP. In the latter case, the STA uses the Remote Request Broker (RRB) function of the current AP to relay the requests to the RRB function of the target AP, where they are processed. Any advance resource reservation is held by the AP 2 until the STA completes a (re)association, or the (re)association deadline expires.

The resource reservation and security association with AP2 are completed with the (re)association frame exchange. Fast BSS-Transition is completed by a successful (re)association.

Acronym added to clause 4 "MDC  Mobility Domain Controller"

Figure 7A modified to include boxes inside each AP labeled "R0 Key Holder" and "R1 Key Holder". Arc from "R0 Key Holder" in AP1 to "R1 Key Holder" in AP2 labeled "R1 Key Distribution". Arc between RRBs labeled "Remote Request/Response"