IEEE P802.11
Wireless LANs

| Network Selection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-02-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ronny Kim | LG Electronics, Inc. | 533 Hogye-1dong Dongan-gu Anyang-shi Kyongki-do Korea | +82-31-450-1879 | ronnykim@lge.com |
| Sook Hyun Yang | LG Electronics, Inc. | 16 Woomyeon-dong Seocho-gu Seoul Korea | +82-2-526-4198 | dimanche@lge.com |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This document specifies partial proposal to address the requirements in the Network Selection Cluster for TGu. The approach of “Interworking Information Broker” is introduced to allow the STA to interact with an entity which can has interface with SSPNs.

# Contents

Contents	2

1	Overview	3

1.1	Introduction	3

2	Interworking information acquisition	4

2.1	Interworking information broker address advertisement and Action frames	4

2.2	Frame Formats	4

2.2.1	Frame Formats for Interworking information broker address advertisement	4

2.2.1.1	Beacon frame	4

2.2.2	Action frames for Interworking information broker address advertisement	5

2.2.2.1	Interworking information action frame	5

2.2.3	Information elements for Interworking information broker address advertisement	6

2.2.3.1	Interworking Information Broker IE	6

2.2.3.2	Remote Request /Response frame definition	6

3	Reference	6

Figure

Figure 1 Network Topology for interworking information	3

Figure 2 Interworking Information Acquisition	4

Figure 3 Interworking Information Request action frame	5

Figure 4 Interworking Information Response action frame	5

Figure 5 Interworking Information Broker IE	6

Table

Table 1 Beacon frame body	4

Table 2 Interworking Information action field values	5

Table 3 Remote request/response frame	6

# Overview

## Introduction

The purpose of this document is to propose the mechanism for interworking information delivery.

In an 802.11r draft, action frames and their delivery mechanism on the DS are defined. By using existing mechanism, we can achieve interworking information delivery.

One centralized entity, namely Interworking information Broker, is suggested because making all APs have interworking functionality with all SSPNs causes scalability problem and complexity. The one of possible network topologies is shown in figure 1. Interworking information broker is logical entity which is in charge of interworking information with SSPNs. Interworking information broker can be one of APs. It is assumed that interworking information broker has the architecture to be communicable with APs over the DS. This architecture does not prevent each AP from being Interworking Information Broker.

Figure 1 Network Topology for interworking information

If this topoplogy is not supported, this mechanism stops at current AP and communication between AP and Interworking information broker is left to implementor’s choice.

Figure 2 Interworking Information Acquisition

# Interworking information acquisition

## Interworking information broker address advertisement and Action frames

In order for STA to communicate with Interworking information broker, AP shall advertise interworking information broker MAC addresses. These addresses are ones that AP can communicate with over the DS.

If these addresses are not included in beacon, interworking information broker are assumed to be current AP itself or exist in the network with which AP can communicate in some other way. Communication between AP and the network entities which are not in the same DS is out of scope of this document.

Once interworking information broker and MAC addresses are acquired, STA can communicate with interworking information broker by exchanging action frames. AP relays action frames to/from interworking information broker by looking into STA address and destination address of action frames.

## Frame Formats

### Frame Formats for Interworking information broker address advertisement

#### Beacon frame

The Beacon frame can include Interworking Information Broker IE in order to deliver the MAC Address of Interworking Information Broker.

| Order | Information |
| --- | --- |
| 11 | Interworking Information Broker IE |

Table 1 Beacon frame body

### Action frames for Interworking information broker address advertisement

#### Interworking information action frame

Two Action Frames are defined to retrieve Interworking Information for SSPNs. If the AP receives Interworking Information Request Action Frame with destination address setting from the STA, the AP can relay the frame to the Interworking Information Broker. Also, the AP can deliver the interworking information for the requested SSPNs through Interworking Information Response Action Frame. The action frame values associated with each frame format within the Interworking Information Action frame category are defined in table 2.

| Action field value | Description |
| --- | --- |
| 0 | Reserved |
| 1 | Interworking Information Request |
| 2 | Interworking Information Response |
| 3-255 | Reserved |

Table 2 Interworking Information action field values

Interworking Information Request Action Frame

The Interworking Information Request Action frame is sent by the STA to its associated AP in order to request the interworking information (e.g. roaming partner with the current SSPN, cost, supported services). The AP can send this action frame to the Interworking information Broker that is one entity to be able to provide interworking information requested by the STA.

The Interworking Information Request Aciton Frame format is as follows;

Figure 3 Interworking Information Request action frame

The Category field shall be set to the value for Interworking Informaiton Request Action frame.

The Action field shall be set to 1.

The STA Address shall be set to the STA MAC address.

Destination Address shall be set to the value of MAC Address of Interworking information Broker that can provide the interworking information for the requested SSPNs.

Interworking Information Request IE includes the request information.

Interworking Information Response Action Frame

The interworking information response action frame is responded to Interworking Information Request Action Frame. The AP associated with the STA can send the interworking information by using this response action frame.

The Interworking Informaiton Response Aciton Frame format is as follows;

Figure 4 Interworking Information Response action frame

The Category field shall be set to the value for Interworking Informaiton Request Action frame.

The Action field shall be set to 2.

The STA Address shall be set to the STA MAC address.

Destination Address shall be set to the value of MAC Address of Interworking information Broker that can provide the interworking information for the requested SSPNs.

Interworking Information Response IE includes the interworking information for the requested SSPNs.

### Information elements for Interworking information broker address advertisement

#### Interworking Information Broker IE

The Interworking Information Broker IE is used to notify AP’s reachable interworking information Broker’s MAC address. Delivery of this IE indicates STA can communicate with interworking information broker with action frames.

Figure 5 Interworking Information Broker IE

#### Remote Request /Response frame definition

Remote request/response frame which AP use to relay interworking information action frame to interworking information broker.

| Size | Field | Description |
| --- | --- | --- |
| 1 | Protocol Version | Shall be 0x01 |
| 1 | Packet Type | Fast BSS Transition Remote Request : 0x00 Fast BSS Transition Remote Response : 0x01 Interworking Information Request: 0x02 Interworking Information Response: 0x03 |
| 2 | Action Length | Unsigned number representing the length in octets of the Action Frame field |
| 1 | Request Identifier | An identifier that wil be used to match a remote request with a remote response |
| 6 | AP Address | Shall be set to the MAC address of the current AP |
| Variable | Action Frame | The content of Action Frames |

Table 3 Remote request/response frame

# Reference

[1]	IEEE 802.11-05/0822r9 – TGu Requirements