IEEE P802.11
Wireless LANs

| Strawman 11n PICS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-03-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Simon Black | Nokia |  |  | EXT-simon.3.black @nokia.com |
| Vincenzo Scarpa | ST |  |  | vincenzo.scarpa @st.com |
| Kaberi Banerjee | Philips |  |  | kaberi.banerjee @philips.com |
| David Cypher | NIST |  |  | david.cypher @nist.gov |
| Srinivas Kandala | Airgo Neworks, Inc |  |  | srini @airgonetworks.com |
| Ali Raissinia | Airgo Neworks, Inc |  |  | aliraissinia @airgo.com |
| Adrian Stephens | Intel |  |  | Adrian.p.stephens @intel.com |

Abstract

This is a strawman PICS for discussion based on draft IEEE802.11n/D0.02.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Annex A

## A.4 PICS proforma–IEEE Std 802.11, 2006 Edition9

### A.4.3 IUT configuration

Change PICS items CF10 as follows and insert CF15 at the end of the table:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF10 | Is spectrum management operation supported? | 7.3.1.4, 11.6 | (CF6 OR CF15):O |  |
| *CF15 | Enhancements for Higher Throughput |  | O | Yes, No |

Insert the following new clause after clause A.4.14:

### A.4.15 Enhancements for Higher Throughput

#### A4.15.1 MAC Enhancements for Higher Throughput

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following MAC protocol enhancements supported? |  |  |  |
| HTM1 | HT capabilities signaling |  |  |  |
| HTM1.1 | HT capabilities element | 7.3.2.41.1 | CF15:M | Yes, No, N/A |
| HTM1.2 | Signaling of STA capabilities in Probe Request, Association Request and Reassociation Request frames | 7.3.2.41, 7.2.3.8, 7.2.3.4, 7.2.3.6, 11.1.3.2.2 | (CF15 AND CF2):M | Yes, No, N/A |
| HTM1.3 | Signaling of STA and BSS capabilities in Beacon, Probe Response, Association Response and Reassociation Response frames | 7.3.2.41, 7.2.3.1, 7.2.3.9, 7.2.3.5, 7.2.3.7, 11.1.3.2.2 | (CF15 AND CF1):M | Yes, No, N/A |
| HTM2 | Signaling of additional HT information | 7.3.2.42 | (CF15 AND CF1):M | Yes, No, N/A |
| HTM3 | MPDU aggregation |  |  |  |
| HTM3.1 | Reception of A-MPDU | 7.3.2.41.3, 8.8, 11.15, A.1.17.2 | CF15:M | Yes, No, N/A |
| HTM3.2 | A-MPDU format | 7.1.2.2, 7.4A.1 | CF15:M | Yes, No, N/A |
| HTM3.3 | A-MPDU contents | 7.4A.2 | CF15:M | Yes, No, N/A |
| HTM3.4 | A-MPDU frame exchange sequences | 9.9.1.4, 9.12 | CF15:M | Yes, No, N/A |
| HTM3.5 | Transmission of A-MPDU | 7.3.2.41.3, 8.8, 9.10.7.7, 11.15 | CF15:O | Yes, No, N/A |
| HTM4 | MSDU aggregation |  |  |  |
| HTM4.1 | Reception of A-MSDUs | 7.1.2.1, 7.1.3.5, 7.2.2.1, A1.17.2 | CF15:M | Yes, No, N/A |
| HTM4.2 | A-MSDU format | 7.1.2.1 | CF15:M | Yes, No, N/A |
| HTM4.3 | A-MSDU content | 7.1.2.1, 7.2.2.1 | CF15:M | Yes, No, N/A |
| HTM4.4 | Transmission of A-MSDUs | 7.1.2.1, 7.1.3.5, 7.2.2.1 | CF15:O | Yes, No, N/A |
| HTM5 | Block Acknowledgement |  |  |  |
| HTM5.1 | Block-ACK mechanism | 7.2.1.7, 7.2.1.8, 7.3.1.14, 7.3.1.15, 7.3.2.29, 7.4.4, 9.10, 11.5 | CF15:M | Yes, No, N/A |
| HTM5.2 | Use of Compressed Bitmap between HT STAs | 7.3.1.14, 9.10.6 | CF15:M | Yes, No, N/A |
| HTM5.3 | N-Immediate Block-ACK extensions | 9.10.7 | CF15:M | Yes, No, N/A |
| HTM5.4 | N-Delayed Block-ACK extensions | 9.10.8 | CF15:O | Yes, No, N/A |
| HTM5.5 | Multiple TID Block ACK | 7.2.1.7.1, 7.2.1.7.2, 7.2.1.8.2, 9.18.3, 11.5. 2 | HTM10:M | Yes, No, N/A |
| HTM6 | Protection mechanisms for different HT PHY options |  |  |  |
| HTM6.1 | Protection of RIFS PPDUs in the presence of non-HT STAs | 9.14.1 | CF15:M | Yes, No, N/A |
| HTM6.2 | Protection of Green Field PPDUs in the presence of non-HT STAs | 9.14.2 | HTP1.3:M | Yes, No, N/A |
| *HTM7 | L-SIG TXOP protection mechanism | 9.15 | CF15:O | Yes, No, N/A |
| HTM7.1 | Update NAV according to L-SIG | 9.15.3 | HTM7:M | Yes, No, N/A |
| HTM8 | Long NAV protection mechanism | 9.16.2 | CF15:O | Yes, No, N/A |
| HTM8.1 | Truncation of TXOP | 9.16.3 | CF15:O | Yes, No, N/A |
| *HTM9 | Reverse direction aggregation exchanges | 9.17 | CF15:O | Yes, No, N/A |
| HTM9.1 | Constraints regarding responses | 9.17.3 | HTM9:M | Yes, No. N/A |
| HTM10 | Power Save Multi-Poll (PSMP) | 7.4.7.10, 9.18.1.1 | CF15:O | Yes, No, N/A |
| *HTM10.1 | Scheduled PSMP | 9.18.1, 11.4.4A | HTM10:M | Yes, No, N/A |
| HTM10.1.1 | PSMP additions to TSPEC | 7.3.2.29, 11.4.4A.2 | HTM10.1:M | Yes, No, N/A |
| HTM10.1.2 | AP role in scheduled PSMP sequence | 9.18.1.1.1, 9.18.1.1.2 | (HTM10.1 AND CF1):M | Yes, No, N/A |
| HTM10.1.3 | STA role in scheduled PSMP sequence | 9.18.1.1.1, 9.18.1.1.2, 9.18.1.1.3 9.18.3.1 | (HTM10.1 AND CF2):M | Yes, No, N/A |
| *HTM10.2 | Unscheduled PSMP | 9.18.4, 11.4.4B | CF15:M | Yes, No, N/A |
| HTM10.2.1 | PSMP additions to TSPEC | 7.3.2.29, 11.4.4A.2 | HTM10.2:O | Yes, No, N/A |
| HTM10.3 | Creation, scheduling and transmission of PSMP Action frame | 7.4.7.10, 9.18.1.1 | (HTM10 AND CF1):M | Yes, No, N/A |
| HTM10.4 | Reception and interpretation of PSMP Action frame | 7.4.7.10 | (HTM10 AND CF2):M | Yes, No, N/A |
| HTM10.5 | Multiple TID Block ACK (MTBA) rules in PSMP sequence | 7.2.1.7.2, 7.2.1.8.2, 9.18.3, 11.5.4 | HTM10: M | Yes, No. N/A |
| HTM10.6 | Multi-phase PSMP |  |  |  |
| HTM10.6.1 | AP support for multi-phase PSMP | 7.1.3.5.6, 9.18.2 | (HTM10 AND CF1):O | Yes, No. N/A |
| HTM10.6.2 | STA support for multi-phase PSMP | 7.1.3.5.6, 9.18.2 | (HTM10 AND CF2):O | Yes, No. N/A |
| HTM11 | Link Adaptation |  |  |  |
| HTM11.1 | Use of HT Control field for Link Adaptation in immediate response exchange. | 7.1.3.8, 9.19.2, 9.19.3 | CF15:O | Yes, No, N/A |
| HTM11.2 | Link adaptation using explicit feedback mechanism | 7.2.2.2, 9.19.4 | CF15:O | Yes, No, N/A |
| HTM12 | Transmit Beam forming |  |  |  |
| *HTM12.1 | Transmission of beam formed PPDUs | 9.20 | CF15:O | Yes, No, N/A |
| *HTM12.2 | Reception of beam formed PPDUs | 9.20 | CF15:O | Yes, No, N/A |
| HTM12.3 | Initiate transmit beam forming frame exchange with implicit feedback | 9.20.2 | HTM12.1:O | Yes, No, N/A |
| HTM12.3.1 | Reception of sounding PPDUs | 9.20.2 | (HTM12.3 OR HTM13):M | Yes, No, N/A |
| HTM12.4 | Response to transmit beam forming frame exchange with implicit feedback | 9.20.2 | HTM12.2:O | Yes, No, N/A |
| HTM12.5 | Initiate transmit beam forming frame exchange with explicit feedback | 7.4.7.6, 7.4.7.7, 9.20.3 | HTM12.1:O | Yes, No, N/A |
| HTM12.5.1 | Transmission of sounding PPDUs | 9.20.3 | (HTM12.5 OR HTM13):M | Yes, No, N/A |
| HTM12.6 | Respond to transmit beam forming frame exchange with explicit feedback | 7.4.7.6, 7.4.7.7, 9.20.3 | HTM12.2:O | Yes, No, N/A |
| HTM12.6.1 | Transmission of QoS Null+_HTC frame including management action paylod of type CSI | 9.20.3 | HTM12.6:O.1 | Yes, No, N/A |
| HTM12.6.2 | Transmission of QoS Null+_HTC frame including management action paylod of type “uncompressed steering matrices” | 9.20.3 | HTM12.6:O.1 | Yes, No, N/A |
| HTM12.6.3 | Transmission of QoS Null+_HTC frame including management action paylod of type “compressed steering matrices” | 9.20.3 | HTM12.6:O.1 | Yes, No, N/A |
| *HTM12.7 | Calibration procedure | 7.2.2.2, 7.4.7.4, 7.4.7.5, 9.20.2.1 | HTM12:O | Yes, No, N/A |
| HTM13 | Antenna Selection | 7.1.3.8, 7.3.2.41.7, 7.4.7.9, 9.21 | CF15:O | Yes, No, N/A |
| *HTM14 | Zero Length Sounding Frame | 9.22 | CF15:O | Yes, No, N/A |
| HTM15 | Space-Time Block Coding (STBC) support |  |  |  |
| HTM15.1 | Secondary Beacon transmission | 11.1.2.1.1 | HTP2.11:O | Yes, No, N/A |
| HTM15.2 | Dual CTS protection | 7.3.2.42, 7.2.5.4.1 | HTP2.11:O | Yes, No, N/A |
| HTM16 | MIMO power save support |  |  |  |
| *HTM16.1 | AP Support for dynamic and static MIMO power save mode | 11.2.3.1 | CF15 AND CF1:M | Yes, No, N/A |
| HTM16.2 | STA Support for dynamic and static MIMO power save mode | 11.2.3.1 | CF15 AND CF2:O | Yes, No, N/A |
| HTM16.3 | Transmit MIMO Power Save state information using HT capabilities, or MIMO Power Save Action frame | 7.4.7.2, 11.2.3.1 | (HTM16.1 OR HTM16.2):M | Yes, No, N/A |
| HTM16.4 | Receive MIMO Power Save state information and support frame exchanges with MIMO Power Save STAs | 11.2.3.1 | CF15:M | Yes, No, N/A |
| HTM17 | Mechanisms for coexistence of 20MHz and 40MHz channels | 9.23 | CF15:M | Yes, No, N/A |
| HTM18 | Channel selection methods for 20/40MHz operation | 11.10.8 | (HTP2.3.4 AND CF1):M | Yes, No, N/A |
| HTM19 | 40/20MHz Operation | 11.15.1 | HTP2.3.4:M | Yes, No, N/A |
| HTM20 | Phased Coexistence Operation (PCO) |  |  |  |
| *HTM20.1 | PCO capability at AP | 11.16 | (CF15 AND CF1):O | Yes, No, N/A |
| HTM20.1.1 | Rules for operation at a PCO AP | 7.4.7.3, 11.16.1 | HTM20.1:M | Yes, No, N/A |
| HTM20.2 | STA support for PCO mode | 11.16 | (CF15 AND CF2):O | Yes, No, N/A |
| HTM20.2.1 | Rules for operation at PCO STA | 7.4.7.3, 11.16.2 | HTM20.2:M | Yes, No, N/A |
| HTM21 | Management Information Base (MIB) |  |  |  |
| HTM21.1 | dot11PhyHTComplianceGroup | Annex D | CF15:M | Yes, No, N/A |
| HTM21.2 | dot11PhyMCSGroup | Annex D | CF15:M | Yes, No, N/A |

#### A4.15.2 PHY Enhancements for Higher Throughput

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following PHY protocol enhancements supported? |  |  |  |
| HTP1 | PHY Operating Modes |  |  |  |
| HTP1.1 | Operation according to Clause 17 and Clause 19 | 20.1.3 | CF15:M | Yes, No, N/A |
| HTP1.2 | Mixed mode | 20.1.3 | CF15:M | Yes, No, N/A |
| *HTP1.3 | Green Field (GF) mode | 20.1.3 | CF15:O | Yes, No, N/A |
| HTP2 | PLCP frame format |  |  |  |
| HTP2.1 | Mixed mode PLCP format | 20.3.2 | CF15:M | Yes, No, N/A |
| HTP2.2 | Green Field PLCP format | 20.3.2 | HTP1.3:M | Yes, No, N/A |
| HTP2.3 | Modulation and Coding Schemes (MCS) |  |  |  |
| HTP2.3.1 | MCS 0 though MCS 7 in 20MHz with 800ns guard interval | 20.3.2.3, 20.7 | CF15:M | Yes, No, N/A |
| HTP2.3.2 | MCS 8 through MCS 15 in 20MHz with 800ns guard interval | 20.3.2.3, 20.7 | (CF15 AND CF1):M | Yes, No, N/A |
| HTP2.3.3 | Transmit and receive support for 400ns guard interval | 20.3.2.3, 20.7 | CF15:O | Yes, No, N/A |
| *HTP2.3.4 | Operation at 40MHz | 20.3.2.3, 2.7 | CF15:O | Yes, No, N/A |
| HTP2.3.5 | Support for MCS with indices 16 though 76 | 20.3.2.3, 20.7 | CF15:O | Yes, No. N/A |
| HTP2.4 | PHY timing parameters |  |  |  |
| HTP2.4.1 | Values in legacy 20MHz channel | 20.3.2.4 | CF15:M | Yes, No. N/A |
| HTP2.4.2 | Values in 20MHz HT channel | 20.3.2.4 | CF15:M | Yes, No. N/A |
| HTP2.4.3 | Values in 40MHz channel | 20.3.2.4 | HTP2.3.4:M | Yes, No. N/A |
| HTP2.5 | HT Preamble field definition and coding |  |  |  |
| HTP2.5.1 | Mixed mode preamble | 20.3.3.2 | CF15:M | Yes, No, N/A |
| HTP2.5.2 | Green Field preamble | 20.3.3.3 | HTP1.3:M | Yes, No. N/A |
| HTP2.6 | HT Data field definition and coding | 20.3.4 | CF15:M | Yes, No, N/A |
| HTP2.6.1 | Use of LDPC codes | 20.3.4.3.3 | CF15:O | Yes, No, N/A |
| HTP2.7 | Beam forming | 20.3.5 | CF15:O | Yes, No, N/A |
| HTP2.8 | Sounding PPDUs |  |  |  |
| HTP2.8.1 | HT preamble format for sounding PPDUs | 20.3.6 | CF15:O | Yes, No, N/A |
| HTP2.8.2 | Sounding with a zero length packet | 20.3.6.1 | HTM14:M | Yes, No, N/A |
| HTP2.8.3 | Sounding PPDU for calibration | 20.3.6.2 | HTM12.7:M | Yes, No, N/A |
| HTP2.9 | Channel numbering and channelization |  |  |  |
| HTP2.9.1 | Channel allocation for 20MHz channels at 5GHz | 17.3.8.3 | CF15:M | Yes, No, N/A |
| HTP2.9.2 | Channel allocation for 20MHz channels at 2.4GHz | 19.4.2 | CF15:M | Yes, No, N/A |
| HTP2.9.3 | Channel allocation for 40MHz channels at 5GHz | 20.3.8.1 | HTP2.3.4:M | Yes, No, N/A |
| HTP2.9.4 | Channel allocation for 40MHz channels at 2.4GHz | 20.3.8.2 | HTP2.3.4:M | Yes, No, N/A |
| HTP2.10 | PMD transmit specification |  |  |  |
| HTP2.10.1 | PMD transmit specification for 20MHz channel | 20.3.14 | CF15:M | Yes, No, N/A |
| HTP2.10.2 | PMD transmit specification for 40MHz channel | 20.3.14 | HTP2.3.4:M | Yes, No, N/A |
| HTP2.11 | Space-Time Block Coding (STBC) | 20.3.4.5.1 | CF15:O | Yes, No, N/A |

References: