IEEE P802.11
Wireless LANs

| Remote Request Broker Error Handling |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-02-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | Chantry Networks | 1900 Minnesota Ct, Suite 125, Mississauga, ON. CANADA L5N 3C9 | +1-905-363-6413 | michael.montemurro@siemens.com |

Abstract

This document updates the text in clause 7.3.1.9 and clause 8A.5 of IEEE 802.11r draft 1.1 with information on remote request broker error handling.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### 7.3.1.9 Remote request/response message definition

Modify the status code table as follows:

| Status | Code Meaning |
| --- | --- |
| 52 | Invalid Fast BSS Transition Action Frame Count |
| 53 | Expected a reservation to precede Fast BSS Transition |
| 54 | Invalid PMKID |
| 55 | Invalid EAPKIE |
| 56 | Invalid MDIE |
| 57 | Target AP Unreachable Remote Request Timeout |
| 58 | Request Denied for Policy Reasons |
| 59-65535 | Reserved |

Table 14—Presence of challenge text information elements Authentication Algorithm Authentication transaction sequence number Status Code Challenge Text Presence of fields 4- 12

### 8A.5.2 Remote Request Broker (RRB)

Change the paragraphs following Table 35B as follows:

The Remote Request Broker resides on the AP, and has two functions depending on its mode of operation:

- Respond to Remote Requests targeted to itself;

- Forward Remote Requests to the target AP across the DS.

If the Remote Request Broker at the target AP receives a remote request frame directed to itself from the current  AP, it will interact with the MAC SME to process the request and respond with a remote response frame, through the Remote Request Broker on the current AP, back to the requesting STA.

If the Remote Request Broker at the current AP receives a request from a STA directed to another AP in the same Mobility Domain (and therefore supporting "over-the-DS" communications) the AP will forward the Request to that target AP. The AP will track the Request until it receives a Response. The AP will maintain a timer while it waits for the Response. If no response is received in the dot11FTRemoteRequestTimeout period, the AP shall respond with an FT Action frame to the STA with the status code “Remote Request Timeout” (see clause 7.3.1.9), indicating that the remote request has timed out with no response from the target AP.

If the Remote Request Broker receives a request from a STA that either exceeds the dot11FTPendingRemoteRequestLimit, or is directed to a target AP that is not part of the Mobility Domain, the Remote Request Broker shall respond with an FT Action frame response with the status code set to the value “Request Denied for Policy Reasons” (see clause 7.3.1.9).

The target AP and the current AP need to reside in the same mobility domain to successfully exchange Remote Request frames. The RRB on the current AP shall transmit Remote Request frames to the target AP based on the BSSID of the target AP supplied in the FT Action frames using the same procedures as pre-authentication, as described in clause 8.4.6.1.