IEEE P802.11
Wireless LANs

| LB78 Section 7-4 Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-FEB-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tim Olson | Cisco Systems | 170 West Tasman Dr. San Jose, CA 95137 | 408-853-4527 | tolson@cisco.com |

Abstract

This document contains normative text changes for letter ballot 78 comments for section 7.4.  This document resolves comments: 364, 940, 1377, 1437, 365, 1378, 89, 117, 368, 369, 426, 427, 582, 583, 744, 1069, 1284, 1285, 1381, 1382.

The submission is referenced to draft D3.2 and replaces all editor instructions as well as actual base draft modification.  The change markings in this text identify the changes to be made against the D3.2 draft.

All new text relative to the 802.11 base is identified by underlining and all deleted text is identified by strikethrough.

#### 7.4.5.1 Radio Measurement Request frame format

The Radio Measurement Request frame uses the Action frame body format. It is transmitted by a STA requesting another STA to make one or more measurements on one or more channels. The format of the frame is shown in Figure k1.

|  | Category | Action | Dialog Token | Number of Repetitions | Measurement Request Elements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | variable |

Figure k1—Radio Measurement Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to indicate a Measurement Request according to Error: Reference source not found in 7.4.5.

The Dialog Token field shall be set equal to a non-zero value chosen by the STA sending the radio measurement request to identify the request/report transaction.

The Number of Repetitions field contains the requested number of repetitions for all the Measurement Request elements in this frame. A value of zero in the Number of Repetitions field indicates Measurement Request elements are executed once without repetition.  A value of 0xffff in the Number of Repetitions field indicates Measurement Request elements are repeated until the measurement is cancelled.

The Measurement Request Elements field shall contain zero or more of the Measurement Request elements described in 7.3.2.21. The number and length of the Measurement Request elements in a Measurement Request frame is limited by the maximum allowed MMPDU size.

#### 7.4.5.2 Radio Measurement Report frame format

The Measurement Report frame uses the Action frame body format. It is transmitted by a STA in response to a Measurement Request frame or by a STA providing a triggered autonomous measurement report. The format of the frame is shown in Figure k2.

|  | Category | Action | Dialog Token | Measurement Report Elements |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure k2—Measurement Report frame body format

The Category field shall be set to indicate the Radio Measurement category according to Table 19a.

The Action field shall be set to indicate a Measurement Report according to Error: Reference source not found.

The Dialog Token field shall be set equal to the value in the corresponding Measurement Request frame. If the Radio Measurement Report frame is not being transmitted in response to a Radio Measurement Request frame then the Dialog token shall be set equal to zero.

The Measurement Report Elements field shall contain one or more Measurement Report elements described in 7.3.2.22. The number and length of the Measurement Report elements in a single Radio Measurement Report frame is limited by the maximum allowed MMPDU size.  Section 11.11.6 describes the required behavior for multi-frame Radio Measurement Report frame responses.

#### 7.4.5.5 Neighbor Report Request frame format

The Neighbor Report Request frame uses the Action frame body format and is transmitted by a STA requesting information in the Neighbor Report about neighboring AP’s. The format of the Neighbor Report Request frame body is shown in Figure k3.

|  | Category | Action | Dialog Token | Neighbor Report Request Types | SSID element (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 2-34 |

Figure k3—Neighbor Report Request frame body format

The Category field shall be set to the value indicating the Radio Measurement category, as specified in Table 19a in 7.3.1.11.

The Action field shall be set to the value indicating Neighbor Report Request, as specified in Error: Reference source not found in 7.4.5.

The Dialog Token field shall be set to a non-zero value chosen by the STA sending the measurement request to identify the request/report transaction.

The Neighbor Report Request Types field shall be one octet in length and shall contain the subfields as shown in Figure k4.

|  | B0 | B1 B7 |
| --- | --- | --- |
|  | Neighbor TSF offset Request | Reserved |
| Bits: | 1 | 7 |

Figure k4—Neighbor Report Request Types Subfield

Neighbor TSF offset Request – This bit is set to 1 to request TSF offset information be provided in neighbor list entries if available. When this bit is set to 0 the TSF Info field shall not be included in any neighbor list entries.

All other bits are reserved and shall be set to 0 on transmission and ignored on reception.

The SSID element is defined in 7.3.2.1. An SSID may optionally be included in a Neighbor Report Request frame to indicate a request for a neighbor list for the specified SSID in the SSID Element. The absence of a SSID element indicates neighbor report for the current ESS.  Inclusion of the SSID element is determined by the evaluating the length of the Neighbor Report Request frame.