IEEE P802.11
Wireless LANs

| LB78 Neighbor Report Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-02-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tim Olson | Cisco Systems, Inc. | 170 Tasman Dr San Jose, Ca | 408-853-4527 | tolson@cisco.com |
| Bernard Aboba | Microsoft Corp | One Microsoft Way Redmond, Wa 98052 | 425-706-6605 | bernarda@microsoft.com |

Abstract

This document is intended to be normative text for clause 7.3.2.27 to be added by the editor.  The intent of the document is to use Microsoft Word’s “track changes” from the tools menu to convey the changes to be made.

The changes made in this document address gracefully handling future upgrades of the Neighbor Report.  Comments 84, 576, 1274.

The source for the draft amendment was the word version in the group’s directory on the 802.11 web site with a file name of P802.11k-D3.2.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

#### 7.3.2.27 Neighbor Report element

The format of the Neighbor Report element is shown in Figure k1.

|  |  |  |  |  |  |  |  | Zero or more optional extensions |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | BSSID | BSSID information | Regulatory Class | Channel Number | PHY Type | Extensions |  |
| Octets: | 1 | 1 | 6 | 4 | 1 | 1 | 1 | Variable |  |

Figure k1—Neighbor Report element format

The Element ID field is set to the Neighbor Report value in Table 20.

The value of Length field is dependent on the number and length ofoptional extensions. Each Neighbor Report element describes an AP and consists of BSSID, BSSID Information, Regulatory Class, Channel Number, PHY Type, and optionally may include extensions. The minimum value of the Length field is 13 (i.e., with no optional extensions in the Neighbor Report element).

.

The BSSID is the BSSID of the BSS being reported. The subsequent fields in the Neighbor Report element pertain to this BSS.

The BSSID Information field may be used to help determine neighbor BSS transition candidates. It shall be four octets in length and shall contain the subfields as shown in Figure .

|  | B0 | B1 | B2 | B3 | B4 | B9 | B10 | B31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AP Reachability |  | Security | Authenticator | Capabilities |  | Reserved |  |
| Bits: | 2 |  | 1 | 1 | 6 |  | 22 |  |

Figure k32—BSSID information field

The AP Reachability field indicates whether the AP identified by this BSSID is reachable by the STA that requested the Neighbor Report.  For example the AP identified by this BSSID is reachable for the exchange of preauthentication frames as described in clause 8.4.6.1. The values are shown in Table k1.

Table k1—Reachability field

| Value | Reachability | Usage |
| --- | --- | --- |
| 0 | Reserved | n/a |
| 1 | Not Reachable | A station sending a preauthentication frame to the BSSID will not receive a response even if the AP represented by the BSSID is capable of preauthentication. |
| 2 | Unknown | The AP is unable to determine if the value Reachable or Not Reachable is to be returned. |
| 3 | Reachable | The station sending a preauthentication frame to the BSSID can receive a response from an AP that is capable of preauthentication. |

The Security bit, if set to 1 it, indicates that the AP identified by this BSSID supports the current security association of the STA . If the bit is set to 0, it indicates that either the AP can only provide less security than the current association, as defined by clause 8, or the information is not available to the AP at this time.

The Authenticator bit, when set to 1, indicates the AP represented by this BSSID has the same authenticator as the AP sending the report. If this bit is  set to 0, it indicates a distinct authenticator or the information is not available.

The Capabilities Subfield contains selected capability information for the AP represented by this BSSID. The bit fields within this subfield shall have the same meaning and be set equal to the equivalent bits within the Capability Information field being sent in the beacons by the AP being reported. The format of the Capabilities subfield is as in Figure 3.

|  | B4 | B5 | B6 | B7 | B8 | B9 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Spectrum Management | QoS | APSD | Radio Measurement | Delayed Block Ack | Immediate Block Ack |
| Bits: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure k33—Capabilities Subfield

Bits 10 – 31 are reserved and are ignored on reception.

Channel Number indicates the current operating channel of the AP represented by the BSSID in this Neighbor Report element.

Regulatory Class contains an enumerated value from Error: Reference source not found specifying the frequency band in which the Current Channel is valid.

Channel Number indicates the last known operating channel of the AP represented by the BSSID in this neighbor list entry.

The PHY 
Type field indicates the PHY type of the AP being reported.  It shall have an integer value coded according to the value of the dot11PHYType.. Figure k5.

The format of the optional extensions is shown in Figure k34.

|  | Sub-Element ID | Length | Data |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable |

Figure k34—Sub-Element format

The values of the Sub-Element ID field are shown in Table k11.

The value of Length field is dependent on the length of the sub-element..

Table k11—Optional Extensions

| Sub-Element ID | Name |
| --- | --- |
| 0 | Reserved |
| 1 | TSF Information |

The TSF Information sub-element contains TSF Offset and Beacon Interval subfields

|  | TSF Offset | Beacon Interval |
| --- | --- | --- |
| Octets: | 2 | 2 |

Figure k6—TSF Information sub-element data field format

The value of the TSF Information sub-element length field is 4.

The TSF Offset subfield is 2 octets long and contains the neighbor AP’s TSF timer offset. This is the time difference, in TU units, between the serving AP and a neighbor AP. This offset is given modulo the neighbor AP’s Beacon Interval and rounded to the nearest TU boundary.

The Beacon Interval subfield is the beacon interval of the Neighbor AP being reported. Thissub field is defined in clause 7.3.1.3 and illustrated in Figure 26.