IEEE P802.11
Wireless LANs

| Submission to address TGn draft omissions in clause 7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-02-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sanjiv Nanda | Qualcomm, Inc. | 5755 Morehouse Drive San Diego, CA 92121 |  | snanda@qualcomm.com |

Abstract

This document contains proposed changes to the TGn Draft to address omissions from the document.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Omissions Covered

This document addresses the following omissions identified in document 11-06-0263-00-000n-tgn-draft-omissions.xls:

- OM023 through OM029.

- OM037 and OM038.

This document defines the order of insertion of HT Capability information element and HT Additional Information element in beacon, association request/response, reassociation request/response, probe request/response and DLS request/response frames.

We assume that 11r draft 1.0 defines the IE order values consumed by 11r and earlier amendments thus far. Therefore, we define the order values for the IEs for TGn appropriately following those values already consumed by 11r and earlier amendments.

## Interpretation of a Motion to Adopt

A motion to adopt the changes defined in this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, and OMnnn headings are not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

# OM023

      - Beacon frame format

Insert the following text at the end of Section 7.2.3.1:

If the dot11HTCapabilityImplemented attribute is true, a STA shall include a HT Capability information element in the transmission of Beacon frames. In addition, the AP in an infrastructure BSS shall also include the Additional HT Information Element. The HT Capabilities information element and the Additional HT Information element are included in the order shown in Table 8.

TGn Editor:  Add the following additional rows in Table 8 and renumber the Vendor Specific IE to 32:

| Order | Information | Notes |
| --- | --- | --- |
| 30 | HT Capabilities | The HT Capabilities information element shall be present when dot11HTCapabilityImplemented attribute is true |
| 31 | Additional HT Information | The Additional HT Information element shall be included by an AP when dot11HTCapabilityImplemented attribute is true |

# OM024

      - Association Request frame format

Insert the following text at the end of Section 7.2.3.4:

If the dot11HTCapabilityImplemented attribute is true, a STA shall include a HT Capability information element in the transmission of Association Request frames.

TGn Editor:  Add the following additional row in Table 10 and renumber the Vendor Specific IE to 16:

| Order | Information | Notes |
| --- | --- | --- |
| 15 | HT Capabilities | The HT Capabilities information element shall be present when dot11HTCapabilityImplemented attribute is true |

# OM025

      - Association Response frame format

Insert the following text at the end of Section 7.2.3.5:

If the dot11HTCapabilityImplemented attribute is true, a STA shall include a HT Capability information element in the transmission of Association Response frames. In addition, the AP in an infrastructure BSS shall also include the Additional HT Information Element. The HT Capabilities information element and the Additional HT Information element are included in the order shown in Table 11.

TGn Editor:  Add the following additional rows in Table 11 and renumber the Vendor Specific IE to 17:

| Order | Information | Notes |
| --- | --- | --- |
| 15 | HT Capabilities | The HT Capabilities information element shall be present when dot11HTCapabilityImplemented attribute is true |
| 16 | Additional HT Information | The Additional HT Information element shall be included by an AP when dot11HTCapabilityImplemented attribute is true |

# OM026

      - Reassociation Request frame format

Insert the following text at the end of Section 7.2.3.6:

If the dot11HTCapabilityImplemented attribute is true, a STA shall include a HT Capability information element in the transmission of Reassociation Request frames.

TGn Editor:  Add the following additional row in Table 12 and renumber the Vendor Specific IE to 17:

| Order | Information | Notes |
| --- | --- | --- |
| 16 | HT Capabilities | The HT Capabilities information element shall be present when dot11HTCapabilityImplemented attribute is true |

# OM027

      - Ressociation Response frame format

Insert the following text at the end of Section 7.2.3.7:

If the dot11HTCapabilityImplemented attribute is true, a STA shall include a HT Capability information element in the transmission of Reassociation Response frames. In addition, the AP in an infrastructure BSS shall also include the Additional HT Information Element. The HT Capabilities information element and the Additional HT Information element are included in the order shown in Table 13.

TGn Editor:  Add the following additional rows in Table 13 and renumber the Vendor Specific IE to 17:

| Order | Information | Notes |
| --- | --- | --- |
| 15 | HT Capabilities | The HT Capabilities information element shall be present when dot11HTCapabilityImplemented attribute is true |
| 16 | Additional HT Information | The Additional HT Information element shall be included by an AP when dot11HTCapabilityImplemented attribute is true |

# OM028

      - Probe Request frame format

Insert the following text at the end of Section 7.2.3.8:

If the dot11HTCapabilityImplemented attribute is true, a STA shall include a HT Capability information element in the transmission of Probe Request frames.

TGn Editor:  Add the following additional row in Table 14 and renumber the Vendor Specific IE to 7:

| Order | Information | Notes |
| --- | --- | --- |
| 6 | HT Capabilities | The HT Capabilities information element shall be present when dot11HTCapabilityImplemented attribute is true |

# OM029

      - Probe Response frame format

Insert the following text at the end of Section 7.2.3.9:

If the dot11HTCapabilityImplemented attribute is true, a STA shall include a HT Capability information element in the transmission of Probe Response frames. In addition, the AP in an infrastructure BSS shall also include the Additional HT Information Element. The HT Capabilities information element and the Additional HT Information element are included in the order shown in Table 15.

TGn Editor:  Add the following additional rows in Table 15 and renumber the Vendor Specific IE to 29 and the Requested IEs to 30-n:

| Order | Information | Notes |
| --- | --- | --- |
| 27 | HT Capabilities | The HT Capabilities information element shall be present when dot11HTCapabilityImplemented attribute is true |
| 28 | Additional HT Information | The Additional HT Information element shall be included by an AP when dot11HTCapabilityImplemented attribute is true |

# OM037

      - DLS Request frame format

Insert the following text at the end of Section 7.4.3.1:

If the dot11HTCapabilityImplemented attribute is true, a STA shall include a HT Capability information element in the transmission of DLS Request frames.

TGn Editor:  Add the following additional row in Table 51:

| Order | Information | Notes |
| --- | --- | --- |
| 9 | HT Capabilities | The HT Capabilities information element shall be present when dot11HTCapabilityImplemented attribute is true |

# OM038

      - DLS Response frame format

Insert the following text at the end of Section 7.4.3.2:

If the dot11HTCapabilityImplemented attribute is true, a STA shall include a HT Capability information element in the transmission of DLS Response frames.

TGn Editor:  Add the following additional row in Table 52:

| Order | Information | Notes |
| --- | --- | --- |
| 9 | HT Capabilities | The HT Capabilities information element shall be present when dot11HTCapabilityImplemented attribute is true |