IEEE P802.11
Wireless LANs

| 40MHz channels switching |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-02-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Solomon Trainin | Intel Corporation |  |  | solomon.trainin@intel.com |
| Adrian P Stephens | Intel Corporation |  |  | adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the TGn Draft to address omissions from the document related to channel switching under DFS

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Omissions Covered

This document addresses the following omissions identified in document 11-06-0263-00-000n-tgn-draft-omissions.xls: OM034 OM036 OM070 OM071

# OM034 OM036 OM070 OM071

Insert the Element ID assigned by the 802.11 ANA below

Add Element ID of New Extension Channel Offset element to the Table 26— Element IDs

Add the following sublause after 7.3.2.20:

7.3.2.20A  New Extension Channel Offset element

The New Extension Channel Offset element is used by an AP in a BSS or a STA in an IBSS together with Channel Switch Announcement element when changing to a new 40MHz channel. The format of the   New Extension Channel Offset element is shown in Figure n75.

|  | Element ID | Length | New Extension Channel offset |
| --- | --- | --- | --- |
| Octets | 1 | 1 | 1 |

Figure n75— New Extension Channel Offset element format

The Length field shall be set to 1.

New Extension Channel offset represents the position of the extension channel relatively to the control channel. The New Extension Channel offset set to 1 means that the extension channel is above the control channel; a value 3 represents extension channel below the control channel; value 0 indicates that no extension channel is present.

The New Extension Channel Offset element is included in Channel Switch Announcement frames, as described in 7.4.1.5, and may be included in Beacon frames, as described in 7.2.3.1, and Probe Response frames, as described in 7.2.3.9.

7.2.3.1 Beacon frame format

Append the following row to the Table 8—Beacon frame body

| TBD | Extension Channel Offset | New Extension Channel Offset element may be present if dot11SpectrumManagementRequired is true and the BSS/IBSS is 40/20MHz capable |
| --- | --- | --- |

7.2.3.9 Probe Response frame format

Append the following row to the Table 15—Probe Response frame body

| TBD | New Extension Channel Offset | New Extension Channel Offset element may be present if dot11SpectrumManagementRequired is true and the BSS/IBSS is 40/20MHz capable |
| --- | --- | --- |

7.4.1.5 Channel Switch Announcement frame format

Change the Figure 117 following way:

| Category | Action Value | Channel Switch Announcement element | New Extension Channel Offset element |
| --- | --- | --- | --- |

Octets		1		1			5			3

Figure 117— Channel Switch Announcement frame body format

Append the subcaluse with following sentence:

The New Extension Channel Offset element shall be set as described in 7.3.2.20A. This element shall be present when switching to a 40MHz channel.  It may be present when switching to a 20MHz channel (in which case the extension channel offset is set to 0).

10.3.15.1.2 Semantics of the service primitive

Text as follows:

The primitive parameters are as follows:

MLME-CHANNELSWITCH.request(

Mode,

Channel Number,

New Extension Channel Offset

Channel Switch Count

)

10.3.15.1.3 When generated

Change the subclause

This primitive is generated by the SME to schedule a channel switch and announce this switch to peer entities in the BSS.

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Mode | Integer | 0, 1 | Channel switch mode, as defined for the Channel Switch Announcement element. |
| Channel Number | Integer | As defined in 17.3.8.3.3 | Specifies the new channel number. |
| New Extension Channel Offset | Integer | 0, 1, 3 | Specifies the position of extension channel in relation to the control channel. The 40/20 capable BSS/IBSS shall insert this parameter. |
| Channel Switch Count | Integer | 0–255 | Specifies the number of TBTTs until the channel switch event, as described for the Channel Switch Announcement element. |

The parameter of New Extension Channel Offset may be present for the HT Stations.

10.3.15.3.1 Function

Make the following changes:

MLME-CHANNELSWITCH.indication(

Peer MAC Address,

Mode,

Channel Number,

New Extension Channel Offset,

Channel Switch Count

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the Measurement Report frame was received. |
| Mode | Integer | 0, 1 | Channel switch mode, as defined for the Channel Switch Announcement element. |
| Channel Number | Integer | As defined in 17.3.8.3.3 | Specifies the new channel number. |
| New Extension Channel Offset | Integer | 0, 1, 3 | Specifies the position of extension channel in relation to the control channel. The 40/20 capable BSS/IBSS shall insert this parameter. |
| Channel Switch Count | Integer | 0–255 | Specifies the number of TBTTs until the channel switch event, as described for the Channel Switch Announcement element. |

10.3.15.4.2 Semantics of the service primitive

Make the following changes:

MLME-CHANNELSWITCH.response(

Mode,

Channel Number,

New Extension Channel Offset

Channel Switch Count

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Mode | Integer | 0, 1 | Channel switch mode, as defined for the Channel Switch Announcement element. |
| Channel Number | Integer | As defined in 17.3.8.3.3 | Specifies the new channel number. |
| New Extension Channel Offset | Integer | 0, 1, 3 | Specifies the position of extension channel in relation to the control channel. The 40/20 capable BSS/IBSS shall insert this parameter. |
| Channel Switch Count | Integer | 0–255 | Specifies the number of TBTTs until the channel switch event, as described for the Channel Switch Announcement element. |