IEEE P802.11
Wireless LANs

| Submission to address TGn draft omissions mainly in clause 5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-03-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Daqing Gu | Mitsubishi Electric | 201 Broadway Cambridge, MA 02139 | 617-621-7543 | dgu@merl.com |
| Srinivas Kandala | Airgo Networks |  |  | srini@airgonetworks.com |
| Sanjiv Nanda | Qualcomm, Inc. |  |  | snanda@qualcomm.com |

Abstract

This document contains proposed changes to the TGn Draft to address omissions mainly in clause 5.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Omissions Covered

This document addresses the following omissions identified in document 11-06-0263-00-000n-tgn-draft-omissions.xls:

OM002, OM003, OM004, OM005, OM006, OM140

## Interpretation of a Motion to Adopt

A motion to adopt the changes defined in this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, and OMnnn headings are not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or set in context the response to the omission.

# OM002

5.3 Logical service interfaces

Change paragraph 3 of 5.3 as follows:

The complete set of IEEE 802.11 architectural services are as follows:

a) Authentication

b) Association

c) Deauthentication

d) Disassociation

e) Distribution

f) Integration

g) Data confidentiality

h) Reassociation

i) MSDU delivery

j) DFS

k) TPC

l) Higher layer timer synchronization (QoS facility only)

m) QoS traffic scheduling (QoS facility only)

n)	Radio measurement

o) Link adaptation

p) Transmit beamforming

q) Antenna selection

# OM003

5.3.1 SS

Change paragraph 3 of 5.3.1 as follows:

The SS is as follows:

a) Authentication

b) Deauthentication

c) Data confidentiality

d) MSDU delivery

e) DFS

f) TPC

g) Higher layer timer synchronization (QoS facility only)

h) QoS traffic scheduling (QoS facility only)

i) Radio measurement

j) Link adaptation

k) Transmit beamforming

l) Antenna selection

# OM004

5.4 Overview of the services

Change paragraph 1 of 5.4 as follows:

There are many services specified by IEEE 802.11. Six of the services are used to support medium access control (MAC) service data unit (MSDU) delivery between STAs. Three of the services are used to control IEEE 802.11 LAN access and confidentiality. Three of the services are used to provide link management. Two of the services are used to provide spectrum management. One of the services provides support for LAN applications with QoS requirements. One of the services provides support for higher layer timer synchronization.

# OM005

Insert the following new subclsuese after 5.4.8.2

5.4.9 Radio link management

Three services are provided to manage radio links and improve the link performance. These three services are link adaptation, transmit beamforming and antenna selection.

5.4.9.1 Link adaptation

Feedback based link adaptation allows the transmitter to select a modulation and coding scheme to optimize link performance. As the radio channel changes dynamically, the receiving STA provides feedback to the transmitting STA to allow the transmtting STA to select the modulation and coding scheme to maximize radio link performance.

5.4.9.2 Transmit beamforming

MIMO technology is used to meet the high throughput requirements. Combining transmit beamforming with MIMO can achieve higher throughput than basic MIMO at the same SNR, or conversely, higher range at the same data rate. Transmit beamforming provides robust link performance and improved system performance.

5.4.9.3 Antenna selection

Antenna selection is a possibly time-variant mapping of the signals at the RF chains onto a set of antenna elements, when the number of RF chains is smaller than the number of antenna elements at a STA. The mapping can be chosen based on instantaneous or averaged channel state information. It is a diversity technique to improve the link performance.

# OM006

5.6 Relationships between services

a) 	Class 1 frames (permitted from within States 1, 2, and 3):

b) Class 2 frames (if and only if authenticated; allowed from within States 2 and 3 only):

c) Class 3 frames (if and only if associated; allowed only from within State 3):

Change the following paragraph as follows:

1) 	Data frames

i) 	Data subtypes: Data frames allowed. That is, either the “To DS” or “From DS” FC bits may 			be set to true to utilize the DSS.

ii) 	QoS data subtypes allowed to/from non-AP QSTA(s) that are associated with QAP(s).

iii) Data frames between QSTAs in a QBSS with FC bits “To DS” and “From DS” both false.

iv)	Zero length farme (ZLF).

Change the following paragraph as follows:

2)	Management frames

i)	Deauthentication: Deauthentication notification when in State 3 implies disassociation as 				well, changing the STA’s state from 3 to 1. The station shall become authenticated again 				prior to another association.

ii)	QoS, DLS and Block Ack Action

iii)	Radio Measurement Action

iv)	HT action frames

# OM0140

Insert the following new subclause after 5.2.6

Submission note: the author notices that the subclause number 5.2.6 is used in both documents of IEEE P802.11-REVma/D5.1 and IEEE P802.11k/D3.2, it is expected that one of them will be changed to 5.27. So, the new subclaue will be labeled as 5.2.8.

5.2.8 HT STA

The IEEE 802.11 HT capabilities and features provide MAC and PHY enhancements to support LAN applications with high throughput requirements. The enhancements that distinguish HT STAs from non-HT STAs and HT APs from non-HT APs are collectively termed as the HT features. These HT features are listed in Annex A PICS in this amendement. Some of these HT features are mandatory, and some are optional. A STA with at least all the mandatory HT features is called a HT STA.  Since the PICS also require support for the QoS features, an HT STA is also a QSTA.