IEEE P802.11
Wireless LANs

| Broadcast and Multicast Enhancements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-03-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jari Jokela | Nokia |  | +358504860445 | jari.jokela @nokia.com |
| Jarkko Kneckt | Nokia |  |  | Jarkko.kneckt @nokia.com |
| Mikko Jaakkola | Nokia |  |  | Mikko Jaakkola @nokia.com |
| Simon Black | Nokia |  |  | EXT-Simon.3.Black@nokia.com |

Abstract

This submission contains the normative text related to broadcast and multicast enhancements.  Specifically the intention is to improve terminals power efficiency (Req. #2010) and provide means to improve the reliability (related to Req. #2070) of the broadcast and multicast services. Furthermore it contains aspects related to advertisements (Req. #1300).

This document is a new revision of document 05/1074r0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

[Insert the following to appropriate place in 2]

IETF RFC 1027, Using ARP to Implement Transparent Subnet Gateways, October 1987

[Make the following changes to 7.2.3.1]

##### 7.2.3.1 Beacon frame format

[Insert the following rows in Table 5]

| Order | Information | Notes |
| --- | --- | --- |
| TBD | Wireless Network Management Capability | Wireless Network Management Capability is present if dot11WirelessManagementImplemented is true |
| TBD | Multicast Service Info | Multicast Service Info is present if dot11WirelessManagementImplemented is true |

[Make the following changes to 7.2.3.9]

##### 7.2.3.9 Probe Response

[Insert the following rows in Table 12]

| Order | Information | Notes |
| --- | --- | --- |
| TBD | Wireless Network Management Capability | Wireless Network Management Capability is present if dot11WirelessManagementImplemented is true |
| TBD | Multicast Service Info | Multicast Service Info is present if dot11WirelessManagementImplemented is true |

##### 7.3.1.9 Status Code field

[Insert the following row in Table 19]

| TBD | Multicast Service Setup denied |
| --- | --- |

[Make the following changes to 7.3.2]

### 7.3.2 Information elements

[Insert the following row into Table 26]

| Information element | Element ID |
| --- | --- |
| Multicast Service Info | x+5 |
| Reserved | x+6, 220 |

[Make the following changes to 7.3.2.35]

[Replace Figurev2 with the following]

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Event Log | Diagnostics | Service Mode | Flexible Multicast Service Interval | Support Proxy ARP | Multicast ACK (informative) | Reserved |  |
| Bits: | 1 | 1 | 1 | 1 | 1 | 1 | 10 |  |

Figure v1—Wireless Network Management Capabilities

- The Event Log bit set to 1 indicates the STA supports Event Log as described in 11.1.5.2. The Event Log bit set to 0 indicates that the STA does not support this service.

- The Diagnostics bit set to 1 indicates the STA supports Diagnostics as described in 11.1.5.3. The Diagnostics bit set to 0 indicates that the STA does not support this service.

- The Service Mode bit set to 1 indicates the AP supports transmission of broadcast and multicast traffic as unicast frames in MAC level as described in 9.2.7.1.2. The Service Mode bit set to 0 indicates the AP does not support transmission of broadcast and multicast traffic as unicast frames in MAC level. The Service Mode bit set to 1 indicates the STA supports broadcast/multicast to unicast conversion. A STA may set the Service Mode bit to 1 only if AP supports the operation. The Service Mode bit set to 0 indicates the STA does not support broadcast/multicast to unicast conversion.

- The Flexible Multicast Service Interval bit set to 1 indicates the AP supports flexible multicast service intervals as described in section 11.2.1.2. The Flexible Multicast Service Interval bit set to 0 indicates the AP does not support flexible multicast service intervals. In this case the normal DTIM listen interval will be used for the multicast service. The Flexible Multicast Service Interval bit set to 1 indicates the STA supports flexible multicast service intervals as described in section 11.2.1.2.  The  Flexible Multicast service interval bit set to 0 indicates the STA does not support flexible multicast service intervals as described in section 11.2.1.2. In this case the normal DTIM listen interval is used for the multicast service.

- The Suppport Proxy ARP bit set to 1 indicates the AP supports Proxy ARP operation for the non-AP STA that have associated to it as described in section 9.2.7.1.4. The Suppport Proxy ARP bit set to 0 indicates the AP does not support Proxy ARP operation.

- The Multicast ACK bit set to 1 indicates the AP supports 802.11n multicast aggregation scheme as described in section 9.2.7.1.3. The Multicast ACK bit set to 0 indicates the AP does not support 802.11n multicast aggregation scheme as described in section 9.2.7.1.3. The Multicast ACK bit set to 1 indicates the STA supports 802.11n multicast aggregation scheme as described in section 9.2.7.1.3. The Multicast ACK bit set to 0 indicates the STA does not support 802.11n multicast aggregation scheme as described in section 9.2.7.1.3.

- All other bits are reserved, and are set to 0 on transmission and ignored on reception.

The lack of a Wireless Network Management Capability element is interpreted as the STA having no advertised Wireless Network Management Capabilities.

[Insert the following after 7.3.2.39]

##### 7.3.2.40 Multicast Service Info information element

The Multicast Service Info gives information about the active multicast groups within the BSS and the broadcast service. Active multicast groups includes all the multicast groups for which one or more non-AP STAs has completed the multicast service setup procedure.

|  | Element ID | Length | Broadcast Group Info | Multicast Group Count | Multicast Group Information | … | Multicast Group Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 8 | … | 8 |

Figure b. Multicast Service Info information element.

The Length field shall be set to 2+n*8, where n indicates the number of active multicast groups.

The Multicast Group Count shall indicate the number of active multicast groups within the BSS. If no active multicast groups are present in the BSS the Multicast Group Count shall be set to 0.

The Broadcast Group Info field is as shown in Figure d.

|  | Broadcast to Unicast | Broadcast Service Mode Count | Buffered Broadcast Data |
| --- | --- | --- | --- |
| Bits: | 1 | 6 | 1 |

Figure c. Broadcast Group Information field

The Broadcast to Unicast bit set to 1 indicates the AP is having the conversion from broadcast to unicast active. If active, the AP shall send all broadcast frames as unicast frames.

The Broadcast Service Mode Count field indicates how many TBTTs (including the current one) appears before the service mode of the broadcast will be changed. In case of immediate change this field is set to 0. In case of no change is planned this field is set to 63.

The Buffered Broadcast Data bit set to 1 indicates the AP has one or more buffered broadcast frames.  The Buffered Broadcast Data bit shall be set to 0 by the AP if the Broadcast to Unicast bit is set to 1.

The Multicast Group Information field is as shown in Figure e.

|  | Multicast Address | Multicast Service Interval Mode | Multicast Service Interval | Multicast Service Interval Count | Unicast Only | Buffered Traffic Indication |
| --- | --- | --- | --- | --- | --- | --- |
| Bits: | 48 | 1 | 7 | 6 | 1 | 1 |

Figure d. Multicast Group Information field

The Multicast Address field indicates the MAC address of the multicast group.

The Multicast Service Interval Mode set to 1 indicates if the Multicast Service Interval field is expressed in Beacon Intervals. The Multicast Service Interval Mode set to 0 indicates if the Multicast Service Interval field is expressed in TUs.

The Multicast Service Interval field indicates the proposed multicast service interval for this particular multicast service. The proposed multicast service interval can be multiple of Beacon intervals or number of TUs depending on the Multicast Service Interval Mode bit.

The Multicast Service Interval Count field indicates how many TBTTs (inlcuding the current one) appears before the next service interval for this multicast group. When the Multicast Service Interval is shorter than beacon interval this field is set to 63.

The Unicast Only bit is set to 1 if all STAs in BSS support multicast enhancements and multicast traffic is delivered only by unicast transmissions. If the Unicast Only bit is set, the Multicast service Interval Mode, Multicast service Interval, and Buffered Traffic Indication fields are reserved.

The Buffered Traffic Indication field indicates whether there are buffered multicast frames for this particular multicast group.  The Buffered Traffic Indication field shall be set to 0 by the AP if the Unicast Only bit is set to 1.

### 7.4.6 Wireless Network Management action details

[Insert the following rows to table v7]

Table v1—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 4 | Multicast Service Setup Request |
| 5 | Multicast Service Setup Response |
| 6 | Multicast Service Termination Request |
| 7 | Multicast Service Termination Response |
| 8 | Multicast Service Change |
| 9-255 | Reserved |

[Insert the following after 7.4.6.4]

##### 7.4.6.5 Multicast Service Setup Request frame format

Multicast Service Setup Request frame is sent by a STA.

|  | Category | Action | Multicast MAC Address | Service Parameters |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 2 |

Figure e. Multicast Service Setup Request frame format.

The Category field shall be set to 6.

The Action field shall be set to 0.

The Multicast MAC Address indicates the multicast group to which the request is intended.

The Service Parameters field is as show in Figure g.

|  | Multicast Service Interval Mode | Multicast Service Interval | Service Mode | Reserved |
| --- | --- | --- | --- | --- |
| Bits: | 1 | 7 | 1 | 6 |

Figure f. Service Parameters field.

The Multicast Service Interval Mode bit shall be set to 1 if the Multicast Service Interval field is expressed in Beacon Intervals. The Multicast Service Interval Mode shall be set to 0 if the Multicast Service Interval field is expressed in TUs.

The Multicast Service Interval field indicates the proposed Multicast Service Interval for this particular multicast service. The proposed Multicast Service Interval can be multiple of Beacon intervals or number of TUs depending on the Multicast Service Interval Mode bit.

The Service Mode bit shall be set to 1 if the STA allows the AP to transmit multicast data belonging to multicast group identified by Multicast MAC Address field as unicast frames. The Service Mode bit shall be set to 0 if the STA does not allow the AP to transmit multicast data as unicast frames. This setting overrules the mode selected during the association phase. This setting is valid only for the multicast MAC address specified in the Multicast Service Setup Request frame.

##### 7.4.6.6 Multicast Service Setup Response frame format

Multicast Service Setup Response frame is sent by an AP.

|  | Category | Action | Status Code | Multicast MAC Address | Service Parameters |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 6 | 2 |

Figure g. Multicast Service Setup Response frame format.

The Category field shall be set to 6.

The Action field shall be set to 1.

The Status Code shall be set according to 7.3.1.9

The Multicast MAC Address indicates the multicast group to which the response is intended.

The Service Parameters field is as show in Figure j.

|  | Multicast Service Interval Mode | Multicast Service Interval | Service Mode | Reserved |
| --- | --- | --- | --- | --- |
| Bits: | 1 | 7 | 1 | 7 |

Figure h. Service Parameters field.

The Multicast Service Interval Mode shall be set to 1 if the Multicast Service Interval field is expressed in Beacon Intervals. The Multicast Service Interval Mode shall be set to 0 if the Multicast Service Interval field is expressed in TUs.

The Multicast Service Interval field indicates the Multicast Service Interval for this particular multicast service. The Multicast Service Interval can be multiple of Beacon intervals or number of TUs depending on the Multicast Service Interval Mode bit.

The Service Mode bit shall be set to 1 if the AP transmit multicast data as unicast frames. The Service Mode bit shall be set to 0 if the AP does not transmit multicast data as unicast frames. This setting overrules the mode selected during the association phase. This setting is valid only for the multicast MAC address specified in the Multicast Service Setup Response frame.

##### 7.4.6.7 Multicast Service Termination Request frame format

Multicast Service Termination Request frame is sent by a non-AP STA. It is used to indicate that the non-AP STA is not going to use the multicast service identified by the Multicast MAC Address field anymore.

|  | Category | Action | Multicast MAC address |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |

Figure I. Multicast Service Termination Request frame format

The Category field shall be set to 6.

The Action field shall be set to 2.

The Multicast MAC address indicates the multicast group which service is terminated.

##### 7.4.6.8 Multicast Service Termination Response frame format

Multicast Service Termination Response frame is sent by an AP.

|  | Category | Action | Multicast MAC address |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |

Figure J. Multicast Service Termination Response frame format

The Category field shall be set to 6.

The Action field shall be set to 3.

The Multicast MAC address indicates the multicast group which service is terminated.

##### 7.4.6.9 Multicast Service Mode Change frame format

Multicast Service Mode Change frame is sent by an AP.

|  | Category | Action | Multicast MAC address | Service Mode Change Parameters |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 |

Figure k. Multicast Service Mode Change frame format.

The Category field shall be set to 6.

The Action field shall be set to 4.

The Multicast MAC address indicates the multicast group to which the change is associated.

The Service Mode Change Parameters field is as show in Figure n.

|  | Service Mode | Service Mode Count |
| --- | --- | --- |
| Bits: | 1 | 7 |

Figure l. Service Mode Change Parameters field.

The Service Mode bit shall be set to 1 if the AP transmit multicast data as unicast frames. The Service Mode bit shall be set to 0 if the AP does not transmit multicast data as unicast frames. This setting overrules the mode selected during the association phase.

The Service Mode Count field indicates the number of multicast service intervals after which the service mode change will be effective. The Service Mode Count field is set to 0 if change will be effective immediately.

### 9.2.7 Broadcast and multicast MPDU transfer procedure

[Change clause 9.2.7 as follows]

##### 9.2.7 Broadcast and multicast MPDU transfer procedure

In the absence of a PCF, when broadcast or multicast MPDUs are transferred from a STA with the ToDS bit clear, only the basic access procedure shall be used. Regardless of the length of the frame, no RTS/CTS exchange shall be used. In addition, no ACK shall be transmitted by any of the recipients of the frame. Any broadcast or multicast MPDUs transferred from a STA with a ToDS bit set shall, in addition to conforming to the basic access procedure of CSMA/CA, obey the rules for RTS/CTS exchange, because the MPDU is directed to the AP. The broadcast/multicast message shall be distributed into the BSS. The STA originating the message shall receive the message as a broadcast/multicast message. Therefore, all STAs shall filter out broadcast/multicast messages that contain their address as the source address. Broadcast and multicast MSDUs shall be propagated throughout the ESS.

By default, there is no MAC-level recovery on broadcast or multicast frames, except for those frames sent with the ToDS bit set. As a result, the reliability of this traffic is reduced, relative to the reliability of directed traffic, due to the increased probability of lost frames from interference, collisions, or time-varying channel properties.

In case of enhanced broadcast/multicast service is used, an AP has the option to convert broadcast or multicast frames to unicast frames per non-AP STA basis. With unicast service mode more reliable service is ensured as normal acknowledgement procedures can be utilised.

Note - In case of  802.11n enhancements are supported the acknowledgements may be used for the multicast frames as described in section 9.2.7.1.3.

[Insert new subclauses after 9.2.7]

### 9.2.7.1 Enhanced broadcast/multicast service

##### 9.2.7.1.1 Enhanced broadcast/multicast service capability indication

An AP supporting enhanced broadcast/multicast shall indicate its support by using Wireless Network Management Capability information element sent in the Beacon and Probe Response frames.

A STA wishing to use enhanced broadcast/multicast services shall indicate it by using Wireless Network Management Capability information element sent in the (re)Association Request frame.

##### 9.2.7.1.2 Enhanced broadcast/multicast service modes

With enhanced broadcast/multicast service broadcast/multicast frames can be sent either normally as broadcast/multicast frames or as unicast frames. By default all the broadcast/multicast frames are sent normally as broadcast or multicast frames.

Broadcast service mode is indicated by the Broadcast to Unicast field of the Multicast Service Info information element sent in the Beacon and Probe Response frames. If broadcast to unicast conversion is enabled then the AP shall send all the broadcast traffic as unicast frames. The AP shall advertise that it is changing the broadcast service mode using the Broadcast Service Mode Count of Multicast Service Info information element.

The non-AP STA that supports multicast enhancements and is wishing to use enhanced multicast services shall complete a multicast service setup procedure for each multicast address for which it desires to receive traffic. By completing this the AP will have knowledge of which non-AP STAs belong to which multicast groups. This enables multicast acknowledgement and multicast to unicast conversion procedures. If the AP denies the usage of multicast service, normal multicast transmission rules apply.

An AP uses the Multicast Service Info field to indicate the active multicast groups within the BSS and the service characteristics of each multicast group. Active multicast groups are those for which one or more non-AP STAs have completed the multicast service setup procedure and for which the service is not yet terminated by the multicast service termination procedure by all the non-AP STAs which have completed the multicast service setup procedure.

The AP shall send all the broadcast and multicast data also normally if there are one or more associated STAs that do not support multicast enhancements.

Non-AP STA may terminate multicast service delivery by transmitting a Multicast Service Termination Request frame. The AP shall respond upon receiving Multicast Service Termination Request by transmitting Multicast Service Termination Response frame to the non-AP STA. After this the AP shall not use multicast to unicast conversion for this non-AP STA and shall use only normal DTIM service interval for this non-AP STA. If the non-AP STAs is the only existing STA using the multicast service and if it terminates the multicast service then the AP shall not advertise that particular multicast group in the Multicast Service Info.

The service mode of each multicast group can be decided per non-AP STA basis and for the same multicast group different service modes can be used for different non-AP STAs. It is possible that within the same BSS there are non-AP STAs using different service modes simultaneously. Furthermore it is possible to deliver the multicast frames using normal multicast procedures. Initially, the multicast service mode is set during the multicast service setup phase. The multicast service mode can be changed dynamically by using Multicast Service Mode Change frame. The decision when to use particular service mode is out of scope of the standard.

For the multicast to unicast conversion the following rules apply:

- The AP shall send all the multicast traffic to the non-AP STAs, to which the service mode is set to enable multicast to unicast conversion, as unicast frames and normal unicast transmission rules shall be applied.

- The AP shall send all the multicast traffic to the non-AP STAs, to which the service mode is not set to enable multicast to unicast conversion, as multicast frames and normal multicast transmission rules shall be applied.

##### 9.2.7.1.3 802.11n multicast/broadcast  ACK mode (informative)

With 802.11n it is possible to create frame sequences that support reliable multicast or broadcast transmissions. With this approach, the AP will schedule the UL acknowledgements and sends the schedule in the PSMP frame before the actual DL multicast frame transmission. The PSMP frame defines starting time of DL TXOP and zero or more starting times for the UL TXOP(s) as explained in 7.4.7.10. If the address of transmitted data in DL TXOP corresponds to multicast address, the specified terminals in UL TXOP shall transmit acknowledgement or block acknowledgement for the transmitted multicast frame(s).

If the multicast frame belongs to multicast service, the acknowledgement is used, if both non-AP STA and AP have set the multicast acknowledgement bit in Radio Resource Management Capability field.

If non-AP STA fails to acknowledge the transmitted multicast frame the AP shall retransmit the frame as multicast transmission until the transmission attempts are exceeding dot11ShortRetryLimit or dot11LongRetryLimit, or the MSDU lifetime have been reached. The AP shall require acknowledgement in retransmission only from the non-AP STA(s) whose acknowledgement was not received correctly by the AP. The AP shall consider that multicast frame(s) delivery to non-AP STAs that were not specified in PSMP frame to transmit acknowledgements was successful.

The AP need not to require acknowledgements from all receivers in multicast transmission.

##### 9.2.7.1.4 Proxy ARP support in AP

AP supporting Proxy ARP, as defined in IETF RFC 1027, shall set Support Proxy ARP bit to 1 in Radio Resource Management Capabilities field.  The Proxy ARP shall respond in behalf of the associated non-AP STAs to ARP queries. The AP shall use MAC level association information to decide activity of the associated non-AP STA and broadcast to non-AP STAs only the requests that it is not able to respond.

##### 10.3.2 Scan

##### 10.3.2.2.2 Semantics of the service primitive

[Insert the following row to the end of BSSDescription]

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Wireless Network Management Capability | As defined in frame format | As defined in frame format | The advertised radio resource management capabilities of the BSS. |

[Insert the following clauses after 10.3.X]

### 10.3.X Multicast Service Setup

The following MLME primitives support the signaling of Multicast Service Setup.

#### 10.3.X.1 MLME-MSERVICESETUP.request

##### 10.3.X.1.1 Function

This primitive requests that a Multicast Service Setup Request frame be sent to the AP with which the STA is associated. It is valid only at a Radio Management capable non-AP STA.

##### 10.3.24.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- MSERVICESETUP.request	(
PeerSTAAddress,

MulticastMACAddress,
ServiceParameters,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MAC Address | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the Multicast Service Setup process. |
| MulticastMACAddress | MACAddress | Any valid multicast MAC address | Specifies the multicast MAC address |
| ServiceParameters | As defined in frame format | As defined in frame format | Specifies the proposed service parameters for the multicast service |

##### 10.3.X.1.3 When Generated

This primitive is generated by the SME to request that a Multicast Service Setup frame be sent to the AP with which the STA is associated.

##### 10.3.X.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Multicast Service Setup Action management frame. The STA then attempts to transmit this to the AP with which it is associated.

#### 10.3.X.2 MLME-MSERVICESETUP.confirm

##### 10.3.X.2.1 Function

This primitive reports the result of a multicast service setup. It is valid only at a Radio Management capable non-AP STA.

##### 10.3.X.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- MSERVICESETUP.confirm	(
ResultCode,
MulticastMACAddress,
ServiceParameters,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-MSERVICESETUP.request. |
| MulticastMACAddress | MACAddress | Any valid multicast MAC address | Specifies the multicast MAC address |
| ServiceParameters | As defined in frame format | As defined in frame format | Specifies service parameters for the multicast service |

##### 10.3.X.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-MSERVICESETUP.request and indicates the results of the request.

This primitive is generated when the MLME-MSERVICESETUP.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a Multicast Service Setup Response frame from the AP.

##### 10.3.X.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode and may use the reported data.

#### 10.3.X.3 MLME-MSERVICESETUP.indication

##### 10.3.X.3.1 Function

This primitive indicates that a Multicast Service Setup Request frame was received from a non-AP STA. It is valid only at a Radio Management capable AP.

##### 10.3.X.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- MSERVICESETUP.indication	(
PeerSTAAddress,
MulticastMACAddress,
ServiceParameters,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Multicast Service Setup Request frame was received. |
| MulticastMACAddress | MACAddress | Any valid multicast MAC address | Specifies the multicast MAC address |
| ServiceParameters | As defined in frame format | As defined in frame format | Specifies the proposed service parameters for the multicast service |

##### 10.3.X.3.3 When Generated

This primitive is generated by the MLME when a valid Multicast Service Setup Request frame is received.

##### 10.3.X.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 9.2.7.1.2.

#### 10.3.X.4 MLME-MSERVICESETUP.response

##### 10.3.X.4.1 Function

This primitive is generated in response to an MLME-MSERVICESETUP.indication requesting a Multicast Service Setup frame be sent to a non-AP STA. It is valid only at a Radio Management capable AP.

##### 10.3.X.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MSERVICESETUP.response	(
PeerSTAAddress,
ResultCode,
MulticastMACAddress,
ServiceParameters,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Multicast Service Setup Request frame was received. |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, REFUSED, UNSPECIFIED_ FAILURE | Indicates the results of the corresponding MLME-MSERVICESETUP.indication. |
| MulticastMACAddress | MACAddress | Any valid multicast MAC address | Specifies the multicast MAC address |
| ServiceParameters | As defined in frame format | As defined in frame format | Specifies service parameters for the multicast service |

##### 10.3.X.4.3 When Generated

This primitive is generated by the SME in response to an MLME-MSERVICESETUP.indication requesting a Multicast Service Setup Response be sent to a non-AP STA. If the returned ResultCode is SUCCESS, the primitive contains the MulticastMACAddress, ServiceParameters and MulticastTriggerConditions to be sent to the STA.

##### 10.3.X.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Multicast Service Setup Response frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

### 10.3.X Multicast Service Termination

The following MLME-primites support the signaling of the multicast service termination.

### 10.3.X. MLME-MSERVICETERM.request

##### 10.3.X.1.1 Function

This primitive requests that a Multicast Service Termination Request be sent to the AP with which the non-AP STA is associated. It is valid only at a Radio Management capable non-AP STA.

##### 10.3.X.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MSERVICETERM.request	(
PeerSTAAddress,
MulticastMACAddress 				              )

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | Specifies the address of the peer MAC entity with which to perform the multicast service termination process. |
| MulticastMACAddress | MACAddress | Any valid multicast MAC address | Specifies the multicast MAC address |

##### 10.3.X.1.3 When Generated

This primitive is generated by the SME to request that a Multicast Service is Terminated.

##### 10.3.X.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Multicast Service Termination Request frame. The non-AP STA then attempts to transmit this to the AP.

#### 10.3.X.2 MLME-MSERVICETERM.confirm

##### 10.3.X.2.1 Function

This primitive reports the result of a multicast service termination. It is valid only at a Radio Management capable non-AP STA.

##### 10.3.X.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- MSERVICETERM.confirm	(
ResultCode,
MulticastMACAddress,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-MSERVICETERM.request. |
| MulticastMACAddress | MACAddress | Any valid multicast MAC address | Specifies the multicast MAC address |

##### 10.3.X.2.3 When Generated

This primitive is generated by the MLME as a result of an MLME-MSERVICETERM.request and indicates the results of the request.

This primitive is generated when the MLME-MSERVICETERM.request contains invalid parameters, when a timeout or failure occurs, or when the STA receives a Multicast Service Termination Response frame from the AP.

##### 10.3.X.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode and may use the reported data.

#### 10.3.X.3 MLME-MSERVICETERM.indication

##### 10.3.X.3.1 Function

This primitive indicates that a Multicast Service Termination Request frame was received from a non-AP STA. It is valid only at a Radio Management capable AP.

##### 10.3.X.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- MSERVICETERM.indication	(
PeerSTAAddress,
MulticastMACAddress
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Multicast Service Termination Request frame was received. |
| MulticastMACAddress | MACAddress | Any valid multicast MAC address | Specifies the multicast MAC address |

##### 10.3.X.3.3 When Generated

This primitive is generated by the MLME when a valid Multicast Service Termination Request frame is received.

##### 10.3.X.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 9.2.7.1.2.

#### 10.3.X.4 MLME-MSERVICETERM.response

##### 10.3.X.4.1 Function

This primitive is generated in response to an MLME-MSERVICETERM.indication requesting a Multicast Service Termination responce frame be sent to a non-AP STA. It is valid only at a Radio Management capable AP.

##### 10.3.X.4.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-MSERVICESTERM.response	(
PeerSTAAddress,
ResultCode,
MulticastMACAddress,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Multicast Service Setup Request frame was received. |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, REFUSED, UNSPECIFIED_ FAILURE | Indicates the results of the corresponding MLME-MSERVICESETUP.indication. |
| MulticastMACAddress | MACAddress | Any valid multicast MAC address | Specifies the multicast MAC address |

##### 10.3.X.4.3 When Generated

This primitive is generated by the SME in response to an MLME-MSERVICETERM.indication requesting a Multicast Service Termination Response be sent to a non-AP STA. If the returned ResultCode is SUCCESS, the primitive contains the MulticastMACAddress to be sent to the STA.

##### 10.3.X.4.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Multicast Service Termination Response frame. The STA then attempts to transmit this to the non-AP STA indicated by the PeerSTAAddress parameter.

### 10.3.X Multicast Service Mode Change

The following MLME primitives support the signaling of Multicast Service Mode Change.

#### 10.3.X.1 MLME-MSERVICEMODEC.request

##### 10.3.X.1.1 Function

This primitive requests that a Multicast Service Mode Change frame be sent to the non-AP STA. It is valid only at a Radio Management capable AP.

##### 10.3.24.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- MSERVICEMODEC.request	(
PeerSTAAddress,

MulticastMACAddress,
ServiceParameters,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerSTAAddress | MACAddress | Any valid individual or multicast MAC Address | The address of the non-AP STA MAC entity to which a Multicast Service Mode Change frame is intended or the address of the non-AP STA MAC entities to which a Multicast Service Mode Change frame is intended |
| MulticastMACAddress | MACAddress | Any valid multicast MAC address | Specifies the multicast MAC address |
| ServiceParameters | As defined in frame format | As defined in frame format | Specifies service parameters for the multicast service |

##### 10.3.X.1.3 When Generated

This primitive is generated by the SME to request that a Multicast Service Mode Change frame be sent to the non-AP STA.

##### 10.3.X.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Multicast Service Mode Change management frame. The AP then attempts to transmit this to the non-AP STA.

#### 10.3.X.3 MLME-MSERVICEMODEC.indication

##### 10.3.X.3.1 Function

This primitive indicates that a Multicast Service Mode Change frame was received from an AP. It is valid only at a Radio Management capable non-AP STA.

##### 10.3.X.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME- MSERVICEMODEC.indication	(
MulticastMACAddress,
ServiceParameters, 
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| MulticastMACAddress | MACAddress | Any valid multicast MAC address | Specifies the multicast MAC address |
| ServiceParameters | As defined in frame format | As defined in frame format | Specifies the proposed service parameters for the multicast service |

##### 10.3.X.3.3 When Generated

This primitive is generated by the MLME when a valid Multicast Service Mode Change frame is received.

##### 10.3.X.3.4 Effect of Receipt

On receipt of this primitive the SME should operate according to the procedure in 9.2.7.1.2.

[Insert the following  clauses after 11.2.1.1 and renumber the following clauses]

##### 11.2.1.2 Enhanced Broadcast/Multicast power management

Using the enhanced multicast service it is possible to create flexible service intervals for multicast services.

The Service Interval for a multicast service is created by using the multicast service setup signaling. A non-AP STA wishing to join the multicast group can propose a multicast service interval to the AP. Multicast service interval can be either multiple of Beacon periods or it can be specified in number of TUs. The AP shall make the selection of the multicast service interval and shall indicate it in the Multicast Service Setup Response frame, in Multicast Service Mode Change frame and in Beacon and Probe Response frames.

The APs shall send a Multicast Service Info element in each Beacon frame. The Multicast Service Info contains information about the broadcast service and active multicast services. It indicates what is the service mode of broadcast and multicast services, indicates the existence of buffered broadcast and/or multicast traffic and includes information about the service intervals of the each active multicast groups. The existence of buffered multicast traffic can be made per group basis by using Buffered Traffic Indication field and the existence of the buffered broadcast traffic is indicated with Buffered Broadcast Data field

If the service mode of the broadcast or multicast service is set to enable conversion to unicast, normal unicast power management rules apply.

If the non-AP STA is using enhanced broadcast/multicast service, it may take into account that the AP shall send broadcast/multicast MSDUs using normal broadcast/multicast transmission rules if not all the associated STAs are broadcast/multicast enhanced.

[Insert the following  text at the end of bullet c in 11.2.1.5]

#### 11.2.1.5 AP operation during the CP

c)

If enhanced multicast service is used the AP shall send Beacon with Multicast Service Info field at every TBTT. Buffered Traffic Indication bit shall be set if there are buffered frames belonging to particular multicast group. Buffered Broadcast Data bit shall be set to 1 if there are buffered broadcast traffic.

[Insert the following  text at the end of bullet f  in 11.2.1.5]

f)

If enhanced multicast service is used the AP will send all multicast frames belonging to particular multicast group after the service interval which is specified for the multicast service. If all the associated STAs are broadcast/multicast enhanced then The More Data field of each multicast frame shall be set to indicate the presence of further buffered multicast MSDUs from the multicast address. If all the associated STAs are not broadcast/multicast enhanced then The More Data field of each multicast frame shall be set to indicate the presence of further buffered broadcast/multicast MSDUs. If the AP is unable to transmit all the buffered multicast MSDUs before the next TBTT, the AP shall indicate that it will continue to deliver the multicast MSDUs by setting the Buffered Traffic Indication bit in Multicast Service Info field to 1. Buffered Traffic Indication bit shall be set to 1 until all buffered multicast frames have been transmitted. If all the associated STAs are broadcast/multicast enhanced then The More Data field of each broadcast frame shall be set to indicate the presence of further buffered broadcast MSDUs. If all the associated STAs are not broadcast/multicast enhanced then The More Data field of each broadcast frame shall be set to indicate the presence of further buffered broadcast/multicast MSDUs.  The Buffered Broadcast Data bit shall be set to 1 to indicate the presence of further buffered broadcast MSDUs.

[Insert the following  text at the end of bullet e  in 11.2.1.7]

#### 11.2.1.7 Receive operation for STAs in PS mode during the CP

e)	If the non-AP STA is using enhanced multicast service and if the service mode is multicast, the non-AP STA shall wake up before the specified multicast service interval. A non-AP STA shall remain awake until the More Data field of the multicast MSDU indicates there are no further buffered broadcast/multicast MSDUs, or until Multicast Service is not listed in Multicast Service Info field or Buffered Traffic Indication field indicates that there are no further buffered multicast MSDUs.

Insert this new clause after clause A.4.13:

### A.4.14 Radio Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME5 RME5.1 RME5.2 RME5.3 RME5.4 RME5.5 RME5.6 | Broadcast/Multicast Enhancements Multicast Service Info information element Multicast Service Setup Request frame Multicast Service Setup Response frame Multicast Service Termination Request frame Multicast Service Termination Response frame Multicast Service Mode Change frame | 9.2.7.1 7.3.2 7.4.6.5 7.4.6.6 7.4.6.7 7.4.6.8 7.4.6.9 | CFv:O (CF1&RME5):M (CF2&RME5):M (CF1&RME5):M (CF2&RME5):M (CF1&RME5):M (CF1&RME5):M | Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A Yes, No, N/A |

##### References: