IEEE P802.11
Wireless LANs

| A method of avoiding packets loss during a transition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2005-12-31 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Du Hanmei | Huawei | Huawei Longgang Production Base, Shenzhen 518129 P.R.China | +86-755-28972654 | Duhanmei61564@huawei.com |

Abstract

This document proposes a method of avoiding packets loss during a fast BSS transition in 802.11r.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Background

As described in the Nokia’s contribution of 11-05-1105-00-000r-ready-to-transition-clear-to-transition

- When station makes a transition, packets may be lost for one of two reasons:

  - They are queued in the old AP awaiting transmission

  - They are misdirected to the wrong AP by the DS during the transition process

AP1

AP2

DS

STA

Packet “left

behind”

AP1

AP2

DS

STA

Packet delivered

to old AP

# Solution

## Solution summary

Pick up data response

（Re）association Resp

FT Action ACK or response

（Re）association Req

FT Action confirm or request

(RTT Req)

AP1

AP2

STA

Successful (secure)session &Data transition

成功会话和数据传输

Successful (secure) session & Data transition

FT Action confirm or request

Pick up data request

Pick up data ACK

As show in the figure above, we add a Buffer  IE  to the last  Authentication messege, ie, FT Action confirm messege in the pre-reservation mechanism or FT Action response messege in the Base mechanism.

The Buffer IE is defined below:

| Element ID | Length | Buffer identify |
| --- | --- | --- |

octet：1                                                        1                                                    1

The Buffer Identify   is set below:

| 00 | No buffer |
| --- | --- |
| 01 | Forward the data to target AP to buffer over DS |
| 10 | Buffer the data at current AP |
| …… | …… |

- When AP1 has MSDU for the STA queued  for future delivery, the Buffer Identify IE will be contained in the last authentication messege sent to STA. if the Buffer Identify is set to 01, AP1 will send the data to target AP over DS. If the Buffer Identify is set to 10, AP1 will buffer the data. Otherwise, if the Buffer Identify is set to 00, message shall flow like before as the darft described.

The reassociation request message shall also contain the Bffer IE, when target AP receives this messege, it identifies the Buffer Identify, if it is set to 01, and target AP will send the data which the STA didn’t receive to the STA, if it is set to “10”, target AP send a messege to old AP to pick up data which buffered at the old AP.

In addition, the reassociation request messege could contain a “SN IE”, the “SN IE” can be set like below:

| Element ID | Length | SN |
| --- | --- | --- |

Octet ： 1                                                          1                                                        4

The “SN” represents the data sequence number that the STA has received. AP2 could know the data which STA hasn’t received via contrast with the “SN”. So that, AP2 could only sent the data which STA hasn’t received or the AP2 could sent “pick up data” messege which contains “SN IE” to AP1, in this case AP1 need only sent the data which STA hasn’t received to AP2.