IEEE P802.11
Wireless LANs

| Coexistence of Two Overlapping BSSs in Adjacent Channels Testing |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: March 2006 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Neeraj K. Sharma | Intel | 2111 NE 25th Avenue, Hillsboro, OR 97124 | 858-385-4112 | neeraj.k.sharma@intel.com |
| Jim Darnell | Intel | 13250 Avenue of Science, San Diego, CA 92128 | 858-385-4477 | jim.darnell@intel.com |
| Uriel Lemberger | Intel | PO Box 1659, Matam Industrial Park, Haifa 31015 Israel | +972-4-865-5701 | uriel.lemberger@intel.com |
| Alexander Tolpin | Intel | PO Box 1659, Matam Industrial Park, Haifa 31015 Israel | +972-4-865-5430 | alexander.tolpin@intel.com |
| Guy Halperin | Intel | PO Box 1659, Matam Industrial Park, Haifa 31015 Israel | +972-4-465-4321 | guy.halperin@intel.com |
| Nir Alon | Intel | PO Box 1659, Matam Industrial Park, Haifa 31015 Israel | +972-4-865-6621 | nir.alon@intel.com |
| Fahd Pirzada | Dell | One Dell Way Round Rock, TX 78682 | 512-338-4400 | Fahd_Pirzada@dell.com |
| Pratik Mehta | Dell | One Dell Way Round Rock, TX 78682 | 512-338-4400 | Pratik_Mehta@dell.com |
| Jason Trachewsky | Broadcom | 190 Mathilda Place Sunnyvale, CA 94086 U.S.A. | +1-408-543-3319 | jat@broadcom.com |
| Dalton Victor | Broadcom | 190 Mathilda Place Sunnyvale, CA 94086 U.S.A. | +1-408-922-5824 | dvictor@broadcom.com |
| Mark Kobayashi | Broadcom | 190 Mathilda Place Sunnyvale, CA 94086 U.S.A. | +1-408-543-3318 | mmk@broadcom.com |

Abstract

This document introduces test metric and methodology for coexistence of two overlapping BSSs on adjacent channels in an Over-the-air indoor line of sight test environment as part of Recommended Practice for the Evaluation of 802.11 Wireless Performance. It is being provided to the 802.11T task group as a draft text in conjunction with a companion presentation.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Coexistence of Two Overlapping BSSs in Adjacent Channels in OTA Indoor LOS Test Environment

## Introduction and Purpose

The purpose of the coexistence of two overlapping BSSs on adjacent channels test is to depict an end user’s expected throughput performance in a situation where another 802.11 device is nearby on an adjacent channel. The proposed test uses OTA indoor line of sight test environment. The test metric defined for measuring the effect of coexistence is a primary metric which directly impacts an end-user’s experience as expressed by throughput verses range in a real life environment.

The test is applicable to wireless clients and provides the basic measure of throughput degradation in the client’s performance due to the presence of an interferer. The test is applicable to IBSS (Independent BSS) as well as infrastructure client configurations.

The setup comprises of a DUT and its associated reference AP (WLCP) and a STA used as a source of interference (Client_Jammer) and its associated AP (AP_Jammer). Indoor LOS locations are chosen for the DUT, Client_Jammer, WLCP, and AP_Jammer such that the DUT and Client_Jammer are separated by 2m ± 1cm and the distance between the WLCP and AP_Jammer is 40m ± 1cm. Test Controllers/ Traffic Generators are utilized at the WLCP and AP_Jammer to generate traffic from WLCP to DUT and from AP_Jammer to Client_Jammer on top of Layer 2. The DUT’s throughput is measured for two minutes without interference, two minutes with interference, and a final two minutes without interference. In the ideal case, the DUT’s throughput is unaffected by the interference from the neighboring device.

## Test Configuration

### Resource Requirements

The following equipment is required to carry out this test:

- A wireless counterpart (WLCP), which is a reference AP for an infrastructure BSS or a reference client for an IBSS. The DUT should be capable of associating with the WLCP.

- AP_Jammer, which is an AP used for generating interference.

- Client_Jammer, which is a STA used for generating interference. The Client_Jammer should be capable of associating with the AP_Jammer.

- Test controllers/Traffic Generators that are automated and controlled by dedicated software, and include the following capabilities:

  - The ability to transfer 1MB file between WLCP and the DUT and between AP_Jammer and the Client_Jammer above the MAC layer (Layer 2).

  - The ability to control time/duration of file transfer between the WLCP and the DUT and between AP_Jammer and the Client_Jammer.

- Rolling carts of appropriate height to move/place the DUT and Client_Jammer.

- Inclinometer or other angle measurement device, and/or a test jig to fix the antenna angle of the DUT and Client_Jammer with 802.11antennae in the lid/LCD.

- Spectrum analyzer to monitor noise level.

The test setup is described in section 1.2.3.

### Test environment

This test uses the Over-the-air Indoor LOS test environment described in [2] in section 5.6. The setup should be properly isolated from external interference and other unwanted signals.

### Test setup

Figure 1 depicts the test setup.

AP_Jammer

Test Station Cart

Spectrum Analyzer

Test Controller/Traffic Generator

Distance Between DUT and Client_Jammer

WLCP

Client_Jammer

DUT

Distance Between WLCP

and DUT

Distance Between

Client_Jammer

and AP_Jammer

Test Stand or wall/ceiling

1200

1200

Test Controller/Traffic Generator

Distance Between WLCP and AP_Jammer

Figure 1 Test Setup

The distances between the test devices are as follows:

- Distance between WLCP and AP_Jammer = 40 meters ± 1cm

- Distance between DUT and Client_Jammer = 2 meters ± 1cm

- Distance between WLCP and DUT = Distance between the AP_Jammer and Client_Jammer.

The positioning of the DUT and Client_Jammer should be as follows:

- For laptops, DUT and Client_Jammer are placed back-to-back as shown in Figure 1.

- For desktops with external antennas, the DUT’s and Client_Jammer’s antennas are placed on top of their respective test station carts.

- For desktops with internal antennas, the desktops are placed on top of their respective test station carts.

The remaining test setup parameters are same as specified in Clause 5.6 of the P802.11.2/D0.5.

### Special capabilities necessary for this test

It is necessary to be able to transfer a fixed size file between the DUT and WLCP, and Client_Jammer and AP_Jammer for a fixed period of time.

### Variance of Measurement Results

Prior to beginning the test, the test equipment described above should be calibrated, and all test software verified. The test setup may be monitored during the test to ensure that the test conditions do not change. The variance of measurement results between multiple runs on the same setup is expected to be within 10% of the percentage of DUT’s throughput degradation defined in section 1.3.2.1

## Approach

### Configuration Parameters

This sub-clause provides a list of configuration parameters applicable to this test.

#### Baseline Configuration

The baseline DUT and Client_Jammer setup that should be configured, measured, and reported whenever this test is performed is as follows:

- Maximum Tx power setting for DUT, WLCP, Client_Jammer, and AP_Jammer with capability to change this parameter. It is necessary to ensure that the Tx power of the Client_Jammer and DUT are less than 20dBm and 17dBm for 2.4GHz and 5GHz, respectively

- Both the DUT and Client_Jammer in adjacent channels in 802.11a-only, 802.11b-only, or 802.11g-only modes.

  - For 802.11b/g mode, DUT/WLCP and Client_Jammer/AP_Jammer channel setup are 1 and 6, respectively, or 2 other channels with offset of 25MHz.

  - For 802.11a mode, DUT/WLCP and Client_Jammer/AP_Jammer channel setup are 48 and 52, respectively, or 2 other channels with offset of 20MHz.

  - The actual channel selected should be recorded.

- No security (Open System).

- Both DUT and Client_Jammer on AC power with all power saving features disabled (WLANs in CAM mode).

#### Modifiers

The baseline DUT and Client_Jammer setup parameters may be modified as follows to enable additional testing to be performed.

- Fixed Tx/Rx data rates for the DUT and Client_Jammer.

- Power saving mode.

- Other operating channels in 802.11a-only, 802.11b-only, or 802.11g-only modes.

- Mixed mode where the DUT is in 802.11g mode while Client_Jammer is in 802.11b mode or vice versa.

- Distance between DUT and Client_Jammer can be varied.

#### Test conditions

The test conditions used while performing this test are as follows:

- RF frequency (channel)

- Mode – 802.11a, 802.11b, 802.11g

- Tx/Rx

### Measurement procedure

To measure the DUT’s throughput degradation due to interference, DUT’s throughput performance is measured for two minutes without interference (Quiet Run-up Period), two minutes with interference (Interference Period), and two minutes without interference (Quiet Run-down Period) as shown in Figure 2. Details are presented in section 1.3.2.2.

The reason to measure the DUT’s throughput performance during the quiet run-down period is to account for throughput impact due to the possible DUT’s graceful rate scaling recovery after the interference.

1 min

2 min

3 min

4 min

Time

Client_Jammer/AP_Jammer Active Period

DUT/WLCP Active Period

Quiet Run-up Period

Interference Period

Quiet Run-down Period

5 min

6 min

Figure 2  Activity of the DUT and Client_Jammer during the test Period

#### Specification of metric

The DUT’s capability to coexist and perform well while communicating in an environment consisting of two overlapping BSSs on adjacent channels is measured by the percentage of DUT’s throughput degradation due to interference:

% of DUT's Throughput Degradation = DUT's Average Throughput with IntereferenceDUT's Average Throughput without Intereference×100%

DUT’s Average Throughput without Interference is determined as follows:

DUT's Average Throughput without Intereference =  DUT's Average Throughput During Quiet Run−up Period + DUT's Average Throughput During Quiet Run−down Period2

#### Procedure

The DUT and the Client_Jammer are setup according to the baseline configuration, using an initial combination of test conditions. The following steps are then performed:

- Setup DUT/WLCP and Client_Jammer/AP_Jammer on appropriate adjacent channels for a/b/g mode.

- Associate DUT with WLCP.

- Enforce DUT to Tx/Rx 1MB file for 6 minutes (DUT using auto rate negotiation).

- Measure DUT’s throughput and record.

- After 2 minutes, associate Client_Jammer with AP_Jammer.

- Enforce Client_Jammer to Tx/Rx 1MB file for 2 minutes (Client_Jammer using auto rate negotiation).

- Measure Client_Jammer’s throughput and record.

- After 2 minutes, turn off Client_Jammer and AP_Jammer.

- Repeat above steps with DUT and Client_Jammer in the following Tx/Rx modes of file transfer:

DUT	Client_Jammer

Tx	Tx

Tx	Rx

Rx	Tx

Rx	Rx

- Calculate % of DUT’s throughput degradation.

- Repeat above steps with different combinations of channels and modes for the DUT and Client_Jammer.

The DUT’s throughput should be recorded separately for Quiet Run-up, Interference, and Quiet Run-down periods and reported as discussed in section 1.3.3. The same test should be repeated and the results reported with the Client_Jammer behaving like a DUT without changing the setup i.e. the Client_Jammer’s OTA throughput is measured for two minutes without interference (from the DUT), two minutes with interference (from the DUT), and a final two minutes without interference.

### Reported Results

The summary of the results should be reported as a table. The table format is shown below.

| Mode | DUT Channel | Client_Jammer Channel | DUT Tx/Rx | Client_Jammer Tx/Rx | DUT’s Average Throughput during Quiet Run-up Period (Mbps) | DUT’s Average, Minimum, and Maximum Throughput during Interference Period (Mbps) | Client_Jammer’s Average, Minimum, and Maximum Throughput during Interference Period (Mbps) | DUT’s Average Throughput during Quiet Run-down Period (Mbps) | % of DUT’s Throughput Degradation |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Figure 3  Metric Reporting Template

The following data should also be reported:

- Ambient temperature

- Relative humidity

- Details of Client_Jammer, AP_Jammer, and WLCP e.g. manufacturer, model, etc.

- Maximal Tx power of each device in the setup should be measured and recorded: DUT; Client_Jammer; WLCP; and AP_Jammer.

- Average RSSI values (average of at least 5 measurements during 1 minute) for each of the RF paths for the DUT and Client_Jammer should be measured and recorded before the test as characteristics of the setup. Assuming DUT and WLCP are operating on channel x and the Client_Jammer and AP_Jammer are operating on channel y, then the RF paths are as follows:

  - WLCP → DUT – RSSI measured on channel x

  - WLCP → Client_Jammer – RSSI measured on channel x

  - AP_Jammer→ DUT – RSSI measured on channel y

  - AP_Jammer→ Client_Jammer – RSSI measured on channel y

  - DUT→ Client_Jammer – RSSI measured on channel x

  - Client_Jammer→ DUT – RSSI measured on channel y

- Results for the case where the Client_Jammer is behaving like a DUT without changing the setup.

The details of the test results could also be included in a graphical format. The graphs should show DUT’s and Client_Jammer’s throughput over a period of 6 and 2 minutes, respectively, for different modes, channels, and Tx/Rx combinations.

# References

- IEEE 802.11-1999.

- P802.11.2-D0.5 - Draft Recommended Practice for the Evaluation of 802.11 Wireless Performance.