IEEE P802.11
Wireless LANs

| Normative Text Changes for BSS Channel Switch |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2006-MAY-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Marian Rudolf | InterDigital Communications | Montreal, QC | 514-9046258 | Marian.Rudolf@InterDigital.com |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | joekwak@sbcglobal.net |

Abstract

This document contains normative text for the BSS channel switch protocol to augment capabilities provided by 802.11h DFS. As explained in the accompanying presentation 06/0388r1, the proposed additions on top of the baseline 11h DFS are simple additions to support minimum service interruption for ongoing sessions when executing the BSS channel switch

The proposal has been drafted with the goal to introduce only a minimum amount of changes necessary compared to the 11h DFS baseline in order to meet above goal. It notably re-uses existing 11h DFS protocol signalling and procedures almost entirely.

This normative text proposal addresses TGv objective #Req2000 Dynamic Channel Selection as a solution which is "able to change the operating channel (and/or band) for the entire BSS during live system operation and be done seamlessly with no intermittent loss of connectivity from the perspective of an associated STA."

Changes since presentation at the March meeting include:

1.  Deletion of Channel Switch Solicitation.

2.  Deletion of Channel Switch Request Type.

3. Simplification of Channel Switch Request/Response formats and content.

4.  Addition of new triggered Link Diagnostic Request to support BSS Channel Switch.

5.  Simplification of the BSS Channel Switch Procedure in section 11.

Note to the editor: normative text additions are marked as changes from the latest TGv draft amendment, 802.11vD0.02. Insertions are marked in blue underscore and deletions are marked as red strikethrough.

.

[NOTE TO EDITOR:  Change bars are used to show changes and additions from draft 802.11vD0.02]

- 7.3.2.21.12 Link Diagnostics Request

The Measurement Request field corresponding to a Link Diagnostics Request is shown Figure x1. The response to a Multicast Diagnostics Request is a Multicast Diagnostics Report.

|  | Randomization Interval | Measurement Duration | Link Triggered Reporting (optional) |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 3 |

- Figure x1—Measurement Request field format for a Multicast Diagnostics Request

Randomization Interval is set to the desired maximum random delay in the measurement start time, expressed in TUs. The use of Randomization Interval is described in 11.11.3. Randomization Interval is not used and is set to 0 when requesting triggered Multicast diagnostics.

The Measurement Duration field shall be set to the preferred or mandatory duration of the requested measurement, expressed in TUs. See 11.11.3.. When setting up a triggered link diagnostics alert, Measurement Duration is not used and is set to 0.

The Link Triggered Reporting field is used to specify trigger conditions and thresholds. It is only present when setting up triggered management diagnostic reporting. The Link Triggered Reporting field is as shown in Figure x2.

|  | Link Trigger Condition | Trigger Threshold | Trigger Timeout |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

- Figure x2—Link Triggered Reporting field

Link Trigger Condition specifies reporting triggers for triggered management diagnostic reporting. The values for the Link Trigger Condition are shown in Table x1.

Table x1—Link Trigger Condition Code Definitions

| Link Trigger Condition Code | Link Trigger |
| --- | --- |
| 0 | Link Diagnostic Report to be generated when the RCPI level for any frame addressed to this STA from the AP is less than the value specified in the Trigger Threshold field. |
| 1 | Link Diagnostic Report to be generated when the RSNI level for any frame addressed to this STA from the AP is less than the value specified in the Trigger Threshold field. |
| 2 | Link Diagnostic Report to be generated when the ANPI level is greater than the value specified in the Trigger Threshold field. |
| 3 | Link Diagnostic Report to be generated when the Channel Load is greater than the value specified in the Trigger Threshold field. |
| 4-255 | reserved |

The Trigger Threshold  field contains a value to be used as the threshold value for the indicated Link Trigger Condition.

The Trigger Timeout field contains a value in units of 100 TU during which a measuring STA shall not generate further triggered Link Diagnostic Reports after a trigger condition has been met.

- 7.3.2.22.12 Link Diagnostics Report

The format of the Measurement Report field of a Link Diagnostics report is shown in Figure x3.

|  | Measurement Start Time | Measurement Duration | RCPI | RSNI | ANPI | Channel Load | Link Trigger Condition (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 1 | 1 | 1 | 1 | 1 |

- Figure x3—Measurement Report field format for a Multicast Diagnostics Report

The Measurement Start Time is set to the value of the STA TSF timer at the time the measurement started. For a triggered Link Diagnostics Report, this is the TSF value at the reporting STA when the trigger condition was met

Measurement Duration shall be set equal to the duration over which the Link Diagnosics Report was measured, expressed in TUs.

RCPI indicates the received channel power of the most recently measured frame addressed to the STA from the AP. RCPI is reported in dBm, as defined in the RCPI measurement clause for the PHY Type.

RSNI indicates the received signal to noise indication in dB for the most recently measured frame addressed to the STA from the AP, as described in 7.3.2.40.

ANPI shall be set to the average noise plus interference power value measured during the indicated Measurement Duration while the indicated channel is idle as described in 11.11.8.4.

Channel Load shall contain the proportion of measurement duration for which the measuring STA determined the channel to be busy. Procedure for Channel Load measurement and definition of channel load values are found in 11.11.8.3.

The Link Trigger Condition field is an optional field indicating the trigger which generated this triggered Link Diagnostics report. . The values for the Link Trigger Condition are shown in Table x1.

- 7.4 Action frame format details

Insert the following after clause 7.4.5:

    - Wireless Network Management action details

Several Action frame formats are defined for Wireless Network Management purposes. An Action field, in the octet field immediately after the Category field, differentiates the formats. The Action field values associated with each frame format are defined in Table v42.

- Table v42—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 0 | Event Log Request |
| 1 | Event Log Report |
| 2 | Diagnostic Request |
| 3 | Diagnostic Report |
| 4 | Presence Request |
| 5 | Presence Response |
| 6 | Presence Configuration Request |
| 7 | Presence Configuration Response |
| X | Channel Switch Request |
| X+1 | Channel Switch Response |
| 8-255 | Reserved |

Insert and renumber new clause.7.4.6.x after 7.4.6.4 as follows:

##### 7.4.6.X Channel Switch Request frame format

The Channel Switch Request frame is used by an AP in a BSS to coordinate the channel switch from the current to a new target channel. The Channel Switch Request frame body format is shown in Figure x5 .

|  | Category | Action | Dialog Token | Regulatory Class | Target Channel Number | Response Time Count | Response Randomization Interval | Channel Switch Announcement (optional)t |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 5 |

Figure x5—Channel Switch Request frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Channel Switch Request frame, as specified in in 7.4.6.

The Dialog Token field is set to a value chosen by the STA sending the event log request to identify the request/report transaction.

Regulatory Class indicates the channel set for this channel switch. Regulatory Class and Channel Number together specify the channel frequency and spacing for this channel switch.   Valid values of Regulatory Class are shown in Annex J.

Target Channel Number indicates the destination channel number for this channel switch. Channel Number is defined within a Regulatory Class as shown in Annex J.

The Response Time Count field shall be set to the number of target beacon transmission times (TBTTs) until the latest opportunity for the STA receiving the Channel Switch Request to send a Channel Switch Response to the requesting STA.. A value of 1 indicates a request for a Channel Switch Response to be sent before the next TBTT. A value of 0 indicates that the Response Time Count is not specified for this management request and that the STA may send a response at any time.

The Response Randomization Interval specifies the upper bound of the random delay to be used just prior to sending the Channel Switch Response for this request. The Response Randomization Interval field is specified in units of TU. See 11.15.X.

The Channel Switch Announcement is an optional field which indicates that the BSS will switch to a new channel.  The Channel Switch Announcement element is described in 7.3.2.20.

##### 7.4.6.(X+1) Channel Switch Response frame format

The Channel Switch Response frame is provided by a STA in response to receiving a Channel Switch Request frame and is used by a non-AP STA in a BSS or a STA in an IBSS to coordinate the channel switch to a new channel. The Channel Switch Response frame body format is shown Figure x6.

|  | Category | Action | Dialog Token | Regulatory Class | Alternative Channel Number | Channel Switch Response Code |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure x6— Channel Switch Response frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Channel Switch Response frame, as specified in 7.4.6.

The Dialog Token field is set to a value chosen by the STA sending the event log request to identify the request/report transaction.

Regulatory Class indicates the channel set for this channel switch. Regulatory Class and Alternative Channel Number together specify the channel frequency and spacing for this channel switch. Valid values of Regulatory Class are shown in Annex J.

The Alternative Channel Number field indicates a STA preference to switch to an alternative target channel number. Alternative Channel Number is defined within a Regulatory Class as shown in Annex J.  A value of 0 indicates the STA does not have a preference for an alternative target channel for this channel switch.

The Channel Switch Response Code field indicates the STA response to a channel switch request as defined in Table x2.

Table x2—Channel Switch Response Code Definitions for a Channel Switch Request

| Channel Switch Response Code | Response Definition for Query Request Type |
| --- | --- |
| 0 | Reporting STA will not switch to the requested channel. |
| 1 | Reporting STA is able to switch to the requested channel at the requested time or has already switched to the requested channel. |
| 2 | Reporting STA will not follow, but would follow on the Alternative Channel Number. |
| 3 | Reporting STA is able to switch to the requested channel at the requested time and may do so if this channel switch request is confirmed or when a this channel switch is announced. |
| 4-255 | reserved |

# 10. Layer management

## 10.3 MLME SAP Interface

Insert the following new clause after Clause 10.3.31 in 802.11v Draft 0.01 as indicated below:

### 10.3.x BSS Channel Switch

### 10.3.x Channel Switch Request

This set of primitives supports the signaling of Channel Switch Request frames between an AP and associated non-AP STAs.

#### 10.3.x.1 MLME-CHANSWITCHREQUEST.request

##### 10.3.x.1.1 Function

This primitive requests transmission of a Channel Switch Request frame to a non-AP STA.

##### 10.3.x.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANSWITCHREQUEST.request	(
PeerMACAddress
DialogToken,
Regulatory Class
Target Channel Number
Channel Switch Mode
Channel Switch Count
Response Time Count
Response Randomization Interval
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Channel Switch Request frame is sent. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify the Channel Switch transaction. Set to 0 for an autonomous Channel Switch Request frame. |
| Regulatory Class | Integer | 0 – 255 | For the Target Channel Number, as defined in AnnexJ. |
| Target Channel Number | Integer | 0 – 255 | Channel number requested for BSS switch, defined within the Regulatory Class in Annex J. |
| Channel Switch Mode | Integer | 0-1 | Channel switch mode, as defined for the Channel Switch Announcement element. |
| Channel Switch Count | Integer | 0 – 255 | Indicates the number of target beacon transmission times (TBTTs) until the requested channel switch to the indicated new channel. A value of 1 indicates a request for a channel switch immediately before the next TBTT. A value of 0 indicates that the Channel Switch Count is not specified for this management request. |
| Response Time Count | Integer | 0 – 255 | Indicates the number of target beacon transmission times (TBTTs) until the latest opportunity for the STA receiving the Channel Switch Solicitation to send a Channel Switch Request to the requesting STA. A value of 1 indicates a request for a Channel Switch Request to be sent before the next TBTT. A value of 0 indicates that the Response Time Count is not specified for this management request and that the STA may send a response at any time. |
| Response Randomization Interval | Integer | 0-65535 | Indicates the upper bound of the random delay to be used just prior to sending the Channel Switch Response for this request. The Response Randomization Interval field is specified in units of TU. |

##### 10.3.x.1.3 When Generated

This primitive is generated by the SME to request that a Channel Switch Request frame be sent to an associated non-AP STA. This request is sent either following the reception of a MLME-CHANSWITCHSOLICIT.indication or may be sent autonomously.

##### 10.3.x.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Channel Switch Request management frame of action type. The STA then attempts to transmit this frame to the indicated non-AP STA.

#### 10.3.x.2 MLME-CHANSWITCHREQUEST.confirm

##### 10.3.x.2.1 Function

This primitive reports the result of a request to send a Channel Switch Request frame to the initiating protocol entity.

##### 10.3.x.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANSWITCHREQUEST.confirm	(
ResultCode,
DialogToken
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-CHANSWITCHREQUEST.request. |
| DialogToken | Integer | As defined in the corresponding MLME-CHANSWITCHREQUEST. request | The Dialog Token to identify this Channel Switch transaction. |

##### 10.3.x.2.3 When Generated

This primitive is generated by the MLME when the request to transmit a Channel Switch Request frame completes and indicates the results of the request.

##### 10.3.x.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.x.3 MLME-CHANSWITCHREQUEST.indication

##### 10.3.x.3.1 Function

This primitive indicates that a Channel Switch Request frame was received from the AP with which the STA is associated.

##### 10.3.x.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANSWITCHREQUEST.indication	(
PeerMACAddress,
DialogToken,
Regulatory Class
Target Channel Number
Channel Switch Mode
Channel Switch Count
Response Time Count
Response Randomization Interval
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the MAC entity from which a Channel Switch Request frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identity this Channel Switch transaction as received in the Channel Switch Request frame. |
| Regulatory Class | Integer | 0 – 255 | For the Target Channel Number, as defined in AnnexJ. |
| Target Channel Number | Integer | 0 – 255 | Channel number requested for BSS switch, defined within the Regulatory Class in Annex J. |
| Channel Switch Mode | Integer | 0-1 | Channel switch mode, as defined for the Channel Switch Announcement element. |
| Channel Switch Count | Integer | 0 – 255 | Indicates the number of target beacon transmission times (TBTTs) until the requested channel switch to the indicated new channel. A value of 1 indicates a request for a channel switch immediately before the next TBTT. A value of 0 indicates that the Channel Switch Count is not specified for this management request. |
| Response Time Count | Integer | 0 – 255 | Indicates the number of target beacon transmission times (TBTTs) until the latest opportunity for the STA receiving the Channel Switch Solicitation to send a Channel Switch Request to the requesting STA. A value of 1 indicates a request for a Channel Switch Request to be sent before the next TBTT. A value of 0 indicates that the Response Time Count is not specified for this management request and that the STA may send a response at any time. |
| Response Randomization Interval | Integer | 0-65535 | Indicates the upper bound of the random delay to be used just prior to sending the Channel Switch Response for this request. The Response Randomization Interval field is specified in units of TU. |

##### 10.3.x.3.3 When Generated

This primitive is generated by the MLME when a valid Channel Switch Request frame is received.

##### 10.3.x.3.4 Effect of Receipt

On receipt of this primitive the SME shall operate according to the procedure in 11.15.4.

### 10.3.x+1 Channel Switch Response

This set of primitives supports the signaling of Channel Switch Response frames between non-AP STAs and an AP.

#### 10.3.x+1.1 MLME-CHANSWITCHRESPONSE.request

##### 10.3.x+1.1.1 Function

This primitive requests transmission of a Channel Switch Response frame to the AP with which the STA is associated.

##### 10.3.x+1.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANSWITCHRESPONSE.request	(
Peer MACAddress

DialogToken,
Regulatory Class
Alternative Channel Number
Channel Switch Response Code
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Channel Switch Response frame is sent. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify the Channel Switch transaction. |
| Regulatory Class | Integer | 0 – 255 | For the Target Channel Number, as defined in AnnexJ. |
| Alternative Channel Number | Integer | 0 – 255 | Channel number indicating alternative target channel for BSS switch, defined within the Regulatory Class in Annex J. |
| Channel Switch Response Code | Integer | 0-3 | Indicates the STA response to a channel switch request as defined in Table x2.The Channel Switch Response Code is a response to the request to switch to the target channel as identified in the Channel Switch Request sent by the AP. |

##### 10.3.x+1.1.3 When Generated

This primitive is generated by the SME to request that a Channel Switch Response frame be sent to the AP with which the STA is associated.

##### 10.3.x+1.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Channel Switch Response management frame of action type. The non-AP STA then attempts to transmit this to the AP with which it is associated.

#### 10.3.x+1.2 MLME-CHANSWITCHRESPONSE.confirm

##### 10.3.x+1.2.1 Function

This primitive reports the result of a request to send a Channel Switch Response frame to the initiating protocol entity.

##### 10.3.x+1.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANSWITCHRESPONSE.confirm	(
ResultCode,
DialogToken
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-CHANSWITCHRESPONSE.request. |
| DialogToken | Integer | As defined in the corresponding MLME-CHANSWITCHRESPONSE. request | The Dialog Token to identify this Channel Switch transaction. |

##### 10.3.x+1.2.3 When Generated

This primitive is generated by the MLME when the request to transmit a Channel Switch Response frame completes and indicates the results of the request.

##### 10.3.x+1.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.x+1.3 MLME-CHANSWITCHRESPONSE.indication

##### 10.3.x+1.3.1 Function

This primitive indicates that a Channel Switch Response frame was received from a non-AP STA.

##### 10.3.x+1.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-CHANSWITCHRESPONSE.indication	(
PeerMACAddress,
DialogToken,
Regulatory Class
Alternative Channel Number
Channel Switch Response Code
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the non-AP STA MAC entity from which a Channel Switch Response frame was received. |
| DialogToken | Integer | 1 – 255 | The Dialog Token to identify this Channel Switch transaction as received in the Channel Switch Response frame. |
| Regulatory Class | Integer | 0 – 255 | For the Target Channel Number, as defined in AnnexJ. |
| Alternative Channel Number | Integer | 0 – 255 | Channel number indicating alternative target channel for BSS switch, defined within the Regulatory Class in Annex J. |
| Channel Switch Response Code | Integer | 0-3 | Indicates the STA response to a channel switch request as defined in Tabel x2.The Channel Switch Response Code is a response to the request to switch to the target channel as identified in the Channel Switch Request sent by the AP. |

##### 10.3.x+1.3.3 When Generated

This primitive is generated by the MLME when a valid Channel Switch Response frame is received.

##### 10.3.x+1.3.4 Effect of Receipt

On receipt of this primitive the SME shall operate according to the procedure in 11.15.4.

- 11  MAC sublayer management entity

- 11.15  Wireless Network Management Procedures

Insert and renumber new clause 11.15.x after 11.15.3 as follows:

- 11.15.X BSS Channel Switch

The Channel Switch Request and Channel Switch Response provide a means and a protocol to coordinate the movement of a BSS to a new channel. It is an objective to minimize service disruption to the BSS  in this process, although a channel switch may not successfully move all STAs to the new target channel.  Figure x7, below, diagrams the Management Actions for BSS Channel Switch procedure, including all optional frames.

An AP may request appropriate triggered measurements in any STA in order to detect conditions which may warrant a BSS channel switch.  A STA so requested will provide a triggered measurement report when the requested conditions are detected. The AP may then use the information in the triggered report to intitiate a BSS Channel Switch.

An AP may initiate a channel switch by sending a Channel Switch Request to some or all of its associated STAs to collect information and feedback from the STAs on the proposed channel switch event.  The Channel Switch Request indicates the target channel number the AP proposes to switch to and a Response Time Count.  The Response Time Count indicates the latest time for the STA to send a Channel Switch Response frame. An AP may further direct a BSS channel switch by including a Channel Switch Announcement in beacon frames or by sending a Channel Switch Request which includes a Channel Switch Announcement to its associated STAs.  Because it can be sent as a unicast frame, the Channel Switch Request allows the AP to confirm STA reception directed channel switch.

A STA receiving a Channel Switch Request frame shall respond by sending a Channel Switch Response frame within the specified Response Time Count.  The Channel Switch Response frame shall indicate the STAs capability and intention to perform the requested channel switch. A STA receiving a Channel Switch Request shall determine if it will or will not switch to the requested channel if the channel switch is announced. If the response time limit for this request permits, the STA may make radio measurements of the current and new channel before sending a Channel Switch Response of type Query. If the STA determines that the target channel is unsuitable for use or when the STA has a preference to switch to a different channel, the station may identify a suggested suitable alternate channel in the Channel Switch Response. When a determination has been made, the STA shall send a Channel Switch Response to the requesting AP and may include a suitable alternate channel.  The response code in the Channel Switch Response pertains to the request to switch to the requested Target Channel Number and not to any suggested Alternate Channel Number.

An AP receiving a Channel Switch Response from one or multiple STAs shall determine whether or not it will proceed with the channel switch. If the AP decides to proceed with the channel switch, it shall send a Channel Switch Announcement in the beacons specifying the target channel and switching. An AP may send a unicast Channel Switch Request which includes a Channel Switch Announcement to certain STAs

If the STA has indicated in a Channel Switch Response frame that it will switch channel (Channel Switch Response Code set to 1), the STA shall switch to the target channel at the specified channel switch time. A STA that switches to the Target Channel shall send a Channel Switch Response after it switches to the new channel.  The STA shall send this Channel Switch Response before the specified Response Time Count and at a randomly selected frame transmission time. The STA shall randomly select a delay time less than the Response Randomization Interval specified in the Channel Switch Request.  At the specified channel switch time, the STA shall switch channel, shall wait the randomly selected delay time and then shall transmit its Channel Switch Response frame.

Figure x7 below diagrams the BSS channel switch procedure.

AP determines if it will proceed with switch

Channel Switch Announcement

(Broadcast)

Channel Switch Response

(unicast)

Channel Switch Request

(

broadcast or unicast

)

Each STA determines

if it can/will switch

Channel Switch Response

(

Unicast)

...

AP

STA

Other STAs

Triggered Alert or Measurement

.

Optional AP-set trigger to start procedure

:

.

Channel Switch Announcement

(Broadcast)

Indicated Channel Switch Time

Figure x7. Management Actions for BSS Channel Switch procedure

Insert the following new rows in clause A.4.14:

### A. 4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following Wireless Network Management capabilities supported? |  |  |  |
| RME5 | BSS Channel Switch | 11.15.X | CFv:M | Yes, No, N/A |
| RME5.1 | Channel Switch Request | 7.4.6.X | (CFv AND CF1):M | Yes, No, N/A |
| RME5.2 | Channel Switch Response | 7.4.6.(X+1) | (CFv AND CF12):M | Yes, No, N/A |